3GPP TSG-RAN WG4 Meeting #90	R4-1901455

Athens, Greece, 25 February – 1 March 2019

Source:	Ericsson, Telstra

Title:	TP to TR 37.716-21-21: Addition of EN-DC configuration DC_28A_n8A-n78A

Agenda item:	9.8.2

Document for:	Approval

# Background

This contribution provides text proposal on DC_28A_n8A-n78A as defined in the revised WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_28_n8-n78

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n8-n78 | 28 | 703 MHz |  | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n8 | 880 MHz |  | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz |  | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28_n8-n78 | DC_28_n8 DC_28_n78 | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 140 |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.x.3	Co-existence studies

Co-existence studies of Band 28 + Band n8 + band n78 are already captured in Table 5.3-1 of TR 37.864-41-21 [2].

### 6.x.4	∆TIB and ∆RIB values

For DC_28_n8-n78, the TIB,c and RIB,c values are derived from CA_8-28-41 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n8-n78 | 28 | 0.5 |
|  | n8 | 0.6 |
|  | n78 | 0.3 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n8-n78 | 28 | 0.1 |
|  | n8 | 0.2 |
|  | n78 | 0 |

### 6.x.5	MSD

Based on the co-existence studies in TR 37.864-41-21 [2], DC_28_n8-n78 need to have MSD added in table 7.3B.2.3.5.2-1 of TS 38.101-3.

Table 6.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28_n8-n78 | 28 | 728 | 5 | 25 | 783 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3458 | 10 | 52 | 3458 | 9.1 | TDD | IMD4 |
|  | 28 | 713 | 5 | 25 | 768 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3762 | 10 | 52 | 3762 | 3.7 | TDD | IMD5 |
|  | 28 | 713 | 5 | 25 | 768 | N/A | FDD | N/A |
|  | n8 | 890 | 5 | 25 | 935 | 4.3 | TDD | IMD5 |
|  | n78 | 3787 | 10 | 52 | 3787 | N/A | FDD | N/A |

##### ---End of changes---

# Reference

[1] 		RP-182302, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics

[2]		TR 37.864-41-21 v15.0.0 ”NR Dual Connectivity (EN-DC) of LTE x Down Link (DL) / 1 Up Link (UL) (x=1,2,3,4) and inter-band NR 2 Down Link (DL) / 1 Up Link”