3GPP TSG RAN WG4 Meeting #90	R4-1901492

Athens, Greece, February 25-March 1, 2019

Agenda Item:	6.10.2.3

Source:	Huawei, HiSilicon

Title:	Comparison of TS38.141-1 and TS38.141-2 base conformation test models

Document for:	Approval

# Introduction

In the last meeting, the technical specifications for the test model generation were completed. While there were efforts to ensure consistency between the two specifications (TS 38.141-1 [1] and TS 38.141-2 [2]), minor differences are present. This contribution identifies the differences between the two specifications and provides proposals to align the text. Accompanying draft CRs capture the changes for TS 38.141-1 [3] and TS 38.141-2 [4], respectively.

# Background

The technical differences in the test models between TS 38.141-1 [1] and TS 38.141-2 [2] are summarized in the following table.

Table 1. Differences in FR1-TM and FR2-TM

| Feature | TS 38.141-1 | TS 38.141-2 |
| --- | --- | --- |
| TDD/FDD support | Both used | Only TDD |
| TDD patterns | 5 ms periodicity, {15, 30, 60} kHz SCS | 1.25 ms periodicity, {60, 120} kHz SCS |
| DM-RS | 2 symbols | 1 symbols |
| PT-RS | Not present | Present |
| TM1.2 | Present | Not present |
| TM2a | Present | Not present |
| TM3.1a | Present | Not present |
| TM3.2 | Present | Not present |
| TM3.3 | Present | Not present |
| Number of users | Sometimes 2 | Always 1 |

# Comparison

Using the word comparison feature, subclauses 4.9.2 of TS 38.141-1 [1] and TS 38.141-2 [2] are compared below.

### 4.9.2	Test models

#### 4.9.2.1	General

The following sections will describe the NR test models needed for BS type 2-O. Note the NR FR1 test models described in TS 38.141-1 [3] are also applicable for BS type 1-O conformance testing.

#### 4.9.2.2	NR FR2 test models

The set-up of physical channels for transmitter tests shall be according to one of the NR test models (NR-- FR2-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	Duration is 2 radio frames for TDD (20 ms)

-	The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier spacing

-	NRB is the maximum transmission bandwidth configuration seen in table 5.3.2-2 in TS 38.104 [2].

-	Normal CP

-	Virtual resource blocks of localized type

For NR FR2 TDD, test models are derived based on the uplink/downlink configuration as shown in the table 4.9.2.2-1 using information element TDD-UL-DL-ConfigCommon.

Table 4.9.2.2-1: Configurations of TDD for BS type 2-O test models

| Field name | Value |  |
| --- | --- | --- |
| referenceSubcarrierSpacing (kHz) | 60 | 120 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 1.25 | 1.25 |
| nrofDownlinkSlots | 3 | 7 |
| nrofDownlinkSymbols | 10 | 6 |
| nrofUplinkSlots | 1 | 2 |
| nrofUplinkSymbols | 2 | 4 |

Common physical channel parameters for all NR FR2 test models are specified in the following tables: table 4.9.2.2-2 for PDCCH, table 4.9.2.2-3 for PDSCH. Specific physical channel parameters for NR FR2 test models are described in sections 4.9.2.2.1 to 4.9.2.2.3.

Table 4.9.2.2-2: Common physical channel parameters for BS type 2-O PDCCH

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in the each symbol | NRB – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control region | 0 dB |

Table 4.9.2.2-3: Common physical channel parameters for BS type 2-O PDSCH

| Parameter | Value |
| --- | --- |
| mapping type | PDSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | ‘pos2’ |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 0 |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |
| PTRS configuration and density | LPT-RS 4 KPT-RS 2 = 00 |
| Ratio of PT-RS EPRE to DM-RS EPRE | 0 dB |

##### 4.9.2.2.1	NR FR2 test model 1.1 (NR-FR2-TM1.1)

This model shall be used for tests on:

-	BS output power

-	TAE

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

-	Transmitter intermodulation

Common physical channel parameters are defined in section 4.9.2.2. Specific physical channel parameters for NR-FR2-TM1.1 are defined in table 4.9.2.2.1-1.

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR2-TM1.1

| Parameter | Value |
| --- | --- |
| # of QPSK PDSCH PRBs | NRB |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |

##### 4.9.2.2.2	NR FR2 test model 2 (NR-

|  | NRBG=minxNRBP,12NRBP-NRBPmod2+1 |
| --- | --- |
| NRBG | NRBG-10,2,4…,2NRBG-2 NRBP-1 |
|  | NRB-PNRBG |
|  | 10log10NRB-103/10PNRBGNRB-PNRBG |

##### FR2-TM2)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Common physical channel parameters are defined in section 4.9.2.2. Specific physical channel parameters for NR-FR2-TM2 are defined in table 4.9.2.2.2-1.

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR2-TM2

| Parameter | Value |
| --- | --- |
| # of 64QAM PDSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1; n=0,…,10×2μ-23-1. |
|  | NRB-1 |

4.9.2.2.3

##### NR FR2 test model 3.1 (NR-FR2-TM3.1)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 64QAM modulation (at max power)

Common physical channel parameters are defined in section 4.9.2.2. Specific physical channel parameters for NR-FR2-TM3.1 shall be defined in table 4.9.2.2.1-1 with all QPSK PDSCH PRBs replaced by 64QAM.

|  | NRBG=minxNRBP,12NRBP-NRBPmod2+1 |
| --- | --- |
| NRBG | NRBG-10,2,4…,2NRBG-2 NRBP-1 |
|  | NRB-PNRBG |
|  | 10log10NRB-10-3/10PNRBGNRB-PNRBG |

|  | NRBG=minxNRBP,12NRBP-NRBPmod2+1 |
| --- | --- |
| NRBG | NRBG-10,2,4…,2NRBG-2 NRBP-1 |
|  | NRB-PNRBG |
|  | 10log10NRB-10-6/10PNRBGNRB-PNRBG |

#### 4.9.2.3	Data content of physical channels and signals for NR-TM

Randomisation of the data content is obtained by utilizing the length-31 Gold sequence scrambling of TS 38.211 [20], subclause 5.2.1 which is invoked by all physical channels prior to modulation and mapping to the RE grid. An appropriate number of ‘0’ bits shall be generated prior to the scrambling.

Initialization of the scrambler and RE-mappers as defined in TS 38.211 [20] use the following additional parameters:

-	 = 0

-	  Antenna ports starting with 1000 for PDSCH

-	Antenna ports starting with 2000 for PDCCH

-	q = 0 (single code word)

-	Rank 1 (single layer)

-   = 0

##### 4.9.2.3.1	PDCCH

-	NsymbCORESET= 2

-	PDCCH modulation to be QPSK as described in TS 38.211 [20], subclause 5.1.3.

-	For each slot the required amount of bits for all PDCCHs is as follows: 1(# of PDCCH) * 1(# of CCE per PDCCH) * 6(REG per CCE) * 9(data RE per REG) * 2(bits per RE) with these parameters according to the NR-FR2-TM definitions in subclause 4.4.9.2.2.

-	Generate this amount of bits according to ‘all 0’ data.

-	1 CCE shall be according to TS 38.211 [20], subclause 7.3.2. PDCCH using non-interleaved CCE-to-REG mapping. PDCCH should occupy first two symbol for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	Perform PDCCH scrambling according to TS 38.211 [20], subclause 7.3.2.3.

-	NID=NIDcellin DM-RS sequence generation in TS 38.211 [20], subclause 7.4.1.3.

-	nRNTI = 0 in scrambling sequence generation in TS 38.211 [20], subclause 7.3.2.3.

-	Perform mapping to REs according to TS 38.211 [20], subclause 7.3.2.5.

##### 4.9.2.3.2	PDSCH

-	For each slot generate the required amount of bits for all PRBs according to ‘all 0’ data.

-	For  = 0 for 1 user PDSCH transmissions distinguished by .

-	Perform user specific scrambling according to TS 38.211 [20], subclause 7.3.1.1.

-	Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS 38.211 [20], subclause7.3.1.2.

nID=NIDcell

-	Perform mapping of the complex-valued symbols to layer according to TS 38.211 [20], subclause 7.3.1.3.   Complex-valued modulation symbols  for codeword  shall be mapped onto the layers ,  where  is equal to 1.

-	Perform PDSCH mapping type A according to TS 38.211 [20].

-	DM-RS sequence generation according to TS 38.211 [20], subclause 7.4.1.1.1 where l is the OFDM symbol number within the slot with symbols indicated by table 4.9.2.2-3.

NIDnSCID=NIDcell

nSCID=0

-	DM-RS mapping according to TS 38.211 [20], subclause 7.4.1.1.2 with parameters listed in table 4.9.2.2-3.

-	For NR-FR2-TM PT-RS sequence generation according to TS 38.211 [20], subclause 7.4.1.2.1, with parameters listed in table 4.9.2.2-3.

-	For NR-FR2-TM PT-RS mapping according to TS 38.211 [20], subclause 7.4.1.2.2, with parameters listed in table 4.9.2.2-3.

****************** END ******************

Observation 1: Although there are some differences, the specifications are mostly consistent.

Proposal 1: The small differences are mostly editorial and can be clarified with some minor work.

# Conclusion

Through the collaboration between companies, the specifications for test models are very consistent.

Observation 1: Although there are some differences, the specifications are mostly consistent.

Proposal 1: The small differences are mostly editorial and can be clarified with some minor work.

# References

TS 38.141-1

TS 38.141-2

R4-1901530, “Corrections to 38.141-1 subclause 4.9.2 base conformation test models”, Huawei, HiSilicon, RAN4#90, Feb. 25 – Mar. 1, 2019.

R4-1901534, “Corrections to 38.141-2 subclause 4.9.2 base conformation test models”, Huawei, HiSilicon, RAN4#90, Feb. 25 – Mar. 1, 2019.