3GPP TSG RAN WG4 Meeting #90	R4-1901500

Athens, Greece, February 25 – March 1, 2019

Agenda Item:	6.10.2.3

Source:	Huawei, HiSilicon

Title:	Discussion of clarifying EVM measurements for test models

Document for:	Approval

# Introduction

During the drafting of the TPs for the test models in subclause 4.9.2.2 of TS 38.141-1 [2] and TS 38.141-2 [3], the description of the EVM measurements in subclauses 6.5.2.3 and 9.6.2.3 of TS 38.104 was modified in [1]. After some offline discussion and review of TS 38.104, some minor modifications are proposed in an accompanying draft CR [5].

# Discussion

## Measurements

The subclauses 6.5.2.3 and 9.6.2.3 of TS 38.104 [4] mix both slots and subframes. While subframe is a valid term, the proper unit is slot. It is noted that with 15 kHz, the duration of a slot and a subframe is the same. The text change is to replace “subframe” with “slot” in subclauses 6.5.2.3 and 9.6.2.3.1 so that the correct time unit is used.

Proposal 1: Replace “subframe” with “slot” in subclauses 6.5.2.3 and 9.6.2.3.1

## Flexible slots

For downlink FR1 slots, the locations of the DMRS are symbols 2 and 11. For downlink FR2 slots, the location of the DMRS is symbol 2, and the locations of the PT-RS symbols are 3, 7, and 11.

There are several issues with the measurements:

- There are several types of flexible slots described in the test models

  - A flexible slot begins with downlink symbols. The slot is either {sequence of downlink symbols, sequence of flexible symbols} or {sequence of downlink symbols, sequence of flexible symbols, sequence of uplink symbols}

  - A flexible slot begins with flexible symbols. The slot is {sequence of flexible symbols, sequence of uplink symbols}

  - To avoid measurements being performed on a flexible slot beginning with flexible symbols a restriction is captured using the sentence “EVM measurements start on the third symbol of a slot (l=2) when the first symbol of that slot is a downlink symbol.”

- However, the number of downlink symbols in a flexible slot vary as a function of numerology and specification, as indicated in Table 1.

Table 1. Reference symbol locations

| Test | Frequency | SCS (kHz) | Number of downlink symbols in a flexible slot | DMRS symbol location, (0 based) | PTRS symbol location, (0 based) |
| --- | --- | --- | --- | --- | --- |
| NR | FR1 | 15 | 10 | 2, 9 | N/A |
| NR | FR1 | 30 | 6 | 2 | N/A |
| NR | FR1 | 60 | 12 | 2, 9 | N/A |
| NR | FR2 | 60 | 10 | 2 | 3, 7 |
| NR | FR2 | 120 | 6 | 2 | 3 |
| MSR | FR1 | 15 | 10 | 2, 9 | N/A |
| MSR | FR1 | 30 | 6 | 2 | N/A |
| MSR | FR1 | 60 | 12 | 2, 9 | N/A |

Several observations:

Observation 1: With 30 kHz SCS, there is only one reference symbol available in the flexible slot.

Observation 2: With 60 kHz SCS (FR2), there are only two PT-RS symbols available in the flexible slot.

Observation 3: With 120 kHz SCS (FR2), there is only PT-RS symbol available in the flexible slot.

One question is whether EVM measurement should be allowed on flexible slots?

- If yes, what is the minimum number of reference symbols allowed? If yes, how should the specification be written?

- If no, how should the specification be written?

- If unsure, leave the specification as is

For the “no” case, a possible text proposal is

EVM measurements start on the third symbol of a downlink slot (l=2).

For the “yes” case and for FR1

- If one reference symbol is sufficient, then no change is needed.

- If two reference symbols are needed, then a possible text proposal to avoid measurements on the 30 kHz flexible slot is

EVM measurements start on the third symbol of a slot (l=2) when the first symbol of that slot is a downlink symbol and when the number of downlink symbols of that slot is greater than 7.

For the “yes” case and for FR2

- If two reference symbols are sufficient, then no change is needed.

- If three reference symbols are needed, then a possible text proposal to avoid measurements on the 120 kHz flexible slot is

EVM measurements start on the third symbol of a slot (l=2) when the first symbol of that slot is a downlink symbol and when the number of downlink symbols of that slot is greater than 7.

Proposal 2: Clarification of the number of reference symbols in a flexible slot is needed for EVM measurements in the test models

If two reference symbols and three reference symbols are sufficient for FR1 and FR2, then

EVM measurements start on the third symbol of a slot (l=2) when the first symbol of that slot is a downlink symbol and when the number of downlink symbols of that slot is greater than 7.

## Reference signal density

As stated in the previous section, the DMRS density and PTRS density was provided for downlink slots. One question is whether the performance should be clarified to include requirements about the DMRS density. Since the test models are used for base station declaration, no additional statement is needed. In addition with the decisions for when EVM measurements are made (in the previous section), the decisions imply the minimum density.

# Conclusion

This contribution clarifies several aspects of the EVM frame structure.

Proposal 1: Replace “subframe” with “slot” in subclauses 6.5.2.3 and 9.6.2.3.1

For the flexible slot, the following observations were made:

Observation 1: With 30 kHz SCS, there is only one reference symbol available in the flexible slot.

Observation 2: With 60 kHz SCS (FR2), there are only two PT-RS symbols available in the flexible slot.

Observation 3: With 120 kHz SCS (FR2), there is only PT-RS symbol available in the flexible slot.

Because certain numerologies have a reduced number of

Proposal 2: Clarification of the number of reference symbols in a flexible slot is needed for EVM measurements in the test models

# References

R4-1816733, “Draft CR for 38.104 EVM requirement Section 6.5.2.3 and section 9.6.2.3”, Ericsson, RAN4#89, Nov. 12-16, 2018.

TS 38.141-1

TS 38.141-2

TS 38.104

R4-1901553, “Corrections to EVM frame structure in subclauses 6.5.2.3 and 9.5.2.3 in 38.104”, Huawei, HiSilicon, RAN4#90, Feb. 25 – Mar. 1, 2019.

# Appendix

Subclauses 6.5.2.2, 6.5.2.3, 9.6.2.2, and 9.6.2.3 are presented from

#### 6.5.2.2	Minimum Requirement for BS type 1-C and BS type 1-H

For BS type 1-C and 1-H, the EVM levels of each NR carrier for different modulation schemes on PDSCH outlined in table 6.5.2.2-1 shall be met using the frame structure described in subclause 6.5.2.3.

Table 6.5.2.2-1: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |

#### 6.5.2.3	EVM frame structure for measurement

EVM shall be evaluated for each NR carrier over all allocated resource blocks and downlink subframes. Different modulation schemes listed in table 6.5.2.2-1 shall be considered for rank 1.

For NR, for all bandwidths, the EVM measurement shall be performed for each NR carrier over all allocated resource blocks and downlink subframes within 10 ms measurement periods. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries.

EVM measurements start on the third symbol of a slot (l=2) when the first symbol of that slot is a downlink symbol.

#### 9.6.2.3	Minimum Requirement for BS type 2-O

For BS type 2-O, the EVM levels of each NR carrier for different modulation schemes on PDSCH outlined in table 9.6.2.3-1 shall be met, following the EVM frame structure described in subclause 9.6.2.3.1.

Table 9.6.2.3-1: EVM requirements for BS type 2-O carrier

| Modulation scheme for PDSCH | Required EVM (%) |
| --- | --- |
| QPSK | 17.5 |
| 16QAM | 12.5 |
| 64QAM | 8 |

##### 9.6.2.3.1	EVM frame structure for measurement

EVM requirements shall apply for each NR carrier over all allocated resource blocks. Different modulation schemes listed in table 9.6.2.3-1 shall be considered for rank 1.

For NR, for all bandwidths, the EVM measurement shall be performed for each NR carrier over all allocated resource blocks and downlink subframes within 10 ms measurement periods. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries.

EVM measurements start on the third symbol of a slot (l=2) when the first symbol of that slot is a downlink symbol.