3GPP TSG-RAN WG4 Meeting #90												  	         R4-1901503

Athens, Greece, 25th of February – 1st of March, 2019

Source: 	Nokia, Nokia Shanghai Bell

Title: 					MU for co-location blocking

Agenda item:			5.1.3.5

Document for:			Discussion

# 1	Introduction

MU for co-location blocking varies depending on frequency range. However, currently the specification is unclear whether the frequency range means refers to wanted signal or interfering signal. In this contribution the MU derivation is revisited following the procedure used for general out of band blocking, which clarifies how both wanted and interferer frequency are taken into account.

# 2	Discussion

Maximum test system uncertainty for receiver tests for eAAS is specified in Table 4.1.2.3-1 in TS 37.145-2 [1]. The approach is different between OTA blocking and co-location blocking. For OTA blocking both wanted and interfering frequency ranges are clearly specified, whereas for co-location blocking there is ambiguity whether the frequency refers to wanted or interfering signal.

Table 1: Extract from Table 4.1.2.3-1 in [1].

| 7.6.2 OTA Blocking | fwanted ≤ 3 GHz 1 MHz < finterferer ≤ 3 GHz: ±2.0 dB 3 GHz < finterferer ≤ 6 GHz: ±2.1 dB 6 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB 3 GHz < fwanted ≤ 4.2GHz: 1 MHz < finterferer ≤ 3 GHz: ±2.0 dB 3 GHz < finterferer ≤ 6 GHz: ±2.1 dB 6 GHz < finterferer ≤ 12.75 GHz: ±3.6 dB | See 3GPP TR 37.843 [28], subclause 10.8. Uncertainty budget contributors as well as uncertainty budget assessment described in annex C. |
| --- | --- | --- |
| 7.6.3 Co-location blocking | ±3.2 dB, f ≤ 3.0 GHz ±3.4 dB, 3.0 GHz < f ≤ 4.2 GHz (NOTE 2) | See 3GPP TR 37.843 [28], subclause 10.6.5. Uncertainty budget contributors as well as uncertainty budget assessment described in annex |

The derivation of MU for OTA blocking has taken into account both the uncertainty for wanted signal and interferer signal, as documented in TR 37.843 [2]. These uncertainties have been combined using the formula:

It is clear that the total MU for co-location blocking is also dependent on both wanted signal and interferer signal. Therefore, we have re-calculated the total MU for co-location blocking using the following MUs as specified in TS 38.141-2 [3] and [1].

MU for wanted signal:

±1.3 dB, f ≤ 3.0 GHz

±1.4 dB, 3.0 GHz < f ≤ 4.2 GHz

±1.6 dB, 4.2 GHz < f ≤ 6.0 GHz

MU for interfering signal:

±3.2 dB, finterferer ≤ 3.0 GHz

±3.4 dB, 3.0 GHz < finterferer ≤ 4.2 GHz

±3.5 dB, 4.2 GHz < finterferer ≤ 6.0 GHz

Also we applied 0.1 dB as Noise effect similarly as used in MU derivation in TR 37.843. Using these values and the MU formula presented earlier, we end up with following MU values

Table 2: MU for co-location blocking

| Co-location blocking MU (dB) |  | Wanted signal operating band |  |  |
| --- | --- | --- | --- | --- |
|  |  | f ≤ 3 GHz | 3 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6 GHz |
| Interferer frequency | 30 MHz < f ≤ 3 GHz | 3.6 | 3.6 | 3.7 |
|  | 3 GHz < f ≤ 4.2 GHz | 3.7 | 3.8 | 3.9 |
|  | 4.2 GHz < f ≤ 6.0 GHz | 3.8 | 3.9 | 3.9 |

We have provided an update to both eAAS and NR specification in companion CRs [4] and [5] which use the values shown in Table 2.

# 3	Conclusion

In this contribution we have provided an updated analysis for co-location blocking MU.

# 4	References

[1]	3GPP TS 37.145-2 v.15.2.0

[2]	3GPP TR 37.843 v. 15.2.0

[3]	3GPP TS 38.141 v. 15.0.0

[4]	R4-1901508 CR to 37.145-2 Corrections on acceptable uncertainty of test requirements, Nokia, Nokia Shanghai Bell

[5]	R4-1901510 CR to 38.141-2 Corrections on Measurement uncertainties and test requirements, Nokia, Nokia Shanghai Bell