3GPP TSG-RAN WG4 Meeting #90	R4-1901565

# Athens, Greece, February 25 – March 1, 2019

Agenda item:	8.1.2

Source: 	Intel Corporation

Title: 	ACS and In-band Blocking for remaining BW Classes in Rel-16

Document for:	Approval

# Introduction

In RAN4#89 it was defined the ACS and IBB for BW Classes C, D and E since the rest of the BW Classes were removed for Rel-15. In this contribution we provide our view for the definition of ACS and IBB for the remaining BW classes for Rel-16.

# Discussion

## Background

- Definition of ACS and In-band Blocking for NR bands single carrier in TS38.101-1 [1] :

ACS for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

|  |  | CA Bandwidth Class |  |  |
| --- | --- | --- | --- | --- |
| Rx Parameter | Units | C | D | E |
| ACS | dB | 33.0 | 25.2 | 24.0 |

Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| Rx Parameter | Units | CA Bandwidth Class |  |  |
| --- | --- | --- | --- | --- |
|  |  | C | D | E |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 31.5 dB | Aggregated power + 23.7 dB | Aggregated power + 22.5 dB |
| BWInterferer | MHz | BWchannel CA | 50 | 50 |
| FInterferer (offset) | MHz | BWchannel CA / -BWchannel CA | 25 + Foffset / -25 -Foffset | 25 + Foffset / -25 -Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 . NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in […] |  |  |  |  |

In-band blocking parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx Parameter | Units | CA Bandwidth Class |  |  |
| --- | --- | --- | --- | --- |
|  |  | C | D | E |
| Pw in Transmission Bandwidth Configuration, per CC | dB | REFSENS + CA Bandwidth Class specific value below |  |  |
|  |  | 6 | 13.8 | 15.0 |
| BWInterferer | MHz | BWchannel CA | 50 | 50 |
| FIoffset, case 1 | MHz | BWchannel CA+ BWchannel CA/2 | 75 | 75 |
| FIoffset, case 2 | MHz | BWInterferer + FIoffset, case 1 | 125 | 125 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4.NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |

In-band blocking for intra-band contiguous CA  with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | -BWchannel CA/2 – FIoffset, case 1 and BWchannel CA/2 + FIoffset, case 1 | ≤ -BWchannel CA/2 – FIoffset, case 2 and ≥ BWchannel CA/2 + FIoffset, case 2 |
|  | Finterferer |  | NOTE 2 | FDL_low – 3BWchannel CA to FDL_high + 3BWchannel CA |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |

## Estimation of ACS and IBB intra-band contiguous CA for all BW classes

In NR spec [1] only BW Class C is considered for intra-band contiguous CA requirements and the requirements for ACS and IBB intra-band contiguous CA has been defined considering the REFSENS per CC. In LTE the definition of ACS intra-band contiguous is based on the aggregated power for the interferer. Another difference when comparing NR and LTE is the jammer BW which for NR is considered to be the same as the aggregated BW for the case of two CCs and for LTE is 5 MHz. However, this raises the concern of the interferer BW for more than two CCs. In the worst case eight CCs will translate into a very wide interferer BW when considering the aggregated BW of all CCs as the jammer BW.

Observation 1: Concern on the jammer BW for more than two CCs, since the current agreement is to have the jammer BW the same as the aggregated BW.

From system perspective it would make sense to follow similar definitions as in LTE. The ACS requirement will be defined in terms of the aggregated power and not per CC, while IBB is added to the REFSENES per CC. Since we need to consider the difference on the spectral efficiency in NR, a modification regarding LTE is required on the ACS and IBB requirements in NR.

The details on the estimation of ACS and IBB intra-band contiguous CA has been provided in [2], in which the ACS is calculated considering the SC ACS level as reference and scaling according to the maximum aggregated BW and the jammer BW of each BW Classes – similar approach was followed for the calculation of IBB. For the calculation the jammer BW was defined as the minimum CC bandwidth of the CCs located on the edges from the corresponding BW Classes.

Observation 1: The ACS intra-band contiguous CA requirement is calculated considering the SC ACS level as reference and scaling according to the maximum aggregated BW and the jammer BW of each BW class.

Observation 2: The IBB intra-band contiguous CA requirement is calculated considering the SC IBB as reference and scaling according to the maximum aggregated BW and the jammer BW of each BW class.

Observation 3: The jammer BW is defined as the minimum CC bandwidth of the CCs located on the edges from the corresponding BW Class

### Summary of proposal for intra-band contiguous CA for all BW classes

In [1] there is not consistency in the calculation of ACS levels between C and D, E BW Classes. RAN4 should have the same methodology to estimate ACS and IBB intra-band contiguous CA for all BW Classes.

We have made similar calculations for the BW Classes B, C, D, E, F, G, H, I, J, K, L. The summary of the results is provided in the following table:

Table 1: ACS and IBB proposal for all BW Classes

| BW Class | AggBW | Max BW (MHz) | Jammer BW (MHz) | ACS1 (dB) | ACS offset proposal, dB | ACS2 sig (dBm) | ACS2 (dBm) | ∆IBB sig | IBB1 (dBm) | IBB2 (dbm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B | 20M-50M | 50 | 5 | 23.0 | 21.5 | -46.5 + 10log(Nc/Nagg) | -25 | 13.0 | -56 | -44 |
| F | 60M-100M | 100 | 20 | 26.0 | 24.5 | -49.5 + 10log(Nc/Nagg) | -25 | 10.0 | -56 | -44 |
| C | 110M-200M | 200 | 50 | 27.0 | 25.5 | -50.5 + 10log(Nc/Nagg) | -25 | 9.0 | -56 | -44 |
| D | 210M-300M | 300 | 50 | 25.2 | 23.7 | -48.7 + 10log(Nc/Nagg) | -25 | 10.8 | -56 | -44 |
| E | 310M-400M | 400 | 50 | 24.0 | 22.5 | -47.5 + 10log(Nc/Nagg) | -25 | 12.0 | -56 | -44 |
| G | 110M-150M | 150 | 40 | 27.3 | 25.8 | -50.8 + 10log(Nc/Nagg) | -25 | 8.7 | -56 | -44 |
| H | 160M-200M | 200 | 40 | 26.0 | 24.5 | -49.5 + 10log(Nc/Nagg) | -25 | 10.0 | -56 | -44 |
| I | 210M-250M | 250 | 40 | 25.0 | 23.5 | -48.5 + 10log(Nc/Nagg) | -25 | 11.0 | -56 | -44 |
| J | 260M-300M | 300 | 40 | 24.2 | 22.7 | -47.7 + 10log(Nc/Nagg) | -25 | 11.8 | -56 | -44 |
| K | 310M-350M | 350 | 40 | 23.6 | 22.1 | -47.1 + 10log(Nc/Nagg) | -25 | 12.4 | -56 | -44 |
| L | 360M-400M | 400 | 40 | 23.0 | 21.5 | -46.5 + 10log(Nc/Nagg) | -25 | 13.0 | -56 | -44 |
| NOTE 1: ACS1 tested with REFSENS+14dB per CC NOTE 2: ACS offset is referred to the aggregated BW power NOTE 3: ∆IBB are added to REFSENS per CC NOTE 4: Jammer BW is equal to the minimum outer CC bandwidth NOTE 5: Jammer BW’s apply to ACS BW and IBB BW |  |  |  |  |  |  |  |  |  |  |

Observation 4: RAN4 should have the same methodology to estimate ACS and IBB intra-band contiguous CA for all BW Classes.

Proposal 1:	The introduction of the ACS and IBB requirement for intra-band contiguous CA for all BW classes as provided in Table 1.

## Definition of ACS intra-band contiguous CA definition for all BW Classes

In this subsection Table 2 gives the ACS levels for all BW Classes. Table 3 and Table 4 are the ACS test parameters for Case 1 and Case 2, respectively.

Table 2: Adjacent channel selectivity level

|  |  | CA Bandwidth Class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C | D | E | F | G |
| ACS | dB | 23.0 | 27.0 | 25.2 | 24.0 | 26.0 | 27.3 |
|  |  | H | I | J | K | L |  |
|  |  | 26.0 | 25.0 | 24.2 | 23.6 | 23.0 |  |

Table 3: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F | G |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 21.5 dB | Aggregated power + 25.5 dB | Aggregated power + 23.7 dB | Aggregated power + 22.5 dB | Aggregated power + 24.5 dB | Aggregated power + 25.8 dB |
| BWInterferer | MHz | 5 | 50 | 50 | 50 | 20 | 40 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 25 + Foffset / -25 - Foffset | 25 + Foffset / -25 -Foffset | 25 + Foffset / -25 -Foffset | 10 + Foffset / -10 - Foffset | 20 + Foffset / -20 - Foffset |
|  |  | H | I | J | K | L |  |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |  |
| PInterferer | dBm | Aggregated power + 24.5 dB | Aggregated power + 23.5 dB | Aggregated power + 22.7 dB | Aggregated power + 22.1 dB | Aggregated power + 21.5 dB |  |
| BWInterferer | MHz | 40 | 40 | 40 | 40 | 40 |  |
| FInterferer (offset) | MHz | 20 + Foffset / -20 - Foffset | 20 + Foffset / -20 - Foffset | 20 + Foffset / -20 -Foffset | 20 + Foffset / -20 -Foffset | 20 + Foffset / -20 - Foffset |  |
| NOTE 1: The transmitter shall be set to 4dB below […]. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in […] |  |  |  |  |  |  |  |

Table 4: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F | G |
| Pw in Transmission Bandwidth Configuration, per CC |  | -46.5 + 10log(Nc/Nagg) | -50.5 + 10log(Nc/Nagg) | -48.7 + 10log(Nc/Nagg) | -47.5 + 10log(Nc/Nagg) | -49.5 + 10log(Nc/Nagg) | -50.8 + 10log(Nc/Nagg) |
| PInterferer | dBm | -25 | -25 | -25 | -25 | -25 | -25 |
| BWInterferer | MHz | 5 | 50 | 50 | 50 | 20 | 40 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 25 + Foffset / -25 - Foffset | 25 + Foffset / -25 -Foffset | 25 + Foffset / -25 -Foffset | 10 + Foffset / -10 - Foffset | 20 + Foffset / -20 - Foffset |
|  |  | H | I | J | K | L |  |
| Pw in Transmission Bandwidth Configuration, per CC |  | -49.5 + 10log(Nc/Nagg) | -48.5 + 10log(Nc/Nagg) | -47.7 + 10log(Nc/Nagg) | -47.1 + 10log(Nc/Nagg) | -46.5 + 10log(Nc/Nagg) |  |
| PInterferer | dBm | -25 | -25 | -25 | -25 | -25 |  |
| BWInterferer | MHz | 40 | 40 | 40 | 40 | 40 |  |
| FInterferer (offset) | MHz | 20 + Foffset / -20 - Foffset | 20 + Foffset / -20 - Foffset | 20 + Foffset / -20 -Foffset | 20 + Foffset / -20 -Foffset | 20 + Foffset / -20 - Foffset |  |
| NOTE 1: The transmitter shall be set to 4dB below […]. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in […] |  |  |  |  |  |  |  |

## Definition of IBB intra-band contiguous CA definition for all BW Classes

In this subsection Table 4 gives the in-band blocking parameters for all BW Classes and Table 5 provides the in-band blocking requirement.

Table 5: In-band blocking parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F | G |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |  |
|  |  | 13 | 9.0 | 10.8 | 12.0 | 10.0 | 8.7 |
| BWInterferer | MHz | 5 | 50 | 50 | 50 | 20 | 40 |
| FIoffset, case 1 | MHz | 15 | 75 | 75 | 75 | 30 | 60 |
| FIoffset, case 2 | MHz | 30 | 125 | 125 | 125 | 50 | 100 |
|  |  | H | I | J | K | L |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |  |
|  |  | 10.0 | 11.0 | 11.8 | 12.4 | 13.0 |  |
| BWInterferer | MHz | 40 | 40 | 40 | 40 | 40 |  |
| FIoffset, case 1 | MHz | 60 | 60 | 60 | 60 | 60 |  |
| FIoffset, case 2 | MHz | 100 | 100 | 100 | 100 | 100 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |  |  |

Table 6: In-band blocking for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | -BWchannel CA/2 – FIoffset, case 1 and BWchannel CA/2 + FIoffset, case 1 | ≤ -BWchannel CA/2 – FIoffset, case 2 and ≥ BWchannel CA/2 + FIoffset, case 2 |
|  | Finterferer |  | NOTE 2 | FDL_low – 3BWchannel CA to FDL_high + 3BWchannel CA |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |

# Conclusion

In this contribution we have provided our view for ACS and IBB intra-band contiguous CA for all BW classes. In summary, we have made the following observations and proposal:

Observation 1: The ACS intra-band contiguous CA requirement is calculated considering the SC ACS level as reference and scaling according to the maximum aggregated BW and the jammer BW of each BW class.

Observation 2: The IBB intra-band contiguous CA requirement is calculated considering the SC IBB as reference and scaling according to the maximum aggregated BW and the jammer BW of each BW class.

Observation 3: The jammer BW is defined as the minimum CC bandwidth of the CCs located on the edges from the corresponding BW Class

Observation 4: RAN4 should have the same methodology to estimate ACS and IBB intra-band contiguous CA for all BW Classes.

Proposal 1:	The introduction of the ACS and IBB requirement for intra-band contiguous CA for all BW classes as provided in Table 1.

# References

- 3GPP TS 38.101-1 V15.4.0 (2018-12)

- R4-1814598 “ACS and IBB for intra-band contiguous CA”, Intel, RAN4 #89