3GPP TSG-RAN4 Meeting #90		R4-1901624

# Athens, Greece, February 25-March 01, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Transmit Timing Requirements under Beam Switch in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To specify that the UE should do abrupt adjustment in UL TX timing when there is RX or TX beam switch. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is specified that when the transmission timing error between the UE and the reference timing exceeds Te, and the transmission timing error is not due to the reception and/or the transmission beam switch in the reference cell then the UE is required to adjust its timing to within Te with adjustments according to the rules defined in section 7.1.2. Otherwise if there is RX or TX beam switch then the UE should adjust its timing wrt the new beam by a factor of 2 times the time difference between the old and the new beam. The UE is also allowes to cause an interruption of 1 symbol if the difference between the old and new beam timings exceed CP length. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behaviour in terms of UE timing adjustment under RX and TX beam switch is unclear. This may lead to degradation of reception at the BS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 7.1.2, 7.1.2.1, 7.1.2.2, B.2.1, B.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGES----------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[2]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[3]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[4]	3GPP TS 38.215: "NR; Physical layer measurements".

[5]	3GPP TS 38.533: "NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM)".

[6]	3GPP TS 38.211: "NR; Physical channels and modulation”.

[7]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[8]	3GPP TS 38. 212 "NR; Multiplexing and channel coding".

[9]	3GPP TS 38.202: "NR; Physical layer services provided by the physical layer".

[10]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[11]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[12]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[13]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[14]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[15]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[16]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[17]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity", Stage 2.

[18]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[19]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[20]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[21]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements".

[22]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[23]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[24]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA); Overall description".

[25]	3GPP TS 36.101: "Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[26]	3GPP TS 38.214: "NR; Physical layer procedures for data".

----------------------NEXT CHANGE----------------------------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [11] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [11].

BWP	Bandwidth Part

CA	Carrier Aggregation

CC	Component Carrier

CP	Cyclic Prefix

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

DC	Dual Connectivity

DL	Downlink

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EN-DC	E-UTRA-NR Dual Connectivity

FDD	Frequency Division Duplex

FR	Frequency Range

HARQ	Hybrid Automatic Repeat Request

HO	Handover

MAC	Medium Access Control

MCG	Master Cell Group

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MIB	Master Information Block

NR	New Radio

NSA	Non-Standalone operation mode

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

PBCH	Physical Broadcast Channel

PCell	Primary Cell

PLMN	Public Land Mobile Network

PRACH	Physical RACH

PSCell	Primary SCell

PSS	Primary Synchronization Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

RACH	Random Access Channel

RAT	Radio Access Technology

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RRC	Radio Resource Control

RRM	Radio Resource Management

RSSI	Received Signal Strength Indicator

SA	Standalone operation mode

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing

SDL	Supplementary Downlink

SFN	System Frame Number

SI	System Information

SIB	System Information Block

SMTC	SSB-based Measurement Timing configuration

SRS	Sounding Reference Signal

SSSS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

SUL	Supplementary Uplink

TA	Timing Advance

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TDD	Time Division Duplex

TTI	Transmission Time Interval

UE	User Equipment

UL	Uplink

----------------------NEXT CHANGE----------------------------

## 7.1	UE transmit timing

### 7.1.1	Introduction

The UE shall have capability to follow the frame timing change of the connected gNB. The uplink frame transmission takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. The reference cell is PSCell in case of EN-DC. The reference cell is PCell in case of NR standalone. If the UE is configured with a pTAG containing the PCell, UE shall use the PCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. If the UE is configured with a psTAG containing the PSCell, UE shall use the PSCell as the reference cell for deriving the UE transmit timing for cells in the psTAG.UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are defined in the following requirements.

### 7.1.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS or it is the PRACH transmission.

The UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell. NTA for PRACH is defined as 0.

(in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA for other channels is not changed until next timing advance is received. The value ofdepends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). is defined in Table 7.1.2-2.

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.1.2-2: The Value of

| Frequency range and band of cell used for uplink transmission | (Unit: TC) |
| --- | --- |
| FR1 FDD band without LTE-NR coexistence case or FR1 TDD band without LTE-NR coexistence case | 25600 (Note 1) |
| FR1 FDD band with LTE-NR coexistence case | 0 (Note 1) |
| FR1 TDD band with LTE-NR coexistence case | 39936 (Note 1) |
| FR2 | 13792 |
| Note 1: The UE identifies based on the information n-TimingAdvanceOffset according to [2]. If UE is not provided with the information n-TimingAdvanceOffset, the default value of is set as 25600 for FR1 band. In case of multiple UL carriers in the same TAG, UE expects that the same value of n-TimingAdvanceOffset is provided for all the UL carriers according to section 4.2 in [3] and the value 39936 of can also be provided for a FDD serving cell.Note 2: Void |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell except when the timing advance in clause 7.3 is applied.

When the transmission timing error between the UE and the reference timing exceeds Te, the UE is required to adjust its timing to within Te, and the transmission timing error is not due to the reception and/or the transmission beam switch in the reference cell. The reference timing shall be  before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp  per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200ms.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 7.1.2-3.

Table 7.1.2-3: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (KHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2 | 60 | 2.5*64*Tc | 2.5*64*Tc |
|  | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

### 7.1.2	UE transmit timing under beam switch in FR2

#### 7.1.2.1	Introduction

The requirements in this section are applicable only for the reference cell on a carrier frequency belonging to FR2.

The reception timing at the UE in the reference cell may change due to the receive beam switching in the UE or due to the downlink transmit beam switching in the reference cell. The UE switches the downlink transmit beam or the receive beam in response to the change in the active TCI state indicated via MAC CE command [7] or via DCI [26]. The UE may also autonomously switch its receive beam.

#### 7.1.2.2	Requirements

The UE upon performing the receive beam switching shall adjust its uplink transmission timing wrt the new beam reception timing by two times the difference between the reception timings of the old receive beam and the new receive beam.

The UE upon detecting the downlink transmit beam switching in the reference cell shall:

- adjust its uplink transmission timing wrt the new transmit beam reception timing by two times the difference between the reception timings of the old transmit beam and the new transmit beam and

- use the new downlink transmit beam for deriving its uplink transmission timing.

The UE shall meet the requirements in this section provided that for each relevant SSB involved in receiver or transmit beam switching the following conditions are met at the UE:

-	SSB_RP and SSB Ês/Iot according to Annex B.2.5 for a corresponding operating Band.

If the magnitude of the difference between the timings of the old receive beam (before the receive beam switch) and the new receive beam (after the receive beam switch) exceeds the CP length then the UE is allowed an interruption of OFDM 1 symbol in uplink and downlink immediately after the receive beam switching at the UE.

If the magnitude of the difference between the reception timings of the old transmit beam (before the transmit beam switch) and the new transmit beam (after the transmit beam switch) exceeds the CP length then the UE is allowed an interruption of 1 OFDM symbol in uplink and downlink immediately after the transmit beam switching at the UE.

---------------------------NEXT CHANGE----------------------------

# B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state

## B.2.1	Introduction

In Annex B.2, the following conditions are specified:

-	UE conditions which shall apply for UE intra-frequency measurements procedures and requirements in Section 9,

UE conditions which shall apply for UE inter-frequency measurements procedures and requirements in Section 9,

-	UE conditions which shall apply for UE intra-frequency measurements performance requirements in Section 10,

-	UE conditions which shall apply for UE inter-frequency measurements performance requirements in Section 10.

- The conditions for UE transmit timing adjustment under receiver or tranmit beam switching requirements in Section 7.1.

---------------------------NEXT CHANGE----------------------------

## B.2.5	Conditions for UE transmit timing under beam switching in FR2

This section defines the following conditions for UE transmit timing adjustment under receiver or tranmit beam switching performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.5-1 for FR2 SSB.

Table B.2.5-1: Conditions for UE transmit timing under beam switching in FR2

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |
| Conditions | NR_TDD_FR2_A | TBD | TBD | TBD |
|  | NR_TDD_FR2_B | TBD | TBD |  |
|  | NR_TDD_FR2_F | TBD | TBD |  |
|  | NR_TDD_FR2_G | TBD | TBD |  |
|  | NR_TDD_FR2_T | TBD | TBD |  |
|  | NR_TDD_FR2_Y | TBD | TBD |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.3. |  |  |  |  |

Editor’s note: The corresponding conducted level is SSB Ês/Iot ≥ -3 dB.

---------------------------END OF CHANGES----------------------------