3GPP TSG-RAN WG4 Meeting 90	R4-1901685

# Athens, Greece, 25th February – 1st March 2019

Source: 	Skyworks Solutions, Inc.

Title: 	MSD Test Point Alignment for SUO Eligible EN-DC Combinations

Agenda Item:	6.7.6 [FR1] Receiver characteristics [ NR_newRAT-Core]

Document for:	Discussion

# Introduction

This contribution focuses solely on single UL operation (SUL) eligible EN-DC combinations for which several discrepancies between MSD test point configurations and reference sensitivity exception levels due to dual uplink operation are present between values specified for EN-DC “two bands” operation in FR1 (Table 7.3B.2.3.5.1-1) and values specified for EN-DC “three bands” operation in FR1 (Table 7.3B.2.3.5.2-1). This contribution discusses whether adopting a single test point configuration and unified MSD levels would be beneficial to improve specification consistency.

# Discussion

## Discrepancies in MSD test point configurations and MSD levels.

Out of the 72 “two-band” ENDC combinations operating in FR1 listed in Table 5.2B.4.1-1 [1], there are only eight combinations eligible for Single UL operation (SUO). These combinations are denoted “parent” SUO combinations throughout this document and are summarized in table 1 below. For each of these parent combination, EN-DC “two-band” reference sensitivity exceptions due to dual uplink operation and associated test point (TP) are defined in Table 7.3B.2.3.5.1-1.

Table 1: SUO Two-band “Parent” Combinations for EN-DC Operation in FR1 – source Table 5.2B.4.1-1.

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1_n773 | 1 | n77 | DC_1_n77 |
| DC_2_n66 | 2 | n66 | DC_2_n66 |
| DC_2_n78 | 2 | n78 | DC_2_n78 |
| DC_3_n773 | 3 | n77 | DC_3_n77 |
| DC_3_n783 | 3 | n78 | DC_3_n78 |
| DC_5_n66 | 5 | n66 | DC_5_n66 |
| DC_20_n8 | 20 | n8 | DC_20_n8 |
| DC_66_n5 | 66 | n5 | DC_66_n5 |
| NOTE 3: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

In Table 7.3B.2.3.5.2-1, the higher order “three band” EN-DC combinations eligible to SUO are subsets of only three of these SUO “parents”: DC_1_n77, DC_3_n77 and DC_3_n78.

Table 2 below compares the DC_1_n77 SUO parent MSD levels and UL/DL TP configurations (highlighted in yellow) to the related ”three-band” EN-DC subsets.

Table 2: SUO “Parent” MSD level and TP Configuration vs MSD for DC_1_n77 “Three-band” EN-DC Subsets for Operation in FR1 – Source Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.2-1.

|  | EN-DC Configuration | EUTRA or NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | IMD order | SUO test point ? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (MHz) | (MHz) | LCRB |  | (dB) |  |  |  |
| SUO “parent” MSD TP | DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD23 | DC_1_n77 |
|  |  |  |  |  |  |  | 32.54 |  |  |  |
|  |  | n77 | 4090 | 10 | 25 | 4090 | N/A | TDD | N/A |  |
| DC_1_n77 ENDC "three band" subsets | DC_1A-3A_n77A | 1 | 1950 | 5 | 25 | 2140 | 31 | FDD | IMD2 | DC_1_n77 |
|  |  | 3 | 1775 | 5 | 25 | 1870 | N/A |  | N/A |  |
|  |  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |  |
|  |  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A | DC_3_n77 |
|  |  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 |  | IMD2 |  |
|  |  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |  |
|  |  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A | No |
|  |  | 3 | 1775 | 5 | 25 | 1870 | 8.5 |  | IMD4 |  |
|  |  | n77 | 3980 | 10 | 50 | 3980 | N/A | TDD | N/A |  |
|  | DC_1A-18A_n77A | 1 | 1930 | 5 | 25 | 2120 | 16.4 | FDD | IMD3 | No |
|  |  | 18 | 825 | 5 | 25 | 870 | N/A |  | N/A |  |
|  |  | n77 | 3770 | 10 | 50 | 3770 | N/A | TDD | N/A |  |
|  | DC_1A-19A_n78A DC_1A-19A_n77A | 1 | 1940 | 5 | 25 | 2130 | 17.8 | FDD | IMD3 | No |
|  |  | 19 | 832.5 | 5 | 25 | 877.5 | N/A |  | N/A |  |
|  |  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |  |
|  | DC_1A-21A_n78A DC_1A-21A_n77A | 1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | FDD | IMD2 | DC_1_n77 |
|  |  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |  |
|  |  | n77, n78 | 3605 | 10 | 50 | 3605 | N/A | TDD | N/A |  |
|  | DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | 15.8 | FDD | IMD3 | No |
|  |  | 28 | 740 | 5 | 25 | 795 | N/A |  | N/A |  |
|  |  | n77 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |  |
|  |  | 1 | 1960 | 5 | 25 | 2150 | N/A | FDD | N/A | No |
|  |  | 28 | 725 | 5 | 25 | 780 | 4.3 |  | IMD5 |  |
|  |  | n77 | 3330 | 10 | 50 | 3330 | N/A | TDD | N/A |  |
|  | DC_1A-41A_n77A | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A | No |
|  |  | n77 | 3400 | 10 | 50 | 3400 |  | TDD |  |  |
|  |  | 41 | 2510 | 5 | 25 | 2510 | 11 | TDD | IMD4 |  |
|  |  | 1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A | No |
|  |  | n77 | 4150 | 10 | 50 | 4150 |  | TDD |  |  |
|  |  | 41 | 2510 | 5 | 25 | 2510 | 3.6 | TDD | IMD5 |  |

Test points allowing corresponding to SUO criteria are highlighted in light blue. It can be seen that the three-band MSD TP are different from the two-band MSD TP, and therefore different MSD levels are specified even though these test cases aim at checking the UE performance for an identical IMD mechanism. In this example, four different MSD levels are defined (29.8, 32.54, 31 and 30.6 dB) and three distinct TP configurations. It should also be noted that several SUO TP are missing.

Similar observation is made in Table 3 which compares the DC_3_n77 and DC_3_n78 SUO parent MSD levels and UL/DL TP configurations (highlighted in yellow) to their respective ”three-band” EN-DC subsets. In this example, there are 4 different MSD levels and three different test point configurations defined to check an MSD due to an identical IMD mechanism. One possible way of addressing these inconsistencies would consist in applying MSD levels and MSD TP configurations from the “two band” Table 7.3B.2.3.5.1-1 to all higher-order EN-DC subsets. This would be useful for future releases as the number of ENDC combinations is expected to grow.

Table 3: SUO “Parent” MSD Level and TP configuration vs MSD for DC_3_n77/DC_3_n78 “Three-band” EN-DC Subsets for Operation in FR1 – Source Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.2-1.

|  | EN-DC Configuration | EUTRA or NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | IMD order | SUO test point ? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (MHz) | (MHz) | LCRB |  | (dB) |  |  |  |
| SUO “parent” MSD TP | DC_3A_n77A DC_3A_n78A | 3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD23 | DC_3_n77 DC_3_n78 |
|  |  |  |  |  |  |  | 28.74 |  |  |  |
|  |  | n77, n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |  |
| DC_3_n77 DC_3_n78 ENDC "three-band" subsets | DC_1A-3A_n77A | 1 | 1950 | 5 | 25 | 2140 | 31 | FDD | IMD2 | DC_1_n77 |
|  |  | 3 | 1775 | 5 | 25 | 1870 | N/A |  | N/A |  |
|  |  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |  |
|  |  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A | DC_3_n77 |
|  |  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 |  | IMD2 |  |
|  |  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |  |
|  |  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A | No |
|  |  | 3 | 1775 | 5 | 25 | 1870 | 8.5 |  | IMD4 |  |
|  |  | n77 | 3980 | 10 | 50 | 3980 | N/A | TDD | N/A |  |
|  | DC_1A-3A_n78A DC_1A-3C_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A | DC_3_n78 |
|  |  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.2 |  | IMD2 |  |
|  |  |  |  |  |  |  |  |  | ¦fB78-fB1¦ |  |
|  |  | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |  |
|  |  | 1 | 1935 | 5 | 25 | 2125 | 2.8 | FDD | IMD5 | No |
|  |  |  |  |  |  |  |  |  | ¦2*fB78-3*fB3¦ |  |
|  |  | 3 | 1775 | 5 | 25 | 1870 | N/A |  | N/A |  |
|  |  | n78 | 3725 | 10 | 50 | 3725 | N/A | TDD | N/A |  |
|  | DC_3C-7C_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | FDD | IMD3 | No |
|  |  |  |  |  |  |  |  |  | ¦fB78-2*fB7¦ |  |
|  |  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |  |
|  |  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |  |
|  |  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | FDD | IMD4 | No |
|  |  |  |  |  |  |  |  |  | ¦2*fB78-2*fB7¦ |  |
|  |  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |  |
|  |  | n78 | 3475 | 10 | 50 | 3475 | N/A | TDD | N/A |  |
|  | DC_3A-28A_n77A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A | No |
|  |  | 28 | 715 | 5 | 25 | 770 | 15.3 | FDD | IMD3 |  |
|  |  | n77 | 4195 | 10 | 50 | 4195 | N/A | TDD | N/A |  |
|  |  | 3 | 1755 | 5 | 25 | 1850 | 17 | FDD | IMD3 | No |
|  |  | 28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |  |
|  |  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |  |
|  | DC_3A-28A_n78A | 3 | 1775 | 5 | 25 | 1870 | 17.3 | FDD | IMD3 | No |
|  |  | 28 | 740 | 5 | 25 | 760 | N/A |  | N/A |  |
|  |  | n78 | 3350 | 10 | 25 | 3350 | N/A | TDD | N/A |  |
|  |  | 3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A | No |
|  |  | n28 | 743 | 5 | 25 | 798 | N/A |  | N/A |  |
|  |  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | TDD | IMD5 |  |
|  | DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A | No |
|  |  | n78 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |  |
|  |  | n79 | 4910 | 40 | 216 | 4910 | 16.3 | TDD | IMD3 |  |
|  |  | 3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A | No |
|  |  | n79 | 4510 | 40 | 216 | 4510 | N/A | TDD | N/A |  |
|  |  | n78 | 3710 | 10 | 50 | 3710 | 4.2 | TDD | IMD5 |  |
|  | DC_3A-7A_n78A DC_3C-7A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | FDD | IMD3 | No |
|  |  |  |  |  |  |  |  |  | ¦fB78-2*fB7¦ |  |
|  |  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |  |
|  |  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |  |
|  |  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | FDD | IMD4 | No |
|  |  |  |  |  |  |  |  |  | ¦2*fB78-2*fB7¦ |  |
|  |  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |  |
|  |  | n78 | 3475 | 10 | 50 | 3475 | N/A | TDD | N/A |  |
|  | DC_3A-20A_n78A DC_3C-20A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | FDD | IMD3 | No |
|  |  |  |  |  |  |  |  |  | ¦fB78-2*fB20¦ |  |
|  |  | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |  |
|  |  | n78 | 3510 | 10 | 50 | 3510 | N/A | TDD | N/A |  |
|  | DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | FDD | N/A | No |
|  |  | 21 | 1459.5 | 5 | 25 | 1507.5 | 8.8 |  | IMD4 |  |
|  |  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |  |
|  | DC_3A-21A_n77A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | FDD | IMD5 | No |
|  |  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |  |
|  |  | n77 | 3935 | 10 | 50 | 3935 | N/A | TDD | N/A |  |

Observation:

MSD levels and test point configurations are inconsistent between Table 7.3B.2.3.5.1-1 and Table 7.3B.2.3.5.2-1 for EN-DC SUO eligible combinations whose parents are DC_1_n77, DC_3_n77 and DC_3_n78. SUO TP are missing for several combinations in Table 7.3B.2.3.5.2-1. This could be resolved by applying the two-band SUO parent reference sensitivity exception specifications of Table 7.3B.2.3.5.1-1 to all higher-order EN-DC subsets defined in Table 7.3B.2.3.5.2-1.

# Conclusions

In this contribution, we observe several discrepancies in MSD levels and MSD TP configurations. This could be resolved by applying the two-band SUO parent specifications of Table 7.3B.2.3.5.1-1 to all higher-order EN-DC subsets defined in Table 7.3B.2.3.5.2-1.

Observation:

MSD levels and test point configurations are inconsistent between Table 7.3B.2.3.5.1-1 and Table 7.3B.2.3.5.2-1 for EN-DC SUO eligible combinations whose parents are DC_1_n77, DC_3_n77 and DC_3_n78. SUO TP are missing for several combinations in Table 7.3B.2.3.5.2-1. This could be resolved by applying the two-band SUO parent reference sensitivity exception specifications of Table 7.3B.2.3.5.1-1 to all higher-order EN-DC subsets defined in Table 7.3B.2.3.5.2-1.

# References

[1]	38.101-3-f40