3GPP TSG-RAN WG4 Meeting #90 	R4-1901702

# Athens, Greece, 25 February – 1 March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-1 | CR | 0136 | rev |  | Current version: | 13.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.145-1 on Correction of unwanted emissions scaling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AAS_BS_LTE_UTRA-Perf |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At RAN4#85, there was an input concerning emission scaling (R4-1712716) which points out that emission scaling is not possible in the U.S. and it is also not in line with present European regulation for spurious emissions. It was therefore agreed to condition the scaling of emissions by a condition that it applies “unless stated differently in regional regulation”. This was implemented for NR in TS 38.104 (approved CR in R4-1714544). Based on present status of regulation, scaling should be conditioned on regulation for spurios emissions, but not for OBUE limits. Scaling of unwanted emissions is also applied for AAS in TS 37.105 and TS 37.141-1/2. For the same reason as for NR, the scaling of spurious emissions should be conditioned on regional regulation and aligned with the text agreed for TS 38.104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The scaling of spurious emissions limits is conditioned that it applies “unless stated differently in regional regulation”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AAS specifications would not be aligned with NR specifications and would in some cases conflict with regional regulation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4, 6.6.6.5.1, 7.6.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.104, TS 37.105 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.145-2, TS 38.141-1, TS 38.141-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 4.4	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or set by local and regional regulation as mandatory requirements. It is normally not stated in the 3GPP specifications under what exact circumstances that the requirements apply, since this is defined by local or regional regulation.

Table 4.4-1 lists all requirements in the present specification that may be applied differently in different regions. Non-AAS requirements are applicable as defined in the present document. In many cases, such requirements include regional requirements that are implicitly referenced from the present specification, and listed in the specification for the specifications concerned [2] [5].

Table 4.4-1: List of regional requirements

| Subclause number | Requirement | Comments |
| --- | --- | --- |
| 4.5 | Operating bands and Band Categories | Some bands may be applied regionally. |
| 6.2.2.2.1 | Base station output power | These requirements apply in Japan for an E-UTRA BS operating in Band 34 and Band 41. |
| 6.6.2 | Occupied bandwidth | The requirement may be applied regionally. There may also be regional requirements to declare the Occupied bandwidth according to the definition. |
| 6.6.4 | Spectrum emission mask | The mask specified may be mandatory in certain regions. In other regions this mask may not be applied. Additional spectrum protection requirements may apply regionally. |
| 6.6.5 | Operating band unwanted emissions | The BS may have to comply with the applicable emission limits established by FCC Title 47 [15], when deployed in regions where those limits are applied and under the conditions declared by the manufacturer. |
| 6.6.5 | Operating band unwanted emissions | The requirements for protection of DTT may apply regionally. |
| 6.6.5 | Operating band unwanted emissions | Regional requirement as defined in TS 37.104, subclause 6.6.2.4.4 [12] may be applied for the protection of systems operating in frequency bands adjacent to Band 1 as defined in TS 37.104, subclause 4.5, [12] in geographic areas in which both an adjacent band service and UTRA and/or E-UTRA are deployed. |
| 6.6.5 | Operating band unwanted emissions | Additional requirements for Band 41 may apply in certain regions as additional Operating band unwanted emission limits. |
| 6.6.5 | Operating band unwanted emissions | Additional Band 32 unwanted emissions requirements may apply in certain regions. |
| 6.6.6 | Spurious emissions | Category A limits are mandatory for regions where Category A limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [35] apply. Category B limits are mandatory for regions where Category B limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [35] apply. |
| 6.6.6 | Spurious emissions | Additional spurious emissions requirements may be applied for the protection of system operating in frequency ranges other than the AAS BS operating band as described in TS 37.104 [12] subclause 6.6.1.3. |
| 6.6.6 | Spurious emissions | In addition to 3GPP requirements, the BS may have to comply with the applicable emission limits established by FCC Title 47 [15], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |
| 6.6.6 | Spurious emissions | The emission limits specified as the basic limit + X (dB) are applicable, unless stated differently in regional regulation. |
| 6.7 | Transmitter intermodulation | Additional requirements may apply in certain regions. |
| 7.6 | Rx spurious emissions | The emission limits specified as the basic limit + X (dB) are applicable, unless stated differently in regional regulation. |

*********************End of change*****************

*********************Next changed section*****************

#### 6.6.6.5	Test requirements

##### 6.6.6.5.1	General

Conformance may be shown to either the measure and sum test requirement or the per TAB connector test requirement:

1)	The spurious emission test requirements for an AAS BS when using the measure and sum alternative are that for each TAB connector TX cell group and each applicable basic limit as specified in this subclause, the power summation of emissions at the TAB connectors of the TAB connector TX cell group shall not exceed a limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell) , unless stated differently in regional regulation.

2)	The spurious emission test requirements for an AAS BS when using the per TAB connector alternative are that for each TAB connector TX cell group and each applicable basic limit as specified in this subclause, the emissions at each of the TAB connectors of the TAB connector TX cell group shall not exceed a limit specified as the basic limit + X - 10log(n) where n is the number of TAB connectors in the TAB connector TX cell group and X = 10log10(NTXU,countedpercell), unless stated differently in regional regulation.

The appropriate table for the basic limit is based on the same power level (PRated,c,sys) as used for the AAS BS rated power limits for BS classes in table 6.2.2.1-1 the same method of scaling the power level using NTXU,counted is used.

*********************End of change*****************

*********************Next changed section*****************

### 7.6.5	Test Requirements

#### 7.6.5.1	General

Conformance may be shown to either the measure and sum test requirement or the per TAB connector test requirement.

1)	The spurious emission test requirements for an AAS BS when using the measure and sum alternative are that for each TAB connector RX cell group and each applicable basic limit as specified in subclause 7.6.5.2, the power summation of emissions at the TAB connectors of the TAB connector RX cell group shall not exceed a limit specified as the basic limit + X, where X = 10log10(NRXU,countedpercell), unless stated differently in regional regulation.

2)	The spurious emission test requirements for an AAS BS when using the per TAB connector alternative are that for each TAB connector RX cell group and each applicable basic limit as specified in subclause 7.6.5.2, the emissions at each of the TAB connectors of the TAB connector RX cell group shall not exceed a limit specified as the basic limit + X - 10log(n) where n is the number of TAB connectors in the TAB connector RX cell group and X = 10log10(NRXU,countedpercell), unless stated differently in regional regulation.