3GPP TSG-RAN WG4 Meeting #90	R4-1901792

# Athens, Greece, February 25th – March 1st 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR Beam Candidate Detection and Latencies section 8.5.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  |  |  |  |  | Date: |  |  | 2018-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction CR to section 8.5.5 Beam candidate detection Procedures |  |  |  |  |  |  |  |  |
| Summary of change: |  | A number of corrections to ensure beam candidate detection latencies is reasonable with reduced system performance impact. - Evaluation of beam candidates in q1 shall be done as if UE is in no DRX - During beam candidate detection UE is not required to perform gap assisted measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Section 8.5.5 allows too relaxed UE requirements leaving beam candidate detection procedure slow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Next Modified Subclause

### 8.5.5	Requirements for SSB based candidate beam detection

#### 8.5.5.1	Introduction

The requirements in this section apply for each SSB resource in the set  configured for a serving cell, provided that the SSBs configured for candidate beam detection are actually transmitted within UE active DL BWP during the entire evaluation period specified in section 8.5.5.2.

#### 8.5.5.2	Minimum requirement

Upn request the UE shall be able to evaluate whether the L1-RSRP measured on the configured SSB resource in set  estimated over the last TEvaluate_CBD_SSB [ms] period becomes better than the threshold Qin_LR.

Upon being requested by higher layers as specified in TS 38.321 [7], the UE shall monitor the configured SSB resources using the evaluation period corresponding to the no DRX mode until the SSB block indexes from the set q1 and the corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold has been delivered to higher layers.

The value of TEvaluate_CBD_SSB is defined in Table 8.5.5.2-1 for FR1 and FR2.

Table 8.5.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1 and FR2

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX | ceil([3]*P) * TSSB |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

### 8.5.6	Requirements for CSI-RS based candidate beam detection

#### 8.5.6.1	Introduction

The requirements in this section apply for each CSI-RS resource in the set  configured for a serving cell, provided that the CSI-RS resources configured for candidate beam detection are actually transmitted within UE active DL BWP during the entire evaluation period specified in section 8.5.6.2.

#### 8.5.6.2	Minimum requirement

Upn request the UE shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in set  estimated over the last TEvaluate_CBD_CSI-RS [ms] period becomes better than the threshold Qin_LR within TEvaluate_CBD_CSI-RS [ms] period.

Upon being requested by higher layers as specified in TS 38.321 [7], the UE shall monitor the configured SSB resources using the evaluation period corresponding to the no DRX mode until the SSB block indexes from the set q1 and the corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold has been delivered to higher layers.

The value of TEvaluate_CBD_CSI-RS is defined in Table 8.5.6.2-1 for FR1 and FR2.

Table 8.5.6.2-1: Evaluation period TEvaluate_CBD_CSI-RS for FR1 and FR2

| Configuration | TEvaluate_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TBD, ceil(MCBD *P) * TCSI-RS) |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

Next Modified Subclause