3GPP TSG-RAN WG4 Meeting #90 	R4-1901961

# Athens, Greece, 25th February – 1st March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6345 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NE-DC in 36.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NE-DC is not supported in the current specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | General introduction of NE-DC in 36.133 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NE-DC is not supported in the current specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.3, 3.6.1; New sections: 3.6.2, 3.6.3, 8.1.2.1.1c, 8.1.2.1.1.c.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [26] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [26].

Any Cell Selection state: as defined in TS 36.304 [1]

Asynchronous Dual Connectivity: As defined in TS 36.331 [2].

Carrier aggregation: aggregation of two or more component carriers in order to support wider transmission bandwidths TS 36.104 [30].

Dual Connectivity: As defined in TS 36.331 [2].

EN-DC:  E-UTRA-NR Dual Connectivity as defined in TS 37.340 [41, Section 4.1.2].

en-gNB: As defined in TS 37.340 [41].

Extended IDLE-mode DRX: extended DRX cycles in IDLE mode as specified in TS 24.008 [34], where one extended DRX cycle is a time period between two first paging occasions within two consecutive PTWs.

Extended CONNECTED-mode DRX: extended DRX cycles in CONNECTED mode as specified in TS 36.331 [2].

FeMBMS/Unicast mixed cell: an MBMS/Unicast cell performing MBMS transmissions as defined in TS 36.300 [25].

Frame Structure 3: frame structure type 3 as defined in TS 36.211 [16]

gNB: as defined in TS 38.300 [46]

High operating band: an operating band with a higher downlink frequency with respect to another, low, operating band.

Inter-band carrier aggregation: carrier aggregation of component carriers in different operating bands TS 36.104 [30].

Intra-band contiguous carrier aggregation: contiguous carriers aggregated in the same operating band TS 36.104 [30].

Intra-band non-contiguous carrier aggregation: non-contiguous carriers aggregated in the same operating band TS 36.104 [30].

IDC autonomous denial subframes: The maximum number of uplink subframes in which the UE is allowed not to transmit E-UTRAN signals when configured with IDC autonomous denial (TS 36.331 [2]).

IDC autonomous denial validity: It is the period over which the autonomous denial subframes are counted (TS 36.331 [2]).

IDC solution: This refers to DRX or IDC autonomous denial configured by eNodeB in response to receiving InDeviceCoexIndication from the UE (TS 36.331 [2]).

Low operating band: an operating band with a lower downlink frequency with respect to another, high, operating band.

Master Cell Group: As defined in TS 36.331 [2].

Master eNB: As defined in TS 36.300 [25].

MBSFN ABS: ABS configured in MBSFN-configurable subframe.

NB-IoT Cell: A cell for NB-IoT.

NB-IoT: As defined in TS 36.331 [2].

ng-eNB: As defined in TS 38.300 [46].

NE-DC: NR-E-UTRA Dual Connectivity as defined in TS 37.340 [41, Section 4.1.3.2].

Non-MBSFN ABS: ABS configured in any downlink subframe.

Normal Performance Group: For UE which supports Increased UE carrier monitoring UTRA or E-UTRA the group of inter-frequency carriers or inter-RAT carriers is divided into two groups. The group which has a better delay performance compared to the other group is refered to as the normal performance group.

Paging Time Window: As defined in TS 24.008 [34].

Primary Cell: As defined in TS 36.331 [2].

ProSe Direct Communication: As defined in TS 23.303 [33]

ProSe Direct Discovery: As defined in TS 23.303 [33]

Primary SCell: As defined in TS 36.331 [2].

Primary Secondary Timing Advance Group: Timing Advance Group containing the PSCell.

Primary Timing Advance Group: Timing Advance Group containing the PCell.

Reduced Performance Group: For UE which supports Increased UE carrier monitoring UTRA or E-UTRA the group of inter-frequency carriers or inter-RAT carriers is divided into two groups. The group which has worse delay performance compared to the other group is refered to as the reduced performance group

Secondary Cell: As defined in TS 36.331 [2].

Secondary eNB: As defined in TS 36.300 [25].

Serving Cell: As defined in TS 36.331 [2].

Secondary Cell Group: As defined in TS 36.331 [2].

Secondary Timing Advance Group: As defined in TS 36.331 [2].

SSB: SS/PBCH block as defined in TS 38.211 [42, section 7.4.3].

sTTI : A transmission time interval (TTI) of either one slot or one subslot transmission duration as defined in TS 36.211 [16] on either uplink or downlink.

Synchronous Dual Connectivity: As defined in TS 36.331 [2].

TDD-FDD carrier aggregation: Carrier aggregation of component carriers in E-UTRA TDD and E-UTRA FDD operating bands TS 36.104 [30].

Timing Advance Group: As defined in TS 36.331 [2].

WLAN RSSI: As defined in TS36.214 [4].

x_RA: x-to-RS EPRE ratio for the channel or physical signal x in all transmitted OFDM symbols not containing RS.

x_RB: x-to-RS EPRE ratio for the channel or physical signal x in all transmitted OFDM symbols containing RS.

--- unchanged sections ---

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [26] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [26].

1x RTT	CDMA2000 1x Radio Transmission Technology

ABS	Almost Blank Subframe

ARQ	Automatic Repeat Request

AP	Access Point

AWGN	Additive White Gaussian Noise

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

CA	Carrier Aggregation

CC	Component Carrier

CCCH SDU	Common Control Channel SDU

CE	Coverage Enhancement

CGI	Cell Global Identifier

CPICH	Common Pilot Channel

CPICH Ec/No	CPICH Received energy per chip divided by the power density in the band

CRS	Cell-specific Reference Signals

C-RNTI	Cell RNTI

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DL	Downlink

DMTC	Discovery signal Measurement Timing Configuration

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

DUT	Device Under Test

E-CID	Enhanced Cell-ID (positioning method)

ECGI	Evolved CGI

eDRX_IDLE	Extended IDLE-mode DRX

eDRX_CONN	Extended CONNECTED-mode DRX

eNB	E-UTRAN NodeB

EN-DC	E-UTRA-NR Dual Connectivity

E-SMLC	Enhanced Serving Mobile Location Centre

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

FDD	Frequency Division Duplex

FS3	Frame Structure type 3

GERAN	GSM EDGE Radio Access Network

GNSS	Global Navigational Satellite System

GSM	Global System for Mobile communication

HARQ	Hybrid Automatic Repeat Request

HD-FDD	Half-Duplex FDD

HO	Handover

HRPD	High Rate Packet Data

IDC	In-Device Coexistence

IEEE	Institute of Electrical and Electronics Engineers

LBT	Listen before talk

LPP	LTE Positioning Protocol

LWA	LTE-WLAN Aggregation

MAC 	Medium Access Control

MCG	Master Cell Group

MeNB	Master eNB

MBMS	Multimedia Broadcast Multicast Service

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MBSFN ABS	MBSFN Almost Blank Subframe

MDT	Minimization of Drive Tests

MGRP	Measurement Gap Repetition Period

MIB	Master Information Block

MPDCCH	MTC Physical Downlink Control Channel

NE-DC	NR-E-UTRA Dual Connectivity

NG-RAN	NG  Radio Access Network

NR	New Radio

NSA	Non-standalone

NPBCH	Narrowband Physical Broadcast CHannel

NPDCCH	Narrowband Physical Downlink Control CHannel

NPDSCH	Narrowband Physical Downlink Shared CHannel

NPRACH	Narrowband Physical Random Access CHannel

NPUSCH	Narrowband Physical Uplink Shared CHannel

NPSS	Narrowband Primary Synchronization Signal

NRS	Narrowband Refernce Signal

NRSRP	Narrowband Reference Signal Received Power

NRSRQ	Narrowband Reference Signal Received Quality

NSCH	Narrowband Synchronization Channel

NSSS 	Narrowband Secondary Synchronization Signal

OCNG	OFDMA Channel Noise Generator

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OTDOA	Observed Time Difference of Arrival

PBCH	Physical Broadcast Channel

P-CCPCH	Primary Common Control Physical Channel

PCell	Primary Cell

PCFICH	Physical Control Format Indicator CHannel

PDCCH	Physical Downlink Control CHannel

PDSCH	Physical Downlink Shared CHannel

PHICH	Physical Hybrid-ARQ Indicator CHannel

PLMN	Public Land Mobile Network

PMCH	Physical Multicast Channel

PRACH	Physical Random Access CHannel

ProSe	Proximity-based Services

PRS	Positioning Reference Signal

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control Channel

PSCell	Primary SCell

PSS	Primary Synchronization SignalPSSCH	Physical Sidelink Shared CHannel

psTAG	Primary Secondary Timing Advance Group

pTAG	Primary Timing Advance Group

PTW	Paging Time Window

PUCCH	Physical Uplink Control CHannel

PUSCH	Physical Uplink Shared Channel

RS-SINR	Reference Signal Signal to Noise and Interference RatioRSCP	Received Signal Code Power

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

QAM	Quadrature Amplitude Modulation

RACH	Random Access Channel

RAT	Radio Access Technology

RNC	Radio Network Controller

RNTI	Radio Network Temporary Identifier

RRC	Radio Resource Control

RRM	Radio Resource Management

SCE	Small Cell Enhancement

SCH	Synchronization Channel

SCell	Secondary Cell

SCG	Secondary Cell GroupSDU	Service Data Unit

SCS	Subcarrier spacing

SeNB	Secondary eNB

SFN	System Frame Number

SI	System Information

SIB	System Information Block

SLSS	SideLink Synchronization Sequence

SON	Self Optimized Network

SPDCCH	Short Physical Downlink Control channel

SPUCCH	Short Physical Uplink Control channel

SRS	Sounding Reference Signal

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSS	Secondary Synchronization Signal

SSTD	SFN and subframe time difference

sTAG	Secondary Timing Advance Group

TAG	Timing Advance Group

TDD	Time Division Duplex

TP	Transmission Point

TTI	Transmission Time Interval

UE	User Equipment

UL	Uplink

UMTS	Universal Mobile Telecommunication System

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

V2V	Vehicle to Vehicle

V2X	Vehicle to Everything

WLAN	Wireless Local Area Network

WB-RSRQ	Wide Bandwith RSRQ

--- unchanged sections ---

## 3.6	General

### 3.6.1	Applicability of requirements in this specification version

In this specification,

-	‘cell’, ‘PCell’, ‘PSCell’ and ‘SCell’ refer to E-UTRA cell, E-UTRA PCell, E-UTRA PSCell and E-UTRA SCell, respectively,

-	NR cells are referred to as ‘NR cell’, ‘NR PCell’, ‘NR PSCell’ and ‘NR SCell’,

-	‘dual connectivity’ refers to Intra-E-UTRA dual connectivity,

-	E-UTRA-NR dual connectivity or EN-DC refer to when E-UTRA is the master,

-	NR-E-UTRA dual connectivity or NE-DC refer to when NR is the master.

-	The requirements for TDD-FDD carrier aggregation are specified for two downlink and one uplink component carriers. The requirements are specified for both cases when the PCell belongs to TDD or FDD.

-	All the requirements for intra-band contiguous and non-contiguous CA apply under the assumption of the same uplink-downlink and special subframe configurations [16] in the PCell and SCell.

-	All the requirements for inter-band CA apply for the same uplink-downlink and special subframe configurations [16] in the PCell and SCell. Different uplink-downlink and special subframe configurations [16] in the PCell and SCell are supported for inter-band CA for UEs which:

- do not support simultaneous reception and transmission for inter-band TDD CA specified in TS 36.331 [2], and

- are compliant to the requirements specified in TS 36.101 for inter-band CA with uplink in one E-UTRA band and without simultaneous Rx/Tx.

-	All the inter-frequency requirements and requirements for measurements on deactivated carrier apply for the same uplink-downlink and special subframe configurations [16] in the PCell and SCell. Different uplink-downlink and special subframe configurations [16] in the PCell and SCell are supported for inter-frequency for UEs which:

-	do not support simultaneous reception and transmission for inter-band TDD CA specified in TS 36.331 [2], and are compliant to the requirements specified in TS 36.101 for inter-band CA with uplink in one E-UTRA band and without simultaneous Rx/Tx.

-	Requirements for E-UTRA carrier aggregation are applicable for the CA capable UE which has been configured with at least one downlink SCell, but:

-	up to four downlink CCs intra-band contiguous and up to three downlink CCs inter-band and one uplink CC for carrier aggregation, or

-	up to three downlink CCs intra-band contiguous and up to four downlink CCs inter-band and one uplink CC for carrier aggregation, or

-	up to four downlink CCs and up to two uplink CCs for inter-band carrier aggregation, or

-	up to five downlink CCs and up to two uplink CCs for intra-band contiguous carrier aggregation, or

-	up to four downlink CCs and one uplink CCs for inter-band carrier aggregation, or

-	up to two downlink/uplink CCs intra-band contiguous and one downlink/uplink inter-band carrier aggregation, or

-	up to two downlink CCs intra-band contiguous and up to three downlink inter-bands and up to two uplink CCs for inter-band carrier aggregation, or

-	up to two downlink CCs intra-band non-contiguous and up to three downlink inter-bands and up to two uplink CCs for inter-band carrier aggregation, or

-	up to three downlink CCs intra-band contiguous and one downlink intra-band non-contiguous and up to two uplink CCs intra-band contiguous for carrier aggregation, or

-	two sub-blocks intra-band non-contiguous with two downlink CCs intra-band contiguous per sub-blocks and up to two uplink CCs intra-band contiguous for carrier aggregation, or

-	two sub-blocks intra-band non-contiguous with two downlink CCs intra-band contiguous and three downlink CCs intra-band contiguous per sub-block  and up to two uplink CCs intra-band contiguous for carrier aggregation, or

-	two downlink CCs intra-band contiguous and three down link CCs inter-band and one uplink CC for carrier aggregation, or

-	two downlink CCs intra-band contiguous and two downlink CCs intra-band contiguous and one downlink CC inter-band and one uplink CC for carrier aggregation, or

-	up to two downlink CCs and up to two uplink CCs for intra-band non-contiguous carrier aggregation, or

-	up to two downlink CCs and up to two uplink CCs for inter-band carrier aggregation, or

-	up to two downlink CCs intra-band contiguous and one downlink inter-band and one uplink CC for carrier aggregation, or

-	up to two downlink CCs intra-band non-contiguous and one downlink inter-band and one uplink CC for carrier aggregation, or

-	up to two downlink CCs and one uplink CC for intra-band non-contiguous carrier aggregation.

-	Requirements for E-UTRA carrier aggregation for discovery signal measurements are applicable for CA capable UE which has been configured with at least one downlink SCell, but:

-	up to four downlink CCs intra-band contiguous and up to three downlink CCs inter-band and one uplink CC for carrier aggregation, or

-	up to three downlink CCs intra-band contiguous and up to four downlink CCs inter-band and one uplink CC for carrier aggregation, or

-	up to four downlink CCs and up to two uplink CCs for inter-band carrier aggregation, or

-	up to five downlink CCs and up to two uplink CCs for intra-band contiguous carrier aggregation, or

-	up to four downlink CCs and up to one uplink CCs for inter-band carrier aggregation, or

-	up to two downlink/uplink CCs intra-band contiguous and one downlink/uplink inter-band carrier aggregation, or

-	up to two downlink CCs intra-band contiguous and up to three downlink inter-band and up to two uplink CCs for inter-band carrier aggregation, or

-	up to two downlink CCs intra-band non-contiguous and up to three downlink inter-band and up to two uplink CCs for inter-band carrier aggregation, or

-	up to three downlink CCs intra-band contiguous and one downlink intra-band non-contiguous and up to two uplink CCs intra-band contiguous for carrier aggregation, or

-	two sub-blocks intra-band non-contiguous with two downlink CCs intra-band contiguous per sub-blocks and up to two uplink CCs intra-band contiguous for carrier aggregation, or

-	two sub-blocks intra-band non-contiguous with two downlink CCs intra-band contiguous and three downlink CCs intra-band contiguous per sub-block  and up to two uplink CCs intra-band contiguous for carrier aggregation, or

-	two downlink CCs intra-band contiguous and three down link CCs inter-band and one uplink CC for carrier aggregation, or

-	two downlink CCs intra-band contiguous and two downlink CCs intra-band contiguous and one downlink CC inter-band and one uplink CC for carrier aggregation, or

-	up to two downlink CCs and up to two uplink CCs for intra-band non-contiguous carrier aggregation, or

-	up to two downlink CCs and up to two uplink CCs for inter-band carrier aggregation, or

-	up to two downlink CCs intra-band contiguous and one downlink inter-band and one uplink CC for carrier aggregation, or

-	up to two downlink CCs intra-band non-contiguous and one downlink inter-band and one uplink CC for carrier aggregation, or

-	up to two downlink CCs and one uplink CC for intra-band non-contiguous carrier aggregation.

-	Requirements for E-UTRA carrier aggregation, where the PCell is FDD PCell or TDD PCell and at least one downlink SCell follows the frame structure 3 and no UL SCell following the frame structure type 3 [16], are applicable for the CA capable UE, which has been configured with at least one downlink SCell but:

-	up to four downlink CCs intra-band contiguous and up to three downlink CCs inter-band and one uplink CC for carrier aggregation, or

-	up to three downlink CCs intra-band contiguous and up to four downlink CCs inter-band and one uplink CC for carrier aggregation, or

-	up to five downlink CCs and up to two uplink CCs for inter-band carrier aggregation.

-	Requirements for E-UTRA carrier aggregation, where the PCell is FDD PCell or TDD PCell, and at least one downlink SCell and one uplink SCell follow the frame structure type 3 [16], are applicable for the CA capable UE, which has been configured with at least one downlink SCell and at least one uplink SCell but:

-	up to five downlink CCs and two uplink CCs for inter-band carrier aggregation.

-	The requirements for UE configured with eDRX_CONN cycle do not apply for CA requirements and dual connectivity requirements.

-	The requirements for a UE category 0 are derived assuming UE category 0 [31] and a single antenna receiver.

-	The requirements for UE category M1 are derived assuming: DL Category M1 and Uplink Category M1, operation in any LTE system bandwidth but with a channel bandwidth of 1.4 MHz and transmission bandwidth of 6 PRBs in downlink and uplink, and a single antenna receiver. DL UE category M1 and UL UE category M1 are defined in TS 36.306 [31].

-	The requirements for normal coverage in idle mode shall apply provided the UE category M1 is with the radio condition that SCH Ês/Iot-6 dB and CRS Ês/Iot -6 dB.

-	The requirements for enhanced coverage in idle mode shall apply provided the UE category M1 is with the radio condition that SCH Ês/Iot  -15 dB and CRS Ês/Iot  -15 dB.

-	The requirements for CEMode A shall apply provided the UE category M1 is configured with CEMode A, SCH Ês/Iot  -6 dB and CRS Ês/Iot  -6 dB. The CEMode A and the number of repetition levels for different physical channels are defined in TS 36.213 [3].

-	The requirements for CEMode B shall apply provided the UE category M1 is configured with CEMode B, SCH Ês/Iot  -15 dB and CRS Ês/Iot  -15 dB. The CEMode B and the number of repetition levels for different physical channels are defined in TS 36.213 [3].

-	The requirement for UE category M2 are derived assuming downlink category M2 and uplink category M2, operation in any LTE system bandwidth but with a channel bandwidth not exceeding 5MHz, transmission bandwidth not exceeding 24RB in downlink and 5MHz in uplink, and a single antenna receiver. DL UE category M2 and UL UE category M2 are defined in TS 36.306 [31].

-	The requirements for normal coverage in idle mode shall apply provided the UE category M2 is with the radio condition that SCH Ês/Iot  -6 dB and CRS Ês/Iot  -6 dB.

-	The requirements for enhanced coverage in idle mode shall apply provided the UE category M2 is with the radio condition that SCH Ês/Iot  -15 dB and CRS Ês/Iot  -15 dB.

-	The requirements for CEMode A shall apply provided the UE category M2 is configured with CEMode A, SCH Ês/Iot  -6 dB and CRS Ês/Iot  -6 dB. The CEMode A and the number of repetition levels for different physical channels are defined in TS 36.213 [3].

-	The requirements for CEMode B shall apply provided the UE category M2 is configured with CEMode B, SCH Ês/Iot  -15 dB and CRS Ês/Iot  -15 dB. The CEMode B and the number of repetition levels for different physical channels are defined in TS 36.213 [3].

-	Unless explicitly defined the following additional requirements are applicable to UE category M2:

-	Cell Selection and Re-selection Requirements in section 4.7

-	Handover requirements in section 5.5 and 5.6

-	Random access requirements in section 6.2.3

-	RRC re-establishment requirements in section 6.7

-	RRC connection release with redirection requirements in section 6.8

-	Radio Link monitoring requreiements in section 7.19

-	Timing advance requirements in section 7.28

-	UE timer accuracy requirement in section 7.27

-	E-UTRAN intra frequency measurement requirements in section 8.13.2.1 and 8.13.3.1

-	E-UTRAN inter frequency measurement requirements in section 8.13.2.6 and 8.13.3.5

-	UE measurement capability in section 8.13.2.7 and 8.13.3.6

-	E-UTRAN E-CID measurements requirements in section 8.13.2.5.1, 8.13.2.5.2, 8.13.2.5.3, 8.13.2.5.4, 8.13.2.5.5, 8.13.2.5.6 and 8.13.3.4

-	Measurement accuracy requirements in section 9.1.21.

-	The requirements for non-BL/CE UE are derived assuming: DL and UL category other than Category 0/M1/M2/NB1/NB2, operation in any LTE system bandwidth but with a channel bandwidth not exceeding 20MHz, transmission bandwidth not exceeding 96RB in downlink and 5MHz in uplink, and dual antenna receiver, when in RRC_IDLE mode camped on a cell acquired using SIB1-BR, or in RRC_CONNECTED configured with CE mode A/B. Non-BL/CE UE is defined in [31].

-	The Cat-M2 UE requirements for normal coverage in idle mode shall apply provided the UE is non-BL/CE and with the radio condition that the serving cell SCH Ês/Iot  -6dB and CRS Ês/Iot  -6 dB, unless corresponding individual non-BL/CE requirements are specified.

-	The Cat-M2 UE requirements for enhanced coverage in idle mode shall apply provided the UE is non-BL/CE and with the radio condition that the serving cell -6dB  SCH Ês/Iot  -15dB and -6dB  CRS Ês/Iot  -15 dB, unless corresponding individual non-BL/CE requirements are specified.

-	The Cat-M2 UE requirements for CEMode A shall apply provided the UE is non-BL/CE and is configured with CEModeA, the serving cell SCH Ês/Iot  -6 dB and CRS Ês/Iot  -6 dB, unless corresponding individual non-BL/CE requirements are specified. The CEMode A and the number of repetition levels for different physical channels are defined in [3].

-	The Cat-M2 UE requirements for CEMode B shall apply provided the UE is non-BL/CE and is configured with CEMode B, the serving cell SCH Ês/Iot  -15 dB and CRS Ês/Iot  -15 dB, unless corresponding individual non-BL/CE requirements are specified. The CEMode B and the number of repetition levels for different physical channels are defined in [3].

-	Unless explicitly defined the following additional requirements are applicable to non-BL/CE UE:

-	Cell Selection and Re-selection Requirements in section 4.7

-	Handover requirements in section 5.5 and 5.6

-	Random access requirements in section 6.2.3

-	RRC re-establishment requirements in section 6.7

-	RRC connection release with redirection requirements in section 6.8

-	UE transmit timing requirements in section 7.26

-	Radio Link monitoring requreiements in section 7.19

-	Timing advance requirements in section 7.28

-	UE timer accuracy requirement in section 7.27

-	E-UTRAN intra frequency measurement requirements in section 8.13.2.1 and 8.13.3.1

-	E-UTRAN inter frequency measurement requirements in section 8.13.2.6 and 8.13.3.5

-	UE measurement capability in section 8.13.2.7 and 8.13.3.6

-	E-UTRAN E-CID measurements requirements in section 8.13.2.5.1, 8.13.2.5.2, 8.13.2.5.3, 8.13.2.5.4, 8.13.2.5.5, 8.13.2.5.6 and 8.13.3.4

-	E-UTRAN OTDOA RSTD measurements requirements in section 8.16.2.3, 8.16.2.4, 8.16.3.1 and 8.16.3.2.

-	Measurement accuracy requirements in section 9.1.21 and 9.1.25 if the UE is of category 1bis.

-	Requirements for E-UTRA ProSe Direct Discovery and E-UTRA ProSe Direct Communication are applicable for ProSe operation on either the uplink frequency of PCC, or SCC, or a non-serving carrier, but:

-	with ProSe operation limited to one carrier on a given subframe.

-	Requirements for interruptions due to ProSe Direct Discovery and/or ProSe Direct Communications specified in clause 7.16.3 apply, but:

-	with configured serving carriers of up to two downlink CCs, unless the UE is configured with reception gap for ProSe operation, and

-	with configured serving carriers of up to two uplink CCs, unless the UE is configured with transmission gap for ProSe operation.

-	The requirements for UE category NB1 are derived assuming UE category NB1and a single antenna receiver. UE category NB1 is defined in TS 36.306 [31].

-	The requirements for normal coverage shall apply for UE category NB1 provided that the radio condition of its serving cell are: NSCH Ês/Iot  -6 dB and NRS Ês/Iot  -6 dB.

-	The requirements for enhanced coverage shall apply for UE category NB1 provided that the radio condition of its serving cell are: -15 dB  NSCH Ês/Iot < -6 dB and -15 dB  NRS Ês/Iot < -6 dB.

-	The measurement accuracy requirements in section 9.1.22 for intra-frequency and inter-frequency absolute NRSRQ accuracy for UE Category NB1 apply only in idle mode.

-	The measurement accuracy requirements in section 9.1.22 for intra-frequency absolute NRSRP accuracy for UE Category NB1 apply in idle and connected mode.

-	The measurement accuracy requirements in section 9.1.22 for inter-frequency absolute NRSRP accuracy for UE Category NB1 apply only in idle mode.

-	The requirements for SRS carrier based switching shall apply when the UE capable of SRS carrier based switching is configured to perform SRS carrier based switching for transmitting SRS and/or RACH in one or more CCs in the same or different time resources.

-	The requirements for a UE category 1bis are derived assuming UE category 1bis [31] and a single antenna receiver. Following requirements are applicable to UE category 1bis.

-	Cell re-selection requirements in section 4.2.2.1 to 4.2.2.10

-	Handover requirements in section 5.1, 5.2, 5.3 and 5.4

-	RRC re-establishment requirements in section 6.1

-	Random access requirements in section 6.2

-	RRC connection release with redirection requirements in section 6.3

-	UE transmit timing requirements in section 7.1

-	UE timer accuracy requirements in section 7.2

-	Timing advance requirements in section 7.3

-	Radio link monitoring requirements in section 7.11

-	UE measurement capability in section 8.1.2.1

-	E-UTRAN intra frequency measurement requirements in section 8.5.2.1.1 and 8.5.2.1.3

-	E-UTRAN inter frequency measurement requirements in section 8.1.2.3.1, 8.1.2.3.2, 8.1.2.3.3 and 8.1.2.3.4

-	Inter RAT measurement requirements in section 8.1.2.4

-	OTDOA Intra-Frequency measurement requirements in section 8.1.2.5.3, 8.1.2.5.4

-	OTDOA Inter-Frequency measurement requirements in section 8.1.2.6.5, 8.1.2.6.6, 8.1.2.6.7 and 8.1.2.6.8

-	E-UTRAN E-CID measurement requirements in section 8.1.2.7

-	CGI reading requirements for UE category 0 in section 8.5.2.1.4 and 8.5.2.1.6

-	Intra-frequency RSRP Accuracy Requirements in section 9.1.2.7 and 9.1.2.8

-	Inter-frequency RSRP Accuracy Requirements in section 9.1.3.3 and 9.1.3.4

-	Intra-frequency RSRQ Accuracy Requirements in section 9.1.5.5

-	Inter-frequency RSRQ Accuracy Requirements in section 9.1.6.5 and 9.1.6.6

-	RSTD Intra-Frequency Accuracy Requirement in section 9.1.10.5

-	RSTD Inter-Frequency Accuracy Requirement in section 9.1.10.6

-	UE Rx – Tx time difference measurement accuracy requirements in section 9.1.9.1 and 9.1.9.2

-	The requirements for UE category NB2 are derived assuming UE category NB2 and a single antenna receiver. UE category NB2 is defined in TS 36.306 [31]. Following requirements are applicable to UE category NB2.

-	Cell selection and re-selection requirements in section 4.6.1 and 4.6.2

-	UE Positioning measurement in idle state in section 4.8

-	RRC Re-establishment requirements in section 6.5

-	Random access requirements in section 6.6

-	RRC connection redirection to non-anchor carrier requirements in section 6.9

-	UE transmit timing requirements in section 7.20

-	UE timer accuracy requirements in section 7.21

-	Timing advance requirements in section 7.22

-	Radio link monitoring requirements in section 7.23

-	UE RRC_CONNECTED state measurement requirement in section 8.14

-	UE measurement accuracy requirements in section 9.1.22

-	Power headroom requirements in section 9.1.23

-	All requirements in this specification for UE receiving PMCH in FeMBMS/Unicast-mixed cells apply only for FeMBMS/Unicast-mixed cells configured based on frame structure 1.

-	Requirements for E-UTRA carrier aggregation with one or more FeMBMS/Unicast-mixed SCells shall apply, provided the total number of SCCs, including SCCs with FeMBMS/Unicast-mixed SCells, does not exceed the the maximum number of SCCs the UE is capable of.

-	Requirements for EN-DC operation are applicable for the UE which has been configured with the following number of E-UTRA CCs and NR CCs:

-	up to 5 E-UTRA DL CCs in total with 1 E-UTRA UL CC in MCG. The applicable number of NR CC for EN-DC in the SCG is specified in TS 38.133 [50].

In addition to the requirements explicitly defined for a UE configured with EN-DC the following requirements shall also appyly for the UE configured with EN-DC:

-	Handover requirements in sections 5.1, 5.3.1, 5.3.2 and 5.3.3,

-	RRC Re-establishment requirements in section 6.1,

-	Random access requirements in section 6.2,

-	RRC connection release with redirection requirements in section 6.3,

-	UE transmit timing requirements defined in section 7.1 for UE configured with only pTAG,

-	UE timer accuracy requirements in section 7.2,

-	Timing advance requirements defined in section 7.3 for 1ms TTI and 4 subframe HARQ processing,

-	Radio link monitoring requirements in section 7.6,

-	SCell activation and deactivation delay requirements for E-UTRA carrier aggregation defined in section 7.7 for 1ms TTI and 4 subframe HARQ processing except those for CA with frame structure # 3,

-	Requirements on received time difference between the PCell and SCell or between SCells defined in sections 7.9 except those defined for CA with frame structure # 3, .

-	E-UTRAN intra frequency measurement requirements in section 8.1.2.2, except requirements specified for UE configured with highSpeedEnhancedMeasFlag

-	E-UTRAN OTDOA intra-frequency RSTD measurements requirements defined in section 8.1.2.5 except those for UE category 1bis,

-	E-UTRAN OTDOA inter-frequency RSTD measurements requirements defined in section 8.1.2.6 except those for UE category 1bis,

-	E-UTRAN E-CID measurements requirements in section 8.1.2.7,

-	Requirements on measurements for E-UTRA carrier aggregation in section 8.3,

-	OTDOA RSTD measurement requirements for E-UTRAN carrier aggregation in section 8.4

--- unchanged sections ---

### 3.6.2	Applicability of requirements for EN-DC operation

Requirements for EN-DC operation are applicable for the UE which has been configured with the following number of E-UTRA CCs and NR CCs:

-	up to 5 E-UTRA DL CCs in total with 1 E-UTRA UL CC in MCG. The applicable number of NR CC for EN-DC in the SCG is specified in TS 38.133 [50, section 3.6.2].

In addition to the requirements explicitly defined for a UE configured with EN-DC the following requirements shall also apply for the UE configured with EN-DC:

-	Handover requirements in sections 5.1, 5.3.1, 5.3.2 and 5.3.3,

-	RRC Re-establishment requirements in section 6.1,

-	Random access requirements in section 6.2,

-	RRC connection release with redirection requirements in section 6.3,

-	UE transmit timing requirements defined in section 7.1 for UE configured with only pTAG,

-	UE timer accuracy requirements in section 7.2,

-	Timing advance requirements defined in section 7.3 for 1ms TTI and 4 subframe HARQ processing,

-	Radio link monitoring requirements in section 7.6,

-	SCell activation and deactivation delay requirements for E-UTRA carrier aggregation defined in section 7.7 for 1ms TTI and 4 subframe HARQ processing except those for CA with frame structure # 3,

-	Requirements on received time difference between the PCell and SCell or between SCells defined in sections 7.9 except those defined for CA with frame structure # 3, .

-	E-UTRAN intra frequency measurement requirements in section 8.1.2.2, except requirements specified for UE configured with highSpeedEnhancedMeasFlag

-	E-UTRAN OTDOA intra-frequency RSTD measurements requirements defined in section 8.1.2.5 except those for UE category 1bis,

-	E-UTRAN OTDOA inter-frequency RSTD measurements requirements defined in section 8.1.2.6 except those for UE category 1bis,

-	E-UTRAN E-CID measurements requirements in section 8.1.2.7,

-	Requirements on measurements for E-UTRA carrier aggregation in section 8.3,

-	OTDOA RSTD measurement requirements for E-UTRAN carrier aggregation in section 8.4,

-	Requirements in Section 9 for intra-frequency RSRP, RSRQ, RS-SINR, and RSTD measurements accuracy for PCell carrier frequency,

-	Requirements in Section 9 for inter-frequency RSRP, RSRQ, RS-SINR, and RSTD measurements accuracy for non-serving E-UTRA carrier frequencies,

-	Requirements in Section 9 for carrier aggrgation RSRP, RSRQ, RS-SINR, and RSTD measurements accuracy for PCC, SCC, or both,

-     Requirements in Section 9 for inter-RAT E-UTRAUTRA measurements accuracy and inter-RAT E-UTRAGSM measurements accuracy for UTRA and GSM carriers,

-	Power headroom requirements in Section 9 for PSCell and SCell(s).

### 3.6.3	Applicability of requirements for NE-DC operation

Requirements for NE-DC operation are applicable for the UE which has been configured with the following number of E-UTRA CCs and NR CCs:

-	up to 5 E-UTRA DL CCs in total with 1 E-UTRA UL CC in SCG. The applicable number of NR CC for EN-DC in the SCG is specified in TS 38.133 [50, section 3.6.2].

In addition to the requirements explicitly defined for a UE configured with NE-DC the following requirements shall also apply for the UE configured with NE-DC:

-	Random access requirements in section 6.2 for random access procedures on PSCell and activated SCell(s),

-	UE transmit timing requirements in section 7.1 for psTAG using PSCell as a reference cell and sTAG(s),

-	UE timer accuracy requirements in section 7.2 for UE operation on PSCell and SCell(s),

-	Timing advance requirements in section 7.3 for 1ms TTI and 4 subframe HARQ processing,

-	Radio link monitoring requirements in section 7.6 for PSCell,

-	SCell activation and deactivation delay requirements for E-UTRA carrier aggregation in section 7.7 for 1ms TTI and 4 subframe HARQ processing except those for CA with frame structure # 3,

-	E-UTRAN intra-frequency measurement requirements in section 8.1.2.2 for PSCC and SCC, except requirements specified for UE configured with highSpeedEnhancedMeasFlag,

-	E-UTRAN inter-frequency measurement requirements in section 8.1.2.3 for non-serving E-UTRA carrier frequencies, except requirements specified for UE configured with highSpeedEnhancedMeasFlag,

-	E-UTRAN OTDOA intra-frequency RSTD measurements requirements in section 8.1.2.5 for PSCC and SCC, except those for UE category 1bis,

-	E-UTRAN OTDOA inter-frequency RSTD measurements requirements in section 8.1.2.6 for non-serving E-UTRA carrier frequencies, except those for UE category 1bis,

-	E-UTRAN E-CID measurements requirements in section 8.1.2.7.3 for PSCell and SCell carrier frequencies, and in section 8.1.2.7.4 for non-serving E-UTRA carrier frequencies,

-	Requirements in Section 9 for intra-frequency RSRP, RSRQ, RS-SINR, and RSTD measurements accuracy on PSCC and SCC,

-	Requirements in Section 9 for inter-frequency RSRP, RSRQ, RS-SINR, and RSTD measurements accuracy on non-serving E-UTRA carrier frequencies,

-	Power headroom requirements in Section 9 for PSCell and SCell(s).

--- End of change 1 ---

--- Start of change 2 ---

##### 8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)

The requirements in this section are applicable for UE capable of and configured with the NE-DC operation mode.

When monitoring of multiple inter-frequency E-UTRAN carriers as configured by PSCell, inter-RAT E-UTRAN carriers as configured by NR PCell, and inter-frequency NR carriers as configured by NR PCell using gaps (or without using gaps provided the UE supports such capability) is configured, the UE shall be capable of performing one measurement of the configured measurement type (NR SS-RSRP, NR SS-RSRQ, NR SS-SINR, SFTD, RSRP, RSRQ, and RS-SINR measurements, etc.) of detected cells on all the layers.

For UE configured with the NE-DC operation, the effective total number of frequencies excluding the frequencies of the NR PCell, NR SCells, PSCell, and SCells being monitored is Nfreq, NE-DC, which is defined as:

Nfreq, NE-DC = Nfreq, NE-DC, NR + Nfreq, NE-DC, E-UTRA,

where

Nfreq, NE-DC, NR is the number of NR inter-frequency carriers being monitored as configured by NR PCell.

Nfreq, NE-DC, E-UTRA ≤ Nfreq, NE-DC, E-UTRA, inter-RAT + Nfreq, NE-DC, E-UTRA, inter-freq

where

Nfreq, NE-DC, E-UTRA, inter-RAT is the number of E-UTRA inter-RAT carriers (FDD and TDD) excluding E-UTRA serving carrier(s) being monitored as configured by PSCell [15] or via LPP [22],

Nfreq, NE-DC, E-UTRA, inter-freq is the number of E-UTRA inter-frequency carriers (FDD and TDD) being monitored as configured by NR PCell or via LPP [22].

###### 8.1.2.1.1c.1	NE-DC: Maximum allowed layers for multiple monitoring

If a UE is configured with NE-DC operation, the UE shall be capable of monitoring at least:

-	Depending on UE capability, 7 NR inter-frequency carriers configured by NR PCell [50], and

-	Depending on UE capability, 6 E-UTRA TDD inter-frequency carriers configured by PSCell, and

-	Depending on UE capability, 6 E-UTRA FDD inter-frequency carriers configured by PSCell, and

-	Depending on UE capability, 1 E-UTRA FDD inter-frequency carrier for RSTD measurements configured via LPP [57], and

-	Depending on UE capability, 1 E-UTRA TDD inter-frequency carrier for RSTD measurements configured via LPP [57].

In addition to the requirements defined above, the UE shall be capable of monitoring a total of at least 13 effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD, and E-UTRA TDD layers. The UE shall be capable of monitoring a total of at least 6 effective E-UTRA carrier frequency layers, excluding E-UTRA serving carrier(s), comprising of any above defined combination of E-UTRA inter-RAT carriers excluding E-UTRA serving carrier(s) configured by NR PCell and E-UTRA inter-frequency carriers configured by PSCell.

--- End of change 2 ---