3GPP TSG-RAN WG4 Meeting #90    	R4-1902077

Athens, GR, 25th Feb – 1st March, 2019

Agenda item:	6.14.2.3.1

Source:	Samsung

Title:	Simulation results for NR PUCCH

Document for:	Discussion

# 1	Introduction

During RAN#82 meeting, the status report of Rel-15 NR WI is presented [1]. As mentioned in the report, for NR BS performance requirements, due to lack of simulation results, some requirements, test parameters and configurations are still remaining TBD. Additional work is needed to further alignment simulation results with aiming to specify the requirement, and correct the requirement due to unachievable performance measures.

In this contribution, both ideal and impairment simulation results are provided.

# 2	Simulation Results

## 2.1	Test Metric

RAN4 will encourage companies to investigate the test metric NACK to ACK requirement for format 2, whether it is the limiting factor compared with ACK missed detection requirement. Based on our simulation results, it is clearly shown that the NACK to ACK is not the bottleneck compared with ACK missed detection for PUCCH format 2. As agreed in the WF [2], the test metric of PUCCH format 2 will be applied DTX to ACK and ACK missed detection under condition that the number of UCI bits is less than 11.

## 2.2	Results

In this subsection, both ideal simulation results and impairment results are provided in this contribution, as shown in table 1 to table 13, respectively.

Table 1: Ideal and impairment simulation result for NR PUCCH for format 0 in FR1, TDLC300-100

| SCS and BW | Number of Rx | Symbol | ACK missed detection (<=1%) (DTX to ACK 1%) Ideal | ACK missed detection (<=1%) (DTX to ACK 1%) Impairment |
| --- | --- | --- | --- | --- |
| 15KHz 5MHz | 2 | 1 | 8.0 | 10.0 |
|  | 4 | 1 | 1.6 | 3.6 |
|  | 8 | 1 | -2.5 | -0.5 |
|  | 2 | 2 | 1.7 | 3.7 |
|  | 4 | 2 | -2.4 | -0.4 |
|  | 8 | 2 | -5.5 | -3.5 |
| 15KHz, 10MHz | 2 | 1 | 7.9 | 9.9 |
|  | 4 | 1 | 1.8 | 3.8 |
|  | 8 | 1 | -2.5 | -0.5 |
|  | 2 | 2 | 2.7 | 4.7 |
|  | 4 | 2 | -1.8 | 0.2 |
|  | 8 | 2 | -5.2 | -3.2 |
| 15KHZ 20MHz | 2 | 1 | 7.8 | 9.8 |
|  | 4 | 1 | 1.7 | 3.7 |
|  | 8 | 1 | -2.5 | -0.5 |
|  | 2 | 2 | 1.8 | 3.8 |
|  | 4 | 2 | -2.5 | -0.5 |
|  | 8 | 2 | -5.6 | -3.6 |
| 30KHz, 10MHz | 2 | 1 | 8.5 | 10.5 |
|  | 4 | 1 | 2.1 | 4.1 |
|  | 8 | 1 | -2.4 | -0.4 |
|  | 2 | 2 | 3.2 | 5.2 |
|  | 4 | 2 | -1.5 | 0.5 |
|  | 8 | 2 | -5.1 | -3.1 |
| 30KHz, 20MHz | 2 | 1 | 8.7 | 10.7 |
|  | 4 | 1 | 1.9 | 3.9 |
|  | 8 | 1 | -2.2 | -0.2 |
|  | 2 | 2 | 2.1 | 4.1 |
|  | 4 | 2 | -2.2 | -0.2 |
|  | 8 | 2 | -5.3 | -3.3 |
| 30KHz, 40MHz | 2 | 1 | 8.6 | 10.6 |
|  | 4 | 1 | 2.1 | 4.1 |
|  | 8 | 1 | -2.2 | -0.2 |
|  | 2 | 2 | 2.4 | 4.4 |
|  | 4 | 2 | -2.0 | 0.0 |
|  | 8 | 2 | -5.3 | -3.3 |
| 30KHz, 100MHz | 2 | 1 |  |  |
|  | 4 | 1 |  |  |
|  | 8 | 1 |  |  |
|  | 2 | 2 |  |  |
|  | 4 | 2 |  |  |
|  | 8 | 2 |  |  |

Table 2: Ideal and impairment simulation result for NR PUCCH for format 1 in FR1, TDLC300-100

| SCS and BW | Number of Rx | ACK missed detection (<=1%)(DTX to ACK 1%) Ideal | NACK2ACK (<0.1%) (DTX toACK1%) Ideal | ACK missed detection (<=1%)(DTX to ACK 1%) Impairment | NACK2ACK (<0.1%) (DTX toACK1%) Impairment |
| --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | 2 | -6.4 | -5.9 | -4.4 | -3.9 |
|  | 4 | -10.4 | -9.9 | -8.4 | -7.9 |
|  | 8 | -13.5 | -13.3 | -11.5 | -11.3 |
| 15KHz, 10MHz | 2 | -5.6 | -5.2 | -3.6 | -3.2 |
|  | 4 | -9.9 | 9.6 | -7.9 | -7.6 |
|  | 8 | -13.2 | -12.9 | -11.2 | -10.9 |
| 15KHz, 20MHz | 2 | -6.4 | -5.5 | -4.4 | -3.5 |
|  | 4 | -10.5 | -9.9 | -8.5 | -7.9 |
|  | 8 | -13.5 | -13.2 | -11.1 | -10.7 |
| 30KHz, 10MHz | 2 | -5.3 | -4.6 | -3.3 | -2.6 |
|  | 4 | -9.6 | -9.3 | -7.6 | -7.3 |
|  | 8 | -13.1 | -12.7 | -11.1 | -10.7 |
| 30KHz 20MHz | 2 | -6.1 | -5.6 | -4.1 | -3.6 |
|  | 4 | -10.3 | -9.8 | -8.3 | -7.8 |
|  | 8 | -13.3 | -13.1 | -11.3 | -11.1 |
| 30KHz 40MHz | 2 | -5.0 | -4.4 | -3.0 | -2.4 |
|  | 4 | -9.5 | -8.7 | -7.5 | -6.7 |
|  | 8 | -13.0 | -12.7 | -11.0 | -10.7 |
| 30KHz 100MHz | 2 |  |  |  |  |
|  | 4 |  |  |  |  |
|  | 8 |  |  |  |  |

Table 3: Ideal and impairment simulation result for NR PUCCH for format 2 in FR1 (4, 1, 4), TDLC300-100,

| SCS and BW | Number of Rx | ACK missed detection (<=1%) (DTX to ACK 1%) Ideal | ACK missed detection (<=1%)(DTX to ACK 1%) Impairment |
| --- | --- | --- | --- |
| 15KHz, 5MHz | 2 |  |  |
|  | 4 |  |  |
|  | 8 |  |  |
| 15KHz, 10MHz | 2 | 3.2 | 5.2 |
|  | 4 | -1.7 | 0.3 |
|  | 8 | -5.3 | -3.3 |
| 15KHz, 20MHz | 2 |  |  |
|  | 4 |  |  |
|  | 8 |  |  |
| 30KHz, 10MHz | 2 |  |  |
|  | 4 |  |  |
|  | 8 |  |  |
| 30KHz, 20MHz | 2 |  |  |
|  | 4 |  |  |
|  | 8 |  |  |
| 30KHz, 40MHz | 2 | 2.2 | 4.2 |
|  | 4 | -2.2 | -0.2 |
|  | 8 | -5.4 | -3.4 |
| 30KHz, 100MHz | 2 |  |  |
|  | 4 |  |  |
|  | 8 |  |  |

Table 4: Ideal and impairment simulation result for NR PUCCH for format 2 (22, 2, 9) n FR1, TDLC300-100,()

| SCS and BW | Number of Rx | BLER (<=1%) Ideal | BLER (<=1%) Impairment |
| --- | --- | --- | --- |
| 15KHz, 5MHz | 2 |  |  |
|  | 4 |  |  |
|  | 8 |  |  |
| 15KHz, 10MHz | 2 |  |  |
|  | 4 |  |  |
|  | 8 |  |  |
| 15KHz, 20MHz | 2 |  |  |
|  | 4 |  |  |
|  | 8 |  |  |
| 30KHz, 10MHz | 2 |  |  |
|  | 4 |  |  |
|  | 8 |  |  |
| 30KHz, 20MHz | 2 |  |  |
|  | 4 |  |  |
|  | 8 |  |  |
| 30KHz, 40MHz | 2 |  |  |
|  | 4 |  |  |
|  | 8 |  |  |
| 30KHz, 100MHz | 2 |  |  |
|  | 4 |  |  |
|  | 8 |  |  |

Table 5: Ideal and impairment simulation result for NR PUCCH for format 3 (6, 14, 1) in FR1, TDLC300-100 ()

| SCS and BW | Number of Rx | Without additional DMRS (ideal) | With additional DMRS (ideal) | Without additional DMRS(impairment) | With additional DMRS(impairment) |
| --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | 2 | -2.3 | -2.3 | 0.2 | 0.2 |
|  | 4 | -6.2 | -6.6 | -3.7 | -4.1 |
|  | 8 | -9.4 | -9.9 | -6.9 | -7.4 |
| 15KHz, 10MHz | 2 | -1.3 | -1.5 | 1.1 | 1.0 |
|  | 4 | -5.6 | -6.0 | -3.2 | -3.6 |
|  | 8 | -9.1 | -9.6 | -6.6 | -7.1 |
| 15KHz, 20MHz | 2 | -2.2 | -2.4 | 0.3 | 0.1 |
|  | 4 | -6.3 | -6.7 | -3.8 | -4.2 |
|  | 8 | -9.4 | -9.9 | -6.9 | -7.4 |
| 30KHz, 10MHz | 2 | -0.9 | -1.1 | 1.6 | 1.4 |
|  | 4 | -5.4 | -5.7 | -2.9 | -3.2 |
|  | 8 | -8.8 | -9.4 | -6.3 | -6.9 |
| 30KHz, 20MHz | 2 | -1.8 | -2.0 | 0.7 | 0.5 |
|  | 4 | -6.0 | -6.4 | -3.5 | -3.9 |
|  | 8 | -9.2 | -9.7 | -6.7 | -7.2 |
| 30KHz, 40MHz | 2 | -0.8 | -0.8 | 1.7 | 1.7 |
|  | 4 | -5.2 | -5.5 | -2.7 | -3.0 |
|  | 8 | -8.8 | -9.2 | -6.3 | -6.7 |
| 30KHz, 100MHz | 2 |  |  |  |  |
|  | 4 |  |  |  |  |
|  | 8 |  |  |  |  |

Table 6: Ideal and impairment simulation result for NR PUCCH for format 4 (22, 14, 1) in FR1, TDLC300-100)

| SCS and BW | Number of Rx | Without additional DMRS (ideal) | With additional DMRS (ideal) | Without additional DMRS (impairment) | With additional DMRS (impairment) |
| --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | 2 | -0.3 | -0.2 | 2.2 | 2.3 |
|  | 4 | -4.6 | -4.8 | -2.1 | -2.3 |
|  | 8 | -8.0 | -8.3 | -5.5 | -5.8 |
| 15KHz, 10MHz | 2 | 0.5 | 0.5 | 3.0 | 3.0 |
|  | 4 | -4.1 | -4.4 | -1.6 | -1.9 |
|  | 8 | -7.7 | -8.1 | -5.2 | -5.6 |
| 15KHz, 20MHz | 2 | -0.2 | -0.3 | 2.3 | 2.2 |
|  | 4 | -4.7 | -5.0 | -2.2 | -2.5 |
|  | 8 | -8.0 | -8.3 | -5.5 | -5.8 |
| 30KHz, 10MHz | 2 | 1.0 | 1.1 | 3.5 | 3.6 |
|  | 4 | -3.7 | -3.8 | -1.2 | -1.3 |
|  | 8 | -7.5 | -7.9 | -5.0 | -5.4 |
| 30KHz, 20MHz | 2 | 0.1 | 0.1 | 2.6 | 2.6 |
|  | 4 | -4.5 | -4.7 | -2.0 | -2.2 |
|  | 8 | -7.8 | -8.1 | -5.3 | -5.6 |
| 30KHz, 40MHz | 2 | 1.3 | 1.4 | 3.8 | 3.9 |
|  | 4 | -3.6 | -3.7 | -1.1 | -1.2 |
|  | 8 | -7.4 | -7.7 | -4.9 | -5.2 |
| 30KHz,100MHz | 2 |  |  |  |  |
|  | 4 |  |  |  |  |
|  | 8 |  |  |  |  |

Table 7: Ideal and Impairment simulation result for NR PUCCH for format 0 in FR2, TDLA30-300

| SCS and BW | Number of Rx | Symbol | ACK missed detection (<=1%)(DTX to ACK 1%) Ideal | ACK missed detection (<=1%)(DTX to ACK 1%) Impairment |
| --- | --- | --- | --- | --- |
| 60KHz, 50MHz | 2 | 1 | 7.12 | 9.62 |
|  | 2 | 2 | 2.40 | 4.90 |
| 60KHz, 100MHz | 2 | 1 | 7.15 | 9.65 |
|  | 2 | 2 | 1.90 | 4.40 |
| 120KHz, 50MHz | 2 | 1 | 7.35 | 9.85 |
|  | 2 | 2 | 2.33 | 4.83 |
| 120KHz, 100MHz | 2 | 1 | 7.26 | 9.76 |
|  | 2 | 2 | 1.92 | 4.42 |
| 120KHz, 200MHz | 2 | 1 | 7.21 | 9.71 |
|  | 2 | 2 | 1.29 | 3.79 |

Table 8: Ideal and impairment simulation result for NR PUCCH for format 1 (2,14,1) in FR2, TDLA30-300

| SCS and BW | Number of Rx | ACK missed detection (<=1%)(DTX to ACK 1%) Ideal | NACK2ACK (<0.1%) (DTX toACK1%) Ideal | ACK missed detection (<=1%)(DTX to ACK 1%) Impairment | NACK2ACK (<0.1%) (DTX toACK1%) Impairment |
| --- | --- | --- | --- | --- | --- |
| 60KHz, 50MHz | 2 | -6.64 | -3.19 | -4.14 | -1.41 |
| 60KHz, 100MHz | 2 | -7.04 | -4.77 | -4.54 | -2.27 |
| 120KHz, 50MHz | 2 | -6.62 | -4.14 | -4.12 | -1.64 |
| 120KHz, 100MHz | 2 | -7.05 | -4.78 | -4.55 | -2.28 |
| 120KHz, 200MHz | 2 | -7.55 | -5.16 | -5.05 | -2.66 |

Table 9: Ideal and impairment simulation result for NR PUCCH for format 2 (4, 1, 4) in FR2, TDLA30-300

| SCS and BW | Number of Rx | ACK missed detection (<=1%)(DTX to ACK 1%) Ideal | ACK missed detection (<=1%)(DTX to ACK 1%) Impairment |
| --- | --- | --- | --- |
| 60KHz, 50MHz | 2 | 2.15 | 4.65 |
| 60KHz, 100MHz | 2 | 2.36 | 4.86 |
| 120KHz, 50MHz | 2 | 2.61 | 5.11 |
| 120KHz, 100MHz | 2 | 2.63 | 5.13 |
| 120KHz, 200MHz | 2 | 2.80 | 5.30 |

Table 10: Ideal and impairment simulation result for NR PUCCH for format 2 (22, 2, 9) in FR2, TDLA30-300

| SCS and BW | Number of Rx | BLER(<=%1) Ideal | BLER(<=%1) Impairment |
| --- | --- | --- | --- |
| 60KHz, 50MHz | 2 | 0.13 | 2.63 |
| 60KHz, 100MHz | 2 | -0.73 | 1.77 |
| 120KHz, 50MHz | 2 | 0.60 | 3.10 |
| 120KHz, 100MHz | 2 | -0.80 | 1.7 |
| 120KHz, 200MHz | 2 | -0.63 | 1.87 |

Table 11: Ideal and impairment simulation result for NR PUCCH for format 3 (16, 14, 1) in FR2, TDLA30-300

| SCS and BW | Number of Rx | BLER(<=%1) Without additional DMRS (ideal) | BLER(<=%1) Without additional DMRS(impairment) |
| --- | --- | --- | --- |
| 60KHz, 50MHz | 2 | -0.75 | 1.75 |
| 60KHz, 100MHz | 2 | -1.34 | 1.16 |
| 120KHz, 50MHz | 2 | -0.81 | 1.69 |
| 120KHz, 100MHz | 2 | -1.18 | 1.32 |
| 120KHz, 200MHz | 2 | -1.79 | 0.71 |

Table12: Ideal and impairment simulation result for NR PUCCH for format 3 (16, 4, 3) in FR2, TDLA30-300

| SCS and BW | Number of Rx | BLER(<=%1) Without additional DMRS (ideal) | BLER(<=%1) Without additional DMRS (impairment) |
| --- | --- | --- | --- |
| 60KHz, 50MHz | 2 | 0.27 | 2.77 |
| 60KHz, 100MHz | 2 | -0.32 | 2.18 |
| 120KHz, 50MHz | 2 | 0.53 | 3.03 |
| 120KHz, 100MHz | 2 | -0.03 | 2.47 |
| 120KHz, 200MHz | 2 | -0.46 | 2.04 |

Table 13: Ideal and impairment simulation result for NR PUCCH for format 4 (22, 14, 1) in FR2, TDLA30-300

| SCS and BW | Number of Rx | BLER(<=%1) Without additional DMRS (ideal) | BLER(<=%1) Without additional DMRS (impairment) |
| --- | --- | --- | --- |
| 60KHz, 50MHz | 2 | 3.61 | 6.11 |
| 60KHz, 100MHz | 2 | 3.34 | 5.84 |
| 120KHz, 50MHz | 2 | 3.67 | 6.17 |
| 120KHz, 100MHz | 2 | 3.48 | 5.84 |
| 120KHz, 200MHz | 2 | 2.97 | 5.47 |

# 3	Conclusion

In this contribution, based on the agreement of WF [2] on the NR BS PUCCH demodulation, both ideal and impairment simulation results are provided.

# References

RP-182805,  Status report to TSG for NR Access technology, NTT DOCOMO

R1-1816348, WF on NR PUCCH demodulation requirements in Rel-15, Huawei