3GPP TSG-RAN WG4 Meeting #90	R4-1902100

# Athens, Greece, February 25th – March 1st 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to Link Recovery Procedures introduction section 8.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction CR to section 8.5 Link Recovery Procedures |  |  |  |  |  |  |  |  |
| Summary of change: |  | A number of corrections to clarify and align the introdcution of the Link Recovery procedure with the RAN1 specification. - UE is configured with or derives a number of resource configurations for set q0 - Upon request the UE shall Indexes and L1-RSRP to higher layers. UE is not required to measure q1 resource configurations before requested. - Included Qin_LR - Clarification related to UE requirement related to performing CSI-RS L1-RSRP measurements on same PRBs and symbols as SSB symbols. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Section 8.5 on Link Recovery Procedures is inaccurate and not aligned with agreements in other WG’s |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Next Modified Subclause

## 8.5	Link Recovery Procedures

### 8.5.1	Introduction

The UE shall assess the downlink link quality of resource on a serving cell based on the reference signals in the set  as specified in TS 38.213 [3] in order to detect beam failure instance. The RS resources in the set  can be periodic CSI-RS resources and/or SSBs. UE is not required to perform beam failure detection outside the active DL BWP.

On each RS resource in the set , the UE shall estimate the radio link quality and compare it to the threshold Qout_LR for the purpose of accessing downlink radio link quality of the resources in set .

The threshold Qout_LR is defined as the level at which the downlink radio level link of a given resources in set cannot be reliably received and shall correspond to the BLERout=10% block error rate of a hypothetical PDCCH transmission. For SSB based beam failure detection, Qout_LR_SSB is derived based on the hypothetical PDCCH transmission parameters listed in Table 8.5.2.1-1. For CSI-RS based beam failure detection, Qout_LR_CSI-RS is derived based on the hypothetical PDCCH transmission parameters listed in Table 8.5.3.1-1.

Upon request the UE shall deliver configuration indexes from the the set , as specified in TS 38.213 [3], to higher layers, and the corresponding L1-RSRP measurement, provided that L1-RSRP is equal to or better than the threshold indicated by higher layer parameter Qin,LR (rsrp-ThresholdSSB or rsrp-ThresholdCSI-rs). The UE evaluates the Qin,LR threshold to the L1-RSRP measurement obtained from a SSB. The UE applies the Qin,LR threshold to the L1-RSRP measurement obtained for a CSI-RS resource after scaling a respective CSI-RS reception power with a value provided by higher layer parameter powerControlOffsetSS. The RS resource configurations in the set  can be periodic CSI-RS resources or SSB resources or both SSB and CSI-RS resources. UE is not required to perform candidate beam detection outside the active DL BWP.

The UE is not required to receive CSI-RS for beam failure detection measurements in the PRBs that overlap with an SSB but is required to receive CSI-RS for beam failure detection in PRBs not overlapping with SSB PRBs. However, if the CSI-RS and SSB are in the same OFDM symbol, the UE is not required to perform beam failure detection if the SCS of the CSI-RS used for beam failure detection and the SCS of the SSB are different, if the UE is not capable of simultaneousRxDataSSB-DiffNumerology.

If the CSI-RS used for beam failure detection and if the SSB is used for RLM with N=[8] as specified in section 8.1.2, the UE is allowed additional relaxation.

Next Modified Subclause