3GPP TSG-RAN WG4 Meeting #90												  	        R4-1900074

Athens, Greece, 25th of February – 1st of March, 2019

Source: 	Nokia, TMO US

Title: 					TP to TR 37.716-21-21: DC_66A_n71A-n261A

Agenda Item: 			9.8.3

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.716-21-21to introduce DC_66A_n71A-n261A which is listed in WID

### EN-DC of LTE inter band CA for x=1 bands DL with 1 band UL + NR inter-band CA for 2 bands DL with 1 band UL

| combo | EN-DC configuration | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes and intra contiguous/non-contiguous CA (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 66_n71-n261 | DL_66A_n71A-n261A_UL_66A_n71A | Rel-15 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom, Samsung | new | 2B_66A_n71A_UL_66A_n71A (completed) |
| 66_n71-n261 | DL_66A_n71A-n261A_UL_66A_n261A | Rel-15 | Nelson Ueng, T-Mobile USA | nelson.ueng@T-Mobile.com | Ericsson, Nokia, Deutsche Telekom, Samsung | new | 2B_66A_n261A_UL_66A_n261A (completed) |

*********************** Start of the TP ***************************************************

## 6.X	DC_66A-n71A-n261

### 6.X.1	Operating bands for DC

Table 6.X.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66A-n71A-n261 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | Uplink Configurations |
| --- | --- |
| DC_66A_n71A-n261A | DC_66A_n71A DC_66A_n261A |

### 6.X.3	Co-existence studies

Co-existence studies of DC_66A-n71A are captured in TR 37.863-01-01 and for EN-DC between E-UTRA and FR2 DC_66A_n261A there will no requirements specified.

### 6.X.4	∆TIB and ∆RIB values

For DC_66A_n71A-n261A, the TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66A_n71A-n261A | 66 | 0.3 |
|  | n71 | 0.3 |
|  | n261 | 0 |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66A_n71A-n261A | 66 | 0 |
|  | n71 | 0 |
|  | n261 | 0 |

### 6.X.5	MSD

Based on the co-existence study in 37.716-21-21, MSD does not need to be defined.

*********************** End of the TP ***************************************************