3GPP TSG-RAN WG4 Meeting #90												  	         R4-1902144

Athens, Greece, 25th of February – 1st of March, 2019

Source: 	Nokia

Title: 					TP to TR 36.716-01-01: A-MPR for CA_48C

Agenda item:			7.1.2

Document for:			Approval

# 1	Introduction

This contribution presents A-MPR simulation results for CA_48C and proposes a TP to capture the study into the TR.

# 2	Discussion

## 2.1	FCC Requirements

FCC has released new CBRS UE requirements as follows.

(ii) Except as otherwise specified in paragraph (e)(2) of this section, for channel and frequency assignments made by a CBSD to End User Devices, the conducted power of any End User Device emission outside the fundamental emission (whether in or outside of the authorized band) shall not exceed -13 dBm/MHz within 0 to B megahertz (where B is the bandwidth in megahertz of the assigned channel or multiple contiguous channels of the End User Device) above the upper CBSD-assigned channel edge and within 0 to B megahertz below the lower CBSD-assigned channel edge. At all frequencies greater than B megahertz above the upper CBSD assigned channel edge and less than B megahertz below the lower CBSD-assigned channel edge, the conducted power of any End User Device emission shall not exceed -25 dBm/MHz. Notwithstanding the emission limits in this paragraph, the Adjacent Channel Leakage Ratio for End User Devices shall be at least 30 dB.

(2) Additional protection levels. Notwithstanding paragraph (e)(1) of this section, for CBSDs and End User Devices, the conducted power of emissions below 3540 MHz or above 3710 MHz shall not exceed -25 dBm/MHz, and the conducted power of emissions below 3530 MHz or above 3720 MHz shall not exceed −40dBm/MHz.

(3) Measurement procedure. (i) Compliance with this provision is based on the use of measurement instrumentation employing a resolution bandwidth of 1 megahertz or greater. However, in the 1 megahertz bands immediately outside and adjacent to the licensee’s authorized frequency channel, a resolution bandwidth of no less than one percent of the fundamental emission bandwidth may be employed. A narrower resolution bandwidth is permitted in all cases to improve measurement accuracy provided the measured power is integrated over the full reference bandwidth (i.e., 1 MHz or 1 percent of emission bandwidth, as specified). The fundamental emission bandwidth is defined as the width of the signal between two points, one below the carrier center frequency and one above the carrier center frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power.

Figure 1: CBRS UE requirements

## 2.2	CA_48C

3GPP is defining requirements for LTE uplink CA for band 48 i.e. CA_48C [1]. Applicable CC combinations are presented below [2]

Table 5.X.1-1: Bandwidth combinations for Intra band contiguous CA configurations

| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | CBW [MHz] | CBW [MHz] | CBW [MHz] | CBW [MHz] | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_48C | CA_48C | 48 | 5, 10, 15, 20 | 20 |  |  | 40 | 0 |
|  |  |  | 20 | 5, 10, 15 |  |  |  |  |

FCC rules can be presented as follows:

Band 48 3550 MHz – 3700 MHz 3550 MHz – 3700 MHz TDD

Table 3: SEM

|  | Spectrum emission limit (dBm) / measurement bandwidth for each channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB MHz | 25+100RB (24.95MHz) | 50+100RB (29.9 MHz) | 75+100RB (34.85 MHz) | 100+100RB (39.8 MHz) | Measurement bandwidth |
| ± 0 - 1 | -13 |  |  |  | 1 % channel bandwidth |
| ± 1 - X | -13 |  |  |  | 1 MHz |
| < – X or > X when 3540 MHz < ΔfOOB < 3710 MHz | -25 |  |  |  |  |
| NOTE: X is aggregated channel bandwidth as defined in clause 5.6A |  |  |  |  |  |

Table 4: Additional Spurious emissions

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 40 MHz |  |
| 9 kHz – 3530 MHz | -40 | 1 MHz |
| 3530 MHz – 3540 MHz | -25 |  |
| 3710 MHz – 3720 MHz | -25 |  |
| 3720 MHz – 12.75 GHz | -40 |  |

Table 5: General requirements for CA E-UTRAACLR

|  | CA bandwidth class / CA E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B, C and D |
| CA E-UTRAACLR | 30 dB |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |

No UTRA ACLR is assumed.

## 2.2	A-MPR concept

As can be seen from Figure 1 the emission requirements are tighter outside of the band compared to inside the band with an exception that first 10 MHz on high side of that band has same -13 dBm requirement as within the band. Therefore, it would be beneficial to define two different A-MPRs firstly the band edge A-MPR which has higher -25 dBm...-40 dBm emission requirement on IMD3 region and secondly an inner-band A-MPR which would have lower A-MPR as -13 dBm requirement would apply on IMD3 region. This concept is presented in Figure 2.

Figure 2: Two different A-MPRs per CC combination

## 2.3	Simulation assumptions

Simulation assumptions were as follows

IQ-Image and LO leakage = 28 dBc

CIM3 = 60 dBc

PA calibration point was 20 MHz, 15 kHz, QPSK, DFT-S-OFMA, 100 RB at lower channel edge with 0.5 dB MPR

SEM, ACLR, spurious emission and additional CBRS emission limits are calculated for A-MPR.

The simulated A-MPR is assumed applied as max(A-MPR, MPR), where MPR is the 1CC MPR.

## 2.4	Results for Contiguous CA Resource Allocations

The A-MPR regions illustrated in Figure 2 are defined in Table 6.

Table 6: A-MPR regions for CA_48C

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | A-MPR |
| --- | --- | --- |
| 20+5 / 5 + 20 | FL + BWCA/2 ≤ FC ≤ FH + BWCA/2 | Table 7 |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC ≤ FH - 3*BWCA/2 - 10 MHz | Table 8 |
| 20+10 / 10 + 20 | FL + BWCA/2 ≤ FC ≤ FH + BWCA/2 | Table 7 |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC ≤ FH - 3*BWCA/2 - 10 MHz | Table 8 |
| 20+15 / 15 + 20 | FL + BWCA/2 ≤ FC ≤ FH + BWCA/2 | Table 7 |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC ≤ FH - 3*BWCA/2 - 10 MHz | Table 8 |
| 20+20 | FL + BWCA/2 ≤ FC ≤ FH + BWCA/2 | Table 7 |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC ≤ FH - 3*BWCA/2 - 10 MHz | Table 8 |
| NOTE: FL = 3550 MHz, FH = 3700 MHz and BWCA is the combined bandwidth of the contiguous CCs in the CA combination indicated. |  |  |

A-MPR at the band edge positions defined in Table 6 is given in Table 7. For the lower A-MPR region, the results are given in Table 8.

Herein, applied A-MPR should be considered as max(A-MRP, MPR), where A-MPR is given in Table 7 and Table 8 and MPR for single CC scenario in Table 6.2.3-1 of LTE specification 36.101.

Table 7: A-MPR regions for CA_48C at the band edge

| BWs [MHz] | RB_start | L_CRB | A-MPR [dB] |
| --- | --- | --- | --- |
|  |  |  | max(mods) |
| 20+5 / 5 + 20 | 0 – 7 and 117 - 124 |  | 12 |
|  | 5 - 25 | <85 | 5,5 |
|  |  | >85 | 6,5 |
|  | 26 - 120 |  | 5 |
| 20+10 / 10 + 20 | 0 - 13 and 135 - 149 |  | 11 |
|  | 14 - 33 | <85 | 5 |
|  |  | >85 | 7 |
|  | 34 - 134 |  | 6 |
| 20+15 / 15 + 20 | 0 - 22 and 152 -174 |  | 11 |
|  | 23 - 42 | <95 | 5,5 |
|  |  | >95 | 7 |
|  | 43 - 151 |  | 6 |
| 20+20 | 0 - 31 and 165 - 199 |  | 11 |
|  | 31 - 51 | <100 | 5,5 |
|  |  | >100 | 7 |
|  | 52 - 164 | <100 | 4,5 |
|  |  | >100 | 6 |

Table 8: A-MPR regions for CA_48C at the band center (“range for lower A-MPR”)

| BWs [MHz] | RB_start | L_CRB | A-MPR [dB] |
| --- | --- | --- | --- |
|  |  |  | max(mods) |
| 20+5 / 5 + 20 | 0 – 7 and 117 - 124 |  | 4 |
|  | 5 - 25 | <85 | 2 |
|  |  | >85 | 2,5 |
|  | 26 - 120 |  | 2 |
| 20+10 / 10 + 20 | 0 - 13 and 135 - 149 |  | 4,5 |
|  | 14 - 33 | <85 | 1,5 |
|  |  | >85 | 2,5 |
|  | 34 - 134 |  | 1,5 |
| 20+15 / 15 + 20 | 0 - 22 and 152 -174 |  | 4,5 |
|  | 23 - 42 | <95 | 1 |
|  |  | >95 | 2,5 |
|  | 43 - 151 |  | 1 |
| 20+20 | 0 - 31 and 165 - 199 |  | 4,5 |
|  | 31 - 51 | <100 | 1 |
|  |  | >100 | 1,5 |
|  | 52 - 164 | <100 | 1 |
|  |  | >100 | 1 |

Furthermore, an example A-MPR scenario of 20+20 MHz CA combination with QPSK modulation is shown in Figure 3 with contiguous CA allocations, considering both the band edge scenario (“0 Hz offset”) and the lower A-MPR region (“29.8 MHz offset”).

A complete set of A-MPR triangles with contiguous CA resource allocations is included in the Appendix.

Figure 3: The A-AMPR triangles for contiguous resource allocations of 20+20 MHz CA combination at the lower band edge (left) and at the lower edge of the range for lower A-MPR (right). EVM and IBE are not consider for A-MPR.

## 2.5	Results for Non-contiguous CA Resource Allocations

As illustrated, with an example scenario of 20+20 MHz CA combination, in Figure 4, with non-contiguous CA allocations higher A-MPR is needed. In here, non-contiguous channel edge allocations are simulated as a worst-case-scenario.

For the channel edge positions indicated in Figure 2 and Table 6 (“0 Hz offset” scenario in Figure 4), 18 dB A-MPR is needed for the smallest allocation ratio resource allocations, i.e. the narrowest non-contiguous edge allocations.

For the lower A-MPR range indicated in Figure 2 and Table 6 (“29.8 MHz offset” scenario in Figure 4), 12 dB A-MPR is needed for the smallest allocation ratio resource allocations, i.e. the narrowest non-contiguous edge allocations.

Figure 4: The A-AMPR for non-contiguous edge allocations of 20+20 MHz CA combination at the lower band edge (left) and at the lower edge of the range for lower A-MPR (right). EVM and IBE are not consider for A-MPR.

Complete set of A-MPR results with non-contiguous CA resource allocations is included in the Appendix. Considering the maximum A-MPR needed in the case of small allocation ratio, the results for other BW combinations are quite consistent with 20+20 MHz scenario presented in Figure 3.

Based on these results, the following formulation for A-MPR is proposed for the edge scenario:

A-MPR = CEIL {MA, 0.5}

where MA is defined as follows

MA =      18.00 - 10.00 A; 			0 ≤ A < 0.05

18.50 - 20.00 A;			0.05 ≤ A < 0.2

15.50  - 5.00 A;			0.2 ≤ A < 1

where A = NRB_alloc / NRB_agg.

Furthermore, the following formulation for A-MPR is proposed for the center scenario:

A-MPR = CEIL {MA, 0.5}

where MA is defined as follows

MA =      11.50 - 10.00 A; 			0 ≤ A < 0.15

10.88 -  5.88*A;			0.15 ≤ A < 1

where A = NRB_alloc / NRB_agg.

# 3	Conclusion

In this contribution we have proposed a way to define A-MPR for CA_48C and presented our simulation results.

# 4	Reference

[1]	181548, “Revised WID on Rel-16 LTE intra-band Carrier Aggregation for x CC DL/y CC UL including contiguous and non-contiguous spectrum (x>=y)”, Ericsson

[2] R4-1815822, TP to TR 36.716-02-02: Addition of CA configuration for 2DL_48C_2UL_48C_BCS0, 3DL_48A-48C_2UL_48C_BCS0 and 4DL_48C-48C_2UL_48C_BCS0, Ericsson

# Appendix

A-MPR triangles for contiguous CA resource allocations

A-MPR vs allocation ratio for noncontiguous CA resource allocations at the outer channel edges

********************************* Start of the TP ****************************************

### 5.x.5	Vendor B, A-MPR for uplink CA bandwidth class C

## 2.X.5.1	A-MPR concept

Emission requirements are tighter outside of the band compared to inside the band with an exception that first 10 MHz on high side of that band has same -13 dBm requirement as within the band. Therefore, it would be beneficial to define two different A-MPRs firstly the band edge A-MPR which has higher -25 dBm...-40 dBm emission requirement on IMD3 region and secondly an inner-band A-MPR which would have lower A-MPR as -13 dBm requirement would apply on IMD3 region. This concept is presented in Figure 2.X.5.1-1.

Figure 2.X.5.1-1: Two different A-MPRs per CC combination

## 2.X.5.2	Simulation assumptions

Simulation assumptions were as follows

IQ-Image and LO leakage = 28 dBc

CIM3 = 60 dBc

PA calibration point was 20 MHz, 15 kHz, QPSK, DFT-S-OFMA, 100 RB at lower channel edge with 0.5 dB MPR

SEM, ACLR, spurious emission and additional CBRS emission limits are calculated for A-MPR.

The simulated A-MPR is assumed applied as max(A-MPR, MPR), where MPR is the 1CC MPR.

## 2.X.5.3	Results for Contiguous CA Resource Allocations

The A-MPR regions illustrated in Figure2.X.5.1-1 are defined in Table 2.X.5.3-1.

Table 2.X.5.3-1: A-MPR regions for CA_48C

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | A-MPR |
| --- | --- | --- |
| 20+5 / 5 + 20 | FL + BWCA/2 ≤ FC ≤ FH + BWCA/2 | A |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC ≤ FH - 3*BWCA/2 - 10 MHz | B |
| 20+10 / 10 + 20 | FL + BWCA/2 ≤ FC ≤ FH + BWCA/2 | A |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC ≤ FH - 3*BWCA/2 - 10 MHz | B |
| 20+15 / 15 + 20 | FL + BWCA/2 ≤ FC ≤ FH + BWCA/2 | A |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC ≤ FH - 3*BWCA/2 - 10 MHz | B |
| 20+20 | FL + BWCA/2 ≤ FC ≤ FH + BWCA/2 | A |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC ≤ FH - 3*BWCA/2 - 10 MHz | B |
| NOTE: FL = 3550 MHz, FH = 3700 MHz and BWCA is the combined bandwidth of the contiguous CCs in the CA combination indicated. |  |  |

A-MPR at the band edge positions defined in Table 2.X.5.3-1 is given in Table 2.X.5.3-2 this is the A-case. For the lower A-MPR region, the results are given in Table 2.X.5.3-3 this is the B-case.

Herein, applied A-MPR should be considered as max(A-MRP, MPR), where A-MPR is given in Table 2.X.5.3-2 and Table 2.X.5.3-3 and MPR for single CC scenario in Table 6.2.3-1 of LTE specification 36.101.

Table 2.X.5.3-2: A-MPR regions for CA_48C at the band edge, (A)

| BWs [MHz] | RB_start | L_CRB | A-MPR [dB] |
| --- | --- | --- | --- |
|  |  |  | max(mods) |
| 20+5 / 5 + 20 | 0 – 7 and 117 - 124 |  | ≤ 12 |
|  | 5 - 25 | < 85 | ≤ 5.5 |
|  |  | > 85 | ≤ 6.5 |
|  | 26 - 120 |  | ≤ 5 |
| 20+10 / 10 + 20 | 0 - 13 and 135 - 149 |  | ≤ 11 |
|  | 14 - 33 | < 85 | ≤ 5 |
|  |  | > 85 | ≤ 7 |
|  | 34 - 134 |  | ≤ 6 |
| 20+15 / 15 + 20 | 0 - 22 and 152 -174 |  | ≤ 11 |
|  | 23 - 42 | < 95 | ≤ 5.5 |
|  |  | > 95 | ≤ 7 |
|  | 43 - 151 |  | ≤ 6 |
| 20+20 | 0 - 31 and 165 - 199 |  | ≤ 11 |
|  | 31 - 51 | < 100 | ≤ 5.5 |
|  |  | > 100 | ≤ 7 |
|  | 52 - 164 | < 100 | ≤ 4.5 |
|  |  | > 100 | ≤ 6 |

Table 2.X.5.3-3: A-MPR regions for CA_48C at the band center (“range for lower A-MPR”) (B)

| BWs [MHz] | RB_start | L_CRB | A-MPR [dB] |
| --- | --- | --- | --- |
|  |  |  | max(mods) |
| 20+5 / 5 + 20 | 0 – 7 and 117 - 124 |  | ≤ 4 |
|  | 5 - 25 | < 85 | ≤ 2 |
|  |  | > 85 | ≤ 2.5 |
|  | 26 - 120 |  | ≤ 2 |
| 20+10 / 10 + 20 | 0 - 13 and 135 - 149 |  | ≤ 4.5 |
|  | 14 - 33 | < 85 | ≤ 1.5 |
|  |  | > 85 | ≤ 2.5 |
|  | 34 - 134 |  | ≤ 1.5 |
| 20+15 / 15 + 20 | 0 - 22 and 152 -174 |  | ≤ 4.5 |
|  | 23 - 42 | < 95 | ≤ 1 |
|  |  | > 95 | ≤ 2.5 |
|  | 43 - 151 |  | ≤ 1 |
| 20+20 | 0 - 31 and 165 - 199 |  | ≤ 4.5 |
|  | 31 - 51 | < 100 | ≤ 1 |
|  |  | > 100 | ≤ 1.5 |
|  | 52 - 164 | < 100 | ≤ 1 |
|  |  | > 100 | ≤ 1 |

Furthermore, an example A-MPR scenario of 20+20 MHz CA combination with QPSK modulation is shown in Figure 3 with contiguous CA allocations, considering both the band edge scenario (“0 Hz offset”) and the lower A-MPR region (“29.8 MHz offset”).

Figure 2.X.5.3-1: The A-AMPR triangles for contiguous resource allocations of 20+20 MHz CA combination at the lower band edge (left) and at the lower edge of the range for lower A-MPR (right). EVM and IBE are not consider for A-MPR.

## 2.X.5.4	Results for Non-contiguous CA Resource Allocations

As illustrated, with an example scenario of 20+20 MHz CA combination, in Figure 2.X.5.4-1 with non-contiguous CA allocations higher A-MPR is needed. In here, non-contiguous channel edge allocations are simulated as a worst-case-scenario.

For the channel edge positions indicated in Figure 2.X.5.1-1, 18 dB A-MPR is needed for the smallest allocation ratio resource allocations, i.e. the narrowest non-contiguous edge allocations.

For the lower A-MPR range indicated in Figure 2.X.5.1-1, 12 dB A-MPR is needed for the smallest allocation ratio resource allocations, i.e. the narrowest non-contiguous edge allocations.

Figure 2.X.5.4-1: The A-MPR for non-contiguous edge allocations of 20+20 MHz CA combination at the lower band edge (left) and at the lower edge of the range for lower A-MPR (right). EVM and IBE are not consider for A-MPR.

Considering the maximum A-MPR needed in the case of small allocation ratio, the results for other BW combinations are quite consistent with 20+20 MHz scenario presented in Figure 2.X.5.3-1.

Based on these results, the following formulation for A-MPR is proposed for the edge scenario:

A-MPR = CEIL {MA, 0.5}

where MA is defined as follows

MA =      18.00 - 10.00 A; 			0 ≤ A < 0.05

18.50 - 20.00 A;			0.05 ≤ A < 0.2

15.50 - 5.00 A;			0.2 ≤ A < 1

where A = NRB_alloc / NRB_agg.

Furthermore, the following formulation for A-MPR is proposed for the center scenario:

A-MPR = CEIL {MA, 0.5}

where MA is defined as follows

MA =      11.50 - 10.00 A; 			0 ≤ A < 0.15

10.88 -  5.88*A;			0.15 ≤ A < 1

where A = NRB_alloc / NRB_agg.

**************************** End of TP******************************************