3GPP TSG-RAN WG4 Meeting #90 	R4-1902234

# Athens, Greece, 25 February – 1 March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.104 | CR | 0845 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 37.104 on Corrections for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CRs to introduce NR into the MSR specification were approved in December 2018. There are a few issues with the NR introduction: - For bands that apply for NR, a new notation “Band x/nX” has been introduced throughout the specification. This is not in line with the MSR spec principles and is misleading. All bands have an MSR designation (Arabic numerals) as outlined in Tables 4.5-1 and 4.5-2. That table identifies what RATs that are applicable for each band, through the column entries and Table notes. Throughout the spec, the bands should only be identified with the MSR number “X”, regardless of RAT supported (GSM, E-UTRA, UTRA and now also NR). An identification “X/nX” could give the impression that it is an LTE/NR band and implicitly excludes other RATs that are not separately identified. For example, “Band 3/n3” seems to exclude GSM operation in the DCS1800 band and UTRA operation in Band III. - NR operation in Bands n77 and n78 do not have an MSR band designation. For the same reason as in (1) above, they should be identified as Bands 77 and 78 throughout the MSR specification, though MSR requirements do not apply for single- RAT NR operation in those bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - All references to Band “X/nX” are changed to “Band X” - Bands n77 and n78 are given MSR Band designations 77 and 78. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It would remain unclear what combination of Bands and RATs that are referenced for requirements throughout the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.5, 6.6.1.3.1, 6.6.1.4.1, 6.6.2.4.3, 6.6.2.4.7, 7.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.141 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BWChannel	Channel bandwidth (for E-UTRA and NR)

BWConfig	Transmission bandwidth configuration (for E-UTRA), where BWConfig = NRB x 180 kHz in the uplink and BWConfig = 15 kHz + NRB x 180 kHz in the downlink. Transmission bandwidth configuration (for NR), where BWConfig = NRB x SCS x 12.

BWRF 	Base Station RF Bandwidth, where BWRF = FBW RF,high – FBW RF,low

BWRF,max 	Maximum Base Station RF Bandwidth

DwPTS	Downlink part of the special subframe (for E-UTRA TDD operation)

f	Frequency

f	Separation between the Base Station RF Bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency

fmax 	The largest value of f used for defining the requirement

ΔfOBUE 	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

FC	Carrier centre frequency

Ffilter	Filter centre frequency

f_offset 	Separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter

f_offsetmax 	The maximum value of f_offset used for defining the requirement

Fblock,high	Upper sub-block edge, where Fblock,high = FC,block,high + Foffset, RAT

Fblock,low	Lower sub-block edge, where Fblock,low = FC,block,low - Foffset, RAT

FBW RF,high  	Upper Base Station RF Bandwidth edge, where FBW RF,high  = FC,high + Foffset, RAT

FBW RF,low  	Lower Base Station RF Bandwidth edge, where FBW RF,low  = FC,low - Foffset, RAT

FC band, high		Center frequency of the highest transmitted/received carrier in a band.

FC band, low		Center frequency of the lowest transmitted/received carrier in a band.

FC,block, high		Centre frequency of the highest transmitted/received carrier in a sub-block.

FC,block, low		Centre frequency of the lowest transmitted/received carrier in a sub-block.

FC,high		Centre frequency of the highest transmitted/received carrier.

FC,low		Centre frequency of the lowest transmitted/received carrier.

Foffset, RAT	Frequency offset from the centre frequency of the highest transmitted/received carrier to the upper Base Station RF Bandwidth edge, sub-block edge or Inter-RF Bandwidth edge, or from the centre frequency of the lowest transmitted/received to the lower Base Station RF Bandwidth edge, sub-block edge or Inter-RF Bandwidth edge for a specific RAT.

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

PEM,N	Declared emission level for channel N

PEM,B32,B75,B76,ind	Declared emission level in Band 32, Band 75 and Band 76, ind=a, b, c

PEM,B32,ind	Declared emission level in Band 32, ind= d, e

PEM,B50,B74,B75,ind	Declared emission level for Band 50, Band 74 and Band 75, ind=a,b

Pmax	Maximum total output power

Pmax,RAT	Maximum RAT output power

Pmax,c	Maximum carrier output power

PRated,c	Rated carrier output power

PREFSENS		Reference Sensitivity power level

Wgap	Sub-block gap or Inter RF Bandwidth gap size

*********************End of change*****************

*********************Next changed section*****************

## 4.5	Operating bands and Band Categories

MSR requirements are applicable for band definitions and band numbering as defined in the specifications TS 45.005 [5], TS25.104 [2], TS 25.105 [3], TS 36.104 [4] and TS 38.104 [17]. For the purpose of defining the BS requirements, the operating bands are divided into three band categories as follows:

-	Band Category 1 (BC1): Bands for NR FDD, E-UTRA FDD and/or UTRA FDD operation. Bands in this category are also used for NB-IoT operation (all modes)

-	Band Category 2 (BC2): Bands for NR FDD, E-UTRA FDD, UTRA FDD and/or GSM/EDGE operation. Bands in this category are also used for NB-IoT operation (all modes)

-	Band Category 3 (BC3): Bands for NR TDD, E-UTRA TDD and/or UTRA TDD operation. Bands in this category are also used for NB-IoT operation (all modes)

NOTE:	For UTRA TDD, requirements in the present document cover the 1.28 Mcps UTRA TDD option.

The paired and unpaired bands for the three Band Categories are shown in Table 4.5-1 and 4.5-2, together with the corresponding NR, E-UTRA, UTRA and GSM/EDGE band designations. In the present specification, the operating band of an MSR Base Stations is designated using the E-UTRA band number according to the tables.

Table 4.5-1: Paired bands in NR, E-UTRA, UTRA and GSM/EDGE.

| MSR and E-UTRA Band number | NR Band number | UTRA Band number | GSM/EDGE Band designation | Uplink (UL) BS receive UE transmit |  |  | Downlink (DL) BS transmit UE receive |  |  | Band category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n1 | I | - | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | 1 |
| 2 | n2 | II | PCS 1900 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | 2 |
| 3 | n3 | III | DCS 1800 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | 2 |
| 4 |  | IV | - | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | 1 |
| 5 | n5 | V | GSM 850 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | 2 |
| 6 (NOTE 1) |  | VI | - | 830 MHz | – | 840 MHz | 875 MHz | – | 885 MHz | 1 (NOTE 1) |
| 7 | n7 | VII | - | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | 1 (NOTE 3) |
| 8 | n8 | VIII | E-GSM | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | 2 |
| 9 |  | IX | - | 1749.9 MHz | – | 1784.9 MHz | 1844.9 MHz | – | 1879.9 MHz | 1 (NOTE 3) |
| 10 |  | X | - | 1710 MHz | – | 1770 MHz | 2110 MHz | – | 2170 MHz | 1 (NOTE 3) |
| 11 |  | XI | - | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | 1 |
| 12 | n12 | XII | - | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | 1 |
| 13 |  | XIII | - | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | 1 |
| 14 |  | XIV | - | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | 1 |
| 15 |  | XV | - | Reserved |  |  | Reserved |  |  |  |
| 16 |  | XVI | - | Reserved |  |  | Reserved |  |  |  |
| 17 |  | - | - | 704 MHz | – | 716 MHz | 734 MHz | – | 746 MHz | 1 (NOTE 4) |
| 18 |  | - | - | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | 1 (NOTE 4) |
| 19 |  | XIX | - | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | 1 |
| 20 | n20 | XX | - | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | 1 |
| 21 |  | XXI | - | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | 1 |
| 22 |  | XXII | - | 3410 MHz | – | 3490 MHz | 3510 MHz | – | 3590 MHz | 1 (NOTE 3) |
| 238 |  | - | - | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz | 1 (NOTE 2) |
| 24 |  | - | - | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | 1 (NOTE 2) |
| 25 | n25 | XXV | - | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | 1 |
| 26 |  | XXVI | - | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | 1 |
| 27 |  | - | - | 807 MHz | – | 824 MHz | 852 MHz | – | 869 MHz | 1 (NOTE 2) |
| 28 | n28 | - | - | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | 1 (NOTE 4) |
| 29 |  | - | - | N/A |  |  | 717 MHz | – | 728 MHz | 1 (NOTE 2, NOTE 5) |
| 30 |  | - | - | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | 1 (NOTE 2) |
| 31 |  | - | - | 452.5 MHz | – | 457.5 MHz | 462.5 MHz | – | 467.5 MHz | 1 (NOTE 4) |
| 32 (NOTE 5) |  | XXXII (NOTE 6) | - |  | N/A |  | 1452 MHz | – | 1496 MHz | 1 (NOTE 3) |
| 64 |  |  |  | Reserved |  |  |  |  |  |  |
| 65 |  | - | - | 1920 MHz | – | 2010 MHz | 2110 MHz | – | 2200 MHz | 1 (NOTE 2) |
| 66 (NOTE 7) | n66 | - | - | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | 1 (NOTE 4) |
| 67 (NOTE 5) |  | - | - |  | N/A |  | 738 MHz | - | 758 MHz | 1 (NOTE 2) |
| 68 |  | - | - | 698 MHz | – | 728 MHz | 753 MHz | – | 783 MHz | 1 (NOTE 2) |
| 69 |  | - | - |  | N/A |  | 2570 MHz | – | 2620 MHz | 1 (NOTE 2, NOTE 5) |
| 70 (NOTE 9) | n70 | - | - | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | 1 (NOTE 4) |
| 71 | n71 | - | - | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | 1 (NOTE 4) |
| 72 |  | - | - | 451 MHz | – | 456 MHz | 461 MHz | – | 466 MHz | 1 (NOTE 4) |
| 73 |  | - | - | 450 MHz | – | 455 MHz | 460 MHz | – | 465 MHz | 1 (NOTE 4) |
| 74 | n74 | - | - | 1427 MHz | – | 1470 MHz | 1475 MHz | – | 1518 MHz | 1 (NOTE 4) |
| 75 (NOTE 5) | n75 | - | - |  | N/A |  | 1432 MHz | – | 1517 MHz | 1 (NOTE 2) |
| 76 (NOTE 5) | n76 | - | - |  | N/A |  | 1427 MHz | – | 1432 MHz | 1 (NOTE 2) |
| 85 |  | - | - | 698 MHz | – | 716 MHz | 728 MHz | – | 746 MHz | 1 (NOTE 4) |
| NOTE 1: The band is for UTRA only. NOTE 2: The band is for E-UTRA and/or NR only. NOTE 3: The band is for NR, E-UTRA and/or UTRA only. NOTE 4: The band is for NR, E-UTRA and/or NB-IoT only. NOTE 5: Restricted to NR and/or E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 6: Restricted to UTRA operation when dual band is configured (e.g., DB-DC-HSDPA or dual band 4C-HSDPA). The down link frequenc(ies) of this band are paired with the uplink frequenc(ies) of the other FDD band (external) of the dual band configuration. NOTE 7: The range 2180-2200 MHz of the DL operating band is restricted to NR and/or E-UTRA operation when carrier aggregation is configured. NOTE 8: Band 23 is not applicable. NOTE 9: The range 2010-2020 MHz of the DL operating band is restricted to NR and/or E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz. The range 2005-2020 MHz of the DL operating band is restricted to NR and/or E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz. |  |  |  |  |  |  |  |  |  |  |

UTRA FDD can operate with DB-DC-HSDPA for the band configurations listed in subclause 5.2 c) of TS 25.104 [2].

NOTE:	For BS capable of multi-band operation, the supported operating bands may belong to different Band Categories.

Table 4.5-2: Unpaired bands in NR, E-UTRA and UTRA.

| MSR and E-UTRA Band number | NR Band number | UTRA Band number | Uplink (UL) BS receive UE transmit |  |  | Downlink (DL) BS transmit UE receive |  |  | Band category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33 |  | a) | 1900 MHz | – | 1920 MHz | 1900 MHz | – | 1920 MHz | 3 |
| 34 | n34 | a) | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | 3 |
| 35 |  | b) | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz | 3 |
| 36 |  | b) | 1930 MHz | – | 1990 MHz | 1930 MHz | – | 1990 MHz | 3 |
| 37 |  | c) | 1910 MHz | – | 1930 MHz | 1910 MHz | – | 1930 MHz | 3 |
| 38 | n38 | d) | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | 3 |
| 39 | n39 | f) | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | 3 |
| 40 | n40 | e) | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | 3 |
| 41 | n41 | - | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | 3 (NOTE 1) |
| 42 |  | - | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | 3 |
| 43 |  | - | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | 3 |
| 44 |  | - | 703 MHz | – | 803 MHz | 703 MHz | – | 803 MHz | 3 |
| 45 |  | - | 1447 MHz | – | 1467 MHz | 1447 MHz | – | 1467 MHz | 3 |
| 48 |  | - | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | 3 |
| 50 | n50 | - | 1432 MHz | - | 1517 MHz | 1432 MHz | - | 1517 MHz | 3 |
| 51 | n51 | - | 1427 MHz | - | 1432 MHz | 1427 MHz | - | 1432 MHz | 3 |
| 52 |  | - | 3300 MHz | – | 3400 MHz | 3300 MHz | – | 3400 MHz | 3 |
| 77 | n77 | - | 3300 MHz | - | 4200 MHz | 3300 MHz | - | 4200 MHz | 3 (NOTE 2) |
| 78 | n78 | - | 3300 MHz | - | 3800 MHz | 3300 MHz | - | 3800 MHz | 3 (NOTE 2) |
| NOTE 1: The band supports NB-IoT in certain regions NOTE 2: The band is for NR only. |  |  |  |  |  |  |  |  |  |

*********************End of change*****************

*********************Next changed section*****************

##### 6.6.1.3.1	Minimum Requirement

The power of any spurious emission shall not exceed the limits of Table 6.6.1.3.1-1 for a BS where requirements for co-existence with the system listed in the first column apply. For BS capable of multi-band operation, the exclusions and conditions in the Note column of Table 6.6.1.3.1-1 apply for each supported operating band. For BS capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the exclusions and conditions in the Note column of Table 6.6.1.3.1-1 apply for the operating band supported at that antenna connector.

Table 6.6.1.3.1-1: BS Spurious emissions limits for co-existence with systems operating in other frequency bands

| System type to co-exist with |  | Frequency range for co-existence requirement |  | Maximum Level |  | Measurement Bandwidth |  | Note |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSM900 |  | 921 - 960 MHz |  | -57 dBm |  | 100 kHz |  | This requirement does not apply to BS operating in band 8. |  |
|  |  | 876 - 915 MHz |  | -61 dBm |  | 100 kHz |  | For the frequency range 880-915 MHz, this requirement does not apply to BS operating in band 8, since it is already covered by the requirement in sub-clause 6.6.1.2. |  |
| DCS1800 (Note 3) |  | 1805 - 1880 MHz |  | -47 dBm |  | 100 kHz |  | This requirement does not apply to BS operating in band 3. |  |
|  |  | 1710 - 1785 MHz |  | -61 dBm |  | 100 kHz |  | This requirement does not apply to BS operating in band 3, since it is already covered by the requirement in sub-clause 6.6.1.2. |  |
| PCS1900 |  | 1930 - 1990 MHz |  | -47 dBm |  | 100 kHz |  | This requirement does not apply to BS operating in band 2, 25, 36, 70. |  |
|  |  | 1850 - 1910 MHz |  | -61 dBm |  | 100 kHz |  | This requirement does not apply to BS operating in band 2/n2 or 25, since it is already covered by the requirement in sub-clause 6.6.1.2. This requirement does not apply to BS operating in band 35. |  |
| GSM850 or CDMA850 |  | 869 - 894 MHz |  | -57 dBm |  | 100 kHz |  | This requirement does not apply to BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |  |
|  |  | 824 - 849 MHz |  | -61 dBm |  | 100 kHz |  | This requirement does not apply to BS operating in band 5 or 26, since it is already covered by the requirement in sub-clause 6.6.1.2. For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 |  | 2110 - 2170 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 1 or 65, |  |
|  |  | 1920 - 1980 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 1 or 65, since it is already covered by the requirement in sub-clause 6.6.1.2. |  |
| UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 |  | 1930 - 1990 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 2/n2, 25, 70. |  |
|  |  | 1850 - 1910 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 2/n2 or 25, since it is already covered by the requirement in sub-clause 6.6.1.2 |  |
| UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 (Note 3) |  | 1805 - 1880 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 3 or 9. |  |
|  |  | 1710 - 1785 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 3, since it is already covered by the requirement in sub-clause 6.6.1.2. For BS operating in band 9, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in sub-clause 6.6.1.2. |  |
| UTRA FDD Band IV or E-UTRA Band 4 |  | 2110 - 2155 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 4, 10 or 66 |  |
|  |  | 1710 - 1755 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 4, 10 or 66, since it is already covered by the requirement in sub-clause 6.6.1.2. |  |
| UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 |  | 869 - 894 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |  |
|  |  | 824 - 849 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 5 or 26, since it is already covered by the requirement in sub-clause 6.6.1.2. For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 18, 19 |  | 860 - 890 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 6, 18, 19 |  |
|  |  | 815 - 830 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 18 since it is already covered by the requirement in sub-clause 6.6.1.2. |  |
|  |  | 830 - 845 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 6, 19, since it is already covered by the requirement in sub-clause 6.6.1.2. |  |
| UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 |  | 2620 - 2690 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 7. |  |
|  |  | 2500 - 2570 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 7, since it is already covered by the requirement in sub-clause 6.6.1.2. |  |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 |  | 925 - 960 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 8. |  |
|  |  | 880 - 915 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 8, since it is already covered by the requirement in sub-clause 6.6.1.2. |  |
| UTRA FDD Band IX or E-UTRA Band 9 |  | 1844.9 - 1879.9 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 3 or 9. |  |
|  |  | 1749.9 - 1784.9 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 3 or 9, since it is already covered by the requirement in sub-clause 6.6.1.2. |  |
| UTRA FDD Band X or E-UTRA Band 10 |  | 2110 - 2170 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 4, 10, 66 |  |
|  |  | 1710 - 1770 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 10, 66, since it is already covered by the requirement in sub-clause 6.6.1.2. For BS operating in Band 4, it applies for 1755 MHz to 1770 MHz, while the rest is covered in sub-clause 6.6.1.2. |  |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 |  | 1475.9 - 1510.9 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 11, 21, 32, 50, 74, 75 |  |
|  |  | 1427.9 - 1447.9 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 11 or 74, since it is already covered by the requirement in sub-clause 6.6.1.2. This requirement does not apply to BS operating in band 32, 50, 51, 75, 76. |  |
|  |  | 1447.9 – 1462.9 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 21 or 74, since it is already covered by the requirement in sub-clause 6.6.1.2. This requirement does not apply to BS operating in band 32, 50, 75 or n75. |  |
| UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 |  | 729 - 746 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 12 or 85. |  |
|  |  | 699 - 716 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 12 or 85, since it is already covered by the requirement in sub-clause 6.6.1.2. For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 7) |  |
| UTRA FDD Band XIII or E-UTRA Band 13 |  | 746 - 756 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 13. |  |
|  |  | 777 - 787 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 13, since it is already covered by the requirement in sub-clause 6.6.1.2. |  |
| UTRA FDD Band XIV or E-UTRA Band 14 |  | 758 - 768 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 14. |  |
|  |  | 788 - 798 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 14, since it is already covered by the requirement in sub-clause 6.6.1.2. |  |
| E-UTRA Band 17 |  | 734 - 746 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 17. |  |
|  |  | 704 - 716 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 17, since it is already covered by the requirement in subclause 6.6.1.2. For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 7) |  |
| UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 |  | 791 - 821 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 20, 28. |  |
|  |  | 832 - 862 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 20, since it is already covered by the requirement in subclause 6.6.1.2. |  |
| UTRA FDD Band XXII or E-UTRA Band 22 |  | 3510 – 3590 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 22, 42, 48, 49, 77 or 78. |  |
|  |  | 3410 – 3490 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 22, since it is already covered by the requirement in subclause 6.6.1.2. This requirement does not apply to Band 42. |  |
| E-UTRA Band 24 |  | 1525 – 1559 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 24. |  |
|  |  | 1626.5 – 1660.5 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 24, since it is already covered by the requirement in subclause 6.6.1.2. |  |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR Band n25 |  | 1930 - 1995 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 2/n2, 25, 70. |  |
|  |  | 1850 - 1915 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 25, since it is already covered by the requirement in sub-clause 6.6.1.2. For BS operating in Band 2/n2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in sub-clause 6.6.1.2. |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 |  | 859 - 894 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |  |
|  |  | 814 - 849 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 26, since it is already covered by the requirement in sub-clause 6.6.1.2. For BS operating in Band 5, it applies for 814 MHz to 824 MHz, while the rest is covered in sub-clause 6.6.1.2. For BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| E-UTRA Band 27 |  | 852 – 869 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in bands 5, 26 or 27. |  |
|  |  | 807 – 824 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 27, since it is already covered by the requirement in subclause 6.6.1.2. For BS operating in Band 26, it applies for 807 MHz to 814 MHz, while the rest is covered in sub-clause 6.6.1.2. This requirement also applies to BS operating in Band 28, starting 4 MHz above the Band 28 downlink operating band (Note 6). |  |
| E-UTRA Band 28 or NR Band n28 |  | 758 - 803 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 20, 28, 44, 67 or 68. |  |
|  |  | 703 - 748 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 28, since it is already covered by the requirement in sub-clause 6.6.1.2. This requirement does not apply to BS operating in Band 44. For BS operating in Band 67, it applies for 703-736MHz. For E-UTRA BS operating in Band 68, it applies for 728MHz to 733MHz. |  |
| E-UTRA Band 29 |  | 717 – 728 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 29 or 85. |  |
| E-UTRA Band 30 |  | 2350 - 2360 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 30 or 40. |  |
|  |  | 2305 - 2315 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 30, since it is already covered by the requirement in sub-clause 6.6.1.2. This requirement does not apply to BS operating in Band 40. |  |
| E-UTRA Band 31 |  | 462.5 – 467.5 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 31, 72 or 73. |  |
|  |  | 452.5 – 457.5 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 31, since it is already covered by the requirement in sub-clause 6.6.1.2. This requirement does not apply to BS operating in band 72 or 73. |  |
| UTRA FDD Band XXXII or E-UTRA Band 32 |  | 1452 - 1496 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 11, 21, 32, 50, 74, 75. |  |
| UTRA TDD Band a) or E-UTRA Band 33 |  | 1900 - 1920 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 33 |  |
| UTRA TDD Band a) or E-UTRA Band 34 or NR Band n34 |  | 2010 - 2025 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 34 |  |
| UTRA TDD Band b) or E-UTRA Band 35 |  | 1850 – 1910 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 35 |  |
| UTRA TDD Band b) or E-UTRA Band 36 |  | 1930 - 1990 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 2/n2, 25 or 36 |  |
| UTRA TDD Band c) or E-UTRA Band 37 |  | 1910 - 1930 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |  |
| UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38 |  | 2570 – 2620 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 38 or 69. |  |
| UTRA TDD Band f) or E-UTRA Band 39 or NR Band n39 |  | 1880 – 1920MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 39 |  |
| UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40 |  | 2300 – 2400MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 30 or 40 |  |
| E-UTRA Band 41 or NR Band n41 |  | 2496 – 2690MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 41 |  |
| E-UTRA Band 42 |  | 3400 – 3600 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 22, 42 43, 48, 49, 52, 77 or 78 |  |
| E-UTRA Band 43 |  | 3600 – 3800 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 42, 43, 48, 49, 77 or 78 |  |
| E-UTRA Band 44 |  | 703 - 803 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 28 or 44 |  |
| E-UTRA Band 45 |  | 1447 - 1467 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 45 |  |
| E-UTRA Band 46 |  | 5150 - 5925 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 46 |  |
| E-UTRA Band 47 |  | 5855 - 5925 MHz |  | -52 dBm |  | 1 MHz |  |  |  |
| E-UTRA Band 48 |  | 3550 - 3700 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 22, 42, 43, 48, 49, 77 or 78. |  |
| E-UTRA Band 49 |  | 3550 - 3700 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 22, 42, 43, 48, 49, 77 or 78. |  |
| E-UTRA Band 50 or NR Band n50 |  | 1432 - 1517 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 11, 21, 32, 45, 50, 51, 74, 75, 76. |  |
| E-UTRA Band 51 or NR Band n51 |  | 1427 - 1432 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 50, 51, 75, 76. |  |
| E-UTRA Band 52 |  | 3300 – 3400 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 42 or 52 |  |
| E-UTRA Band 65 |  | 2110 - 2200 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 1 or 65, |  |
|  |  | 1920 - 2010 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 65, since it is already covered by the requirement in sub-clause 6.6.1.2. For BS operating in Band 1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in sub-clause 6.6.1.2. |  |
| E-UTRA Band 66 or NR Band n66 |  | 2110 - 2200 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 4, 10, 23, 66. |  |
|  |  | 1710 - 1780 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 66, since it is already covered by the requirement in sub-clause 6.6.1.2. For BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in sub-clause 6.6.1.2. For BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in sub-clause 6.6.1.2. |  |
| E-UTRA Band 67 |  | 738 – 758 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 28 or 67. |  |
| E-UTRA Band 68 |  | 753 -783 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 28, or 68. |  |
|  |  | 698-728 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 68, since it is already covered by the requirement in sub-clause 6.6.1.2. For BS operating in Band 28, it applies between 698 MHz and 703 MHz, while the rest is covered in sub-clause 6.6.1.2. |  |
| E-UTRA Band 69 |  | 2570 - 2620 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 38 or 69. |  |
| E-UTRA Band 70 or NR Band n70 |  | 1995 - 2020 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 2/n2, 25, 70 |  |
|  |  | 1695 – 1710 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 70, since it is already covered by the requirement in sub-clause 6.6.1.2 |  |
| E-UTRA Band 71 or NR Band n71 |  | 617 – 652 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 71 |  |
|  |  | 663 – 698 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 71, since it is already covered by the requirement in sub-clause 6.6.1.2 |  |
| E-UTRA Band 72 |  | 461 - 466 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 31, 72 or 73. |  |
|  |  | 451 - 456 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 72, since it is already covered by the requirement in sub-clause 6.6.1.2. This requirement does not apply to BS operating in band 73. |  |
| E-UTRA Band 73 |  | 460 - 465 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 31, 72 or 73. |  |
|  |  | 450 - 455 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 73, since it is already covered by the requirement in sub-clause 6.6.1.2. |  |
| E-UTRA Band 74 or NR band n74 |  | 1475 – 1518 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 11, 21, 32, 50, 74, 75. |  |
|  |  | 1427 – 1470 MHz |  | -49 dBm |  | 1MHz |  | This requirement does not apply to BS operating in Band 74 or n74, since it is already covered by the requirement in sub-clause 6.6.1.2. This requirement does not apply to BS operating in band 32, 45, 50, 51, 75, 76. |  |
| E-UTRA Band 75 or NR Band n75 |  | 1432 - 1517 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 11, 21, 32, 45, 50, 51, 74, 75, 76. |  |
| E-UTRA Band 76 or NR Band n76 |  | 1427 - 1432 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in Band 50, 51, 75, 76. |  |
| NR Band n77 |  | 3300 MHz – 4200 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 42, 43, 48, 49, 77 or 78 |  |
| NR Band n78 |  | 3300 MHz – 3800 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to BS operating in Band 42, 43, 48, 49, 77 or 78 |  |
| NR Band n80 |  | 1710 - 1785 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 3, since it is already covered by the requirement in sub-clause 6.6.1.2. For BS operating in band 9, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in sub-clause 6.6.1.2. |  |
| NR Band n81 |  | 880 - 915 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 8, since it is already covered by the requirement in sub-clause 6.6.1.2. |  |
| NR Band n82 |  | 832 - 862 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 20, since it is already covered by the requirement in subclause 6.6.1.2. |  |
| NR Band n83 |  | 703 - 748 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 28, since it is already covered by the requirement in sub-clause 6.6.1.2. This requirement does not apply to BS operating in Band 44. For BS operating in Band 67, it applies for 703-736MHz. For E-UTRA BS operating in Band 68, it applies for 728MHz to 733MHz. |  |
| NR Band n84 |  | 1920 - 1980 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 1 or 65, since it is already covered by the requirement in sub-clause 6.6.1.2. |  |
| E-UTRA Band 85 |  | 728 - 746 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 12, 29 or 85. |  |
|  |  | 698 - 716 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 85, since it is already covered by the requirement in sub-clause 6.6.1.2. For BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 7). |  |
| NR Band n86 |  | 1710 - 1780 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to BS operating in band 66, since it is already covered by the requirement in sub-clause 6.6.1.2. For BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in sub-clause 6.6.1.2. For BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in sub-clause 6.6.1.2. |  |
| NOTE 5: Void |  |  |  |  |  |  |  |  |  |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, except for the cases where the noted requirements apply to a BS operating in Band 25, Band 27, Band 28 or Band 29, the co-existence requirements in Table 6.6.1.3.1-1 do not apply for the 10 MHz frequency range immediately outside the downlink operating band (see Tables 4.5-1 and 4.5-2). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.6.1.3.1-1 assumes that two operating bands, where the frequency ranges in Table 4.5-1 or Table 4.5-2 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	For the protection of DCS1800, UTRA Band III, E-UTRA Band 3 or NR Band n3 in China, the frequency ranges of the downlink and uplink protection requirements are 1805 – 1850 MHz and 1710 – 1755 MHz respectively.

NOTE 4:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations (except in Band 46), special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 6:	For Band 28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with Band 27 UL operating band.

NOTE 7:	For Band 29 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with UTRA Band XII or E-UTRA Band 12 UL operating band or E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL operating band.

*********************End of change*****************

*********************Next changed section*****************

##### 6.6.1.4.1	Minimum Requirement

The power of any spurious emission shall not exceed the limits of Table 6.6.1.4.1-1 for a BS where requirements for co-location with a BS type listed in the first column apply, depending on the declared Base Station class. For BS capable of multi-band operation, the exclusions and conditions in the Note column of Table 6.6.1.4.1-1 apply for each supported operating band. For BS capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the exclusions and conditions in the Note column of Table 6.6.1.4.1-1 apply for the operating band supported at that antenna connector.

Table 6.6.1.4.1-1: BS Spurious emissions limits for BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level (WA-BS) | Maximum Level (MR-BS) | Maximum Level (LA-BS) | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- | --- | --- |
| GSM900 | 876-915 MHz | -98 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| DCS1800 | 1710 - 1785 MHz | -98 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| PCS1900 | 1850 - 1910 MHz | -98 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| GSM850 or CDMA850 | 824 - 849 MHz | -98 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 - 1980 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 | 1850 - 1910 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 | 1710 - 1785 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 | 824 - 849 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 19 | 830 - 845 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 | 2500 - 2570 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 | 880 - 915 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 - 1447.9 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 50, 51, 75, 76 |
| UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 699 - 716 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XIV or E-UTRA Band 14 | 788 - 798 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 17 | 704 - 716 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 18 | 815 - 830 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 | 832 - 862 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 – 1462.9 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 32, 50, 75 |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3410 – 3490 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 77 or 78 |
| E-UTRA Band 23 | 2000 - 2020 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 24 | 1626.5 – 1660.5 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR Band n25 | 1850 - 1915 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 814 - 849 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 27 | 807 - 824 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 28 or NR Band n28 | 703 – 748 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 44 |
| E-UTRA Band 30 | 2305 - 2315 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 40 |
| E-UTRA Band 31 | 452.5 – 457.5 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 33 |
| UTRA TDD Band a) or E-UTRA Band 34 or NR Band n34 | 2010 - 2025 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 34 |
| UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 35 |
| UTRA TDD Band b) or E-UTRA Band 36 | 1930 - 1990 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 2, n2 and 36 |
| UTRA TDD Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38 | 2570 – 2620 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 38. |
| UTRA TDD Band f) or E-UTRA Band 39 or NR Band n39 | 1880 – 1920MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 33 and 39 |
| UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40 | 2300 – 2400MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 30 or 40 |
| E-UTRA Band 41 or NR Band n41 | 2496 – 2690MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 41 |
| E-UTRA Band 42 | 3400 – 3600 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 22, 42, 43, 48, 49, 52, 77 or 78 |
| E-UTRA Band 43 | 3600 – 3800 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 43, 48, 49, 77 or 78 |
| E-UTRA Band 44 | 703 – 803 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 28 or 44 |
| E-UTRA Band 45 | 1447 – 1467 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 45 |
| E-UTRA Band 46 | 5150 – 5925 MHz | N/A | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 46 |
| E-UTRA Band 48 | 3550 – 3700 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 43, 48, 49, 77 or 78 |
| E-UTRA Band 49 | 3550 – 3700 MHz | N/A | N/A | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 43, 48, 49, 77 or 78 |
| E-UTRA Band 50 or NR Band n50 | 1432 – 1517 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 11, 21, 32, 51, n51, 74, 75, 76 |
| E-UTRA Band 51 or NR Band n51 | 1427 – 1432 MHz | N/A | N/A | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 50, 75, 76 |
| E-UTRA Band 52 | 3300 – 3400 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 42 or 52 |
| E-UTRA Band 65 | 1920 - 2010 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 66 or NR Band n66 | 1710 – 1780 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 68 | 698 – 728 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 70 or NR Band n70 | 1695 – 1710 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 71 or NR Band n71 | 663 – 698 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 72 | 451 – 456 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 73 | 450 – 455 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 74 or NR band n74 | 1427 – 1470 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 50, 51 |
| NR Band n77 | 3300 MHz – 4200 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 42 43, 48, 49, 77 or 78 |
| NR Band n78 | 3300 MHz – 3800 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 42, 43, 48, 49, 77 or 78 |
| NR Band n80 | 1710 – 1785 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| NR Band n81 | 880 – 915 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| NR Band n82 | 832 – 862 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| NR Band n83 | 703 – 748 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 44 |
| NR Band n84 | 1920 – 1980 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| E-UTRA Band 85 | 698 - 716 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |
| NR Band n86 | 1710 – 1780 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, the co-location requirements in Table 6.6.1.4.1-1 do not apply for the ΔfOBUE frequency range immediately outside the BS transmit frequency range of a downlink operating band (see Tables 4.5-1 and 4.5-2). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [7].

NOTE 2:	Table 6.6.1.4.1-1 assumes that two operating bands, where the corresponding BS transmit and receive frequency ranges in Table 4.5-1 or Table 4.5-2 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	Co-located TDD base stations that are synchronized and using the same or adjacent operating band can transmit without special co-locations requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications.

*********************End of change*****************

*********************Next changed section*****************

##### 6.6.2.4.3	Protection of DTT

In certain regions the following requirement may apply for protection of DTT. For a BS operating in Band 20, the level of emissions in the band 470-790 MHz, measured in an 8 MHz filter bandwidth on centre frequencies Ffilter according to Table 6.6.2.4.3-1, shall not exceed the maximum emission level PEM,N declared by the manufacturer. This requirement applies in the frequency range 470-790 MHz even though part of the range falls in the spurious domain.

Table 6.6.2.4.3-1: Declared emissions levels for protection of DTT

| Filter centre frequency, Ffilter | Measurement bandwidth | Declared emission level [dBm] |
| --- | --- | --- |
| Ffilter = 8*N + 306 (MHz); 21 ≤ N ≤ 60 | 8 MHz | PEM,N |

Note: 	The regional requirement is defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. Compliance with the regional requirement can be determined using the method outlined in Annex G of TS 36.104 [4].

*********************End of change*****************

*********************Next changed section*****************

##### 6.6.2.4.7	Additional band 32, 50, 51, 74, 75 and 76 unwanted emissions

In certain regions, the following requirements may apply to BS operating in Band 32 within 1452-1492 MHz, in Band 75 within 1432-1517 MHz and in Band 76 within 1427-1432 MHz. The level of operating band unwanted emissions, measured on centre frequencies f_offset with filter bandwidth, according to Table 6.6.2.4.7-1, shall neither exceed the maximum emission level PEM,B32,B75,B76,a ,  PEM,B32,B75,B76,b nor PEM,B32,B75,B76,c declared by the manufacturer.

For Band 32, this requirement applies in the frequency range 1452-1492 MHz when non-Mobile/Fixed Communications Network (MFCN) services are deployed in adjacent frequency ranges, while it applies also within 1427-1452 MHz and/or 1492-1517 MHz when MFCN services are deployed in such frequency ranges, even though part of the ranges falls in the spurious domain. For Band 75, this requirement applies in the frequency range 1427-1517 MHz. For Band 76, this requirement applies in the frequency range 1432-1517 MHz even though part of the range falls in the spurious domain.

Table 6.6.2.4.7-1: Declared operating band 32, 75, 76 unwanted emission within 1427-1517 MHz

| Frequency offset of measurement filter centre frequency, f_offset | Declared emission level [dBm] | Measurement bandwidth |
| --- | --- | --- |
| 2.5 MHz | PEM,B32,B75,B76,a | 5 MHz |
| 7.5 MHz | PEM,B32,B75,B76,b | 5 MHz |
| 12.5 MHz ≤ f_offset ≤ f_offsetmax | PEM,B32,B75,B76,c | 5 MHz |
| NOTE: For Band 32, when non-MFCN services are deployed in the adjacent bands, f_offsetmax denotes the frequency difference between the lower Base Station RF Bandwidth edge and 1454.5 MHz, and the frequency difference between the upper Base Station RF Bandwidth edge and 1489.5 MHz for the set channel position. For Band 32, when MFCN services are deployed in the adjacent frequencies, Band 75, n75, 76 and n76, f_offsetmax denotes the frequency difference between the lower Base Station RF Bandwidth edge and 1429.5 MHz, and the frequency difference between the upper Base Station RF Bandwidth edge and 1514.5 MHz for the set channel position. |  |  |

NOTE:	The regional requirement, included in [16], is defined in terms of EIRP per antenna, which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. The assessment of the EIRP level is described in Annex H of TS 36.104 [4].

In certain regions, the following requirement may apply to BS operating in Band 32 within 1452-1492MHz for the protection of non-MFCN services in spectrum adjacent to the frequency range 1452-1492 MHz. The level of emissions, measured on centre frequencies Ffilter with filter bandwidth according to Table 6.6.2.4.7-2, shall neither exceed the maximum emission level PEM,B32,d nor PEM,B32,e declared by the manufacturer. This requirement applies in the frequency range 1429-1518MHz even though part of the range falls in the spurious domain.

Table 6.6.2.4.7-2: Operating band 32 declared emission outside 1452-1492 MHz

| Filter centre frequency, Ffilter | Declared emission level [dBm] | Measurement bandwidth |
| --- | --- | --- |
| 1429.5 MHz ≤ Ffilter ≤ 1448.5 MHz | PEM,B32,d | 1 MHz |
| Ffilter = 1450.5 MHz | PEM,B32,e | 3 MHz |
| Ffilter = 1493.5 MHz | PEM,B32,e | 3 MHz |
| 1495.5 MHz ≤ Ffilter ≤ 1517.5 MHz | PEM,B32,d | 1 MHz |

NOTE:	The regional requirement, included in [15], is defined in terms of EIRP, which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. The assessment of the EIRP level is described in Annex H of TS 36.104 [4].

In certain regions, the following requirement may apply to BS operating in Band 50 and Band 75 within 1492-1517 MHz and in Band 74 within 1492-1518 MHz. The level of emissions, measured on centre frequencies Ffilter with filter bandwidth according to Table 6.6.2.4.7-3, shall neither exceed the maximum emission level PEM,B50,B74,B75,a nor PEM,B50,B74,B75,b declared by the manufacturer.

Table 6.6.2.4.7-3: Operating band 50, 74 and 75 declared emission above 1518 MHz

| Filter centre frequency, Ffilter | Declared emission level [dBm] | Measurement bandwidth |
| --- | --- | --- |
| 1518.5 MHz ≤ Ffilter ≤ 1519.5 MHz | PEM,B50,B74,B75,a | 1 MHz |
| 1520.5 MHz ≤ Ffilter ≤ 1558.5 MHz | PEM,B50,B74,B75,b | 1 MHz |

NOTE:	The regional requirement, included in [16], is defined in terms of EIRP, which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. The assessment of the EIRP level is described in Annex H.

In certain regions, the following requirement may apply to NR or E-UTRA BS operating in Band 50 and Band 75 within 1432-1452 MHz, and in Band 51 and Band 76. Emissions shall not exceed the maximum levels specified in Table 6.6.2.4.7-4.

Table 6.6.2.4.7-4: Additional operating band unwanted emission limits for BS operating in Band 50 and 75 within 1432-1452 MHz, and in Band 51 and 76

| Filter centre frequency, Ffilter | Maximum Level [dBm] | Measurement Bandwidth |
| --- | --- | --- |
| Ffilter = 1413.5 MHz | -42 | 27 MHz |

*********************End of change*****************

*********************Next changed section*****************

### 7.5.2	Co-location minimum requirement

This additional blocking requirement may be applied for the protection of BS receivers when NR, E-UTRA, NB-IoT, UTRA, CDMA or GSM/EDGE BS operating in a different frequency band are co-located with a BS.

The requirements in this subclause assume a 30 dB coupling loss between the interfering transmitter and the BS receiver and are based on co-location with base stations of the same class.

For a wanted and an interfering signal coupled to BS antenna input using the parameters in Table 7.5.2-1, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [4], subclause 7.2.

-	For any UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.104 [2], subclause 7.2.

-	For any UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.105 [3], subclause 7.2.

-	For any GSM/EDGE carrier, the conditions are specified in TS 45.005 [5], Annex P.2.1.

-	For any NB-IoT carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [4], subclause 7.2.

-	For any NR carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 38.104 [17], subclause 7.2.

Table 7.5.2-1: Blocking requirement for co-location with BS in other frequency bands.

| Type of co-located BS | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power for WA BS (dBm) | Interfering Signal mean power for MR BS (dBm) | Interfering Signal mean power for LA BS (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- |
| GSM850 or CDMA850 | 869 – 894 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| GSM900 | 921 – 960 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| DCS1800 | 1805 – 1880 (Note 4) | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| PCS1900 | 1930 – 1990 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 2110 – 2170 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band II or E-UTRA Band 2 or NR Band n2 | 1930 – 1990 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band III or E-UTRA Band 3 or NR Band n3 | 1805 – 1880 (Note 4) | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 – 2155 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band V or E-UTRA Band 5 or NR Band n5 | 869 – 894 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band VI or E-UTRA Band 6 | 875 – 885 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band VII or E-UTRA Band 7 or NR Band n7 | 2620 – 2690 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band VIII or E-UTRA Band 8 or NR Band n8 | 925 – 960 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 – 1879.9 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band X or E-UTRA Band 10 | 2110 – 2170 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XI or E-UTRA Band 11 | 1475.9 - 1495.9 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 729 - 746 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XIIII or E-UTRA Band 13 | 746 - 756 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XIV or E-UTRA Band 14 | 758 - 768 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 17 | 734 - 746 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 18 | 860 - 875 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XIX or E-UTRA Band 19 | 875 - 890 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XX or E-UTRA Band 20 or NR Band n20 | 791 - 821 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XXI or E-UTRA Band 21 | 1495.9 – 1510.9 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510 – 3590 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 23 | 2180 - 2200 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 24 | 1525 – 1559 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XXV or E-UTRA Band 25 or NR Band n25 | 1930 – 1995 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 859 – 894 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 27 | 852 - 869 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 28 or NR Band n28 | 758 – 803 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 29 | 717 – 728 | +16** | +8** | -6** | PREFSENS + 6dB* | CW carrier |
| E-UTRA Band 30 | 2350-2360 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 31 | 462.5 – 467.5 | +16** | +8** | -6** | PREFSENS + 6dB* | CW carrier |
| UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 – 1496 (NOTE 5) | +16** | +8** | -6** | PREFSENS + 6dB* | CW carrier |
| UTRA TDD Band a) or E-UTRA TDD Band 33 | 1900-1920 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA TDD Band a) or E-UTRA TDD Band 34 or NR Band n34 | 2010-2025 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA TDD Band b) or E-UTRA TDD Band 35 | 1850-1910 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA TDD Band b) or E-UTRA TDD Band 36 | 1930-1990 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA TDD Band c) or E-UTRA TDD Band 37 | 1910-1930 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA TDD Band d) or E-UTRA Band 38 or NR Band n38 | 2570-2620 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA TDD Band f) or E-UTRA Band 39 or NR Band n39 | 1880-1920 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| UTRA TDD Band e) or E-UTRA Band 40 or NR Band n40 | 2300-2400 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 41 or NR Band n41 | 2496 - 2690 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 42 | 3400 – 3600 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 43 | 3600 – 3800 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 44 | 703 - 803 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 45 | 1447 - 1467 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 46 | 5150 - 5925 | N/A | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 48 | 3550 – 3700 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 49 | 3550 – 3700 | N/A | N/A | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 50 or NR Band n50 | 1432 – 1517 | +16 | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 51 or NR Band n51 | 1427– 1432 | N/A | N/A | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 52 | 3300 – 3400 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 65 | 2110 – 2200 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 66 or NR Band n66 | 2110 – 2200 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 67 | 738 - 758 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 68 | 753 - 783 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 69 | 2570-2620 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 70 or NR Band n70 | 1995 - 2020 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 71 or NR Band n71 | 617 - 652 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 72 | 461 - 466 | +16** | +8** | -6** | PREFSENS + 6dB* | CW carrier |
| E-UTRA Band 73 | 460 - 465 | +16** | +8** | -6** | PREFSENS + 6dB* | CW carrier |
| E-UTRA Band 74 or NR band n74 | 1475 - 1518 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 75 or NR Band n75 | 1432 - 1517 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 76 or NR Band n76 | 1427 - 1432 | N/A | N/A | -6** | PREFSENS + x dB* | CW carrier |
| NR Band n77 | 3300 - 4200 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| NR Band n78 | 3300 - 3800 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 85 | 728 - 746 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| NOTE 1 (*): PREFSENS depends on the RAT, the BS class and the channel bandwidth, see subclause 7.2. “x” is equal to 3 in case of GSM/EDGE wanted signal and equal to 6 in case of NR or UTRA or E-UTRA or NB-IoT wanted signals. NOTE 2: Except for a BS operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the ΔfOOB immediately outside any of the supported uplink operating band. For a BS operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 3: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD or E-UTRA TDD or NR TDD with E-UTRA FDD or NR FDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [7]. NOTE 4: In China, the blocking requirement for co-location with DCS1800 and Band III BS is only applicable in the frequency range 1805-1850MHz. NOTE 5: For a BS operating in band 11, 21, 74, the requirement for co-location with Band 32 applies for interfering signal within the frequency range 1475.9-1495.9 MHz. NOTE 6: Co-located TDD base stations that are synchronized and using the same or adjacent operating band can receive without special co-location requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications. NOTE 7 (**): For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |  |  |