3GPP TSG-RAN WG4Meeting #90	R4-1902255

# Athens, Greece, 25 February – 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.810 | CR | - | rev | - | Current version: | 16.1.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR38.810 on beam correspondence test procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_test_methods |  |  |  |  |  | Date: |  |  | 2018-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections in this draft CR follows the agreement on beam correspondence requirements in RAN#82 to specify relevant test procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise the test procedure of Tx beam peak search and EIRP spherical coverage to explicitly reflect the beam correspondence requirements based on UE’s beam correspondence capability; Add new test procedure of beam correspondence tolerance for the UE relying on uplink beam sweeping to fulfill minimum peak EIRP and EIRP spherical coverage requirements; Other minor changes on details. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test procedures will be unclear for beam correspondence requriements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.3.7, 5.2.1.3.9(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes 1 >>

##### 5.2.1.3.7	TX Beam Peak direction search and EIRP Spherical Coverage

The beam peak search and spherical coverage test procedure apply to DUTs with and without beam correspondence. The TX beam peak direction is found with a 3D EIRP scan (separately for each orthogonal downlink polarization). The TX beam peak direction search grid points for this single grid approach are defined in Annex G.2. Alternatively, a coarse and fine grid approach could be used according to the definition in Annex G.2.4.

The measurement procedure includes the following steps for each of the points in the grid:

1)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= to form the TX beam towards the measurement antenna.

2)	DUT refines its TX beam toward that direction depending on DUT’s beam correspondence capability reported by UE signalling which should match OEM declaration: if  DUT’s beam correspondence capability (UE feature 2-20) is [bit-1], then DUT chooses the corresponding TX beam using downlink reference signals only; if DUT’s beam correspondence capability (UE feature 2-20) is [bit-0], then DUT chooses the corresponding TX beam using both, the downlink reference signals, and uplink beam sweeping by its network-assisted uplink beam management capability (UE feature 2-30).

3)	Lock the beam.

4)	Measure the mean power Pmeas(PolMeas= PolLink=) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator).

5)	Calculate EIRP (PolMeas= PolLink=)   by adding the composite loss of the entire transmission path for utilized signal path, LEIRP,θ, and frequency to the measured power Pmeas(PolMeas= PolLink=)

6)	Measure the mean power Pmeas (PolMeas= PolLink=) of the modulated signal arriving at the power measurement equipment.

7)	Calculate EIRP (PolMeas= PolLink= )  by adding the composite losses of the entire transmission path for utilized signal path, LEIRP,φ, and frequency to the measured power Pmeas (PolMeas= PolLink=)

8)	Calculate total EIRP(PolLink=)  = EIRP(PolMeas= PolLink=)  + EIRP(PolMeas= PolLink=)

9)	Unlock the beam.

10)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= polarization to form the TX beam towards the measurement antenna.

11)	Repeat steps 2) to 9).

The TX beam peak direction is where the maximum total component of EIRP(PolLink=) or EIRP(PolLink=) is found.

The EIRP results from the TX beam peak search using the minimum number of grid points as described in Annex G.2 can be re-used for EIRP spherical coverage. In case a coarse beam peak grid is used for TX beam peak search, using the minimum number of grid points defined in Annex G.3.3.2.3, the EIRP results can be re-used for EIRP spherical coverage.

In case a separate test is performed for EIRP spherical coverage, the procedure above should be followed using the minimum number of grid points defined in Annex G.3.3.2.3 for spherical coverage.

The EIRPtarget-CDF is then obtained from the Cumulative Distribution Function (CDF) computed using [the maximum total component of EIRP(PolLink=) or EIRP(PolLink=)] for all grid points. When using constant step size measurement grids, a theta-dependent correction shall be applied, i.e., the PDF probability contribution for each measurement point is scaled by sin(θ).

[Editor’s Note:	For spherical coverage test, using the maximum total component of EIRP(PolLink=) or EIRP(PolLink=) for each test point in the grid is considered as the baseline for performance evaluation and test procedure. Decision shall be made on April meeting]

<< End of Changes 1 >>

<< Start of Changes 2>>

##### 5.2.1.3.9	Beam Correspondence Tolerance

The DUT that has beam correspondence capability (UE feature 2-20) as [bit-1] shall meet the minimum peak EIRP and spherical coverage requirement with its autonomously chosen UL beams and without uplink beam sweeping is considered to have met the beam correspondence requirement. Thus, the beam correspondence tolerance test is only applicable tothe DUT that relies on uplink beam sweeping to meet the minimum peak EIRP and spherical coverage requirements.

ΔEIRPBC is introduced for beam correspondence tolerance based on two EIRP measurements (EIRP1 and EIRP2). EIRP1 is calculated based on the beam the UE chooses autonomously (corresponding beam) to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping. EIRP2 is the best EIRP (beam yielding highest EIRP in a given direction) which is based on beam correspondence with relying on UL beam sweeping. ΔEIRPBC shall be calculated over the link angles spanning a subset of the spherical coverage grid points which are corresponding to the top 50% of the EIRP2 measurement.

For each of the points in the grid:

1)	Follow the test procedures specified in subclause 5.2.1.3.7 with uplink beam sweeping disabled, obtain total EIRP1(PolLink=) and total EIRP1(PolLink=EIRP1 is calculated by [EIRP1 = maximum(EIRP1(PolLink=), EIRP1(PolLink=))

2)	Follow the test procedures specified in subclause 5.2.1.3.7, with uplink beam sweeping enabled during DUT Tx beam refinement, obtain total EIRP2(PolLink=) and total EIRP2(PolLink=EIRP2 is calculated by [EIRP2 = maximum(EIRP2(PolLink=), EIRP2(PolLink=))

3)	Calculate the ΔEIRPBC = EIRP2 – EIRP1.

The ΔEIRPtarget-CDF is then obtained from the Cumulative Distribution Function (CDF) computed using [ΔEIRPBC] for each of all top 50% of the EIRP2 measurement points in the grid. When using constant step size measurement grids, a theta-dependent correction shall be applied, i.e., the PDF probability contribution for each measurement point is scaled by sin(θ).

[Editor’s Note:	For beam correspondence tolerance test, using the maximum total component of EIRP1(PolLink=) or EIRP1(PolLink=), and of EIRP2(PolLink=) or EIRP2(PolLink=) for each test point in the grid is considered as the baseline for performance evaluation and test procedure. Decision shall be made on April meeting]

<< End of Changes 2>>