3GPP TSG-RAN4 Meeting # 90	R4-1902306

# Atens, Greece,  25 Feb – 1 Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-1 | CR | 0132 | rev | 1 | Current version: | 13.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections to TS 37.145-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AAS_BS_LTE_UTRA-Perf |  |  |  |  |  | Date: |  |  | 2019-02-28 |
| Category: | D |  |  |  |  |  | Release: |  |  | 13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some editorial mistakes affect the readibility of the specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | A number of editorial changes throughout the document |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possible confusion when reading the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.5.5.5.4 and .6.6.5.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--------- start of change ----------

6.6.5.5.5.4	Basic limits for Wide Area BS Category B (Option 2)

For Category B Operating band unwanted emissions, there are two options for the limits that may be applied regionally, option 2 is as follows.

The limits in this subclause are intended for Europe and may be applied regionally for a TAB connector operating in band 1, 3, 8, 32, 33, 34 or 65.

For a TAB connector operating in band 1, 3, 8, 32, 33, 34 or 65, basic limits are specified in table 6.6.5.5.5.4-1 below for 5, 10, 15 and 20 MHz channel bandwidth.

--------- end of change ----------

--------- start of change ----------

# .6.6.5.2.5	Co-existence with other systems in the same geographical area

These requirements may be applied for the protection of system operating in frequency ranges other than the TAB connector downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the AAS BS, or they may be set by local or regional regulation as a mandatory requirement for an operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in subclause 4.4.

Some requirements may apply for the protection of specific equipment (UE, MS and/or BS) or equipment operating in specific systems (GSM/EDGE, CDMA, UTRA, E-UTRA, etc.) as listed below. The basic limit any spurious emission shall not exceed the limits of table 6.6.6.5.2.5-1for TAB connector(s) where requirements for co-existence with the system listed in the first column apply. For multi-band TAB connector(s), the exclusions and conditions in the Note column of table 6.6.6.5.2.5-1 apply for each supported operating band.

Table 6.6.6.5.2.5-1: Spurious emissions limits for co-existence
with systems operating in other frequency bands

| System type operating in the same geographical area |  | Band for co-existence requirement |  | Maximum Level |  | Measurement Bandwidth |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSM900 |  | 921 - 960 MHz |  | -57 dBm |  | 100 kHz |  | This requirement does not apply to UTRA FDD operating in band VIII. This requirement does not apply to E-UTRA BS operating in band 8 |  |
|  |  | 876 - 915 MHz |  | -61 dBm |  | 100 kHz |  | For the frequency range 880-915 MHz, this requirement does not apply to UTRA FDD operating in band VIII, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For the frequency range 880-915 MHz, this requirement does not apply to E-UTRA BS operating in band 8, since it is already covered by the requirement in subclause 6.6.6.5.2.4. |  |
| DCS1800 |  | 1805 - 1880 MHz |  | -47 dBm |  | 100 kHz |  | This requirement does not apply to UTRA FDD operating in band III. This requirement does not apply to UTRA TDD operating in Band b and c. For UTRA TDD BS operating in Band f, it applies for 1805 - 1850 MHz This requirement does not apply to E-UTRA BS operating in band 3. |  |
|  |  | 1710 - 1785 MHz |  | -61 dBm |  | 100 kHz |  | This requirement does not apply to UTRA FDD operating in band III, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD operating in Band b and c. For UTRA TDD BS operating in Band f, it applies for 1710 - 1755 MHz This requirement does not apply to E-UTRA BS operating in band 3, since it is already covered by the requirement in subclause 6.6.6.5.2.4. |  |
| PCS1900 |  | 1930 - 1990 MHz |  | -47 dBm |  | 100 kHz |  | This requirement does not apply to UTRA FDD BS operating in frequency band II or band XXV. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in frequency band 2, band 25 or band 36. |  |
|  |  | 1850 - 1910 MHz |  | -61 dBm |  | 100 kHz |  | This requirement does not apply to UTRA FDD BS operating in frequency band II or band XXV, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in frequency band 2 or 25, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in frequency band 35. |  |
| GSM850 or CDMA850 |  | 869 - 894 MHz |  | -57 dBm |  | 100 kHz |  | This requirement does not apply to UTRA FDD BS operating in frequency band V or XXVI. This requirement does not apply to E-UTRA BS operating in frequency band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |  |
|  |  | 824 - 849 MHz |  | -61 dBm |  | 100 kHz |  | This requirement does not apply to UTRA FDD BS operating in frequency band V or XXVI, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in frequency band 5 or 26, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For E-UTRA BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| UTRA FDD Band I or E-UTRA Band 1 |  | 2110 - 2170 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band I, This requirement does not apply to E-UTRA BS operating in band 1 or 65. |  |
|  |  | 1920 - 1980 MHz |  | -49 dBm (UTRA TDD -43 dBm for WA BS -40 dBm for LA BS) |  | 1 MHz (UTRA TDD 3.84 MHz) |  | This requirement does not apply to UTRA FDD BS operating in band I, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in band 1 or 65, since it is already covered by the requirement in subclause 6.6.6.5.2.4. |  |
| UTRA FDD Band II or E-UTRA Band 2 |  | 1930 - 1990 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band II or band XXV4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 2 or 25. |  |
|  |  | 1850 - 1910 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band II or band XXV, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 2 or 25, since it is already covered by the requirement in subclause 6.6.6.5.2.4 |  |
| UTRA FDD Band III or E-UTRA Band 3 |  | 1805 - 1880 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band III or band IX For UTRA TDD BS operating in Band f, it applies for 1805- 1850 MHz This requirement does not apply to E-UTRA BS operating in band 3. |  |
|  |  | 1710 - 1785 MHz |  | -49 dBm (UTRA TDD -43 dBm for WA BS -40 dBm for LA BS) |  | 1 MHz (UTRA TDD 3.84 MHz) |  | This requirement does not apply to UTRA FDD BS operating in band III, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For UTRA BS operating in band IX, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in subclause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in band 3 or 9, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For UTRA TDD BS operating in Band f, it applies for 1710- 1755 MHz For E-UTRA BS operating in band 9, it applies for 1710 MHz to 1749.9 MHz and 1784.9 MHz to 1785 MHz, while the rest is covered in subclause 6.6.6.5.2.4. |  |
| UTRA FDD Band IV or E-UTRA Band 4 |  | 2110 - 2155 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band IV or band X This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 4, 10 or 66 |  |
|  |  | 1710 - 1755 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band IV or band X, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 4, 10 or 66, since it is already covered by the requirement in subclause 6.6.6.5.2.4. |  |
| UTRA FDD Band V or E-UTRA Band 5 |  | 869 - 894 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band V or XXVI This requirement does not apply to E-UTRA BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |  |
|  |  | 824 - 849 MHz |  | -49 dBm (UTRA TDD -43 dBm for WA BS -40 dBm for LA BS) |  | 1 MHz (UTRA TDD 3.84 MHz) |  | This requirement does not apply to UTRA FDD BS operating in band V or XXVI, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in band 5 or 26, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For E-UTRA BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| UTRA FDD Band VI or XIX, or E-UTRA Band 6, 18 or 19 |  | 860 - 890 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band VI or XIX For UTRA TDD applicable in Japan This requirement does not apply to E-UTRA BS operating in band 6, 18, 19. |  |
|  |  | 815 - 845 MHz |  | -49 dBm (UTRA TDD -43 dBm) |  | 1 MHz (UTRA TDD 3.84 MHz) |  | This requirement does not apply to UTRA FDD BS operating in band VI or XIX, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For UTRA TDD applicable in Japan This requirement does not apply to E-UTRA BS operating in band 18 between 815-830 MHz, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in band 6, 19 between 830-845 MHz, since it is already covered by the requirement in subclause 6.6.6.5.2.4. |  |
| UTRA FDD Band VII or E-UTRA Band 7 |  | 2620 - 2690 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band VII, This requirement does not apply to E-UTRA BS operating in band 7. This requirement does not apply to E-UTRA BS operating in band 7, since it is already covered by the requirement in subclause 6.6.6.5.2.4. |  |
|  |  | 2500 - 2570 MHz |  | -49 dBm (UTRA TDD -43 dBm for WA BS -40 dBm for LA BS) |  | 1 MHz (UTRA TDD 3.84 MHz) |  | This requirement does not apply to UTRA FDD BS operating in band VII, since it is already covered by the requirement in subclause 6.6.6.5.2.4. |  |
| UTRA FDD Band VIII or E-UTRA Band 8 |  | 925 - 960 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band VIII. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 8. |  |
|  |  | 880 - 915 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band VIII, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 8, since it is already covered by the requirement in subclause 6.6.6.5.2.4. |  |
| UTRA FDD Band IX or E-UTRA Band 9 |  | 1844.9 - 1879.9 MHz |  | -52 dBm (UTRA TDD -43 dBm) |  | 1 MHz (UTRA TDD 3.84 MHz) |  | This requirement does not apply to UTRA FDD BS operating in band III or band IX For UTRA TDD applicable in Japan This requirement does not apply to E-UTRA BS operating in band 3 or 9. |  |
|  |  | 1749. 9 - 1784.9 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band III or band IX, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in band 3 or 9, since it is already covered by the requirement in subclause 6.6.6.5.2.4. |  |
| UTRA FDD Band X or E-UTRA Band 10 |  | 2110 - 2170 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band IV or band X This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 4, 10 or 66 |  |
|  |  | 1710 - 1770 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band X, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For UTRA FDD BS operating in Band IV, it applies for 1755 MHz to 1770 MHz, while the rest is covered in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 10 or 66, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For E-UTRA BS operating in Band 4, it applies for 1755 MHz to 1770 MHz, while the rest is covered in subclause 6.6.6.5.2.4. |  |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 |  | 1475.9 - 1510.9 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XI, XXI, or XXXII. For UTRA TDD applicable in Japan This requirement does not apply to E-UTRA BS operating in band 11, 21 or 32. |  |
|  |  | 1427.9 - 1447.9 MHz |  | -49 dBm (UTRA TDD -43 dBm) |  | 1 MHz (UTRA TDD 3.84 MHz) |  | This requirement does not apply to UTRA FDD BS operating in band XI, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For UTRA BS operating in band XXXII, this requirement applies for carriers allocated within 1475.9MHz and 1495.9MHz. For UTRA TDD applicable in Japan This requirement does not apply to E-UTRA BS operating in band 11, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For E-UTRA BS operating in band 32, this requirement applies for carriers allocated within 1475.9MHz and 1495.9MHz. |  |
|  |  | 1447.9 - 1462.9 MHz |  | -49 dBm (UTRA TDD -43 dBm) |  | 1 MHz (UTRA TDD 3.84 MHz) |  | This requirement does not apply to UTRA FDD BS operating in band XXI, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For UTRA BS operating in band XXXII, this requirement applies for carriers allocated within 1475.9MHz and 1495.9MHz. For UTRA TDD applicable in Japan up to 1462.9MHz. This requirement does not apply to E-UTRA BS operating in band 21, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For E-UTRA BS operating in band 32, this requirement applies for carriers allocated within 1475.9MHz and 1495.9MHz. |  |
| UTRA FDD Band XII or E-UTRA Band 12 |  | 729 - 746 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XII This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 12. |  |
|  |  | 699 - 716 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XII, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 12, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For E-UTRA BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 6) |  |
| UTRA FDD Band XIII or E-UTRA Band 13 |  | 746 - 756 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XIII This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 13. |  |
|  |  | 777 - 787 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XIII, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 13, since it is already covered by the requirement in subclause 6.6.6.5.2.4. |  |
| UTRA FDD Band XIV or E-UTRA Band 14 |  | 758 - 768 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XIV This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 14. |  |
|  |  | 788 - 798 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XIV, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 14, since it is already covered by the requirement in subclause 6.6.6.5.2.4. |  |
| E-UTRA Band 17 |  | 734 - 746 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XII This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 17. |  |
|  |  | 704 - 716 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XII, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 17, since it is already covered by the requirement in subclause 6.6.4.5.3. For E-UTRA BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 6) |  |
| UTRA FDD Band XX or E-UTRA Band 20 |  | 791 - 821 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XX This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 20 or 28. |  |
|  |  | 832 - 862 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XX, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 20, since it is already covered by the requirement in subclause 6.6.6.5.2.4. |  |
| UTRA FDD Band XXII or E-UTRA Band 22 |  | 3510 -3590 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XXII This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 22 or 42. |  |
|  |  | 3410 -3490 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XXII, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 22, since it is already covered by the requirement in subclause 6.6.4.5.3. This requirement does not apply to E-UTRA BS operating in Band 42 |  |
| E-UTRA Band 23 |  | 2180 - 2200 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in band 23 or 66. This requirement does not apply to UTRA TDD |  |
|  |  | 2000 - 2020 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in Band II or XXV, where the limits are defined separately. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 23, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to BS operating in Bands 2 or 25, where the limits are defined separately. |  |
|  |  | 2000 - 2010 MHz |  | -30 dBm |  | 1 MHz |  | This requirement only applies to UTRA FDD BS operating in Band II or Band XXV. This requirement applies starting 5 MHz above the Band XXV downlink operating band. (Note 3) This requirement does not apply to UTRA TDD This requirement only applies to E-UTRA BS operating in Band 2 or Band 25. This requirement applies starting 5 MHz above the Band 25 downlink operating band. (Note 4) |  |
|  |  | 2010 - 2020 MHZ |  | -49 dBm |  | 1 MHz |  |  |  |
| E-UTRA Band 24 |  | 1525 - 1559 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in band 24. This requirement does not apply to UTRA TDD |  |
|  |  | 1626.5 - 1660.5 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in band 24, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD |  |
| UTRA FDD Band XXV or E-UTRA Band 25 |  | 1930 - 1995 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band II or band XXV This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 2 or 25 |  |
|  |  | 1850 - 1915 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XXV, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For UTRA FDD BS operating in Band II, it applies for 1910 MHz to 1915 MHz, while the rest is covered in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 25, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For E-UTRA BS operating in Band 2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in subclause 6.6.6.5.2.4. |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 |  | 859-894 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band V or band XXVI This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 5 or 26. This requirement applies to E-UTRA BS operating in Band 27 for the frequency range 879-894 MHz. |  |
|  |  | 814-849 MHz |  | -49 MHz |  | 1 MHz |  | This requirement does not apply to UTRA FDD BS operating in band XXVI, since it is already covered by the requirements in subclause 6.6.6.5.2.4.For UTRA FDD BS operating in band V, it applies for 814 MHz to 824 MHz, while the rest is covered in subclause 6.6.3.2 This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 26, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For E-UTRA BS operating in Band 5, it applies for 814 MHz to 824 MHz, while the rest is covered in subclause 6.6.6.5.2.4. For E-UTRA BS operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |
| E-UTRA Band 27 |  | 852 - 869 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA BS operating in Band V or XXVI. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in Band 5, 26 or 27. |  |
|  |  | 807 - 824 MHz |  | -49 dBm |  | 1 MHz |  | For UTRA BS operating in Band XXVI, it applies for 807 MHz to 814 MHz, while the rest is covered in subclause 6.6.6.5.2.4. This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in Band 27, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For E-UTRA BS operating in Band 26, it applies for 807 MHz to 814 MHz, while the rest is covered in subclause 6.6.6.5.2.4. This requirement also applies to E-UTRA BS operating in Band 28, starting 4 MHz above the Band 28 downlink operating band (Note 5). |  |
| E-UTRA Band 28 |  | 758 - 803 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in band 20, 28, 44, 67 or 68. This requirement does not apply to UTRA TDD |  |
|  |  | 703 - 748 MHz |  | -49 MHz |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in band 28, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in Band 44. This requirement does not apply to UTRA TDD For E-UTRA BS operating in Band 67, it applies for 703 MHz to 736 MHz. For E-UTRA BS operating in Band 68, it applies for 728MHz to 733 MHz. |  |
| E-UTRA Band 29 |  | 717 - 728 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA TDD. This requirement does not apply to E-UTRA BS operating in Band 29 |  |
| E-UTRA Band 30 |  | 2350 - 2360 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA TDD. This requirement does not apply to E-UTRA BS operating in band 30 or 40. |  |
|  |  | 2305 - 2315 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA TDD . This requirement does not apply to E-UTRA BS operating in band 30, since it is already covered by the requirement in subclause 6.6.6.5.2.4. This requirement does not apply to E-UTRA BS operating in Band 40. |  |
| E-UTRA Band 31 |  | 462.5 -467.5 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA TDD . This requirement does not apply to E-UTRA BS operating in band 31. |  |
|  |  | 452.5 -457.5 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA TDD. This requirement does not apply to E-UTRA BS operating in band 31, since it is already covered by the requirement in subclause 6.6.6.5.2.4. |  |
| UTRA FDD Band XXXII or E-UTRA Band 32 |  | 1452 - 1496 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA BS operating in Band XI, XXI, or XXXII This requirement does not apply to UTRA TDD This requirement does not apply to E-UTRA BS operating in band 11, 21 or 32. |  |
| UTRA TDD in Band a) or E-UTRA Band 33 |  | 1900 - 1920 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in Band 33. |  |
| UTRA TDD in Band a) or E-UTRA Band 34 |  | 2010 - 2025 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in Band 34. |  |
| UTRA TDD Band b) or E-UTRA Band 35 |  | 1850 - 1910 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in Band 35. |  |
| UTRA TDD Band b) or E-UTRA Band 36 |  | 1930 - 1990 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in Band 2 and 36. |  |
| UTRA TDD Band c) or E-UTRA Band 37 |  | 1910 - 1930 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA BS. This is not applicable to E-UTRA BS operating in Band 37. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |  |
| UTRA TDD in Band d) or E-UTRA Band 38 |  | 2570 - 2620 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to E-UTRA BS operating in Band 38. |  |
| UTRA TDD in Band f) or E-UTRA Band 39 |  | 1880 - 1920 MHz |  | -52 dBm |  | 1 MHz |  | Applicable in China for UTRA FDD. This is not applicable to E-UTRA BS operating in Band 39. |  |
| UTRA TDD in Band e) or E-UTRA Band 40 |  | 2300 - 2400 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to E-UTRA BS operating in Band 30 or 40. |  |
| E-UTRA Band 41 |  | 2496 - 2690 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to E-UTRA BS operating in Band 41. |  |
| E-UTRA Band 42 |  | 3400 - 3600 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to E-UTRA BS operating in Band 22, 42 or 43. |  |
| E-UTRA Band 43 |  | 3600 - 3800 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA TDD. This is not applicable to E-UTRA BS operating in Band42 or 43. |  |
| E-UTRA Band 44 |  | 703 - 803 MHz |  | -52 dBm |  | 1 MHz |  | This is not applicable to E-UTRA BS operating in Band 28 or 44 |  |
| E-UTRA Band 45 |  | 1447 - 1467 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA BS. This is not applicable to E-UTRA BS operating in Band 45 |  |
| E-UTRA Band 46 |  | 5150 - 5925 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA BS. This is not applicable to E-UTRA BS operating in Band 46. |  |
| E-UTRA Band 65 |  | 2110 - 2200 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in band 1 or 65, |  |
|  |  | 1920 - 2010 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in band 65, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For E-UTRA BS operating in Band 1, it applies for 1980 MHz to 2010 MHz, while the rest is covered in subclause 6.6.6.5.2.4. |  |
| E-UTRA Band 66 |  | 2110 - 2200 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in band 4, 10, 23 or 66. |  |
|  |  | 1710 - 1780 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in band 66, since it is already covered by the requirement in subclause 6.6.4.5.3. For E-UTRA BS operating in Band 4, it applies for 1755 MHz to 1780 MHz, while the rest is covered in subclause 6.6.6.5.2.4. For E-UTRA BS operating in Band 10, it applies for 1770 MHz to 1780 MHz, while the rest is covered in subclause 6.6.6.5.2.4. |  |
| E-UTRA Band 67 |  | 738 - 758 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in Band 28 or 67. |  |
| E-UTRA Band 68 |  | 753 -783 MHz |  | -52 dBm |  | 1 MHz |  | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in band 28, or 68. |  |
|  |  | 698-728 MHz |  | -49 dBm |  | 1 MHz |  | This requirement does not apply to UTRA BS. This requirement does not apply to E-UTRA BS operating in band 68, since it is already covered by the requirement in subclause 6.6.6.5.2.4. For E-UTRA BS operating in Band 28, it applies between 698 MHz and 703 MHz, while the rest is covered in subclause 6.6.6.5.2.4. |  |
| NOTE 1: The co-existence requirements do not apply for the 10 MHz frequency range immediately outside the downlink operating band (see subclause 4.5.). Emission limits for this excluded frequency range may be covered by local or regional requirements. NOTE 2: The table above assumes that two operating bands, where the frequency ranges would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |  |  |  |  |  |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, except for the cases where the noted requirements apply to a BS operating in Band 25 or Band 29, the co-existence requirements in table 6.6.6.5.2.5-1do not apply for the 10 MHz frequency range immediately outside the downlink operating band (see tables 4.4-1 and 4.4-2). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.6.6.5.2.5-1assumes that two operating bands, where the frequency ranges in table 4.4-1 or table 4.4-2 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	For the protection of DCS1800, UTRA Band III or E-UTRA Band 3 in China, the frequency ranges of the downlink and uplink protection requirements are 1805 - 1850 MHz and 1710 - 1755 MHz respectively.

NOTE 4:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 5:	For Band 28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with Band 27 UL operating band.

NOTE 6:	For Band 29 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with UTRA Band XII or E-UTRA Band 12 UL operating band or E-UTRA Band 17 UL operating band.

The following requirement may be applied for the protection of PHS. This requirement is also applicable at specified frequencies falling between 10 MHz below the lowest BS transmitter frequency of the downlink operating band and 10 MHz above the highest BS transmitter frequency of the downlink operating band.

The basic limit for any spurious emission shall not exceed.

Table 6.6.6.5.2.5-2: Spurious emissions limits for co-existence with PHS

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 1884.5 - 1915.7 MHz | -41 dBm | 300 kHz | Applicable for co-existence with PHS system operating in 1884.5-1915.7MHz |
| NOTE: The requirement is not applicable in China. |  |  |  |

--------- end of change ----------