3GPP TSG-RAN WG4 Meeting RAN4#90                                                                                                 R4-1902382

Athens,Greece, 25 Feburary -1 March, 2019

Agenda Item:   6.14.1

Source:        Samsung

Title:          Ad hoc minutes for UE demodulation and CSI requirements

Document for:  Approval

Ad-hoc attendees: [Samsung, Intel, Huawei, Qualcomm, MTK, Ericsson, CATT, NTT DoCoMo, CMCC, ANRITSU , Keysight]

Information (this week NR UE performance handling arrangement):

- Already allocated Ad-hoc

  - 1st round ad-hoc:

    - Monday afternoon Ad-hoc session (Theta) 17:00~21:00

    - Tuesday Morning Ad-hoc session (Theta)  8:30~13:00

    - Try to review all the open issues and assign working arrangement for offline

  - 2nd round ad-hoc: Wednesday morning Ad-hoc session (Theta) 8:30 ~13:00

    - Treat WF and revised draft CRs

- On-Line session (Chaired by RAN4 chair)

  - 1st round on-line Thursday 8:30 – 13:00 (Theta)

  - 2nd round on-line Friday Morning 8:30~10:00 Omikron 1

- Intel leading offline discussion for generic parameters and PDSCH

- NTT DoCoMo leading offline discussion for SDR test cases

- Ericsson leading offline discussion for PDCCH test cases

- CMCC leading offline discussion for PBCH test cases

- Samsung Leading offline discussion for CSI test cases

# Rel-15 NR UE performance scope (6.1.4)

List of contributions:

| R4-1900121 | Scope of Rel-15 NR UE Demodulation/CSI requirements | Intel Corporation |
| --- | --- | --- |
| R4-1900313 | Rel-15 NR UE performance requirements work scope summary | Samsung |
| R4-1901015 | Test scope of NR UE demodulation in Rel-15 | NTT DOCOMO, INC. |
| R4-1901806 | Discussion on Rel-15 RAN4 demodulation/CSI scope | Huawei, HiSilicon |

Proposals:

| Intel | Proposal #1: Prioritize the work on the completion of the existing open issues identified in Section 2.2. Proposal #2: Extend the inter-working requirements scenarios coverage for NE-DC cases. Proposal #3: Do not introduce any further requirements for the optional features, deprioritized features or for additional scenarios in R15 scope on top of currently agreed set of requirements. |
| --- | --- |
| Huawei | Proposal 1: No new UE demodulation or CSI requirement, which was not discussed in the previous meetings, should be introduced in Rel-15. - The proposal for new requirement, which was under discussion but no decision was made in the previous meetings, can be discussed further. |
| Samsung | Proposal 1: Introducing CA requirements (normal test cases) under Rel-16. Proposal 2: Performance requirements for Rel-15 late drop (NR-NR DC, NE-DC) can be discussed after 2019 June |

Discussion：

Intel: what’s the existing scope?

HST test for single tap belongs to Rel-15 scope or not?

NTT DoCoMO/Huawei: We think should belong to Rel-15.

Agreement:

No new UE demodulation or CSI requirement, which was not discussed in the previous meetings, should be introduced in Rel-15.

  - The proposal for new requirement, which was under discussion but no decision was made in the previous meetings, can be discussed further.

For Rel-15 late drop (NR-NR DC and NE DC) will be further discussed after June 2019 with target timeline of Dec 2019.

TDD DL-UL configuration already agreed with 2nd priority -> further discuss under PDSCH demod part

- Huawei/Intel: we already down-selection among them

- NTT DoCoMo/Ericsson: 6 TDD DL-UL, rest of 3 not introduced

# General (1 hour)

List of contributions

| R4-1900051 | Applicability of Requirements and General Parameters for NR Demodulation Performance Tests | Qualcomm Incorporated |
| --- | --- | --- |
| R4-1900084 | Editorial cleanup of FR2 Radiated Requirements General section | ANRITSU LTD |
| R4-1900085 | Editorial cleanup of FR2 Radiated Requirements General section | ANRITSU LTD |
| R4-1900122 | SNR, Es and Noc setup for NR FR1 and FR2 UE performance requirements | Intel Corporation |
| R4-1900365 | NR UE demodulation and CSI reporting requirements applicability | Intel Corporation |
| R4-1900366 | Draft CR on NR UE demodulation requirements applicability | Intel Corporation |
| R4-1900419 | Draft CR on General Applicability of Requirements | Qualcomm Incorporated |
| R4-1901559 | DL power allocation for NR UE requirements | Intel Corporation |
| R4-1901561 | Draft CR of DL power allocation | Intel Corporation |

## Es, SNR ,Noc  and DL power allocation

Proposals from companies

| Companies | Proposals |
| --- | --- |
| Intel | Proposal #1: Further study how to ensure minimum SNR degradation due to UE RF noise - Option 1: Use band-agnostic high Noc value - Option 2: Adjust the SNR to compensate the degradation - Option 3: Use a per-band variable Noc level in a way to ensure a fixed SNR error Proposal #2: Further study how to minimize UE RF noise impacts for noise-free conditions - Option 1: Use per-band variable Es level in a way to ensure that effective SNR > [35] dB - Option 2: Use a band-agnostic high Es power level Proposal #3: For FR2 Noc setup for multi-band devices increase the Noc power level by ΣMBP defined in TS 38.101-2 Table 6.2.1.3-4. |
| Intel | Proposal #1: Define “EPRE ratio of PBCH/PDCCH/PDSCH to SSS” instead of “EPRE ratio of PBCH_DMRS/PDCCH_DMRS/PDSCH_DMRS to SSS” for NR DL power allocation configuration. |

### Issue 1: Noc Level for FR1

Previous agreement: band agnostic as [-142dBm/Hz] in spec

Options for discussion: (Intel)

Further study how to ensure minimum SNR degradation due to UE RF noise

- Option 1: Use band-agnostic high Noc value ->?

  - Option 1a: [-142 dBm/Hz]

  - Option 1b: -139 dBm/Hz

- Option 2: Adjust the SNR to compensate the degradation

- Option 3: Use a per-band variable Noc level in a way to ensure a fixed SNR error

QC: prefer op3, op1 require to increase Noc for all bands which have impact on SNR range

R&S: op3 will come to per UE depending since REFSENS depending on UE supporting for multi-band combination conditions

QC: The equation is clear.

Huawei: For REFSENS, for different SCS/BW, it’s different.

R&S: value still in the range, maximum Noc increased, maximum SNR maybe achievable. Maximum input level for UE has limitation to -25dBm in total.

Further discussion above option 3 with details and checking with TE vendors for the feasibility.

### Issue 2: Es Level for noise free condition in FR1

Options for discussion:

- Option 1: Use per-band variable Es level in a way to ensure that effective SNR > [35] dB

- Option 2: Use a band-agnostic high Es power level

  - Es = -120 dBm/Hz

QC: prefer op1, to ensure achieve high SNR upper bound

Intel: based on REFSENS to generate Es with offset

Further discussion above option 1 with details and checking with TE vendors for the feasibility.

### Issue 3: Noc for multi-band supporting in FR2

Options for discussion

- Option 1: For FR2 Noc setup for multi-band devices increase the Noc power level by ΣMBP defined in TS 38.101-2 Table 6.2.1.3-4. (Intel)

For FR2 Noc setup for multi-band devices increase the Noc power level by ΣMBP defined in TS 38.101-2 Table 6.2.1.3-4.

Ericsson: how to choose this ΣMBP, upper bound?

Intel: using upper bound.

Issue 4：DL Power allocation setting

Previous agreement:

- EPRE of PBCH_DMRS, PDCCH_DMRS, PDSCH_DMRS, PSS, and NZP-CSI-RS are set relative to SSS EPRE

- EPRE of physical channels (PBCH, PDCCH, PDSCH) are set relative to the EPRE of associated DMRS (e.g., PDSCH to PDSCH_DMRS)

Candidate option:

- Option 1: SSS EPRE->DMRS->Physical channel (Previous agreement)

- Option 2:  Define “EPRE ratio of PBCH/PDCCH/PDSCH to SSS” instead of “EPRE ratio of PBCH_DMRS/PDCCH_DMRS/PDSCH_DMRS to SSS” (Intel)

Need to align among UE RF,RRM and Demod specifications for power allocation specified in Annex

No objection in Ad-hoc for op2 approach, further discussed in this week

## Requirements applicable rules

| Companies | Proposals |
| --- | --- |
| Qualcomm | Proposal 1: All minimum performance requirements defined in Sections 5-8 are applicable to both SA and NSA unless otherwise explicitly stated in Section 9 and 10. Proposal 2: All minimum performance requirements defined in Sections 5-10 are applicable to all UE power classes unless otherwise stated. Proposal 3: If maximum achievable SNR in the TE chamber for certain test conditions is less than the defined SNR requirement for those tests, those tests will not be tested. Proposal 4: Use the following values for missing test parameters: - Number of PDCCH candidates = 1 - Slots for PDCCH monitoring = each slot |
| Intel | Proposal #1: Specify FR2 frequency band applicability rules in the TS 38.101-4 Proposal #2: Further clarify in TS 38.101-4 that for FR2 requirements “In case the required SNR is larger than the SNR upper bound that can be emulated by test system, the corresponding requirement can currently not be tested.” Proposal #3: Explicitly specify the WI code in the title of the requirements section and applicability section or at least in applicability section in the TS 38.101-4. |

Draft CRs

| R4-1900366 | Draft CR on NR UE demodulation requirements applicability | Intel Corporation |
| --- | --- | --- |
| R4-1900419 | Draft CR on General Applicability of Requirements | Qualcomm Incorporated |

### Issue 1: Requirements applicable rules for SA and NSA

Previous agreements:

- For SA/NSA Normal demodulation performance requirements only verified NR carrier requirements, additional EN-DC specific requirements can be further discussed after normal requirements finalized.

- For EN-DC Normal demodulation performance requirements and CSI requirements:

  - Only verify NR carrier requirements

  - Reusing same test parameters and requirements for NR carrier under SA and NSA mode

Proposals:

- Proposal 1: All minimum performance requirements defined in Sections 5-8 are applicable to both SA and NSA unless otherwise explicitly stated in Section 9 and 10. (QC)

Intel： this maybe already captured in related sections.

- Proposal 2: All minimum performance requirements defined in Sections 5-10 are applicable to all UE power classes unless otherwise stated. (QC)

Capture below agreements into specification:

- Proposal 1: All minimum performance requirements defined in Sections 5-8 are applicable to both SA and NSA unless otherwise explicitly stated in Section 9 and 10.

- Proposal 2: All minimum performance requirements defined in Sections 5-10 are applicable to all UE power classes unless otherwise stated.

### Issue 2: Test applicability in FR2 considering TE achievable SNR limitation

Proposal: Further clarify in TS 38.101-4 that for FR2 requirements “In case the required SNR is larger than the SNR upper bound that can be emulated by test system, the corresponding requirement can currently not be tested.”  (Intel, QC)

R&S: value captured in

Intel: Achievable SNR maybe pending on methods and different test cases.

Keysight: Values maybe different pending on test methods.

QC: Assume worst cases? Value can be different pending bands

R&S: Do we need to some clarification for this?

Open issue: How to judge SNR upper bound?

Potential agreement:

Further clarify in TS 38.101-4 that for FR2 requirements “In case the required SNR is larger than the SNR upper bound that can be emulated by test system, the corresponding requirement can currently not be tested.”

- Note: wording need to be further discussed.

Open issues: Testable SNR upper bound?

- Op1: declaration based on TE , at least no worse than values captured in TR 38.810

- Op2: Values capture TR 38.810

How to deal with PC, multi-band and CA supporting in TR 38.810?

### Issue 3: Requirements applicable rules for FR2 Bands

Proposal #1:	Specify FR2 frequency band applicability rules in the TS 38.101-4 (Intel)

Huawei: in future, if new frequency band proposed above 40GHz, how to deal?

Intel: we need to do some analysis work case by case; for high order modulation test cases.

Further discussion for the applicable rules in future if high frequency bands above 40GHz introduced in core specification.

Specify FR2 frequency band applicability rules in the TS 38.101-4

### Issue 4：WI handling

Intel: Explicitly specify the WI code in the title of the requirements section and applicability section or at least in applicability section in the TS 38.101-4.

Further offline

# PDSCH (1 hour)

## Normal PDSCH test cases

| R4-1900236 | Updated PDSCH simulation and impairment results | CMCC |  |
| --- | --- | --- | --- |
| R4-1900048 | Views on PDSCH Demodulation Performance Tests | Quaclomm | Proposal 1: Define at least one PDSCH demodulation performance test each for FR1 and FR2 with DCI based dynamic TDD configuration determination. Proposal 2: Do not define PDSCH or SDR test cases with 8 (FDD) / 16 (TDD) HARQ processes except 30% TP test cases. Proposal 3: Include BW/SCS information in the tables for minimum performance requirements instead of tables for Test Parameters. Proposal 5: Use the following values for missing PDSCH and SDR test parameters: - Number of PDCCH candidates = 1 - PDCCH aggregation level = 8 - PDCCH DCI format = 1-0 - K1 for FDD test cases = 2 - Minimum TB Success Rate duration in SDR tests = 3000ms (same as LTE) - HARQ ACK/NACK Bundling = Multiplexed - Slots for PDCCH monitoring (FR2) = each slot - TCI state for SSB: Parameter; TCI State 0; TCI State 1 (TRS); TCI State 2 (NZP CSI-RS). tci-StateId; Id0; Id1; Id2. qcl-Type1; typeC; typeA; typeA. qcl-Type2; typeD; typeD; typeD. referenceSignal; SSB0; CSI-RS0; CSI-RS1. - QCL Info for TRS and NZP CSI-RS: qcl-InfoPeriodicCSI-RS = Id0 |
| R4-1900358 | NR PDSCH UE demodulation requirements | Intel Corporation | Proposal #1: Do not define PDSCH requirements with 70% test point and 16 HARQ processes for TDD and 8 HARQ processes for FDD. Proposal #2: Use TCI configuration from Table 1 to define Rel-15 NR PDSCH requirements. Table 1. TCI configuration for Rel-15 PDSCH tests TCI parameters; FR1 tests; FR2 tests. TCI state #1; QCL info Type 1; Reference signal; Index of NZP CSI RS resource from TRS. QCL type; Type A. QCL info Type 2; Reference signal; N/A; Index of NZP CSI RS resource from TRS. QCL type; N/A; Type D. Proposal #3: Use 2 slots TRS configuration for Rel-15 Normal and SDR requirements |
| R4-1900359 | NR PDSCH simulation results | Intel Corporation |  |
| R4-1900360 | Summary of NR PDSCH demodulation simulation results (FR1 FDD) | Intel Corporation |  |
| R4-1900361 | Summary of NR PDSCH demodulation simulation results (FR1 TDD) | Intel Corporation |  |
| R4-1900362 | Summary of NR PDSCH demodulation simulation results (FR2) | Intel Corporation |  |
| R4-1900363 | NR single-tap HST requirements | Intel Corporation | Proposal #1: Use LTE Rel-8 single tap HST channel model to define Rel-15 NR demodulation requirements. Proposal #2: Define only Normal PDSCH requirements for Rel-15 NR HST requirements. Proposal #3: Define minimum PDSCH performance for Rel-15 HST scenarios under assumption that UE uses only TRS signals for CFO tracking. Proposal #4: Define Rel-15 HST PDSCH requirements under the following conditions: - Channel bandwidth and SCS: - FDD: 10 MHz, SCS 15 kHz - TDD: 40 MHz, SCS 30 kHz - PDSCH mapping: Type A, Start symbol 2, Duration 12 - DMRS configuration: Type 1, Single symbol, 1 additional DMRS - TDD UL-DL pattern: 7D1S2U, S = 6D:4G:4U - TRS configuration: 10ms, 2 slots, offset 1 - FRC: QPSK Rank 1 (MCS 4) |
| R4-1900601 | Views on HST tests | NTT DOCOMO, INC. | Proposal: For FR1, HST tests should be introduced that is at least equivalent to LTE. Proposal: Introduce HST test for Rel. 15 NR downlink with the same model as in LTE and following parameters. Parameter; Value. Ds; 300 m. Dmin; 2 m. v; 350 km/h. fd; 875 Hz for 15 kHz SCS test and 1167 Hz for 30 kHz SCS test. Observation: Necessity of HST-SFN test can be studied in future releases |
| R4-1900602 | Simulation results for normal PDSCH demodulation test | NTT DOCOMO, INC. |  |
| R4-1900866 | Views on NR PDSCH demodulation performance requirement | MediaTek inc. | Proposal 1: Introduce TDL-D and TDL-E channel in PDSCH test case for both FR1 and FR2. Proposal 2: Introduce PDSCH demodulation test with 2 TCI states. The performance can be the same as 1 TCI state by using the same DL TX and channel settings for the 2 TCI states and 2 TCI states are only differentiated by time offset. Proposal 3: Introduce PDSCH demodulation test with more than 1 AoA in Release 16 for FR2. |
| R4-1900867 | NR PDSCH simulation result | MediaTek inc. |  |
| R4-1901017 | Remaining issues on PDSCH requirements | NTT DOCOMO, INC. | Proposal 1: For CSI-RS for UE Rx beam refinement, density of 3 shall be used. Proposal 2: For FR2 test cases, - TRS is associated with SSB with respect to QCL-type C and D (RRC). - Regarding TCI indication for PDSCH DM-RS, we propose two options with our preference on option 2. - Option 1: PDSCH DM-RS is associated with TRS with respect to QCL-types A and D (RRC+MAC CE). - Option 2: UE reuses TCI information for PDCCH DM-RS (RRC + MAC CE) for PDSCH demodulation. Proposal 3: For FR1 test cases, reuse TCI indication for FR2 by removing QCL-type D. Proposal 4. To cover the mandated number of analogue beams, i.e. 64, change the current TRS configuration to “20ms with 1 slot and OFDM symbol 6, 10” in one normal FR2 PDSCH demodulation test with TDD configuration of {DDDSU} for each Rx port. Proposed test cases are as follows: - 1Rx port: FFS (currently the section is void) - 2Rx port: Test case 1-1 in the Table 7.2.2.2.1-3 Proposal 5. Apply 8/16 HARQ processes for SDR requirements for FDD/TDD. |
| R4-1901018 | TDD configuration for NR PDSCH requirements | NTT DOCOMO, INC. | Observation 1: For TDD configuration {DDDSU} of 120kHz SCS, the K1 values could be slightly changed as follows (highlighted in red) to have wider testing coverage. Note that this modification doesn’t have impact on the PDSCH demodulation performance, so the re-simulation is not needed. K1 = [4] if mod(i,5) = 0 K1 = [3] if mod(i,5) = 1 K1 = [2] if mod(i,5) = 2 K1 = [5] if mod(i,5) = 3 Proposal 1: For 30kHz SCS and {SU} with S = {12D, 2G}, apply K1 values = {3} and 8 HARQ process. Proposal 2: For 120kHz SCS and {DSUU}with S = {12D:2G}, apply K1 values = {3, 5} and 10 HARQ process. |
| R4-1901342 | Simulation results for NR UE PDSCH demodulation tests | Ericsson |  |
| R4-1901768 | NR PDSCH Demodulation Performance Simulation Results | Qualcomm Incorporated |  |
| R4-1901807 | Discussion on dynamic TDD UL/DL configuration test | Huawei, HiSilicon | Proposal 1: Define the RAN4 performance requirements definition for dynamic TDD UL/DL configuration in Rel-16 CLI WI. Proposal 2: Limit the dynamic TDD UL/DL configuration test to FR2 only if operator has strong concern on it. |
| R4-1901808 | Discussion on K1 values selection for NR PDSCH performance tests | Huawei, HiSilicon | Proposal 1: Use maximum 8 HARQ processes and K1 = {1, 2, 3, 4} for UL/DL configuration {DDDSU} with 15kHz subcarrier spacing. Proposal 2: Use K1 = {2, 3, 4, 5, 6, 7, 8} for UL/DL configuration {DDDDDDDSUU} with 30kHz subcarrier spacing for both 8 and 16 HARQ processes. Proposal 3: Use K1 = {1, 2, 3, 4} for UL/DL configuration {DDDSUDDSUU} with 30kHz subcarrier spacing and 8 HARQ processes. Proposal 4: Use K1 = {1, 2, 3, 4} for UL/DL configuration {DDDSU} with 30kHz subcarrier spacing and 8 HARQQ processes. Proposal 5: Use K1 = {2, 3, 5} for UL/DL configuration {DDSU} with 60kHz subcarrier spacing and 10 HARQ processes. Proposal 6: Use K1 = {2, 3, 4, 6} for UL/DL configuration {DDDSU} with 120kHz subcarrier spacing for both maximum 8 and 16 HARQ processes. Proposal 7: Use K1 = {2, 3, 5} for UL/DL configuration {DDSU} with 120kHz subcarrier spacing and 10 HARQ processes. |
| R4-1901809 | Simulation results for NR PDSCH performance requirements | Huawei, HiSilicon |  |
| R4-1901341 | NR UE performance test open issues | Ericsson | Proposal 1: Adapt some/all RAN4 UE PDSCH demodulation tests with TDD 30kHz with the new alternative config Option 2. No new simulation is needed for this alternative config. A draft CR is prepared in [2]. Proposal 2: It’s OK to use the same approach not to model phase noise for 64QAM tests, based on the condition that only the lowest possible MCSs will be used for 64QAM tests. Proposal 3: Specify additional LTE-NR coexistence tests with the correct test config. Collect alignment results. |

Open issues list:

- Additional soft buffer test for PDSCH mapping type A

- Dynamic TDD DL-UL configuration test

- HST test

- TCI and QCL configurations

- Other missing parameters

- Simulation results and SNR requirements derivation

### Additional soft buffer test for PDSCH mapping type A

Previous agreements:

- FFS whether to introduce additional requirement with 70% test point with 8 HARQ process for FDD and 16 HARQ process for TDD

  - Option 1: No Test since soft buffer test has been defined (Intel, QC)

  - Option 2: Introduce PDSCH demodulation tests with 70% test point with 8 HARQ processes for FDD and 16 HARQ processes for TDD

Candidate options for discussion:

- Option 1: No Test (Intel, Qualcomm)

- Option 2:  Apply 8/16 HARQ processes for SDR requirements for FDD/TDD. (NTT DoCoMo)

NTT DoCoMo: intention to verify soft buffer processing, in additional this purpose verification for performance of soft combing.

Huawei:  Simulation already done for both options

Intel: from performance, no performance among options. Soft combing behavior belongs to UE implementation, which cannot be verified. In 70%, no much retransmission. For SDR, most likely no retransmission, with 85% or high relative TP

Huawei: for high Doppler shift, we may observe performance difference considering time diversity.

QC: Same view as Intel, no performance gain meantime increase latency with larger number of HARQ process

NTT DoCoMo: From performance aspect, no difference. Our concern for test coverage of combination of high MCS + high HARQ +70% TP.  If UE only support upper 8 in such condition and NW may suffer scheduling limitation.

Further offline for above options

### Dynamic TDD DL-UL configuration test

Previous agreements:

- Way forward

  - Option 1: Use dynamic UL/DL determination for some existing PDSCH demodulation test case(s).

  - Option 2: Do not define test

- Next steps

  - Further make downselection in RAN4 #90

Candidate options for discussion:

- Option 1: Use dynamic UL/DL determination for some existing PDSCH demodulation test case(s)-both FR1 and FR2 (Qualcomm,Ericsson, NTT DoCoMo)

- Option 2: Use dynamic UL/DL determination for some existing PDSCH demodulation test case(s)-  FR2 only (Huawei, MTK)

- Option 3: Define the RAN4 performance requirements definition for dynamic TDD UL/DL configuration in Rel-16 CLI WI. (Huawei)

QC: If we limit to FR2, we will test coverage issue for UE which only supporting FR1.

Huawei: From feature list, it’s mandatory meanwhile not clear for FR1 considering inter-interference coordination issue has not resolved in ran1 in Rel-15 timeframe. There is a new Rel-16 on going WI to handle interference issue.

Ericsson: We should not mixed with CRL WI. The purpose is clear to ensure UE mornitor DL/UL per TTI.

TDD DL-UL configuration is fixed in test; just indicate through DCI other than RRC. We can clarification test purpose for that.

NTT DoCoMo: we support Op1, use case is isolated cell which can be used in both FR1 and FR2.

Further offline for below options:

- Option 1: Use dynamic UL/DL determination for some existing PDSCH demodulation test case(s)-both FR1 and FR2 (Qualcomm, Ericsson, NTT DoCoMo)

- Option 2: Use dynamic UL/DL determination for some existing PDSCH demodulation test case(s)-  FR2 only (Huawei, MTK)

- Option 3: Define the RAN4 performance requirements definition for dynamic TDD UL/DL configuration in Rel-16 CLI WI. (Huawei)

### HST Test

Previous agreement：

RAN4 #88 Agreement: Use HST model as described on section B3 TS36.101 for FR1 for initial simulation purposes (max Doppler shift specified in table) - FFS introduction of new requirements - This topic will be discussed in agenda for PDSCH demodulation and other related agendas in the next meeting. RAN #81 The UE and BS demodulation performance requirements under HST scenarios are treated in TEI15 after December 2018.

Issue 1: Whether introduce HST test in Rel-15?

- Introduce HST for FR1  in Rel-15 for single tap channel model as specified in Section B3 TS36.101

  - For normal PDSCH demodulation test case(s) only

Issue 2:  Maximum Doppler shift

- FDD 15kHz

  - Option 1: Same as LTE, 750Hz (Intel, QC, Ericsson,Huawei)

  - Option 2: 875 Hz (NTT DoCoMo, MTK)

- TDD 30kHz

  - Option 1: Same as LTE, 750Hz (Intel, QC, Ericsson,Huawei)

  - Option 2: 1167 Hz (NTT DoCoMo, MTK)

Huawei: for UL, no conclusion for maximum Doppler shift values

NTT: We have performance gap among DL and UL. Target 350km/h, two test cases for 15kHz and 30kHz with different values.

Ericsson: what’s frequency bands?

NTT:  For FDD same as LTE, For TDD as 3.6GHz.

MTK: for both FDD(15kHz) and TDD (30kHz) ?

Intel: Yes

Further discussion below options for maximum Doppler shift:

- FDD 15kHz

  - Option 1: Same as LTE, 750Hz (Intel, QC, Ericsson)

  - Option 2: 875 Hz (NTT DoCoMo, MTK)

- TDD 30kHz

  - Option 1: Same as LTE, 750Hz (Intel, QC, Ericsson)

  - Option 2: 1167 Hz (NTT DoCoMo, MTK)

Issue 3: Detailed test configuration for simulation purpose

- CHBW &SCS:

  - FDD: 15kHz +10MHz

  - TDD:30kHz+40MHz, FR1.30-1 (7D1S2U)

- PDSCH scheduling

  - Type A with front symbol 2

  - DMRS pattern:

    - Option1:1 additional DMRS

    - Option2:2 additional DMRS

- MCS

  - Option 1:QPSK and rank1

  - Option 2:16QAM and rank1

- TRS configuration:

  - Option1: 10ms periodicity, offset 1, 2 slots

  - Option2: 20ms periodicity, offset 10,2 slots

- Baseline receiver assumption for frequency offset tracking (for initial simulation purpose only)

  - Option1: TRS based on tracking

  - Option2: SSB+TRS based on tracking

    - SSB with 20ms periodicity

### TCI and QCL configurations

Previous agreements:

- QCL behaviour Type A and  TypeD (FR2 only)

Candidate options:

Intel: TRS and PDSCH

| TCI parameters |  |  | FR1 tests | FR2 tests |
| --- | --- | --- | --- | --- |
| TCI state #1 | QCL info Type 1 | Reference signal | Index of NZP CSI RS resource from TRS |  |
|  |  | QCL type | Type A |  |
|  | QCL info Type 2 | Reference signal | N/A | Index of NZP CSI RS resource from TRS |
|  |  | QCL type | N/A | Type D |

Qualcomm:

- TCI state for SSB:

| Parameter | TCI State 0 | TCI State 1 (TRS) | TCI State 2 (NZP CSI-RS) |
| --- | --- | --- | --- |
| tci-StateId | Id0 | Id1 | Id2 |
| qcl-Type1 | typeC | typeA | typeA |
| qcl-Type2 | typeD | typeD | typeD |
| referenceSignal | SSB0 | CSI-RS0 | CSI-RS1 |

- QCL Info for TRS and NZP CSI-RS: qcl-InfoPeriodicCSI-RS = Id0

NTT DoCoMo:

Proposal 2: For FR2 test cases,

- TRS is associated with SSB with respect to QCL-type C and D (RRC).

- Regarding TCI indication for PDSCH DM-RS, we propose two options with our preference on option 2.

  - Option 1: PDSCH DM-RS is associated with TRS with respect to QCL-types A and D (RRC+MAC CE). (Huawei)

  - Option 2: UE reuses TCI information for PDCCH DM-RS (RRC + MAC CE) for PDSCH demodulation.  (NTT DoCoMO,Intel)

Proposal 3: For FR1 test cases, reuse TCI indication for FR2 by removing QCL-type D.

Open issues:

Further discuss for below issues:

TCI configuration for TRS

TCI configuration for NZP CSI-RS

TCI configuration for DMRS of PDCCH

Whether TCI needed for DMRS of PDSCH

  - Option 1: PDSCH DM-RS is associated with TRS with respect to QCL-types A and D (RRC+MAC CE).

  - Option 2: UE reuses TCI information for PDCCH DM-RS (RRC + MAC CE) for PDSCH demodulation.  (NTT DoCoMO, Intel)

TCI configuration for NZP CSI-RS acquisition

TCI configuration for CSI-RS configured for Rx beam refinement purpose

Whether TCI needed for FR1

### TRS configurations for FR2

Previous agreements:

- Option 1: Do not change TRS configuration

- Option 2: 20ms with 1 slot duration and OFDM symbol indexes 5, 9

- Option 3: 40ms with 1 slot duration

- Other options are not precluded

- TRS configuration change can be applicable for one of test cases. Exact test cases is FFS.

- FFS whether to introduce configuration for normal demodulation or SDR test cases

Options: Further offline

- Option 1: No change: Use 2 slots TRS configuration for Rel-15 Normal and SDR requirements (Intel)

- Option 2: Replace FR2 Test 1-1 with TRS configuration: 20ms with 1 slot and OFDM symbol 6, 10 (NTT DoCoMo)

Intel: New simulation required, no 64 TCI state configured.  This already covered by REFSENS test.

NTT DoCoMo: no test for 64 TCI state configurations. 1 slot, TRS 10ms in REFSENS test

Intel: Do we need to configure 64 TIC states in test cases?

### Others

Issue 1: Common test parameters for PDSCH and CSI test cases

Proposals from Qualcomm

- Number of PDCCH candidates = 1

- PDCCH aggregation level = 8

- PDCCH DCI format =

  - Option1:1-0

  - Option2:1-1

NTT DoCoMo: Actual value in CORESET or ..

Ericsson：Do we need this in RAN4? In LTE phase these configuration belong to RAN5

- FFS：Minimum TB Success Rate duration in SDR tests whether needed in RAN4

- K1 for FDD test cases = 2

- HARQ ACK/NACK Bundling = Multiplexed

- Slots for PDCCH monitoring (FR2) = each slot

Issue 1: Common test parameters for PDSCH (Normal and SDR) and CSI test cases

- Number of PDCCH candidates = 1

- PDCCH aggregation level = 8

- PDCCH DCI format

  - Option1:1-0

  - Option2:1-1

- FFS：Minimum TB Success Rate duration in SDR tests whether needed in RAN4

- K1 for FDD test cases = 2

- HARQ ACK/NACK Bundling = Multiplexed

- Slots for PDCCH monitoring (FR2) = each slot

Issue 2: K1 values

NTT DoCoMo: Revised K1 for FR2.120-1:

K1 = [4] if mod(i,5) = 0
K1 = [3] if mod(i,5) = 1
K1 = [2] if mod(i,5) = 2
K1 = [6] if mod(i,5) = 3     K1 = [5] if mod(i,5) =3

Taking offline with Intel, Huawei, NTT DoCoMo

Issue 3: TDD DL-UL pattern for FR1 30kHz

- Option 1. {DDDDDDDSUU} with 3ms shift (Adapted as 3GPP RAN4 config)

- Option 2. {DDDSUU} + {DDDD} with 0ms shift (Alternative config for RAN4 tests)->Ericsson proposal

Ericsson: Adopted option2 for some/all test requirements

Intel: How to capture in specification? since will have different configurations; LTE also need to be configured in different ways

Huawei: Based on agreement, this is new TDD DL-UL pattern, first time proposed. But we already agreed the scope of Rel-15 for TDD DL-UL pattern in previous meeting.

Ericsson: we already provide CRs, no impact on UE implementation with this modification. For NW perspective, this alternative way may simple. This is not new pattern, no additional simulation work. CR is co-signed by KDDI, NTT DoCoMo and Softbank.

Intel: no concern for the pattern. Concern for the consistent if we only change part of test cases either apply for all test cases or no change.

Further offline for potential impact with op2 and the benefit of op2.

Issue 4: BW/SCS information

QC: Include BW/SCS information in the tables for minimum performance requirements instead of tables for Test Parameters.

->Further offline with Intel for PDSCH changes

->Further clarify the assumption of PRB bundling size and CSI-RS rate matching for simulation from companies

## SDR Test

| R4-1900364 | NR SDR performance requirements | Intel Corporation | Proposal #1: Use the following maximum number of HARQ process for SDR requirements for NR carriers: 4 for FDD and 8 for TDD Proposal #2: Use AL 16 for PDCCH configuration in SDR requirements: Proposal #3: Use the set which has the smallest aggregated channel bandwidth for SDR testing in case multiple sets of CA bandwidth combinations and UE capabilities have the same largest data rate. Proposal #4: Adopt Table 1 to define MCS look up table for FR1 SDR requirements. Table 1. Look up table to derive MCS for FR1 UE capability index. 1; 1; 8; 1; 26; 26; 26. 2; 1; 8; 0.8; 21; 21. 3; 1; 8; 0.75; 20; 20. 4; 1; 8; 0.4; 11; 11. 5; 1; 6; 1; 27; 27; 27. 6; 1; 6; 0.8; 23; 23. 7; 1; 6; 0.75; 22; 22. 8; 1; 6; 0.4; 14; 14. 9; 1; 4; 1; 16; 16; 16. 10; 1; 4; 0.8; 16; 16. 11; 1; 4; 0.75; 16; 16. 12; 1; 4; 0.4; 10; 10. 13; 1; 2; 1; 9; 9; 9. 14; 1; 2; 0.8; 9; 9. 15; 1; 2; 0.75; 9; 9. 16; 1; 2; 0.4; 4; 4. 17; 2; 8; 1; 26; 26; 26. 18; 2; 8; 0.8; 21; 21. 19; 2; 8; 0.75; 20; 20. 20; 2; 8; 0.4; 11; 11. 21; 2; 6; 1; 27; 27; 27. 22; 2; 6; 0.8; 23; 23. 23; 2; 6; 0.75; 22; 22. 24; 2; 6; 0.4; 14; 14. 25; 2; 4; 1; 16; 16; 16. 26; 2; 4; 0.8; 16; 16. 27; 2; 4; 0.75; 16; 16. 28; 2; 4; 0.4; 10; 10. 29; 2; 2; 1; 9; 9; 9. 30; 2; 2; 0.8; 9; 9. 31; 2; 2; 0.75; 9; 9. 32; 2; 2; 0.4; 4; 4. 33; 4; 8; 1; 27; 26; 26. 34; 4; 8; 0.8; 22; 22. 35; 4; 8; 0.75; 21; 21. 36; 4; 8; 0.4; 12; 12. 37; 4; 6; 1; 28; 28; 28. 38; 4; 6; 0.8; 24; 24. 39; 4; 6; 0.75; 23; 23. 40; 4; 6; 0.4; 14; 14. 41; 4; 4; 1; 16; 16; 16. 42; 4; 4; 0.8; 16; 16. 43; 4; 4; 0.75; 16; 16. 44; 4; 4; 0.4; 11; 11. 45; 4; 2; 1; 9; 9; 9. 46; 4; 2; 0.8; 9; 9. 47; 4; 2; 0.75; 9; 9. 48; 4; 2; 0.4; 5; 5. Proposal #5: For FR2 SA SDR requirements, adopt Table 3 to define MCS look up table for and take into account results from Table 5 to define “SNR to MCS+Rank” look up table. Table 3. Look up table to derive upper bound MCS for FR2 UE capability index; UE capability index. 1; 1; 6; 1; 27; 13; 2; 6; 1; 27. 2; 1; 6; 0.8; 23; 14; 2; 6; 0.8; 23. 3; 1; 6; 0.75; 22; 15; 2; 6; 0.75; 22. 4; 1; 6; 0.4; 14; 16; 2; 6; 0.4; 14. 5; 1; 4; 1; 16; 17; 2; 4; 1; 16. 6; 1; 4; 0.8; 16; 18; 2; 4; 0.8; 16. 7; 1; 4; 0.75; 16; 19; 2; 4; 0.75; 16. 8; 1; 4; 0.4; 10; 20; 2; 4; 0.4; 10. 9; 1; 2; 1; 9; 21; 2; 2; 1; 9. 10; 1; 2; 0.8; 9; 22; 2; 2; 0.8; 9. 11; 1; 2; 0.75; 9; 23; 2; 2; 0.75; 9. 12; 1; 2; 0.4; 4; 24; 2; 2; 0.4; 4. Table 5. Impairment simulation results for FR2 MCS; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15; 16. Rank 1; -2.5; -1.7; -0.7; 0.2; 1.2; 2.0; 2.9; 3.5; 4.5; 5.6; 6.4; 7.4; 8.0. Rank 2; 0.5; 1.4; 2.2; 3.3; 4.2; 5.0; 5.8; 6.5; 7.5; 8.5; 9.4; 10.4; 11.1. MCS; 17; 18; 19; 20; 21; 22; 23; 24; 25; 26; 27. Rank 1; 9.2; 9.7; 10.7; 11.6; 12.5; 13.7; 14.4; 15.5; 16.8; 17.6; 18.5. Rank 2; 12.2; 12.8; 13.8; 14.8; 15.6; 16.8; 17.6; 18.9; 20.2; 21.2; 22.3. Proposal #6: Use the following methodology for EN-DC SDR requirements: - Step 1: Select one EN-DC bandwidth combination among all supported EN-DC configurations and set of per component carrier (CC) UE capabilities among all supported UE capabilities that provides the largest data rate [TS 38.306 [14, Section 4.1.2]]. - Set of per NR CC UE capabilities includes channel bandwidth, subcarrier spacing, number of PDSCH MIMO layers, modulation format and scaling factor [TS 38.306 [14, Section 4.1.2]]. - Set of per EUTRA CC UE capabilities includes channel bandwidth, number of PDSCH MIMO layers and modulation format [TS 38.306 [14, Section 4.1.2]]. - Step 2: For each NR CC in EN-DC bandwidth combination, use MCSs from FR1 or FR2 SA SDR requirements depending on EN-DC scenario. For each EUTRA CC in EN-DC bandwidth combination, reuse LTE SDR FRCs [TS 36.101, Sections 8.7]. |
| --- | --- | --- | --- |
| R4-1900868 | NR PDSCH SDR simulation result | MediaTek inc. |  |
| R4-1901019 | On SDR requirements for Rel.15 NR | NTT DOCOMO, INC. | Proposal: Choose the highest configurable MCS for each CBW/SCS combination as upper bound MCS for SDR requirement. The specific values are summarized in the attachment. - Note that the assumption for TBS determination is as follows Parameters; FR1; FR2. PDSCH duration (L); 13 OFDM symbols. # of additional DMRS symbol; 1. FDM of DMRS/PDSCH (Rank 1/2); Yes. Overhead for TBS determination; 0; 6. |
| R4-1901801 | NR PDSCH SDR Performance Simulation Results | Qualcomm Incorporated | Proposal 1: Remove rows with maximum modulation format as 2 and 4 from Table 5.5A-5 in TS 38.101-4. |
| R4-1901920 | views on NR SDR demod requirement | Huawei, HiSilicon | Observation 1: For different bandwidth/SCS combinations, the single MCSupperbound shall be defined as min{ MCSupperbound,1 , MCSupperbound,2 ,….. MCSupperbound,i …. MCSupperbound,n }, where MCSupperbound,i is MCSupperbound for each bandwidth/SCS combination. Proposal 1: The upperbound MCS and practical MCS on FR1 shall be defined as in Table 4. Observation 2: The maximum SNR range for demodulation test depends on the aggregated bandwidth and the specific test method(DNF/DFF/IFF). Proposal 2: SNR level agreed for DNF shall be used for the practical MSC determination for demodulation test. Proposal 3: The upperbound MCS and practical MCS for 100MHz channel bandwidth on FR2 shall be defined as in Table 4. Proposal 4: The maximum SNR level for bandwidth larger than 100MHz shall be scaled down by 3dB/100MHz. |

Open issues:

- Number of HARQ process

- Achievable MCS levels and tables for FR1

- Achievable MCS levels and tables for FR2

- SDR test covering FR1 +FR2 for NR CA and EN-DC

### Issue 1: Number of HARQ process

Previous agreements:

- Proposal:

  - Option 1：If normal PDSCH demodulation requirement with 16 (TDD)/8 (FDD) HARQ processes targeting for higher Rank/MCS are introduced, the number of HARQ process for SDR test is 8 (TDD) / 4 (FDD)； Otherwise, 16 (TDD) / 8 (FDD) processes

  - Option 2: 8 for TDD and 4 for FDD

Candidate options:

- Option 1: 4 for FDD and 8 TDD (Intel)

- Option 2: 8 for FDD and 16 for TDD (NTT DoCoMo)

### Issue 2: MCS levels and tables for FR1

Issue 2-1: MCS upper bound selection among CHBW/SCS combination

Previous agreement:

- Single upper bound applicable for all CHBW and SCS

Discussion

- Option 1: single upper bound (Intel, QC)

- Option 2: per CHBW /SCS combination (NTT DoCoMo)

QC: we need to update table once we have new combinations of CHBW and SCS.

Intel: MCS will be limited by practical values, probably no matter.

NTT DoCoMo: we already provide table following approach 2.

->Offline for above two approaches.

Issue 2-2: Detailed MCS table, How to decide practical MCS based on companies’ results?

Intel volunteers to provide template of collection results from companies.

Intel and QC have results for FR1 and FR2.

### Issue 3: MCS levels and tables for FR2

What’s the assumption for achievable SNR considering test equipment characteristics?

MCS range for evaluation?

QC: do we need to have limitation for MCS levels to provide results?

### Issue 4: SDR test for EN-DC

Proposal from Intel:

Use the following methodology for EN-DC SDR requirements:

- Step 1: Select one EN-DC bandwidth combination among all supported EN-DC configurations and set of per component carrier (CC) UE capabilities among all supported UE capabilities that provides the largest data rate [TS 38.306 [14, Section 4.1.2]].

  - Set of per NR CC UE capabilities includes channel bandwidth, subcarrier spacing, number of PDSCH MIMO layers, modulation format and scaling factor [TS 38.306 [14, Section 4.1.2]].

  - Set of per EUTRA CC UE capabilities includes channel bandwidth, number of PDSCH MIMO layers and modulation format [TS 38.306 [14, Section 4.1.2]].

- Step 2: For each NR CC in EN-DC bandwidth combination, use MCSs from FR1 or FR2 SA SDR requirements depending on EN-DC scenario. For each EUTRA CC in EN-DC bandwidth combination, reuse LTE SDR FRCs [TS 36.101, Sections 8.7].

QC: for RMC?

->Intel already provide draft CR for EN-DC SDR test case, companies are encouraged to check and comeback this later

->QC already provide draft CRs for RMC, further offline with Intel whether these RMCs needed or not in RAN4, LS may be needed to check with RAN5

### Issue 5: Procedure for FR2 SDR test

QC proposal:

Adopt the following procedure for FR2 SDR test cases:

-	Step 1: For all supported CA configurations and set of per component carrier (CC) UE capabilities among all supported UE capabilities:

-	Use the table defining the MCS to determine the MCS (=MCS1) achieving the largest data rate [TS 38.306 [14, Section 4.1.2]] based on UE capabilities.

-	Use the table defining the mapping between MCS and SNR required to achieve that MCS to determine the largest MCS (=MCS2) requiring SNR below test equipment maximum achievable SNR for each CC in that CA configuration.

-	Compute the aggregate throughput for CA configuration using the MCS = min(MCS1,MCS2) for each CC in CA bandwidth combination.

-	Step 2: Choose the CA bandwidth combination among all supported CA configurations that achieves maximum aggregate throughput in step 1 among all UE capabilities.

-	Set of per CC UE capabilities includes channel bandwidth, subcarrier spacing, number of PDSCH MIMO layers, modulation format and scaling factor [TS 38.306 [14, Section 4.1.2]].

-	When there are multiple sets of CA bandwidth combinations and UE capabilities (channel bandwidth, subcarrier spacing, number of MIMO layer, modulation format, scaling factor) with same aggregate throughput, select TBD

-	Step 3: For each CC in chosen CA bandwidth combination, use determined MCS for each CC in step 1 for that CA configuration based on test parameters and indicated UE capabilities.

-> Offline for procedure in FR2

Issue 6: Precoder for FR2 test cases

Further clarify for this

# PDCCH and PBCH (1 hour)

- PDCCH

  - Test case for Aggregation level (AL 16)  of FR1 4Rx

  - Test cases with 4Rx with interleaved CCE-to-REG mapping

- PBCH Test

  - Necessity of 4Rx test

  - SSB sequence identification assumption

| R4-1900117 | NR PDCCH simulation results | Intel Corporation |  |
| --- | --- | --- | --- |
| R4-1901341 | NR UE performance test open issues | Ericsson | Proposal 5: Define PDCCH TC7 4Rx test with bundle size 2 as Option 1, as good performance can be achieved by proper implementation on noise covariance matrix estimation. Proposal 6: Define AL=16 with 4Rx with medium A correlation for PDCCH. |
| R4-1900118 | NR PDCCH UE Demodulation Requirements | Intel Corporation | Observation #1: For 4Rx with AL=16, operating SNR is reasonable with TDLA30-10Hz channel and Medium-A antenna correlation Proposal #1: Introduce 4Rx test cases with AL=16 with channel model as TDLA30-0Hz and Medium-A antenna correlation Observation #2: Performance for 4Rx test cases with interleaved CCE-to-REG mapping is not significantly different between REG bundle sizes of 2 and 6 Proposal #2: Introduce 4Rx tests with interleaved CCE-to-REG mapping with REG bundle size of 2 |
| R4-1900119 | NR PBCH UE Performance Requirements | Intel Corporation | Observation #1: Operating SNR for some test cases is even further reduced without PBCH-DMRS sequence detection Proposal #1: For PBCH demodulation do not introduce requirements without PBCH-DMRS sequence detection |
| R4-1900237 | Updated PDCCH simulation and impairment results | CMCC | Proposal: It is proposed to introduce PDCCH AL=16 with 4Rx test cases for test 14 and test 15 A. |
| R4-1900238 | Further discussion on PBCH demodulation requirements | CMCC | Proposal 1: It is proposed to specify both PBCH demodulation requirements with the knowledge of SSB index and without the knowledge of SSB index. Proposal 2: It is proposed to defne 4Rx PBCH demodulation requirements in Rel-15. |
| R4-1900239 | Summary results for alignment and impairments of NR PBCH demodulation tests in Rel-15 | CMCC |  |
| R4-1900483 | Ideal and impairment results for NR PDCCH | CATT |  |
| R4-1900771 | Propagation condition for 4Rx PDCCH demodulation requirements with AL16 | China Telecom | Proposal 1: For 4Rx AL16 PDCCH test with 30 kHz SCS, use TDL-C with ULA medium A or TDL-A with ULA medium A channel, and the former one is slightly preferred. Proposal 2: For 15 kHz SCS, introduce 4Rx AL16 PDCCH test under TDL-A ULA low or TDL-A ULA medium A channel, and the former one is slightly preferred. |
| R4-1900772 | Further discussion on 4Rx PBCH demodulation requirements | China Telecom | Observation 1: With obvious performance improvement, it is not reasonable to preclude the implementation of 4Rx PBCH. Proposal 1: Introduce 4Rx PBCH demodulation requirements. |
| R4-1900869 | NR PDCCH simulation result | MediaTek inc. |  |
| R4-1900870 | NR PBCH simulation result | MediaTek inc. |  |
| R4-1901274 | Simulation results of NR PBCH demodulation | Ericsson | Proposal 1: RAN4 specify BPCH demodulation requirement without SSB index knowledge in addition to the existing PBCH demodulation requirements with SSB index knowledge. |
| R4-1901343 | Simulation results for NR UE PDCCH demodulation tests | Ericsson |  |
| R4-1901345 | Summary of alignment and impairment results for NR UE PDCCH demodulation tests | Ericsson |  |
| R4-1901655 | NR PDCCH Demodulation Performance Simulation Results | Qualcomm Incorporated |  |
| R4-1901914 | Discussion on NR PDCCH AL16 demod perf | Huawei, HiSilicon | Proposal 1: Define 4Rx AL16 PDCCH requirements under TDL-A 30ns 10Hz channel with ULA medium A correlation for 15 kHz and 30 kHz SCS. |
| R4-1901917 | remaining issue for NR PBCH demodulation requirement. | Huawei, HiSilicon | Observation 1: To achieve the same performance, the PBCH demodulation performance with 4Rx is 2dB better than that with 2Rx. Observation 2: Under the same SNR level, the less combination is needed with 4Rx demodulation to achieve the same performance and it would translate into less MIB decoding delay. Proposal 1: For FR1, define 4Rx PBCH demodulation requirement without the knowledge of SSB index. |
| R4-1901918 | Simulation results for NR PDCCH demod perf | Huawei, HiSilicon |  |
| R4-1901967 | NR PBCH Demodulation Performance Simulation Results | Qualcomm Incorporated |  |

## PDCCH

### Issue 1：Test cases for Aggregation level (AL 16) +4Rx

Previous agreements:

- For FR1 15 kHz (case 14): further evaluate under TDL-A with MIMO correlation ULA medium A for AL=16 under 4Rx, decide in next RAN4 meeting for whether introducing test cases with AL 16 under 4Rx.

- For FR1 30 kHz (case 15a): Introducing test cases AL 16 with 4Rx

  - TDL-C with ULA medium A(baseline)

  - TDL_A with ULA medium A

  - Decided above two options based on evolution results

Candidate options:

Issue 1-1: Introducing FR1 15kHz test with 4Rx AL16 (test 14)- TDL-A ULA Medium A

- Option 1: Yes  (Intel, Huawei, Ericsson, China Telecomm)

Issue 1-2: MIMO correlation for FR1 30kHz test with 4Rx AL 16 (15a/15b)

- Option1: TDL-A, ULA Medium A (Intel, Huawei, Ericsson, China Telecomm)

### Issue 2: Test cases with 4Rx with interleaved CCE-to-REG mapping

Previous agreements:

- Option 1: REG bundle size =2, CORESET BW = 102

- Option 2: REG bundle size =6, CORESET BW = 90

- Further check the details simulation assumption and results among companies and decide in next meeting.

Candidate option:

- Option 1: using bundle size =2 (test 6a,7a adopted)- (Intel, Ericsson),

### Issue 3: Simulation results and requirements definition

->Companies need to provide results into summary files before Tuesday evening and based on collection results to decide SNR requirements.

## PBCH

### Issue 1: Necessity of 4Rx PBCH test cases

Previous agreements:

- Issue 1: Necessity of 4Rx PBCH test cases

  - Option 1: introducing 4Rx PBCH test cases

  - Option 2: not introducing 4Rx PBCH test cases

- Proposal:

  - Further discuss whether to introduce 4Rx PBCH test cases after collecting results from more companies in next meeting

  - If 4Rx PBCH requirements is introduced,

    - whether to apply 4Rx PBCH requirements is up to UE declaration

    - Note the PBCH demodulation requirements do not need to be tested in RAN5 as same as LTE PBCH demodulation requirements. (i.e. no conformance requirements)

Candidate options:

- Option 1:Introducing 4Rx PBCH test cases (CMCC, China Telecomm, Huawei)

- Option 2: Not introducing 4Rx PBCH test cases (QC)

CMCC: According to RLM discussion in RRM, Out of sync SNR is lower than 4Rx PBCH decoding SNR based on evaluation results.

QC: For RLM, 2Tx assumed.Can not combine 2Tx RLM with 1Tx PBCH together to achieve this SNR.

CMCC: Even with 4Rx PBCH, the decoding SNR will not trigger out of sync.

QC: Introducing 2Tx PBCH may achieve similar performance.

CMCC: 4Rx is mandatory for certain bands.

QC: Previous agreements: 4Rx is only mandatory for demodulation, PBCH related RRM.

CMCC: we have 4Rx RLM requirements.

QC: For RLM, no explicit requirements under 4Rx i.e. different sub-sections

CMCC: we have different understanding for RLM.

### Issue 2: UE assumption on DMRS SSB index acquisition for PBCH demodulation

- Issue 2: UE assumption on DMRS SSB index acquisition for PBCH demodulation

  - Option 1: Derive requirements assuming without  knowledge of  SSB  index

  - Option 2: Derive two kinds of requirements with and without knowledge of SSB index

- Proposal:

  - Introduce PBCH requirements assuming without knowledge of SSB index.

    - PBCH requirements  is derived with the combination of SSB index detection probability and MIB decoding probability

  - Whether to introduce requirements with knowledge of SSB index will be further discussed after collecting results from more companies in next meeting

Candidate options

- Option 1: Introduce requirements with knowledge of SSB index (Ericsson, CMCC)

- Option 2: no need to introduce requirements with knowledge of SSB index (Intel)

CMCC: these requirements applied different scenarios. Huawei, Intel concern?

Intel: we didn’t see much performance difference among these two options.

CMCC: 2 dB performance gap observed

Huawei: op1 also fine with us

CMCC: applied to all test cases with two set of requirements under different assumption

Intel: SNR maybe quite low.

CMCC: For NSA without initial cell search, UE can get information of SSB index. The scenario of UE without knowledge of SSB index is corner case.

Potential agreements:

Introduce two sets of requirements with and without knowledge of SSB index for all PBCH test cases.

Intel will check and come back this week.

### Issue 3: Simulation results handling

Companies can provide results before Wednesday evening, SNR will be captured in draft CR based on companies’ results.

How handle results to derive SNR requirements can be further offline.

Intel: Gap performance among two assumptions quite diversity between companies: Intel/Huawei ~1dB; CMCC 2dB~, Ericsson 4dB for some cases.

# CSI (1 hour)

List contributions

| R4-1900102 | Simulation results and discussion on NR CQI reporting | Intel Corporation | Proposal 1: for FR1 FDD and TDD WB CQI test cases, reuse LTE requirements that: - a%>20% - gamma>1.05 - BLER>0.02 Proposal 2: for FR2 TDD WB CQI test cases, define requirements that: - a%>2% - gamma>1.02 - BLER>0.01 Proposal 3: Apply “Option 2: 1x2” antenna configuration for FR1 SB CQI test cases. |
| --- | --- | --- | --- |
| R4-1900103 | Simulation results and discussion on NR PMI reporting | Intel Corporation | Observation 1: For FR2 with 2Tx ULA Medium correlation, throughput gain is larger than ULA Low MIMO correlation case. Proposal 1: For FR1 4Tx PMI test, set test requirement as 1.3 for FDD, TDD and 2Rx/4Rx. Proposal 2: For FR1 8Tx PMI test cases, set test requirement as 1.5 for FDD, TDD and 2Rx/4Rx. Proposal 3: For FR2, introducing 2Tx PMI test case with ULA Medium correlation. |
| R4-1900104 | Simulation results and discussion on NR RI reporting | Intel Corporation | Observation 1: For FR1 FDD and TDD RI test cases, large throughput gain of “Follow RI vs. Fixed Rank2” can be observed at 0dB, and “Follow RI vs. Fixed Rank1” at 22dB. Observation 2: For FR2 TDD RI test case, large throughput gain of “Follow RI vs. Fixed Rank2” can be observed at 0dB, and “Follow RI vs. Fixed Rank1” at 22dB. Proposal 1: Set “Follow RI vs. Fixed Rank2” SNR test point at 0dB, and “Follow RI vs. Fixed Rank1” SNR test point at 22dB. |
| R4-1900105 | Draft CR on NR CSI reporting | Intel Corporation |  |
| R4-1900167 | Views on NR UE CSI Reporting Tests | Qualcomm Incorporated | Proposal 1: Use 2Tx antennas for defining frequency selective fading CQI test cases. Proposal 2: Change parameter k in beam steering approach to 2-μ. Proposal 3: Use the following values for missing test parameters: - Number of PDCCH candidates = 1 - Slots for PDCCH monitoring = each slot - PDCCH Aggregation Level = 8 - PDCCH DCI Format = 1-0 |
| R4-1900305 | Simulation results for CQI test cases | Samsung | Wideband Fading CQI test cases Proposal1: For FR1 2Rx wideband fading CQI test cases (FDD and TDD): - Test point: 6/7dB +12/13 dB - Test requirements: - Aifa>20% - BLER>2% - Gamma>1.05 Proposal2: For FR1 4Rx wideband fading CQI test cases (FDD and TDD): - Test point: 3/4dB +9/10 dB - Test requirements: - Aifa>5% - BLER>2% - Gamma>1.05 Proposal3: For FR2 2Rx wideband fading CQI test cases: - Test point: 6/7dB +12/13 dB - Test requirements: - Aifa>5% - BLER>2% - Gamma>1.05 Sub-band CQI test cases Proposal4: Using 1Tx port to introduce frequency-selective CQI test case Proposal5: For FR2 2Rx sub-band CQI test cases (FDD and TDD): - Test point: 9/10dB +14/15 dB - Test requirements: - Aifa>2% - Beta>55% - Gamma>1.05 - BLER>0.05 Proposal6: For FR2 4Rx sub-band CQI test cases (FDD and TDD): - Test point: 6/7dB +11/12dB - Test requirements: - Aifa>2% - Beta>55% - Gamma>1.05 - BLER>0.05 |
| R4-1900306 | Simulation results for PMI test cases | Samsung | FR1 PMI test cases Proposal 1: For FR1 8Tx PMI test cases, test requirement as 1.5 for FDD, TDD and 2Rx/4Rx. FR2 PMI test cases Observation 1: For FR2, with 2Tx ULA Medium correlation, throughput gain is larger than ULA Low MIMO correlation cases. Proposal 2: For FR2, introducing 2Tx PMI test case under ULA Medium correlation. Proposal 3: For FR2 PMI test case with TDD DL-UL pattern FR2.120-2, test requirement can 1.1 Proposal 4: For FR2 PMI test case with TDD DL-UL pattern FR2.120-1, test requirement can 1.1 |
| R4-1900307 | Simulation results for RI test cases | Samsung | FR1: 2Rx (FDD and TDD) Test Number; Test 1 (2X2); Test 2 (2X2); Test 3 (2X2). MIMO correlation; ULA Low; ULA Low; ULA High for FR1. Metric; Gamma 2-1.0; Gamma 1-1.05; Gamma 1 -0.9. SNR; 0 dB; 20 dB; 20 dB. FR1: 4Rx (FDD and TDD) Test Number; Test 1 (2X4); Test 2 (2X4); Test 3 (2X4); Test 4 (4X4). MIMO correlation; ULA Low; ULA Low; ULA High; ULA Low. Metric; Gamma 2-1.0; Gamma 1-1.05; Gamma 1-0.9; Gamma 2-1.05. SNR; -4dB; 16dB; 16dB; 24dB. FR2: 2Rx (TDD) Test Number; Test 1 (2X2); Test 2 (2X2); Test 3 (2X2). MIMO correlation; ULA Low; ULA Low; XP High for FR1. Metric; Gamma 2-1.0; Gamma 1-1.05; Gamma 1 -0.9. SNR; -2 dB; 20 dB; 20 dB. |
| R4-1900308 | Simulation assumption for Rel-15 NR CSI requirements | Samsung |  |
| R4-1900309 | Summary of FR1 FDD CSI simulation results | Samsung |  |
| R4-1900310 | Summary of FR1 TDD CSI simulation results | Samsung |  |
| R4-1900311 | Summary of FR2 TDD CSI simulation results | Samsung |  |
| R4-1900314 | Draft CR for PMI test cases: 6.2, 8.2, A.3.2.2.2, A.3.2.2.5 | Samsung |  |
| R4-1900315 | Test case design for measurement restrcition | Samsung |  |
| R4-1900603 | Simulation results for PMI test in FR2 | NTT DOCOMO, INC. | Observation: Both test points, i.e., 70 and 90%, are testable. Proposal 1: For FR2 test 4.6 (ULA Low), test requirement shall be 1.2. Proposal 2: For FR2 test 4.6 (ULA Med.), test requirement shall be 1.3. |
| R4-1900871 | NR CSI CQI simulation result | MediaTek inc. |  |
| R4-1900872 | NR CSI PMI simulation result | MediaTek inc. |  |
| R4-1900873 | NR CSI RI simulation result | MediaTek inc. |  |
| R4-1901344 | Simulation results for NR UE CSI tests | Ericsson |  |
| R4-1901805 | Simulation results of NR performance of CQI test under static and fading | Huawei, HiSilicon |  |
| R4-1901885 | Draft CR on FR2 CSI Reporting Tests | Qualcomm Incorporated |  |
| R4-1901924 | NR CSI Reporting Simulation Results | Qualcomm Incorporated |  |
| R4-1902003 | Draft CR on FR1 CSI Reporting Tests | Qualcomm Incorporated |  |

## Generic test configurations

- Issue 1: CS-RS resource/CSI reporting interval and slot offset & minimum CSI delay for aperiodic CSI reporting

  - FR1 FDD 15 kHz: 5/1

    - For 4Tx wideband PMI test case 4.1, minimum CSI delay 6 ms (6 slots)

    - For 8Tx wideband PMI test case 4.3, , minimum CSI delay 8 ms (8 slots)

  - FR1 TDD 30 kHz: 10/1

    - For 4Tx wideband PMI test case 4.2, minimum CSI delay 5.5 ms (11 slots)

    - For 8Tx wideband PMI test case 4.4, minimum CSI delay 6.5 ms (13 slots)

Issue 2: K1 values-> check offline and discuss how to update in specification

  - FR1 FDD 15kHz: [2]

  - FR1 TDD 30kHz with TDD DL-UL pattern  FR1.30-1:

[7] if mod(i,10) = 0
[5] if mod(i,10) = 2
[5] if mod(i,10) = 3
[4] if mod(i,10) = 4
[3] if mod(i,10) = 5
[3] if mod(i,10) = 6

I is slot index per radio frame with 0~19

  - FR2 TDD 120kHz with TDD DL-UL pattern  FR2.120-2:

[11] if mod(i,8) = 0
[7] if mod(i,8) = 4
[6] if mod(i,8) = 5
I is slot index per radio frame with 0~79

  - FR2 TDD 120kHz with TDD DL-UL pattern  FR2.120-1:

[3] if mod(i,5) = 0
[6] if mod(i,5) = 2
I is slot index per radio frame with 0~79

Issue 3: k for beam steering

QC: Change parameter k in beam steering approach to 2-μ.

Intel, Samsung, QC ,Ericsson, and MTK provided results for this meeting, companies are encouraged to put results into summary files before Wednesday .

## CQI reporting

### Test SNR points for static CQI test cases

Previous agreements:

  - FR1 2Rx FDD:[8/9]dB, [14/15]dB

  - FR1 4Rx FDD :[5/6]dB, [11/12]dB

  - FR1 2Rx TDD:[8/9]dB, [14/15]dB

  - FR1 4Rx TDD :[5/6]dB, [11/12]dB

  - FR2 2Rx:[8/9]dB, [14/15]dB

Proposals:

### Wideband CQI test cases

Previous agreements:

- Test metric:

  - a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least a % of the time;

  - b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ g ;

  - c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to  TBD.

- Test SNR points and test requirements for FR1 FDD:

- Test SNR points and test requirements for FR1 TDD:

  - Candidate SNR points: [6/7]dB], [12/13] dB for 2Rx; [3/4]dB], [9/10] dB for 4Rx

- Test SNR points and test requirements for FR2 TDD:

  - Candidate SNR points: [6/7]dB], [12/13] dB for 2Rx;

- Including these values into 38.101-4 with [] for FR1 FDD , the requirements were introduced based on current results from companies; these requirements can be revised based on more results from companies.

Requirements for FR1

|  | FR1 2*2 |  |  |  | FR2 2*4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Open issues | Test SNR points | BLER requirements | CQI distribution requirements | Through ration requirements | Test SNR points | BLER requirements | CQI distribution requirements | Through ration requirements |
| Samsung | 6/7dB , 12/13 dB | 0.02 | 20% | 1.05 | 3/4dB , 9/10 dB | 0.02 | 5% | 1.05 |
| QC |  |  |  |  |  |  |  |  |
| Intel |  | 0.02 | 20% | 1.05 |  | 0.02 | 20% | 1.05 |
| Ericsson |  |  |  |  |  |  |  |  |
| Huawei |  |  |  |  |  |  |  |  |
| Agreement |  |  |  |  |  |  |  |  |

Requirements for FR2

|  | FR1 2*2 |  |  |  |
| --- | --- | --- | --- | --- |
| Open issues | Test SNR points | BLER requirements | CQI distribution requirements | Through ration requirements |
| Samsung | 6/7dB , 12/13 dB | 0.02 | 5% | 1.05 |
| QC |  |  |  |  |
| Intel |  | 0.01 | 2% | 1.02 |
| Ericsson |  |  |  |  |
| Huawei |  |  |  |  |
| Agreement |  |  |  |  |

Further offline to decide requirements for wideband CQI based on collection results.

### Sub-band CQI test cases

Previous agreements：

- Number of Tx

  - Option 1： 1Tx

  - Option 2： 2Tx  (baseline)

- Companies are encouraged to bring results among 0~20 dB with 2dB step size:

  - Relative throughput ratio between following CQI on selected sub-band and median wideband CQI on a random selected sub-band with full size

  - BLER with following CQI on selected sub-band

  - The percentile of reported sub-band differential CQI offset  level  equals to 0 for each sub-band with full size

- Decide test points and test requirements in RAN4#90 based on companies’ results

MIMO TX

Option 1: 1Tx (Intel, Samsung)

Option 2: 2Tx (Qualcomm）

QC: with our analysis, results should be same. Need to take offline.

Requirements:

|  | FR1 2*2 |  |  |  | FR2 2*4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Open issues | Test SNR points | BLER requirements | CQI distribution requirements | Through ration requirements | Test SNR points | BLER requirements | CQI distribution requirements | Through ration requirements |
| Samsung | 9/10 dB 14/15 dB | 0.05 | Afia =2 Beta = 55% | 1.05 | 6/7dB 11/12dB | 0.05 | Afia =2 Beta = 55% | 1.05 |
| QC |  |  |  |  |  |  |  |  |
| Intel |  |  |  |  |  |  |  |  |
| Ericsson |  |  |  |  |  |  |  |  |
| Huawei |  |  |  |  |  |  |  |  |
| Ericsson |  |  |  |  |  |  |  |  |
| Agreement |  |  |  |  |  |  |  |  |

Further offline for the assumption and requirements.

## PMI reporting

Previous agreements:

- Test metric: Relative Throughput ratio between following PMI and random PMI at SNR point corresponding to X% TP with follow PMI.

    - X= 90

- Test requirements

  - FR1 FDD:

    - 8T2R, 8T4R: TBD

    - 4T2R,4T4R: [1.3]

    - Other options not excluded

  - FR1 TDD:

    - 8T2R, 8T4R: TBD

    - 4T2R,4T4R:[1.3]

  - FR2 TDD:

    - 2T2R: TBD

- MCS and Rank for FR2 2Tx PMI test case:

  - MCS 13 and Rank1

- Beam steering approach applied for 4Tx and 8Tx PMI test cases, the methodology can be refer to TS36.101 B.2.3A.4, update speed as k =1 for 15kHz and k=0.5 for 30kHz and The step of phase variation as per rand per ms

- Including these values into 38.101-4 with [] for FR1 FDD and FR1 TDD , the requirements were introduced based on current results from companies; these requirements can be revised based on more results from companies.

  - Decide test points and test requirements in RAN4#90 based on companies’ results

- MIMO correlation for FR2 2Tx PMI test case:

  - Option 1: ULA Low

  - Option 2: ULA medium

| Issues: | FR1 8Tx PMI | FR2 2Tx PMI |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | requirements |  | MIMO correlation | Requirements For FR2.120-2 | Requirements For FR2.120-2 |  |
| Samsung | 1.5 |  | ULA Medium | 1.1 | 1.1 |  |
| QC |  |  |  |  |  |  |
| Intel | 1.5 |  | ULA Medium |  |  |  |
| Ericsson |  |  |  |  |  |  |
| NTT DoCoMO |  |  |  | 1.2/1.3 |  |  |
| Huawei |  |  |  |  |  |  |
| Agreement |  |  |  |  |  |  |

Further offline to decide requirements based on results.

## RI Test

Previous agreements:

- Test metric:

  - a) The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ g1;

  - b) The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ g2;

- Test requirements:

  - FR1 FDD 2Rx : (Baseline))

| Test Number | Test 1 (2X2) ULA Low |  | Test 2 (2X2) ULA Low |  | Test 3 (2X2) ULA High for FR1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SNR | Gamma 2 | SNR | Gamma 1 | SNR | Gamma 1 |
|  | [0 dB] | [1.0] | TBD | TBD | TBD | TBD |

    - Other options not excluded

  - FR1 FDD 4Rx:

    - TBD

  - FR1 TDD 2Rx : TBD

  - FR1 TDD 4Rx:

    - TBD

  - FR2 TDD 2Rx:

    - TBD

FR1 Test requirements for 2Rx

| Test Number | Test 1 (2X2) ULA Low |  | Test 2 (2X2) ULA Low |  | Test 3 (2X2) ULA High for FR1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SNR | Gamma 2 | SNR | Gamma 1 | SNR | Gamma 1 |
| Samsung | 0 dB | 1.0 | 20 dB | 1.05 | 20 dB | 0.9 |
| QC |  |  |  |  |  |  |
| Intel |  |  |  |  |  |  |
| Ericsson |  |  |  |  |  |  |
| Huawei |  |  |  |  |  |  |
| Ericsson |  |  |  |  |  |  |
| Agreement |  |  |  |  |  |  |

FR1 Test requirements for 4Rx

| Test Number | Test 1 (2X4) ULA Low |  | Test 2 (2X4) ULA Low |  | Test 3 (2X4) ULA High |  | Test 4 (4X4) ULA Low |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR | Gamma 2 | SNR | Gamma 1 | SNR | Gamma 1 | SNR | Gamma 2 |
| Samsung | -4dB | 1.0 | 16 dB | 1.05 | 16 dB | 0.9 | 24 | 1.05 |
| QC |  |  |  |  |  |  |  |  |
| Intel |  |  |  |  |  |  |  |  |
| Ericsson |  |  |  |  |  |  |  |  |
| Huawei |  |  |  |  |  |  |  |  |
| Ericsson |  |  |  |  |  |  |  |  |
| Agreement |  |  |  |  |  |  |  |  |

FR2 Test requirements for 2Rx

| Test Number | Test 1 (2X2) ULA Low |  | Test 2 (2X2) ULA Low |  | Test 3 (2X2) XP High for FR2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SNR | Gamma 2 | SNR | Gamma 1 | SNR | Gamma 1 |
| Samsung | -2 dB | 1.0 | 20 dB | 1.05 | 20 dB | 0.9 |
| QC |  |  |  |  |  |  |
| Intel |  |  |  |  |  |  |
| Ericsson |  |  |  |  |  |  |
| Huawei |  |  |  |  |  |  |
| Ericsson |  |  |  |  |  |  |
| Agreement |  |  |  |  |  |  |

Further offline to decide SNR points and requirements.

# Channel Model

| R4-1900120 | Channel models for NR UE demodulation and CSI requirements | Intel Corporation | Observation #1: The PDPs for FR1 channel models generated based on new methodology are different from those captured in 38.101-4 Observation #2: The simplified channel models in 38.101-4 are already used for simulation alignment Observation #3: There might be no necessity to re-generate simplified models for those already defined in 38.101-4. Observation #4: Minimal impact to PDSCH performance with new channel model PDP Observation #5: There is some non-negligible impact to PDCCH performance with new channel model PDP in some cases Proposal #1: Do not re-define the channel models for those already defined in 38.101-4. Proposal #2: Capture new methodology for simplification of additional channel models in the future Proposal #3: Clarify simplification procedure for channel models TDLA10, TDLB100, TDLC300 and TDLC60 in 38.101-4 Proposal #4: Update Step 6 in new methodology for case when more than 1 weakest path is present |
| --- | --- | --- | --- |
| R4-1900166 | Channel modelling considerations for beamforming in high mobility scenarios in FR2 | Azimuth | Proposal 1: Consider longer Track-BBU distance than the one suggested in Figure 6.1.5-2 in [2] Proposal 2: Use dynamic CDL models for HST scenarios |
| R4-1901501 | Information for tap generation for the delay profile | Huawei, HiSilicon | Proposal 1: Decide whether to update the tables with the new values or to retain the current values. A note may be needed to indicate the new procedure was not used to generate the current values. Proposal 2: Consider placing the algorithm from the technical specification to a technical report. Proposal 3: All future tap generation values should use the revised procedure. |

Issue 1: whether update the procedure for simplified Tap generation

QC: when we update the procedure, current agreed PDP values will keep as it is with some clarification?

Update the procedure for simplified Tap generation based on draft CR from Huawei

No change on the PDP values for existing specified channel models and note to clarify these values didn’t follow the new procedure.

Need to align among BS demod, UE demod and RRM for fading channel generations.

Need to update simulation assumption, companies should follow the simplified channel model defined in TS 38.101-4 annex to run simulation for NR UE performance requirements.

Issue2: whether update PDP values for existing specified channel models according to update procedure

# Draft CRs for TS 38.101-4 (1 hour)

## Draft CRs for generic sections

SNR definition for FR2

| R4-1900084 | Editorial cleanup of FR2 Radiated Requirements General section | ANRITSU LTD |
| --- | --- | --- |
| R4-1901561 | Draft CR of DL power allocation | Intel Corporation |

Requirements applicable rules

| R4-1900366 | Draft CR on NR UE demodulation requirements applicability | Intel Corporation |
| --- | --- | --- |

QC: we already have test purpose in corresponding sections, with these applicable tables. We need to avoid duplicated work.

Intel: In test purpose, we just list test applied scenarios, no differiate among mandatory and optional features.

Huawei:

Table 5.1.1.2-1: Requirements applicability, 3rd row can be merged with 2rd row.

Add WI into section, how to handle test cases introduced in TEI

RAN5 will rely on Release. If following this logic, we have sections for corresponding WIs. These sub-sections hard to manage considering more and more WI will be added in future release.

->Taking offline to see how merge these applicable rules and test purpose tables.

->R4-1900366 need to be return to

| R4-1900419 | Draft CR on General Applicability of Requirements | Qualcomm Incorporated |
| --- | --- | --- |

->Ericsson: in general agree the principles, would like to further discussion the wording for SNR limitation.

NO concern for first two sentences.

Return to

## Draft CRs for PDSCH

| R4-1900368 | Draft CR on FR1 SDR requirements | Intel Corporation |
| --- | --- | --- |
| R4-1900414 | Draft CR on DL RMC for SDR Tests | Qualcomm Incorporated |
| R4-1900506 | Draft CR on FR2 SDR Requirements | Qualcomm Incorporated |
| R4-1901276 | Addition of alternative TDD configuration for UE demodulation requirements | Ericsson, NTT DOCOMO, KDDI, SoftBank |
| R4-1901560 | Draft CR on EN-DC SDR requirements | Intel Corporation |

Above CRs not treated in AD-HOC

| R4-1900367 | Draft CR on FR1 normal PDSCH demodulation requirements | Intel Corporation |
| --- | --- | --- |

Duplex mode in Table 5.2.2.2.3-2: Test parameters should be TDD

TCI state, PDCCH configurations and SNR requirements need to be updated based on further offline output

9th change need to be check wording for channel centre

CHBW for CSI-RS signal

CSI-RS for Rx beam refinement need to be removed for FR1

->Need to revise

| R4-1900369 | Draft CR on PDSCH FRC | Intel Corporation |
| --- | --- | --- |

No comments from ad-hoc

Recommend to endorse

| R4-1900438 | Draft CR on FR2 PDSCH Requirements | Qualcomm Incorporated |
| --- | --- | --- |

Intel: Test purpose need to be aligned, do we to have different row for different ranks.

TCI and CSI-RS for Rx refinement need to be added

CHBW/SCS into table need to be offline

PDCCH config. Need to be update

MTK: CSI-RS BW configurations applied for CSI and PDSCH

SNR requirements need to be update

->Recommend to Revise

## Draft CRs for PDCCH

| R4-1900370 | Draft CR on PDCCH FRC | Intel Corporation |
| --- | --- | --- |

Recommend  to endorse

| R4-1900439 | Draft CR on FR2 PDCCH Requirements | Qualcomm Incorporated |
| --- | --- | --- |

Contents in this CR agreed, these changes will be included in CATT CR

->Recommend to note

| R4-1900484 | Draft CR for updating FR2 PDCCH performance requirements in TS38.101-4 section 7.3 | CATT |
| --- | --- | --- |

Need to include change from QC CR and update SNR requirements

Recommend to revise

Huawei will ask a new t-doc for FR1 PDCCH test cases to capture agreements and update SNR requirements

## Draft CRs for PBCH

| R4-1901275 | Correction of NR PBCH demodulation requirements | Ericsson |
| --- | --- | --- |

Recommend to note this CR, contents can be merged in CMCC CRs

| R4-1900240 | Draft CR on 2Rx PBCH demodulation requirement for FR1 | CMCC |
| --- | --- | --- |
| R4-1900241 | Draft CR on 4Rx PBCH demodulation requirements for FR1 | CMCC |
| R4-1900242 | Draft CR on 2Rx PBCH demodulation requirement for FR2 | CMCC |

Recommend return to above CRs depending on offline discussion for 4Rx and requirements with knowdege of SSB index; also need to update SNR requirements

## Draft CRs for CSI

| R4-1900105 | Draft CR on NR CSI reporting | Intel Corporation |
| --- | --- | --- |

Recommend to endorse

| R4-1900314 | Draft CR for PMI test cases: 6.2, 8.2, A.3.2.2.2, A.3.2.2.5 | Samsung |
| --- | --- | --- |

Recommend to revise to capture offline agreements for requirements and MIMO correlation forFR2 2Tx PMI test cases. NO comments for the contents in this draft CR

Need to update general parameters for CSI test cases to update TCI configuration, CSI-RS for Rx beam refinement in FR2, PDCCH configurations.

Power ratio among PDSCH and NZP CSI-RS still missing for CSI need to added for each test cases

| R4-1901885 | Draft CR on FR2 CSI Reporting Tests | Qualcomm Incorporated |
| --- | --- | --- |

Beamforming model will be specified in Annex

DCI format for PDCCH need to revised as 1-1

TCI, CSI-RS configuration for Rx beam refinement missing in general parameters

->Revise to capture general part for QC CR in FR2

->Need a new T-doc for FR2 CQI test cases to address remaining issues depending on offline discussion ->Intel

->QC will ask a new T-doc to introduce Beamforming model in annex

| R4-1902003 | Draft CR on FR1 CSI Reporting Tests | Qualcomm Incorporated |
| --- | --- | --- |

->Revise to capture general part for QC CR in FR1

Need a new T-doc for FR1 CQI test cases to address remaining issues depending on offline discussion ->Huawei

QC will ask a new t-doc for RI test cases (FR1 +FR2)

## Draft CRs for Annex B propagation

| R4-1901554 | Corrections to 38.101-4 subclause B.2.1 Delay profile calculation | Huawei, HiSilicon |
| --- | --- | --- |

->Return to considering some clarification for existing PDP values didn’t follow the procedure