3GPP TSG-RAN WG4 Meeting #90	R4-1902387

Athens, GR, 25 Feb - 1 Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial CR for BS conducted demodulation test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Correlation level for the propgation channel was missing for PUSCH and PRACH tests. - Cluase number for the propgation condition was TBD for some cases. - To align with TS 38.104, the applicability of different physcial channels can be captured in a common sub-clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the propagation correlation level for PUSCH and PRACH tests. - Updated the cluase number for the propgation condition. - Removed the applicability in the requirements for each channel. - Other editorial modifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No need to introduce these editorial corrections. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Start of first change >

8		Conducted performance characteristics

8.1		General

Conducted performance requirements specify the ability of the BS type 1-C or BS type 1-H to correctly demodulate signals in various conditions and configurations. Conducted performance requirements are specified at the antenna connector(s) (for BS type 1-C) and at the TAB connector(s) (for BS type 1-H).

Conducted performance requirements for the BS are specified for the fixed reference channels and the propagation conditions defined in TS 38.104 [2] annex A and annex G, respectively. The requirements only apply to those FRCs that are supported by the BS.

Unless stated otherwise, performance requirements apply for a single carrier only. Performance requirements for a BS supporting CA are defined in terms of single carrier requirements.

For FDD operation the requirements in clause 8 shall be met with the transmitter units associated with antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) in the operating band turned ON.

NOTE:	In normal operating conditions antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) in FDD operation are configured to transmit and receive at the same time. The associated transmitter unit(s) may be OFF for some of the tests.

The SNR used in this clause is specified based on a single carrier and defined as:

# SNR = S / N

Where:

S 		is the total signal energy in a slot on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H).

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a slot.

8.2	Performance requirements for PUSCH

8.2.1	Performance requirements for PUSCH with transmission precoding disabled

8.2.1.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ re-transmissions.

8.2.1.2	Minimum Requirement

The minimum requirement is in TS 38.104 [2] subclause 8.2.1.

8.2.1.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

8.2.1.4	Method of test

8.2.1.4.1	Initial Conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested:	M; see subclause 4.9.1.

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in Annex D.5 and D.6 for BS type 1-C and type 1-H respectively.

8.2.1.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth, defined in table 8.2.1.4.2-1.

# Table 8.2.1.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 kHz | 5 | [-83.5] dBm / 4.5MHz |
|  | 10 | [-80.3] dBm / 9.36MHz |
|  | 20 | [-77.2] dBm / 19.08MHz |
| 30 kHz | 10 | [-80.7] dBm / 8.64MHz |
|  | 20 | [-77.4] dBm / 18.36MHz |
|  | 40 | [-74.2] dBm / 38.16MHz |
|  | 100 | [-70.1] dBm / 98.28MHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in table 8.2.1.4.2-2.

# Table 8.2.1.4.2-2: Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DMRS | DMRS configuration type | 1 |
|  | Maximum number of OFDM symbols for front loaded DMRS | 1 |
|  | Number of additional DMRS symbols | 0, 1 |
|  | Number of DMRS CDM group(s) without data | 2 |
|  | EPRE ratio of PUSCH to DMRS | -3 dB |
|  | DMRS port | {0}, {0, 1} |
|  | DMRS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | A |
|  | PUSCH starting symbol index | 0 |
|  | PUSCH symbol length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex G.

4)	Adjust the equipment so that required SNR specified in table 8.2.1.5-1 to 8.2.1.5-7 is achieved at the BS input.

5)	For each of the reference channels in table 8.2.1.5-1 to 8.2.1.5-7 applicable for the base station, measure the throughput.

8.2.1.5	Test Requirement

The throughput measured according to subclause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in table 8.2.1.5-1 to 8.2.1.5-7.

# Table 8.2.1.5-1 Test requirements for PUSCH, 5 MHz Channel Bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-1 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-8 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-1 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-8 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-1 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-8 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-1 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-8 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-1 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-8 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-1 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-8 | 1+1 | [TBD] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-1 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-8 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-1 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-8 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-1 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-8 | 1+1 | [TBD] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-15 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-22 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-15 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-22 | 1+1 | [TBD] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-15 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-22 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-15 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-22 | 1+1 | [TBD] |

# Table 8.2.1.5-2 Test requirements for PUSCH, 10 MHz Channel Bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-2 | 1+0 | [1.8] |
|  |  |  |  |  | G-FR1-A3-9 | 1+1 | [-1.6] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-2 | 1+0 | [13.7] |
|  |  |  |  |  | G-FR1-A4-9 | 1+1 | [11.1] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-2 | 1+0 | [13.8] |
|  |  |  |  |  | G-FR1-A5-9 | 1+1 | [13.4] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-2 | 1+0 | [-1.8] |
|  |  |  |  |  | G-FR1-A3-9 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-2 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-9 | 1+1 | [7.4] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-2 | 1+0 | [9.4] |
|  |  |  |  |  | G-FR1-A5-9 | 1+1 | [9.2] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-2 | 1+0 | [-5.2] |
|  |  |  |  |  | G-FR1-A3-9 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-2 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-9 | 1+1 | [3.9] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-2 | 1+0 | [5.9] |
|  |  |  |  |  | G-FR1-A5-9 | 1+1 | [5.9] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-16 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-23 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-16 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-23 | 1+1 | [TBD] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-16 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-23 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-16 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-23 | 1+1 | [TBD] |

# Table 8.2.1.5-3 Test requirements for PUSCH, 20 MHz Channel Bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-3 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-10 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-3 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-10 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-3 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-10 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-3 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-10 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-3 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-10 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-3 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-10 | 1+1 | [TBD] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-3 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-10 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-3 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-10 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-3 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-10 | 1+1 | [TBD] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-17 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-24 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-17 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-24 | 1+1 | [TBD] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-17 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-24 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-17 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-24 | 1+1 | [TBD] |

# Table 8.2.1.5-4 Test requirements for PUSCH, 10 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-4 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-11 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-4 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-11 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-4 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-11 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-4 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-11 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-4 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-11 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-4 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-11 | 1+1 | [TBD] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-4 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-11 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-4 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-11 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-4 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-11 | 1+1 | [TBD] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-18 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-25 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-18 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-25 | 1+1 | [TBD] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-18 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-25 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-18 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-25 | 1+1 | [TBD] |

# Table 8.2.1.5-5 Test requirements for PUSCH, 20 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-5 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-12 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-5 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-12 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-5 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-12 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-5 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-12 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-5 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-12 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-5 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-12 | 1+1 | [TBD] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-5 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-12 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-5 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-12 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-5 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-12 | 1+1 | [TBD] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-19 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-26 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-19 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-26 | 1+1 | [TBD] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-19 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-26 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-19 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-26 | 1+1 | [TBD] |

# Table 8.2.1.5-6 Test requirements for PUSCH, 40 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-6 | 1+0 | [-1.2] |
|  |  |  |  |  | G-FR1-A3-13 | 1+1 | [-1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-6 | 1+0 | [11.5] |
|  |  |  |  |  | G-FR1-A4-13 | 1+1 | [11.0] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-6 | 1+0 | [12.7] |
|  |  |  |  |  | G-FR1-A5-13 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-6 | 1+0 | [-3.4] |
|  |  |  |  |  | G-FR1-A3-13 | 1+1 | [-5.0] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-6 | 1+0 | [7.5] |
|  |  |  |  |  | G-FR1-A4-13 | 1+1 | [7.3] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-6 | 1+0 | [9.2] |
|  |  |  |  |  | G-FR1-A5-13 | 1+1 | [8.9] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-6 | 1+0 | [-7.0] |
|  |  |  |  |  | G-FR1-A3-13 | 1+1 | [-7.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-6 | 1+0 | [3.9] |
|  |  |  |  |  | G-FR1-A4-13 | 1+1 | [3.6] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-6 | 1+0 | [6.0] |
|  |  |  |  |  | G-FR1-A5-13 | 1+1 | [5.7] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | 1+1 | [1.7] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-20 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-27 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-20 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-27 | 1+1 | [TBD] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-20 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-27 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-20 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-27 | 1+1 | [TBD] |

# Table 8.2.1.5-7 Test requirements for PUSCH, 100 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-7 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-14 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-7 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-14 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-7 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-14 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-7 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-14 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-7 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-14 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-7 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-14 | 1+1 | [TBD] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-7 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-14 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-7 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-14 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-7 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-14 | 1+1 | [TBD] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-21 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-28 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-21 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-28 | 1+1 | [TBD] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-21 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-28 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-21 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-28 | 1+1 | [TBD] |

8.2.2	Performance requirements for PUSCH with transmission precoding enabled

8.2.2.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ re-transmissions.

8.2.2.2	Minimum Requirement

The minimum requirement is in TS 38.104 [2] subclause 8.2.2.

8.2.2.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

8.2.2.4	Method of test

8.2.2.4.1	Initial Conditions

Test environment: Normal, see annex B.2.

RF channels to be tested: M; see subclause 4.9.1.

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and type 1-H respectively.

8.2.2.4.2	Procedure

1)	Adjust the AWGN generator, according to the SCS and channel bandwidth, defined in table 8.2.2.4.2-1.

# Table 8.2.2.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth [MHz] | AWGN power level |
| --- | --- | --- |
| 15 kHz | 5 | [-83.5] dBm / 4.5MHz |
| 30 kHz | 10 | [-80.7] dBm / 8.64MHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in table 8.2.2.4.2-2.

# Table 8.2.2.4.2-2: Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DMRS | DMRS configuration type | 1 |
|  | Maximum number of OFDM symbols for front loaded DMRS | 1 |
|  | Number of additional DMRS symbols | 0, 1 |
|  | Number of DMRS CDM group(s) without data | 2 |
|  | EPRE ratio of PUSCH to DMRS | -3 dB |
|  | DMRS port | 0 |
|  | DMRS sequence generation | NID=0, group hopping and sequence hopping are disabled |
| Time domain resource | PUSCH mapping type | A |
|  | PUSCH starting symbol index | 0 |
|  | PUSCH symbol length | 14 |
| Frequency domain resource | RB assignment | 15 kHz SCS: 25 PRBs in the middle of the test bandwidth 30 kHz SCS: 24 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex G.

4)	Adjust the equipment so that required SNR specified in table 8.2.2.5-1 to 8.2.2.5-2 is achieved at the BS input.

5)	For each of the reference channels in table 8.2.2.5-1 to 8.2.2.5-2 applicable for the base station, measure the throughput.

8.2.2.5	Test Requirement

The throughput measured according to subclause 8.2.2.4.2 shall not be below the limits for the SNR levels specified in table 8.2.2.5-1 to 8.2.2.5-2.

# Table 8.2.2.5-1 Test requirements for PUSCH, 5 MHz Channel Bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-29 | 1+0 | [2.8] |
|  |  |  |  |  | G-FR1-A3-31 | 1+1 | [TBD] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-29 | 1+0 | [-1.8] |
|  |  |  |  |  | G-FR1-A3-31 | 1+1 | [TBD] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-29 | 1+0 | [-5.2] |
|  |  |  |  |  | G-FR1-A3-31 | 1+1 | [TBD] |

# Table 8.2.2.5-2 Test requirements for PUSCH, 10 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-30 | 1+0 | [-1.1] |
|  |  |  |  |  | G-FR1-A3-32 | 1+1 | [-2.0] |
|  | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-30 | 1+0 | [-4.3] |
|  |  |  |  |  | G-FR1-A3-32 | 1+1 | [-5.1] |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-30 | 1+0 | [-7.6] |
|  |  |  |  |  | G-FR1-A3-32 | 1+1 | [-8.0] |

8.3	Performance requirements for PUCCH

8.3.1	Performance requirements for PUCCH format 0

8.3.1.1	Definition and applicability

The performance requirement of single user PUCCH format 0 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

8.3.1.2	Minimum Requirement

The minimum requirements are in TS 38.104 [2] subclause 8.3.1 and 8.3.2.

8.3.1.3	Test purpose

The test shall verify the receiver’s ability to detect ACK under multipath fading propagation conditions for a given SNR.

8.3.1.4	Method of test

8.3.1.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested: single carrier (SC) M; see subclause 4.9.1.

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and type 1-H respectively.

8.3.1.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth and sub-carrier spacing defined in table 8.3.1.4.2-1.

# Table 8.3.1.4.2-1: AWGN power level at the BS input

| Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 kHz | 5 | [-83.5] dBm / 4.5 MHz |
|  | 10 | [-80.3] dBm / 9.36 MHz |
|  | 20 | [-77.2] dBm / 19.08 MHz |
| 30 kHz | 10 | [-80.7] dBm / 8.64 MHz |
|  | 20 | [-77.4] dBm / 18.36 MHz |
|  | 40 | [-74.2] dBm / 38.16 MHz |
|  | 100 | [-70.1] dBm / 98.28 MHz |

2)	The characteristics of the wanted signal shall be configured according to TS 38.211 [17] and the specific test parameters are configured as mentioned in table 8.3.1.4.2-2:

# Table 8.3.1.4.2-2: Test Parameters

| Parameter | Test |
| --- | --- |
| nrofBits | 1 |
| nrofPRBs | 1 |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| initialCyclicShift | 0 |
| startingSymbolIndex | 13 for 1 symbol 12 for 2 symbols |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex G.

4)	Adjust the equipment so that the SNR specified in table 8.3.1.5-1 or table 8.3.1.5-2 is achieved at the BS input during the ACK transmissions.

5)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.1.4.2-1. The following statistics are kept: the number of ACKs detected in the idle periods and the number of missed ACKs.

# Figure 8.3.1.4.2-1: Test signal pattern for single user PUCCH format 0 demodulation tests

8.3.1.5	Test Requirement

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in table 8.3.1.5-1 and in table 8.3.1.5-2.

# Table 8.3.1.5-1 Test requirements for PUCCH format 0 and 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Number of OFDM symbols | Channel bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | TDLC-300-100 Low | 1 | TBD | [9.7] | TBD |
|  |  |  | 2 | TBD | TBD | TBD |
| 1 | 4 | TDLC-300-100 Low | 1 | TBD | TBD | TBD |
|  |  |  | 2 | TBD | TBD | TBD |
| 1 | 8 | TDLC-300-100 Low | 1 | TBD | TBD | TBD |
|  |  |  | 2 | TBD | TBD | TBD |

# Table 8.3.1.5-2 Test requirements for PUCCH format 0 and 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex G) | Number of OFDM symbols | Channel bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | TDLC-300-100 Low | 1 | TBD | TBD | [11.5] | TBD |
|  |  |  | 2 | TBD | TBD | TBD | TBD |
| 1 | 4 | TDLC-300-100 Low | 1 | TBD | TBD | TBD | TBD |
|  |  |  | 2 | TBD | TBD | TBD | TBD |
| 1 | 8 | TDLC-300-100 Low | 1 | TBD | TBD | TBD | TBD |
|  |  |  | 2 | TBD | TBD | TBD | TBD |

8.3.2	Performance requirements for PUCCH format 1

8.3.2.1	NACK to ACK detection

8.3.2.1.1	Definition and applicability

The performance requirement of PUCCH format 1 for NACK to ACK detection is determined by the two parameters: probability of false detection of the ACK and the NACK to ACK detection probability. The performance is measured by the required SNR at probability of the NACK to ACK detection equal to 0.1% or less. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK at particular bit position when input is only noise. Each false bit detection is counted as one error.

The NACK to ACK detection probability is the probability of detecting an ACK bit when an NACK bit was sent on particular bit position. Each NACK bit erroneously detected as ACK bit is counted as one error. Erroneously detected NACK bits in the definition do not contain the NACK bits which are mapped from DTX, i.e. NACK bits received when DTX is sent should not be considered.

8.3.2.1.2	Minimum Requirement

The minimum requirement is in TS 38.104 [2] subclause 8.3.3.

8.3.2.1.3	Test purpose

The test shall verify the receiver’s ability not to falsely detect NACK bits as ACK bits under multipath fading propagation conditions for a given SNR.

8.3.2.1.4	Method of test

8.3.2.1.4.1	Initial Conditions

Test environment: Normal; see annex B.2.

RF channels to be tested: for single carrier (SC): M; see sub-clause 4.9.1.

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in Annex D.5 and D.6 for BS type 1-C and type 1-H respectively.

8.3.2.1.4.2	Procedure

1)	Adjust the AWGN generator, according to the combinations of SCS and channel bandwidth defined in table 8.3.2.1.4.2-1.

# Table 8.3.2.1.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 kHz | 5 | [-83.5] dBm / 4.5 MHz |
|  | 10 | [-80.3] dBm / 9.36 MHz |
|  | 20 | [-77.2] dBm / 19.08 MHz |
| 30 kHz | 10 | [-80.7] dBm / 8.64 MHz |
|  | 20 | [-77.4] dBm / 18.36 MHz |
|  | 40 | [-74.2] dBm / 38.16 MHz |
|  | 100 | [-70.1] dBm / 98.28 MHz |

2)	The characteristics of the wanted signal shall be configured according to TS 38.211 [17], and the specific test parameters are configured as below:

# Table 8.3.2.1.4.2-2: Test parameters

| Parameter | Values |
| --- | --- |
| nrofBits | 2 |
| nrofPRBs | 1 |
| nrofSymbols | 14 |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| initialCyclicShift | 0 |
| startingSymbolIndex | 0 |
| Index of orthogonal sequence (time-domain-OCC) | 0 |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex G.

4)	Adjusting the equipment so that the SNR specified in table 8.3.2.1.5-1 and table 8.3.2.1.5-2 is achieved at the BS input during the transmissions.

5)	The signal generator sends random codeword from applicable codebook, in regular time periods. The following statistics are kept: the number of ACK bits detected in the idle periods and the number of NACK bits detected as ACK.

8.3.2.1.5	Test Requirement

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of NACK bits falsely detected as ACK shall be less than 0.1% for the SNR listed in tables 8.3.2.1.5-1 and table 8.3.2.1.5-2.

# Table 8.3.2.1.5-1 Required SNR for PUCCH format 1 with 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel Bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low |  |  |  |
|  | 4 | Normal | TDLC300-100 Low |  |  |  |
|  | 8 | Normal | TDLC300-100 Low |  |  |  |

# Table 8.3.2.1.5-2 Required SNR for PUCCH format 1 with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel Bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | Normal | TDLC300-100 Low |  |  |  |  |
|  | 4 | Normal | TDLC300-100 Low |  |  |  |  |
|  | 8 | Normal | TDLC300-100 Low |  |  |  |  |

8.3.2.2	ACK missed detection

8.3.2.2.1	Definition and applicability

The performance requirement of PUCCH format 1 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

8.3.2.2.2	Minimum Requirement

The minimum requirement is in TS 38.104 [2] subclause 8.3.3.

8.3.2.2.3	Test purpose

The test shall verify the receiver’s ability to detect ACK bits under multipath fading propagation conditions for a given SNR.

8.3.2.2.4	Method of test

8.3.2.2.4.1	Initial Conditions

Test environment: Normal; see annex B.2.

RF channels to be tested: for single carrier (SC): M; see sub-clause 4.9.1.

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in Annex D.5 and D.6 for BS type 1-C and type 1-H respectively.

8.3.2.2.4.2	Procedure

1)	Adjust the AWGN generator, according to the combinations of SCS and channel bandwidth defined in table 8.3.2.2.4.2-1.

# Table 8.3.2.2.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 kHz | 5 | [-83.5] dBm / 4.5 MHz |
|  | 10 | [-80.3] dBm / 9.36 MHz |
|  | 20 | [-77.2] dBm / 19.08 MHz |
| 30 kHz | 10 | [-80.7] dBm / 8.64 MHz |
|  | 20 | [-77.4] dBm / 18.36 MHz |
|  | 40 | [-74.2] dBm / 38.16 MHz |
|  | 100 | [-70.1] dBm / 98.28 MHz |

2)	The characteristics of the wanted signal shall be configured according to TS 38.211 [17], and the specific test parameters are configured as below:

# Table 8.3.2.2.4.2-2: Test parameters

| Parameter | Values |
| --- | --- |
| nrofBits | 2 |
| nrofPRBs | 1 |
| nrofSymbols | 14 |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| initialCyclicShift | 0 |
| startingSymbolIndex | 0 |
| Index of orthogonal sequence (time-domain-OCC) | 0 |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex G.

4)	Adjusting the equipment so that the SNR specified in table 8.3.2.2.5-1 and table 8.3.2.2.5-2 is achieved at the BS input during the transmissions.

5) The signal generator sends a test pattern with the pattern outlined in figure 8.3.2.2.4.2-1. The following statistics are kept: the number of ACKs detected in the idle periods and the number of missed ACKs.

# Figure 8.3.2.2.4.2-1: Test signal pattern for PUCCH format 1 demodulation tests

8.3.2.2.5	Test Requirement

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of correctly detected ACK bits shall be larger than 99% for the SNR listed in tables 8.3.2.2.5-1 and table 8.3.2.2.5-2.

# Table 8.3.2.2.5-1 Required SNR for PUCCH format 1 with 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel Bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low |  |  |  |
|  | 4 | Normal | TDLC300-100 Low |  |  |  |
|  | 8 | Normal | TDLC300-100 Low |  |  |  |

# Table 8.3.2.2.5-2 Required SNR for PUCCH format 1 with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel Bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | Normal | TDLC300-100 Low |  |  |  |  |
|  | 4 | Normal | TDLC300-100 Low |  |  |  |  |
|  | 8 | Normal | TDLC300-100 Low |  |  |  |  |

8.3.3	Performance requirements for PUCCH format 2

8.3.3.1	ACK missed detection

8.3.3.1.1	Definition and applicability

The performance requirement of PUCCH format 2 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

8.3.3.1.2	Minimum requirements

The minimum requirements are in TS 38.104 [2] subclause 8.3.4.

8.3.3.1.3	Test purpose

The test shall verify the receiver’s ability to detect ACK bits under multipath fading propagation conditions for a given SNR.

8.3.3.1.4	Method of test

8.3.3.1.4.1	Initial Condition

Test environment: Normal, see annex B.2.

RF channels to be tested: for single carrier (SC): M; see subclause 4.9.1

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and type 1-H respectively.

8.3.3.1.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth defined in table 8.3.3.1.4.2-1.

# Table 8.3.3.1.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 | 5 | [-83.5] dBm / 4.5 MHz |
|  | 10 | [-80.3] dBm / 9.36 MHz |
|  | 20 | [-77.2] dBm / 19.08MHz |
| 30 | 10 | [-80.7] dBm / 8.64 MHz |
|  | 20 | [-77.4] dBm / 18.36 MHz |
|  | 40 | [-74.2] dBm / 38.16 MHz |
|  | 100 | [-70.1] dBm / 98.28 MHz |

2)	The characteristics of the wanted signal shall be configured according to TS 38.211 [17], and the specific test parameters are configured as blow:

# Table 8.3.3.1.4.2-2: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation | QPSK |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| nrofPRBs | 4 |
| nrofSymbols | 1 |
| the number of UCI bits | 4 |
| startingSymbolIndex | 13 |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex G.

4)	Adjust the equipment so that the SNR specified in table 8.3.3.1.5-1 and table 8.3.3.1.5-2 is achieved at the BS input during the UCI transmissions.

5)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.3.1.4.2-1. The following statistics are kept: the number of ACKs detected in the idle periods and the number of missed ACKs.

# Figure 8.3.3.1.4.2-1: Test signal pattern for PUCCH format 2 demodulation tests

8.3.3.1.5	Test requirements

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in table 8.3.3.1.5-1 and table 8.3.3.1.5-2.

# Table 8.3.3.1.5-1: Required SNR for PUCCH format 2 with 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |
|  | 4 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |
|  | 8 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |

# Table 8.3.3.1.5-2: Required SNR for PUCCH format 2 with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel Bandwidth/ SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10MHz | 20MHz | 40MHz | 100MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |
|  | 4 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |
|  | 8 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |

8.3.3.2	UCI BLER performance requirements

8.3.3.2.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. All UCI information shall be decoded.

8.3.3.2.2	Minimum Requirement

The minimum requirement is TS 38.104 [2] subclause 8.3.4.

8.3.3.2.3	Test purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

8.3.3.2.4	Method of test

8.3.3.2.4.1	Initial Condition

Test environment: Normal, see annex B.2.

RF channels to be tested: for single carrier (SC): M; see subclause 4.9.1

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and BS type 1-H respectively.

8.3.3.2.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth defined in table 8.3.3.2.4.2-1.

# Table 8.3.3.2.4.2-1: AWGN power level at the BS input

| Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 | 5 | [-83.5] dBm / 4.5MHz |
|  | 10 | [-80.3] dBm / 9.36MHz |
|  | 20 | [-77.2] dBm / 19.08MHz |
| 30 | 10 | [-80.7] dBm / 8.64MHz |
|  | 20 | [-77.4] dBm / 18.36MHz |
|  | 40 | [-74.2] dBm / 38.16MHz |
|  | 100 | [-70.1] dBm / 98.28MHz |

2)	The characteristics of the wanted signal shall be configured according to TS 38.211 [17], and the specific test parameters are configured as blow:

# Table 8.3.3.2.4.2-2: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation | QPSK |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| nrofPRBs | 9 |
| nrofSymbols | 2 |
| the number of UCI bits | 22 |
| startingSymbolIndex | 12 |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex G.

4)	Adjust the equipment so that the SNR specified in table 8.3.3.2.5-1 or table 8.3.3.2.5-2 is achieved at the BS input during the UCI transmissions.

5)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.3.2.4.2-1. The following statistics are kept: the number of incorrectly decoded UCI.

# Figure 8.3.3.2.4.2-1: Test signal pattern for PUCCH format 2 demodulation tests

8.3.3.2.5	Test requirements

The fraction of incorrectly decoded UCI shall be less than 1% for the SNR listed in table 8.3.3.2.5-1 and table 8.3.3.2.5-2.

# Table 8.3.3.2.5-1: Required SNR for PUCCH format 2 with 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |
|  | 4 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |
|  | 8 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |

# Table 8.3.3.2.5-2: Required SNR for PUCCH format 2 with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Channel Bandwidth/ SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10MHz | 20MHz | 40MHz | 100MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |
|  | 4 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |
|  | 8 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |

8.3.4	Performance requirements for PUCCH format 3

8.3.4.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. All UCI information shall be decoded.

8.3.4.2	Minimum requirement

The minimum requirement is in TS 38.104 [2] subclause 8.3.5.

8.3.4.3	Test purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

8.3.4.4	Method of test

8.3.4.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see sub-clause 4.9.1

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and BS type 1-H respectively.

8.3.4.4.2	Procedure

1)	Adjust the AWGN generator, according to the subcarrier spacing and channel bandwidth defined in table 8.3.4.4.2-1.

# Table 8.3.4.4.2-1: AWGN power level at the BS input

| Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 | 5 | [-83.5] dBm / 4.5MHz |
|  | 10 | [-80.3] dBm / 9.36MHz |
|  | 20 | [-77.2] dBm / 19.08MHz |
| 30 | 10 | [-80.7] dBm / 8.64MHz |
|  | 20 | [-77.4] dBm / 18.36MHz |
|  | 40 | [-74.2] dBm / 38.16MHz |
|  | 100 | [-70.1] dBm / 98.28MHz |

2)	The characteristics of the wanted signal shall be configured according to TS 38.211 [17]. The specific test parameters are configured as below:

# Table 8.3.4.4.2-2: Test parameters

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation | QPSK |  |
| startingPRB | 0 |  |
| intraSlotFrequencyHopping | enabled |  |
| secondHopPRB | The largest PRB index - nrofPRBs |  |
| nrofPRBs | 1 | 3 |
| nrofSymbols | 14 | 4 |
| the number of UCI bits | 16 | 16 |
| startingSymbolIndex | 0 | 0 |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex G.

4)	Adjust the equipment so that the SNR specified in table 8.3.4.5-1 or table 8.3.4.5-2 is achieved at the BS input during the UCI transmissions.

5)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.4.4.2-1. The following statistics are kept: the number of incorrectly decoded UCI.

# Figure 8.3.4.4.2-1: Test signal pattern for PUCCH format 3 demodulation tests

8.3.4.5	Test requirement

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.4.5-1 and table 8.3.4.5-2.

# Table 8.3.4.5-1: Required SNR for PUCCH format 3 with 15kHz SCS

| Test Number | Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DMRS configuration | Channel Bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | [2.0] | TBD |
|  |  |  |  |  | Additional DMRS | TBD | [1.7] | TBD |
|  |  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD |
|  |  |  |  |  | Additional DMRS | TBD | TBD | TBD |
|  |  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD |
|  |  |  |  |  | Additional DMRS | TBD | TBD | TBD |
| 2 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD |
|  |  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD |
|  |  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD |

# Table 8.3.4.5-2: Required SNR for PUCCH format 3 with 30kHz SCS

| Test Number | Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DMRS configuration | Channel Bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD | TBD |
|  |  |  |  |  | Additional DMRS | TBD | TBD | [1.3] | TBD |
|  |  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD | TBD |
|  |  |  |  |  | Additional DMRS | TBD | TBD | TBD | TBD |
|  |  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD | TBD |
|  |  |  |  |  | Additional DMRS | TBD | TBD | TBD | TBD |
| 2 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD | TBD |
|  |  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD | TBD |
|  |  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD | TBD |

8.3.5	Performance requirements for PUCCH format 4

8.3.5.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. All UCI information shall be decoded.

8.3.5.2	Minimum requirement

The minimum requirement is in TS 38.104 [2] sub-clause 8.3.6.

8.3.5.3	Test purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

8.3.5.4	Method of test

8.3.5.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see subclause 4.9.1

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and BS type 1-H respectively.

8.3.5.4.2	Procedure

1)	Adjust the AWGN generator, according to the subcarrier spacing and channel bandwidth defined in table 8.3.5.4.2-1.

# Table 8.3.5.4.2-1: AWGN power level at the BS input

| Subcarrier spacing(SCS) (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 kHz | 5 | [-83.5] dBm / 4.5MHz |
|  | 10 | [-80.3] dBm / 9.36MHz |
|  | 20 | [-77.2] dBm / 19.08MHz |
| 30 kHz | 10 | [-80.7] dBm / 8.64MHz |
|  | 20 | [-77.4] dBm / 18.36MHz |
|  | 40 | [-74.2] dBm / 38.16MHz |
|  | 100 | [-70.1] dBm / 98.28MHz |

2)	The characteristics of the wanted signal shall be configured according to TS 38.211 [17]. The test parameters are configured as below:

# Table 8.3.5.4.2-2: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation | QPSK |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| nrofSymbols | 14 |
| the number of UCI bits | 22 |
| startingSymbolIndex | 0 |
| occ-Length | n2 |
| occ-Index | n0 |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

4)	Adjust the equipment so that the SNR specified in table 8.3.5.5-1 or table 8.3.5.5-2 is achieved at the BS input during the UCI transmissions.

5)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.5.4.2-1. The following statistics are kept: the number of incorrectly decoded UCI.

# Figure 8.3.5.4.2-1: Test signal pattern for PUCCH format 4 demodulation tests

8.3.5.5	Test requirement

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.5.5-1 and table 8.3.5.5-2.

# Table 8.3.5.5-1: Required SNR for PUCCH format 4 with 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DMRS configuration | Channel Bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD |
|  |  |  |  | Additional DMRS | TBD | [3.9] | TBD |
|  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD |
|  |  |  |  | Additional DMRS | TBD | TBD | TBD |
|  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD |
|  |  |  |  | Additional DMRS | TBD | TBD | TBD |

CI idecoded CQIare configuredCQ only tested on the supported band or band combinations, and

# Table 8.3.5.5-2: Required SNR for PUCCH format 4 with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | Additional DMRS configuration | Channel Bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD | TBD |
|  |  |  |  | Additional DMRS | TBD | TBD | TBD | TBD |
|  | 4 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD | TBD |
|  |  |  |  | Additional DMRS | TBD | TBD | TBD | TBD |
|  | 8 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD | TBD |
|  |  |  |  | Additional DMRS | TBD | TBD | TBD | TBD |

8.4	Performance requirements for PRACH

8.4.1	PRACH false alarm probability and missed detection

8.4.1.1	Definition and applicability

The performance requirement of PRACH for preamble detection is determined by the two parameters: total probability of false detection of the preamble (Pfa) and the probability of detection of preamble (Pd). The performance is measured by the required SNR at probability of detection, Pd of 99%. Pfa shall be 0.1% or less.

Pfa is defined as a conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

Pd is defined as conditional probability of detection of the preamble when the signal is present. The erroneous detection consists of several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation.

For AWGN and TDLC300-100, a timing estimation error occurs if the estimation error of the timing of the strongest path is larger than the time error tolerance values given in table 8.4.1.1-1.

# Table 8.4.1.1-1: Time error tolerance for AWGN and TDLC300-100

| PRACH preamble | PRACH SCS (kHz) | Time error tolerance |  |
| --- | --- | --- | --- |
|  |  | AWGN | TDLC300-100 |
| 0 | 1.25 | 1.04 us | 2.55 us |
| A1, A2, A3, B4, C0, C2 | 15 | 0.52 us | 2.03 us |
|  | 30 | 0.26 us | 1.77 us |

The test preambles for normal mode are listed in table A.6-1.

8.4.1.2	Minimum requirement

The minimum requirement is in TS 38.104 [2] subclause 8.4.1.2 and 8.4.2.2.

8.4.1.3	Test purpose

The test shall verify the receiver’s ability to detect PRACH preamble under static conditions and multipath fading propagation conditions for a given SNR.

8.4.1.4	Method of test

8.4.1.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested: for single carrier (SC): M; see subclause 4.9.1.

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and BS type 1-H respectively.

8.4.1.4.2	Procedure

1)	Adjust the AWGN generator, according to the SCS and channel bandwidth.

# Table 8.4.1.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 | 5 | [-83.5] dBm / 4.5MHz |
|  | 10 | [-80.3] dBm / 9.36MHz |
|  | 20 | [-77.2] dBm / 19.08MHz |
| 30 | 10 | [-80.7] dBm / 8.64MHz |
|  | 20 | [-77.4] dBm / 18.36MHz |
|  | 40 | [-74.2] dBm / 38.16MHz |
|  | 100 | [-70.1] dBm / 98.28MHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A.

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

4)	Adjust the frequency offset of the test signal according to table 8.4.1.5-1 or 8.4.1.5-2 or 8.4.1.5-3.

5)	Adjust the equipment so that the SNR specified in table 8.4.1.5-1 or 8.4.1.5-2 or 8.4.1.5-3 is achieved at the BS input during the PRACH preambles.

6) 	The test signal generator sends a preamble and the receiver tries to detect the preamble. This pattern is repeated as illustrated in figure 8.4.1.4.2-1. The preambles are sent with certain timing offsets as described below. The following statistics are kept: the number of preambles detected in the idle period and the number of missed preambles.

# Figure 8.4.1.4.2-1: PRACH preamble test pattern

The timing offset base value for PRACH preamble format 0 is set to 50% of Ncs. This offset is increased within the loop, by adding in each step a value of 0.1us, until the end of the tested range, which is 0.9us. Then the loop is being reset and the timing offset is set again to 50% of Ncs. The timing offset scheme for PRACH preamble format 0 is presented in figure 8.4.1.4.2-2.

# Figure 8.4.1.4.2-2: Timing offset scheme for PRACH preamble format 0

The timing offset base value for PRACH preamble format A1, A2, A3, B4, C0 and C2 is set to 0. This offset is increased within the loop, by adding in each step a value of 0.1us, until the end of the tested range, which is 0.8 us. Then the loop is being reset and the timing offset is set again to 0. The timing offset scheme for PRACH preamble format A1, A2, A3, B4, C0 and C2 is presented in figure 8.4.1.4.2-3.

# Figure 8.4.1.4.2-3: Timing offset scheme for PRACH preamble format A1 A2, A3, B4, C0 and C2

< End of first change >