3GPP TSG-RAN WG4 Meeting #90	R4-1902388

Athens, GR, 25 Feb - 1 Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR |  | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial CR for BS radidated demodulation test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Correlation level for the propgation channel was missing for PUSCH and PRACH tests. - Cluase number for the propgation condition was TBD for some cases. - To align with TS 38.104, the applicability of different physcial channels can be captured in a common sub-clause. - In subcluse 8.1.1, it was stated that up to 1TX-nRX could be tested. However, for NR PUSCH, the demodulation requirements are defined for both 1Tx and 2Tx. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the propagation correlation level for PUSCH and PRACH tests. - Updated the cluase number for the propgation condition. - Removed the applicability in the requirements for each channel. - Removed the limitation on TX number in subcluse 8.1.1. - Other editorial modifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No need to introduce these editorial corrections. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Start of first change >

8	Radiated performance requirements

8.1	General

Radiated performance requirements specify the ability of the BS type 1-O or BS type 2-O to correctly demodulate radiated signals in various conditions and configurations. Radiated performance requirements are specified at the RIB.

Radiated performance requirements for the BS are specified for the fixed reference channels and propagation conditions defined in TS 38.104 [2] annex A and annex J, respectively. The requirements only apply to those FRCs that are supported by the BS.

The radiated performance requirements for BS type 1-O and for the BS type 2-O are limited to two OTA demodulations branches as described in subclause 8.1.1. Conformance requirements can only be tested for 1 or 2 demodulation branches depending on the number of polarizations supported by the BS, with the required SNR applied separately per polarization.

NOTE 1: BS can support more than 2 demodulation branches, however OTA conformance testing can only be performed for 1 or 2 demodulation branches.

Unless stated otherwise, radiated performance requirements apply for a single carrier only. Radiated performance requirements for a BS supporting CA are defined in terms of single carrier requirements.

For BS type 1-O in FDD operation the requirements in clause 8 shall be met with the transmitter units associated with the RIB in the operating band turned ON.

NOTE 2:	BS type 1-O in normal operating conditions in FDD operation is configured to transmit and receive at the same time. The transmitter unit(s) associated with the RIB may be OFF for some of the tests.

In tests performed with signal generators a synchronization signal may be provided from the BS to the signal generator, to enable correct timing of the wanted signal.

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S 		is the total signal energy in a slot on a RIB.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a slot.

8.1.1	OTA demodulation branches

Radiated performance requirements are only specified for up to 2 demodulation branches.

If the BS type 1-O, or the BS type 2-O uses polarization diversity and has the ability to maintain isolation between the signals for each of the demodulation branches, then radiated performance requirements can be tested for up to two demodulation branches (i.e. 1RX or 2RX test setups). When tested for two demodulation branches, each demodulation branch maps to one polarization.

If the BS type 1-O, or the BS type 2-O does not use polarization diversity then radiated performance requirements can only be tested for o a single demodulation branch (i.e. 1RX test setup).

8.2	OTA performance requirements for PUSCH

8.2.1	Performance requirements for PUSCH with transmission precoding disabled

8.2.1.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ re-transmissions.

8.2.1.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.1.1.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.2.1.

8.2.1.3	Test purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

8.2.1.4	Method of test

8.2.1.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested: M, see subclause 4.9.1.

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (D.31).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

8.2.1.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex [E.x.x].

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex [E.x.x]. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 8.2.1.4.2-1.

Table 8.2.1.4.2-1: Test parameters for testing PUSCH

| Parameter |  | BS type 1-O | BS type 2-O |
| --- | --- | --- | --- |
| Transform precoding |  | Disabled |  |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DMRS | DMRS configuration type | 1 |  |
|  | Maximum number of OFDM symbols for front loaded DMRS | 1 |  |
|  | Number of additional DMRS symbols | 0, 1 | 0 |
|  | Number of DMRS CDM group(s) without data | 2 |  |
|  | EPRE ratio of PUSCH to DMRS | -3 dB |  |
|  | DMRS port | {0}, {0,1} |  |
|  | DMRS sequence generation | NID=0, nSCID=0 |  |
| Time domain resource | PUSCH mapping type | A | B |
|  | PUSCH starting symbol index | 0 | 0 |
|  | PUSCH symbol length | 14 | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| TPMI index for 2Tx two layer spatial multiplexing transmission |  | 0 |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| PTRS configuration | Frequency density (KPT-RS) | N.A. | 2 |
|  | Time density (LPT-RS) | N.A. | 1 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.2.1.5.1 and 8.2.1.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in Table 8.2.1.4.2-2.

Table 8.2.1.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | [-83.5] - ΔOTAREFSENS dBm / 4.5MHz |
|  |  | 10 | [-80.3] - ΔOTAREFSENS dBm / 9.36MHz |
|  |  | 20 | [-77.2] - ΔOTAREFSENS dBm / 19.08MHz |
|  | 30 | 10 | [-80.7] - ΔOTAREFSENS dBm / 8.64MHz |
|  |  | 20 | [-77.4] - ΔOTAREFSENS dBm / 18.36MHz |
|  |  | 40 | [-74.2] - ΔOTAREFSENS dBm / 38.16MHz |
|  |  | 100 | [-70.1] - ΔOTAREFSENS dBm / 98.28MHz |
| BS type 2-O | 60 | 50 | TBD |
|  |  | 100 | TBD |
|  | 120 | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 200 | TBD |

8)	For reference channels applicable to the BS, measure the throughput.

8.2.1.5	Test Requirement

8.2.1.5.1	Test requirement for BS type 1-O

The throughput measured according to subclause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in Table 8.2.1.5.1-1 to Table 8.2.1.5.1-7 for 1Tx and for 2Tx two layer spatial multiplexing transmission.

Table 8.2.1.5.1-1 Test requirements for PUSCH, 5 MHz Channel Bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-1 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-8 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-1 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-8 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-1 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-8 | 1+1 | [TBD] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | 1+1 | [TBD] |

Table 8.2.1.5.1-2 Test requirements for PUSCH, 10 MHz Channel Bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-2 | 1+0 | [1.8] |
|  |  |  |  |  | G-FR1-A3-9 | 1+1 | [-1.6] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-2 | 1+0 | [13.7] |
|  |  |  |  |  | G-FR1-A4-9 | 1+1 | [11.1] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-2 | 1+0 | [13.8] |
|  |  |  |  |  | G-FR1-A5-9 | 1+1 | [13.4] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | 1+1 | [TBD] |

Table 8.2.1.5.1-3 Test requirements for PUSCH, 20 MHz Channel Bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-3 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-10 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-3 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-10 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-3 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-10 | 1+1 | [TBD] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | 1+1 | [TBD] |

Table 8.2.1.5.1-4 Test requirements for PUSCH, 10 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-4 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-11 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-4 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-11 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-4 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-11 | 1+1 | [TBD] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | 1+1 | [TBD] |

Table 8.2.1.5.1-5 Test requirements for PUSCH, 20 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-5 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-12 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-5 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-12 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-5 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-12 | 1+1 | [TBD] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | 1+1 | [TBD] |

Table 8.2.1.5.1-6 Test requirements for PUSCH, 40 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-6 | 1+0 | [-1.2] |
|  |  |  |  |  | G-FR1-A3-13 | 1+1 | [-1.9] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-6 | 1+0 | [11.5] |
|  |  |  |  |  | G-FR1-A4-13 | 1+1 | [11.0] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-6 | 1+0 | [12.7] |
|  |  |  |  |  | G-FR1-A5-13 | 1+1 | [TBD] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | 1+1 | [TBD] |

Table 8.2.1.5.1-7 Test requirements for PUSCH, 100 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-7 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A3-14 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-7 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A4-14 | 1+1 | [TBD] |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-7 | 1+0 | [TBD] |
|  |  |  |  |  | G-FR1-A5-14 | 1+1 | [TBD] |
| 2 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | 1+1 | [TBD] |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | 1+1 | [TBD] |

8.2.1.5.2	Test requirement for BS type 2-O

The throughput measured according to subclause 8. 2.1.4.2 shall not be below the limits for the SNR levels specified in table 8. 2.1.5.2-1 to 8.2.1.5.2-5.

Table 8.2.1.5.2-1 Test requirements for PUSCH, 50 MHz Channel Bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-75 Low | 70 % | [TBD] | 1+0 | [TBD] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |

Table 8.2.1.5.2-2 Test requirements for PUSCH, 100 MHz Channel Bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-75 Low | 70 % | [TBD] | 1+0 | [TBD] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |

Table 8.2.1.5.2-3 Test requirements for PUSCH, 50 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-75 Low | 70 % | [TBD] | 1+0 | [TBD] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |

Table 8.2.1.5.2-4 Test requirements for PUSCH, 100 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-75 Low | 70 % | [TBD] | 1+0 | [TBD] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |

Table 8.2.1.5.2-5 Test requirements for PUSCH, 200 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-75 Low | 70 % | [TBD] | 1+0 | [TBD] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |
|  |  | Normal | TDLA30-300 Low | 70 % | [TBD] | 1+0 | [TBD] |

8.2.2	Performance requirements for PUSCH with transmission precoding enabled

8.2.2.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

8.2.2.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.1.2.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.2.2.

8.2.2.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

8.2.2.4	Method of test

8.2.2.4.1	Initial Conditions

Test environment:	Normal, see subclause B.2.

RF channels to be tested:	M, see subclause 4.9.1.

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (D.31).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

8.2.2.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex [E.x.x].

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex [E.x.x]. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in Table 8.2.2.4.2-1.

Table 8.2.2.4.2-1: Test parameters for testing PUSCH

| Parameter |  | BS type 1-O | BS type 2-O |
| --- | --- | --- | --- |
| Transform precoding |  | Enabled |  |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DMRS | DMRS configuration type | 1 |  |
|  | Maximum number of OFDM symbols for front loaded DMRS | 1 |  |
|  | Number of additional DMRS symbols | 0, 1 | 0 |
|  | Number of DMRS CDM group(s) without data | 2 |  |
|  | EPRE ratio of PUSCH to DMRS | -3 dB |  |
|  | DMRS port | 0 |  |
|  | DMRS sequence generation | NID=0, group hopping and sequence hopping are disabled |  |
| Time domain resource | PUSCH mapping type | A | B |
|  | PUSCH starting symbol index | 0 | 0 |
|  | PUSCH symbol length | 14 | 10 |
| Frequency domain resource | RB assignment | 15 kHz SCS: 25 PRBs in the middle of the test bandwidth 30 kHz SCS: 24 PRBs in the middle of the test bandwidth | 30 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| PTRS configuration | Thresholds for frequency density | N.A. | NRB0=0, NRB1=8, NRB2=NRB3=32, NRB4=108 |
|  | Time density (LPT-RS) | N.A. | 1 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.2.2.5.1 and 8.2.2.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in Table 8.2.2.4.2-2.

Table 8.2.2.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | [-83.5] - ΔOTAREFSENS dBm / 4.5MHz |
|  | 30 kHz | 10 | [-80.7] - ΔOTAREFSENS dBm / 8.64MHz |
| BS type 2-O | 60 kHz | 50 | TBD |
|  | 120 kHz | 100 | TBD |

8)	For reference channels applicable to the BS, measure the throughput.

8.2.2.5	Test Requirement

8.2.2.5.1	Test requirement for BS type 1-O

The throughput measured according to subclause 8.2.2.4.2 shall not be below the limits for the SNR levels specified in Table 8.2.2.5.1-1 to Table 8.2.2.5.1-2.

Table 8.2.2.5.1-1 Test requirements for PUSCH, 5 MHz Channel Bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-29 | 1+0 | [2.8] |
|  |  |  |  |  | G-FR1-A3-31 | 1+1 | [TBD] |

Table 8.2.2.5.1-2 Test requirements for PUSCH, 10 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-30 | 1+0 | [-1.1] |
|  |  |  |  |  | G-FR1-A3-32 | 1+1 | [-2.0] |

8.2.2.5.2	Test requirement for BS type 2-O

The throughput measured according to subclause 8.2.2.4.2 shall not be below the limits for the SNR levels specified in Table 8.2.2.5.2-1 to Table 8.2.2.5.2-2.

Table 8.2.2.5.2-1 Test requirements for PUSCH, 50 MHz Channel Bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-11 | 1+0 | [TBD] |

Table 8.2.2.5.2-2 Test requirements for PUSCH, 50 MHz Channel Bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-12 | 1+0 | [TBD] |

8.3	OTA performance requirements for PUCCH

8.3.1	Performance requirements for PUCCH format 0

8.3.1.1	Definition and applicability

The performance requirement of single user PUCCH format 0 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

8.3.1.2	Minimum Requirement

For BS type 1-O, the minimum requirements are in TS 38.104 [2] subclause 11.3.1.1 and 11.3.1.2.

For BS type 2-O, the minimum requirements are in TS 38.104 [2] subclause 11.3.2.1 and 11.3.2.2.

8.3.1.3	Test purpose

The test shall verify the receiver’s ability to detect ACK under multipath fading propagation conditions for a given SNR.

8.3.1.4	Method of test

8.3.1.4.1	Initial Conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested:	single carrier (SC) M; see subclause 4.9.1.

Direction to be tested:

-  For BS type 1-O, receiver target reference direction (D.31).

-  For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

8.3.1.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex [E.x.x].

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex [E.x.x]. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20] and according to additional test parameters listed in Table 8.3.1.4.2-1.

Table 8.3.1.4.2-1: Test Parameters

| Parameter | BS type 1-O | BS type 2-O |
| --- | --- | --- |
| nrofBits | 1 | 1 |
| nrofPRBs | 1 | 1 |
| startingPRB | 0 | 0 |
| intraSlotFrequencyHopping | enabled | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs | The largest PRB index - nrofPRBs |
| initialCyclicShift | 0 | 0 |
| startingSymbolIndex | 13 for 1 symbol 12 for 2 symbols | 13 for 1 symbol 12 for 2 symbols |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.1.5.1 and 8.3.1.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level quoted in Table 8.3.1.4.2-2.

Table 8.3.1.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | TBD |
|  |  | 10 | TBD |
|  |  | 20 | TBD |
|  | 30 kHz | 10 | TBD |
|  |  | 20 | TBD |
|  |  | 40 | TBD |
|  |  | 100 | TBD |
| BS type 2-O | 60 kHz | 50 | TBD |
|  |  | 100 | TBD |
|  | 120 kHz | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 200 | TBD |

8)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.1.4.2-1. The following statistics are kept: the number of ACKs detected in the idle periods and the number of missed ACKs.

Figure 8.3.1.4.2-1: Test signal pattern for single user PUCCH format 0 demodulation tests

8.3.1.5	Test Requirement

8.3.1.5.1	Test requirement for BS type 1-O

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in Table 8.3.1.5.1-1 and in Table 8.3.1.5.1-2.

Table 8.3.1.5.1-1 Test requirements for PUCCH format 0 and 15kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (Annex J) | Number of OFDM symbols | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | TDLC-300-100 Low | 1 | TBD | [9.7] | TBD |
|  |  |  | 2 | TBD | TBD | TBD |

Table 8.3.1.5.1-2 Test requirements for PUCCH format 0 and 30kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (Annex J) | Number of OFDM symbols | Channel Bandwidth / SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | TDLC-300-100 Low | 1 | TBD | TBD | [11.5] | TBD |
|  |  |  | 2 | TBD | TBD | TBD | TBD |

8.3.1.5.2	Test requirement for BS type 2-O

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in Table 8.3.1.5.2-1 and in Table 8.3.1.5.2-2.

Table 8.3.1.5.2-1 Test requirements for PUCCH format 0 and 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (Annex J) | Number of OFDM symbols | Channel Bandwidth / SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | TDLA30-300 Low | 1 | TBD | TBD |
|  |  |  | 2 | TBD | TBD |

Table 8.3.1.5.2-2 Test requirements for PUCCH format 0 and 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (Annex J) | Number of OFDM symbols | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 2 | TDLA30-300 Low | 1 | TBD | TBD | TBD |
|  |  |  | 2 | TBD | TBD | TBD |

8.3.2	Performance requirements for PUCCH format 1

8.3.2.1	NACK to ACK detection

8.3.2.1.1	Definition and applicability

The performance requirement of PUCCH format 1 for NACK to ACK detection is determined by the two parameters: probability of false detection of the ACK and the NACK to ACK detection probability. The performance is measured by the required SNR at probability of the NACK to ACK detection equal to 0.1% or less. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK at particular bit position when input is only noise. Each false bit detection is counted as one error.

The NACK to ACK detection probability is the probability of detecting an ACK bit when an NACK bit was sent on particular bit position. Each NACK bit erroneously detected as ACK bit is counted as one error. Erroneously detected NACK bits in the definition do not contain the NACK bits which are mapped from DTX, i.e. NACK bits received when DTX is sent should not be considered.

8.3.2.1.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.1.3.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.2.3.

8.3.2.1.3	Test purpose

The test shall verify the receiver’s ability not to falsely detect NACK bits as ACK bits under multipath fading propagation conditions for a given SNR.

8.3.2.1.4	Method of test

8.3.2.1.4.1	Initial Conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see sub-clause 4.9.1

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (D.31).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

8.3.2.1.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex [E.x.x].

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex [E.x.x]. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in Table 8.3.2.1.4.2-1.

Table 8.3.2.1.4.2-1: Test parameters

| Parameter | Test |
| --- | --- |
| nrofBits | 2 |
| nrofPRBs | 1 |
| nrofSymbols | 14 |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| initialCyclicShift | 0 |
| startingSymbolIndex | 0 |
| Index of orthogonal sequence (time-domain-OCC) | 0 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.2.1.5.1 and 8.3.2.1.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in Table 8.3.2.1.4.2-2.

Table 8.3.2.1.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | [-83.5] dBm - ΔOTAREFSENS/ 4.5MHz |
|  |  | 10 | [-80.3] dBm - ΔOTAREFSENS/ 9.36MHz |
|  |  | 20 | [-77.2] dBm - ΔOTAREFSENS/ 19.08MHz |
|  | 30 kHz | 10 | [-80.7] dBm - ΔOTAREFSENS/ 8.64MHz |
|  |  | 20 | [-77.4] dBm - ΔOTAREFSENS/ 18.36MHz |
|  |  | 40 | [-74.2] dBm - ΔOTAREFSENS/ 38.16MHz |
|  |  | 100 | [-70.1] dBm - ΔOTAREFSENS/ 98.28MHz |
| BS type 2-O | 60 kHz | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 200 | TBD |

8)	The signal generator sends random codeword from applicable codebook, in regular time periods. The following statistics are kept: the number of ACK bits detected in the idle periods and the number of NACK bits detected as ACK.

8.3.2.1.5	Test Requirement

8.3.2.1.5.1 	Test Requirement for BS type 1-O

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of NACK bits falsely detected as ACK shall be less than 0.1% for the SNR listed in tables 8.3.2.1.5.1-1 and table 8.3.2.1.5.1-2.

Table 8.3.2.1.5.1-1 Required SNR for PUCCH format 1 with 15 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low |  |  |  |

Table 8.3.2.1.5.1-2 Required SNR for PUCCH format 1 with 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | Normal | TDLC300-100 Low |  |  |  |  |

8.3.2.1.5.2 	Test Requirement for BS type 2-O

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of NACK bits falsely detected as ACK shall be less than 0.1% for the SNR listed in Tables 8.3.2.1.5.2-1 and Table 8.3.2.1.5.2-2.

Table 8.3.2.1.5.2-1 Required SNR for PUCCH format 1 with 60 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth / SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | Normal | TDLA30-300 Low |  |  |

Table 8.3.2.1.5.2-2 Required SNR for PUCCH format 1 with 120 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 2 | Normal | TDLA30-300 Low |  |  |  |

8.3.2.2	ACK missed detection

8.3.2.2.1	Definition and applicability

The performance requirement of PUCCH format 1 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

8.3.2.2.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.1.3.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.2.3.

8.3.2.2.3	Test purpose

The test shall verify the receiver’s ability to detect ACK bits under multipath fading propagation conditions for a given SNR.

8.3.2.2.4	Method of test

8.3.2.2.4.1	Initial Conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see sub-clause 4.9.1

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (D.31).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

8.3.2.2.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex [E.x.x].

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex [E.x.x]. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in Table 8.3.2.2.4.2-1.

Table 8.3.2.2.4.2-1: Test Parameters

| Parameter | Test |
| --- | --- |
| nrofBits | 2 |
| nrofPRBs | 1 |
| nrofSymbols | 14 |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| initialCyclicShift | 0 |
| startingSymbolIndex | 0 |
| Index of orthogonal sequence (time-domain-OCC) | 0 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.2.2.5.1 and 8.3.2.2.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in Table 8.3.2.2.4.2-2.

Table 8.3.2.2.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | [-83.5] dBm - ΔOTAREFSENS/ 4.5MHz |
|  |  | 10 | [-80.3] dBm - ΔOTAREFSENS/ 9.36MHz |
|  |  | 20 | [-77.2] dBm - ΔOTAREFSENS/ 19.08MHz |
|  | 30 kHz | 10 | [-80.7] dBm - ΔOTAREFSENS/ 8.64MHz |
|  |  | 20 | [-77.4] dBm - ΔOTAREFSENS/ 18.36MHz |
|  |  | 40 | [-74.2] dBm - ΔOTAREFSENS/ 38.16MHz |
|  |  | 100 | [-70.1] dBm - ΔOTAREFSENS/ 98.28MHz |
| BS type 2-O | 60 kHz | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 200 | TBD |

8) The signal generator sends a test pattern with the pattern outlined in figure 8.3.2.2.4.2-1. The following statistics are kept: the number of ACKs detected in the idle periods and the number of missed ACKs.

Figure 8.3.2.2.4.2-1: Test signal pattern for PUCCH format 1 demodulation tests

8.3.2.2.5	Test Requirement

8.3.2.1.5.1 	Test Requirement for BS type 1-O

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of correctly detected ACK bits shall be larger than 99% for the SNR listed in Tables 8.3.2.2.5-1 and Table 8.3.2.2.5-2.

Table 8.3.2.2.5.1-1 Required SNR for PUCCH format 1 with 15 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low |  |  |  |

Table 8.3.2.2.5.1-2 Required SNR for PUCCH format 1 with 30 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | Normal | TDLC300-100 Low |  |  |  |  |

8.3.2.2.5.2 	Test Requirement for BS type 2-O

The fraction of NACK bits falsely detected as ACK shall be less than 0.1% for the SNR listed in Tables 8.3.2.2.5.2-1 and Table 8.3.2.2.5.2-2.

Table 8.3.2.2.5.2-1 Required SNR for PUCCH format 1 with 60 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth / SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | Normal | TDLA30-300 Low |  |  |

Table 8.3.2.2.5.2-2 Required SNR for PUCCH format 1 with 120 kHz SCS

| Number of TX antennas | Number of Demodulation Branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 2 | Normal | TDLA30-300 Low |  |  |  |

8.3.3	Performance requirements for PUCCH format 2

8.3.3.1	ACK missed detection performance requirements

8.3.3.1.1	Definition and applicability

The performance requirement of PUCCH format 2 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK on the wanted signal. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

8.3.3.1.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2] subclause 11.3.1.4.

For BS type 2-O, the minimum requirement is in TS 38.104 [2] subclause 11.3.2.4.

8.3.3.1.3	Test Purpose

The test shall verify the receiver’s ability to detect ACK bits under multipath fading propagation conditions for a given SNR.

8.3.3.1.4	Method of test

8.3.3.1.4.1	Initial conditions

Test environment:	Normal, see subclause B.2.

RF channels to be tested:	for single carrier (SC): M; see subclause 4.9.1

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (D.31).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

8.3.3.1.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex [E.x.x].

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex [E.x.x]. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in table 8.3.3.1.4.2-1.

Table 8.3.3.1.4.2-1: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation | QPSK |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| nrofPRBs | 4 |
| nrofSymbols | 1 |
| the number of UCI bits | 4 |
| startingSymbolIndex | 13 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.3.1.5.1 and 8.3.3.1.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.3.3.1.4.2-2.

Table 8.3.3.1.4.2-2 AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | [-83.5] - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | [-80.3] - ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | [-77.2] -ΔOTAREFSENS dBm / 19.08MHz |
|  | 30 kHz | 10 | [-80.7] - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | [-77.4] - ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | [-74.2] - ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | [-70.1] - ΔOTAREFSENS dBm / 98.28 MHz |
| BS type 2-O | 60 kHz | 50 | [TBD] dBm / TBD MHz |
|  |  | 100 | [TBD] dBm / TBD MHz |
|  | 120 kHz | 50 | [TBD] dBm / TBD MHz |
|  |  | 100 | [TBD] dBm / TBD MHz |
|  |  | 200 | [TBD] dBm / TBD MHz |

8)	The signal generator sends a test pattern with pattern outlined in figure 8.3.3.1.4.2-1. The following statistics are kept: the number of ACK bits detected in the idle periods and the number of missed ACKs.

Figure 8.3.3.1.4.2-1: Test signal pattern for PUCCH format 2 demodulation tests

8.3.3.1.5	Test requirement

8.3.3.1.5.1	Requirements for BS type 1-O

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in table 8.3.3.1.5.1-1 and table 8.3.3.1.5.1-2.

Table 8.3.3.1.5.1-1: Required SNR for PUCCH format 2 with 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |

Table 8.3.3.1.5.1-2: Required SNR for PUCCH format 2 with 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth/ SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10MHz | 20MHz | 40MHz | 100MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |

8.3.3.1.5.2	Requirements for BS type 2-O

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in table 8.3.3.1.5.2-1 and table 8.3.3.1.5.2.-2

Table 8.3.3.1.5.2-1: Required SNR for PUCCH format 2 with 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth / SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | [TBD] | [TBD] |

Table 8.3.3.1.5.2-2: Required SNR for PUCCH format 2 with 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | [TBD] | [TBD] | [TBD] |

8.3.3.2	UCI BLER performance requirements

8.3.3.2.1	Definition and applicability

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. All UCI information shall be decoded.

8.3.3.2.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2] subclause 11.3.1.4.

For BS type 2-O, the minimum requirement is in TS 38.104 [2] subclause 11.3.2.4.

8.3.3.2.3	Test Purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

8.3.3.2.4	Method of test

8.3.3.2.4.1	Initial conditions

Test environment:	Normal, see subclause B.2.

RF channels to be tested:	for single carrier (SC): M; see subclause 4.9.1

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (D.31).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

8.3.3.2.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex [E.x.x].

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex [E.x.x]. Each of the demodulation branches signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in table 8.3.3.2.4.2-1.

Table 8.3.3.2.4.2-1: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation | QPSK |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| nrofPRBs | 9 |
| nrofSymbols | 2 |
| the number of UCI bits | 22 |
| startingSymbolIndex | 12 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.3.2.5.1 and 8.3.3.2.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.3.3.2.4.2-2.

Table 8.3.3.2.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | [-83.5] - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | [-80.3] - ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | [-77.2] -ΔOTAREFSENS dBm/ 19.08MHz |
|  | 30 kHz | 10 | [-80.7] - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | [-77.4] - ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | [-74.2] - ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | [-70.1] - ΔOTAREFSENS dBm / 98.28 MHz |
| BS type 2-O | 60 kHz | 50 | [TBD] dBm / TBD MHz |
|  |  | 100 | [TBD] dBm / TBD MHz |
|  | 120 kHz | 50 | [TBD] dBm / TBD MHz |
|  |  | 100 | [TBD] dBm / TBD MHz |
|  |  | 200 | [TBD] dBm / TBD MHz |

8)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.3.2.4.2-1. The following statistics are kept: the number of incorrectly decoded UCI.

Figure 8.3.3.2.4.2-1: Test signal pattern for PUCCH format 2 demodulation tests

8.3.3.2.5	Test requirement

8.3.3.2.5.1	Requirements for BS type 1-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.3.2.5.1-1 and table 8.3.3.2.5.1-2.

Table 8.3.3.2.5.1-1: Required SNR for PUCCH format 2 with 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |

Table 8.3.3.2.5.1-2: Required SNR for PUCCH format 2 with 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth/ SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10MHz | 20MHz | 40MHz | 100MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |

8.3.3.2.5.2	Requirements for BS type 2-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.3.2.5.2-1 and table 8.3.3.2.5.2-2.

Table 8.3.3.2.5.2-1: Required SNR for PUCCH format 2 with 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth / SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | [TBD] | [TBD] |

Table 8.3.3.2.5.2-2: Required SNR for PUCCH format 2 with 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | [TBD] | [TBD] | [TBD] |

8.3.4	Performance requirements for PUCCH format 3

8.3.4.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. All UCI information shall be decoded.

8.3.4.2	Minimum requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.1.5.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.2.5.

8.3.4.3	Test purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

8.3.4.4	Method of test

8.3.4.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see sub-clause 4.9.1

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (D.31).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

8.3.4.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex [E.x.x].

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex [E.x.x]. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in Table 8.3.4.4.2-1.

Table 8.3.4.4.2-1: Test parameters

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation | QPSK |  |
| startingPRB | 0 |  |
| intraSlotFrequencyHopping | enabled |  |
| secondHopPRB | The largest PRB index - nrofPRBs |  |
| nrofPRBs | 1 | 3 |
| nrofSymbols | 14 | 4 |
| the number of UCI bits | 16 | 16 |
| startingSymbolIndex | 0 | 0 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.4.5.1 and 8.3.4.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in Table 8.3.4.4.2-2.

Table 8.3.4.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | [-83.5] dBm - ΔOTAREFSENS/ 4.5MHz |
|  |  | 10 | [-80.3] dBm - ΔOTAREFSENS/ 9.36MHz |
|  |  | 20 | [-77.2] dBm - ΔOTAREFSENS/ 19.08MHz |
|  | 30 kHz | 10 | [-80.7] dBm - ΔOTAREFSENS/ 8.64MHz |
|  |  | 20 | [-77.4] dBm - ΔOTAREFSENS/ 18.36MHz |
|  |  | 40 | [-74.2] dBm - ΔOTAREFSENS/ 38.16MHz |
|  |  | 100 | [-70.1] dBm - ΔOTAREFSENS/ 98.28MHz |
| BS type 2-O | 60 kHz | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 200 | TBD |

8.3.4.5	Test requirement

8.3.4.5.1	Test requirement for BS type 1-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.4.5.1-1 and table 8.3.4.5.1-2.

Table 8.3.4.5.1-1: Required SNR for PUCCH format 3 with 15kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DMRS configuration | Channel Bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | [2.0] | TBD |
|  |  |  |  |  | Additional DMRS | TBD | [1.7] | TBD |
| 2 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD |

Table 8.3.4.5.1-2: Required SNR for PUCCH format 3 with 30kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DMRS configuration | Channel Bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD | TBD |
|  |  |  |  |  | Additional DMRS | TBD | TBD | [1.3] | TBD |
| 2 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD | TBD |

8.3.4.5.2	Test requirement for BS type 2-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.4.5.2-1 and table 8.3.4.5.2-2.

Table 8.3.4.5.2-1: Required SNR for PUCCH format 3 with 60kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DMRS configuration | Channel Bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 50 MHz | 100 MHz |
| 1 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | TBD | TBD |
| 2 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | TBD | TBD |

Table 8.3.4.5.2-2: Required SNR for PUCCH format 3 with 120kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DMRS configuration | Channel Bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | TBD | TBD | TBD |
| 2 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | TBD | TBD | TBD |

8.3.5	Performance requirements for PUCCH format 4

8.3.5.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. All UCI information shall be decoded.

8.3.5.2	Minimum requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.1.6.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.2.6.

8.3.5.3	Test purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

8.3.5.4	Method of test

8.3.5.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see subclause 4.9.1

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (D.31).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

8.3.5.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex [E.x.x].

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex [E.x.x]. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in Table 8.3.4.4.2-1.

Table 8.3.5.4.2-1: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation | QPSK |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| nrofSymbols | 14 |
| the number of UCI bits | 22 |
| startingSymbolIndex | 0 |
| occ-Length | n2 |
| occ-Index | n0 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.5.5.1 and 8.3.5.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in Table 8.3.5.4.2-2.

Table 8.3.5.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing(SCS) (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | [-83.5] dBm - ΔOTAREFSENS/ 4.5MHz |
|  |  | 10 | [-80.3] dBm - ΔOTAREFSENS/ 9.36MHz |
|  |  | 20 | [-77.2] dBm - ΔOTAREFSENS/ 19.08MHz |
|  | 30 kHz | 10 | [-80.7] dBm - ΔOTAREFSENS/ 8.64MHz |
|  |  | 20 | [-77.4] dBm - ΔOTAREFSENS/ 18.36MHz |
|  |  | 40 | [-74.2] dBm - ΔOTAREFSENS/ 38.16MHz |
|  |  | 100 | [-70.1] dBm - ΔOTAREFSENS/ 98.28MHz |
| BS type 2-O | 60 kHz | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 200 | TBD |

8.3.5.5	Test requirement

8.3.5.5.1	Test requirement for BS type 1-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.5.5.1-1 and table 8.3.5.5.1-2.

Table 8.3.5.5.1-1: Required SNR for PUCCH format 4 with 15kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DMRS configuration | Channel Bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD |
|  |  |  |  | Additional DMRS | TBD | [3.9] | TBD |

Table 8.3.5.5.1-2: Required SNR for PUCCH format 4 with 30kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DMRS configuration | Channel Bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | TBD | TBD | TBD |
|  |  |  |  | Additional DMRS | TBD | TBD | TBD | TBD |

8.3.5.5.2	Test requirement for BS type 2-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.5.5.2-1 and table 8.3.5.5.2-2.

Table 8.3.5.5.2-1: Required SNR for PUCCH format 4 with 60kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DMRS configuration | Channel Bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | TBD | TBD |

CI idecoded CQIare configuredCQ only tested on the supported band or band combinations, and

Table 8.3.5.5.2-2: Required SNR for PUCCH format 4 with 120kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DMRS configuration | Channel Bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 50 MHz | 100 MHz | 200MHz |
| 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | TBD | TBD | TBD |

< End of first change >