3GPP TSG-RAN WG4 Meeting #90	R4-1902391

# Athens, Greece, 25 Feb – 1 Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | DraftCR | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR for 38.141-2: Radiated test requirements for CP-OFDM based PUSCH in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Demodulation requirements for PUSCH in section 8 are incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This draftCR introduces several editorial changes, removes the applicability rules from chapter 8.2.1.1 and introduced correlation matrix definitions. Tables with test requirements are not yet finished and will be updated during the meeting, when agreements about SNR values and other test settings are available. Agenda Item: 6.14.2.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | 38.104 |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 28.141-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<Start of first change>>

## 8.2	OTA performance requirements for PUSCH

### 8.2.1	Performance requirements for PUSCH with transform precoding disabled

#### 8.2.1.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ re-transmissions.

Which specific test is applicable to BS is based on the test applicability rule defined in section TBD.

#### 8.2.1.2	Minimum Requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.1.1.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.2.2.1.

#### 8.2.1.3	Test purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

#### 8.2.1.4	Method of test

##### 8.2.1.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested: M, see subclause 4.9.1.

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (D.31).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (D.54).

##### 8.2.1.4.2	Procedure

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex [E.x.x].

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex [E.x.x]. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 8.2.1.4.2-1.

Table 8.2.1.4.2-1: Test parameters for testing PUSCH

| Parameter |  | BS type 1-O | BS type 2-O |
| --- | --- | --- | --- |
| Transform precoding |  | Disabled |  |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DMRS | DMRS configuration type | 1 |  |
|  | Maximum number of OFDM symbols for front loaded DMRS | 1 |  |
|  | Number of additional DMRS symbols | 0, 1 | 0 |
|  | Number of DMRS CDM group(s) without data | 2 |  |
|  | EPRE ratio of PUSCH to DMRS | -3 dB |  |
|  | DMRS port | {0}, {0,1} |  |
|  | DMRS sequence generation | NID=0, nSCID=0 |  |
| Time domain resource | PUSCH mapping type | A | B |
|  | PUSCH starting symbol index | 0 | 0 |
|  | PUSCH symbol length | 14 | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| TPMI index for 2Tx two layer spatial multiplexing transmission |  | 0 |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| PTRS configuration | Frequency density (KPT-RS) | N.A. | 2 |
|  | Time density (LPT-RS) | N.A. | 1 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.2.1.5.1 and 8.2.1.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in Table 8.2.1.4.2-2.

Table 8.2.1.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 | 5 | [-83.5] - ΔOTAREFSENS dBm / 4.5MHz |
|  |  | 10 | [-80.3] - ΔOTAREFSENS dBm / 9.36MHz |
|  |  | 20 | [-77.2] - ΔOTAREFSENS dBm / 19.08MHz |
|  | 30 | 10 | [-80.7] - ΔOTAREFSENS dBm / 8.64MHz |
|  |  | 20 | [-77.4] - ΔOTAREFSENS dBm / 18.36MHz |
|  |  | 40 | [-74.2] - ΔOTAREFSENS dBm / 38.16MHz |
|  |  | 100 | [-70.1] - ΔOTAREFSENS dBm / 98.28MHz |
| BS type 2-O | 60 | 50 | TBD |
|  |  | 100 | TBD |
|  | 120 | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 200 | TBD |

8)	For reference channels applicable to the BS, measure the throughput.

#### 8.2.1.5	Test Requirement

##### 8.2.1.5.1	Test requirement for BS type 1-O

The throughput measured according to subclause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in Table 8.2.1.5.1-1 to Table 8.2.1.5.1-7 for 1Tx and for 2Tx two layer spatial multiplexing transmission.

Table 8.2.1.5.1-1: Test requirements for PUSCH, 5 MHz Channel Bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-1 |  | 1+0 |  | [2,68] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A3-8 |  | 1+1 |  | [-1,39] |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-1 |  | 1+0 |  | [13,37] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A4-8 |  | 1+1 |  | [11,14] |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-1 |  | 1+0 |  | [14,39] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A5-8 |  | 1+1 |  | [14,29] |
| 2 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-22 |  | 1+1 |  | [TBD] |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-22 |  | 1+1 |  | [18,89] |

Table 8.2.1.5.1-2: Test requirements for PUSCH, 10 MHz Channel Bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-2 |  | 1+0 |  | [1,67] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A3-9 |  | 1+1 |  | [-1,91] |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-2 |  | 1+0 |  | [13,37] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A4-9 |  | 1+1 |  | [11,14] |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-2 |  | 1+0 |  | [13,82] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A5-9 |  | 1+1 |  | [14,05] |
| 2 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-23 |  | 1+1 |  | [1,2] |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-23 |  | 1+1 |  | [19,1] |

Table 8.2.1.5.1-3: Test requirements for PUSCH, 20 MHz Channel Bandwidth, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-3 |  | 1+0 |  | [TBD] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A3-10 |  | 1+1 |  | [-1,20] |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-3 |  | 1+0 |  | [14,16] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A4-10 |  | 1+1 |  | [11,13] |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-3 |  | 1+0 |  | [14,08] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A5-10 |  | 1+1 |  | [13,73] |
| 2 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-24 |  | 1+1 |  | [TBD] |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-24 |  | 1+1 |  | [19,17] |

Table 8.2.1.5.1-4: Test requirements for PUSCH, 10 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-4 |  | 1+0 |  | [-1,20] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A3-11 |  | 1+1 |  | [-1,89] |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-4 |  | 1+0 |  | [11,32] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A4-11 |  | 1+1 |  | [11,08] |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-4 |  | 1+0 |  | [13,22] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A5-11 |  | 1+1 |  | [13,44] |
| 2 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-25 |  | 1+1 |  | [1,92] |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-25 |  | 1+1 |  | [19,12] |

Table 8.2.1.5.1-5: Test requirements for PUSCH, 20 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-5 |  | 1+0 |  | [-1,43] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A3-12 |  | 1+1 |  | [-2,27] |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-5 |  | 1+0 |  | [11,48] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A4-12 |  | 1+1 |  | [11,00] |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-5 |  | 1+0 |  | [13,39] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A5-12 |  | 1+1 |  | [13,32] |
| 2 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-26 |  | 1+1 |  | [1,80] |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-26 |  | 1+1 |  | [19,88] |

Table 8.2.1.5.1-6: Test requirements for PUSCH, 40 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-6 |  | 1+0 |  | [-1,09] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A3-13 |  | 1+1 |  | [-2,13] |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-6 |  | 1+0 |  | [11,33] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A4-13 |  | 1+1 |  | [10,77] |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-6 |  | 1+0 |  | [12,75] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A5-13 |  | 1+1 |  | [12,73] |
| 2 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-27 |  | 1+1 |  | [1,88] |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-27 |  | 1+1 |  | [19,97] |

Table 8.2.1.5.1-7: Test requirements for PUSCH, 100 MHz Channel Bandwidth, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex J) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-7 |  | 1+0 |  | [-0,49] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A3-14 |  | 1+1 |  | [-2,32] |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-7 |  | 1+0 |  | [11,55] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A4-14 |  | 1+1 |  | [10,88] |
|  |  |  |  | Normal |  | TDLA30-10 Low |  | 70 % |  | G-FR1-A5-7 |  | 1+0 |  | [TBD] |
|  |  |  |  |  |  |  |  |  |  | G-FR1-A5-14 |  | 1+1 |  | [TBD] |
| 2 |  | 2 |  | Normal |  | TDLB100-400 Low |  | 70 % |  | G-FR1-A3-28 |  | 1+1 |  | [1,83] |
|  |  |  |  | Normal |  | TDLC300-100 Low |  | 70 % |  | G-FR1-A4-28 |  | 1+1 |  | [19,47] |

<<End of first change>>