3GPP TSG RAN WG4 #90	R4-1902408

# Athens, Greece, 25 Feb 2019 - 1 Mar, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | CRNum | rev | 0 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Beamforming Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-02-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Beamforming model for CSI-RS is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Beamforming model for CSI-RS was defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for Demodulation and CSI reporting will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-4 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

# B.4	Beamforming Model

## B.4.1	Generic beamforming model

The transmission on antenna port(s) p=p0, p0+1,..., p0+ν-1 is defined by using a precoder matrix  of size NRS×υ, where NRS  is the number of ports for a reference signal and corresponding physical channel (if applicable) configured per test,  p0 is the first port for that reference signal and υ is the number of spatial layers. This precoder takes as an input a block of signals for antenna port(s) p=p0,p0+1,...,p0+ν-1, ypi=yp0i  yp0+1i …   yp0+ν-1iT, , with  being the number of modulation symbols per antenna port including the reference signal symbols, and generates a block of signals ybfqi=ybf0i  ybf1i …   ybfNRS-1iT the elements of which are to be mapped onto the frequency-time index pair as per the test configuration but transmitted on different physical antenna elements:

ybfqi=Wiyp(i)

The precoder matrix is specific to a test case and chosen from PMI matrices defined in Section 5.2.2.2 in TS 38.214 [X].

The physical antenna elements are identified by indices , where NANT is the number of physical antenna elements configured per test.

<< End of changes >>