3GPP TSG-RAN4 Meeting #90	R4-1902413

# Athens, GR,  25 Feb – 1 Mar, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | × | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for PMI test cases: 6.2, 8.2, A.3.2.2.2, A.3.2.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN4#89, one additional FR2 PMI test case with TDD DL-UL pattern FR Following RAN1 specification for minimum CSI calculation time, CSI delay for PMI test cases also need to be updated. The measurement channel for PMI test cases still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add FR2 PMI test case with TDD DL-UL pattern FR2.120-1 (DDDSU) - Revise CSI delay values - FR1 FDD 4Tx PMI test case in 6.3.2.1.1, 6.3.3.1.1 8ms->6ms - FR1 TDD 4Tx PMI test case in 6.3.2.2.1, 6.3.3.2.1 9.5ms->5.5ms - FR1 TDD 8Tx PMI test case in 6.3.2.2.2, 6.3.3.2.2 9.5ms->6.5ms - Add measurement channel for PMI test cases - Update meaurement channel for FR1 TDD PMI test cases, CSI-RS slot interval 5->10 - Add measurement channel for FR2 PMI test cases in A.3.2.2.5 - Add requirements for 8Tx PMI test cases as [1.5] - Add requirements for 2Tx as [1.05] and ULA Low MIMO correlation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PMI requirements cannot be verified properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2, 8.2, A.3.2.2.2, A.3.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | × | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | × |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | × | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< ------------------------------Start of change ------------------------------- >

## 6.3	Reporting of Precoding Matrix Indicator (PMI)

<Editor’s note: The requirements were introduced based on current results from companies; these requirements can be revised based on more results from companies.>

The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reports compared to the case when the transmitter is using random precoding, respectively. When the transmitter uses random precoding, for each PDSCH allocation a precoder is randomly generated and applied to the PDSCH. A fixed transport format (FRC) is configured for all requirements.

The requirements for transmission mode 1 with higher layer parameter codebookType set to ‘typeI-SinglePanel’ are specified in terms of the ratio:

In the definition of γ, for 4TX and 8TX PMI requirements, is [90] % of the maximum throughput obtained at  using the precoders configured according to the UE reports, and is the throughput measured at with random precoding.

### 6.3.1	1RX requirements

(Void)

### 6.3.2	2RX requirements

#### 6.3.2.1	FDD

##### 6.3.2.1.1	Single PMI with 4TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.2.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.1-2.

Table 6.3.2.1.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Duplex Mode |  |  | FDD |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB |  | 52 |
|  | Subcarrier spacing | kHz | 15 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 2 (N1,N2) = (2,1) |
| Beamforming Model |  |  | TBD |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) |
|  | CSI-RS interval and offset |  | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 5/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| CSI-Report interval and offset |  | slot | 5/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.1 FDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-3)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+3)]. Note 3: Randomization of the principle beam direction shall be used as specified in TBD. |  |  |  |

Table 6.3.2.1.1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.3] |

##### 6.3.2.1.2	Single PMI with 8TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.2.1.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.2-2.

Table 6.3.2.1.2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Duplex Mode |  |  | FDD |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB |  | 52 |
|  | Subcarrier spacing | kHz | 15 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 2 (N1,N2) = (4,1) |
| Beamforming Model |  |  | TBD |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 5/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| CSI-Report interval and offset |  | slot | 5/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.2 FDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-4)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+4)]. Note 3: Randomization of the principle beam direction shall be used as specified in TBD. |  |  |  |

Table 6.3.2.1.2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.5] |

#### 6.3.2.2	TDD

##### 6.3.2.2.1	Single PMI with 4TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.2.2.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.2.1-2.

Table 6.3.2.2.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configuration |  |  | FR1.30-1 as specified in Annex A |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB |  | 106 |
|  | Subcarrier spacing | kHz | 30 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 2 (N1,N2) = (2,1) |
| Beamforming Model |  |  | TBD |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 10/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| CSI-Report interval and offset |  | slot | 10/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 5.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.1 TDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot #n based on PMI estimation at a downlink slot not later than slot#[(n-4)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+4)]. Note 3: Randomization of the principle beam direction shall be used as specified in TBD. |  |  |  |

Table 6.3.2.2.1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.3] |

##### 6.3.2.2.2	Single PMI with 8TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.2.2.2-1, and using the downlink physical channels specified in Annex TBD, the minimum requirements are specified in Table 6.3.2.2.2-2.

Table 6.3.2.2.2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB |  | 106 |
|  | Subcarrier spacing | kHz | 30 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 2 (N1,N2) = (4,1) |
| Beamforming Model |  |  | TBD |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 10/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForIChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| CSI-Report interval and offset |  | slot | 10/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.2 TDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-6)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+6)]. Note 3: Randomization of the principle beam direction shall be used as specified in TBD. |  |  |  |

Table 6.3.2.2.2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.5] |

### 6.3.3	4RX requirements

#### 6.3.3.1	FDD

##### 6.3.3.1.1	Single PMI with 4TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.1-2.

Table 6.3.3.1.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Duplex Mode |  |  | FDD |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB |  | 52 |
|  | Subcarrier spacing | kHz | 15 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 4 (N1,N2) = (2,1) |
| Beamforming Model |  |  | TBD |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 5/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| CSI-Report interval and offset |  | slot | 5/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.1 FDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-3)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+3)]. Note 3: Randomization of the principle beam direction shall be used as specified in TBD. |  |  |  |

Table 6.3.3.1.1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.3] |

##### 6.3.3.1.2	Single PMI with 8TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.1.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.3.1.2-2.

Table 6.3.3.1.2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Duplex Mode |  |  | FDD |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB |  | 52 |
|  | Subcarrier spacing | kHz | 15 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 4 (N1,N2) = (4,1) |
| Beamforming Model |  |  | TBD |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 5/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| CSI-Report interval and offset |  | slot | 5/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-6.2 FDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-4)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+4)]. Note 3: Randomization of the principle beam direction shall be used as specified in TBD. |  |  |  |

Table 6.3.3.1.2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.5] |

#### 6.3.3.2	TDD

##### 6.3.3.2.1	Single PMI with 4TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.2.1-1, and using the downlink physical channels specified in Annex TBD, the minimum requirements are specified in Table 6.3.3.2.1-2.

Table 6.3.3.2.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configuration |  |  | FR1.30-1 as specified in Annex A |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB |  | 106 |
|  | Subcarrier spacing | kHz | 30 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 4 (N1,N2) = (2,1) |
| Beamforming Model |  |  | TBD |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (0,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) |
|  | CSI-RS interval and offset |  | 10/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 10/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| CSI-Report interval and offset |  | slot | 10/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 5.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.1 TDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-4)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+4)]. Note 3: Randomization of the principle beam direction shall be used as specified in TBD. |  |  |  |

Table 6.3.3.2.1-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.3] |

##### 6.3.3.2.2	Single PMI with 8TX TypeI-SinglePanel Codebook

For the parameters specified in Table 6.3.3.2.2-1, and using the downlink physical channels specified in Annex TBD, the minimum requirements are specified in Table 6.3.3.2.2-2.

Table 6.3.3.2.2-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |
| Duplex Mode |  |  | TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| DL BWP configuration #1 | First PRB |  | 0 |
|  | Number of contiguous PRB |  | 106 |
|  | Subcarrier spacing | kHz | 30 |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 8 x 4 (N1,N2) = (4,1) |
| Beamforming Model |  |  | TBD |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 8 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |
|  | CSI-RS interval and offset | slot | 10/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | 10/1 |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannnelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| CSI-Report interval and offset |  | slot | 10/1 |
| aperiodicTriggeringOffset |  |  | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 0x FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 6.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.2-8.2 TDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-6)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+6)]. Note 3: Randomization of the principle beam direction shall be used as specified in TBD. |  |  |  |

Table 6.3.3.2.2-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | [1.5] |

< ------------------------------Start of next  change ------------------------------- >

## 8.3	Reporting of Precoding Matrix Indicator (PMI)

The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reports compared to the case when the transmitter is using random precoding, respectively. When the transmitter uses random precoding, for each PDSCH allocation a precoder is randomly generated and applied to the PDSCH. A fixed transport format (FRC) is configured for all requirements.

The requirements for transmission mode 1 with 2TX and higher layer parameter codebookType set to ‘typeI-SinglePanel’ are specified in terms of the ratio

In the definition of γ, for 2TX PMI requirements, is [90] % of the maximum throughput obtained at  using the precoders configured according to the UE reports, and is the throughput measured at with random precoding.

### 8.3.1	1RX requirements

(Void)

### 8.3.2	2RX requirements

#### 8.3.2.1	FDD

(Void)

#### 8.3.2.2	TDD

##### 8.3.2.2.1	Single PMI with 2TX TypeI-SinglePanel Codebook

For the parameters specified in Table 8.3.2.2.1-1, and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified in Table 8.3.2.2.1-2.

Table 8.3.2.2.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 100 | 100 |
| TDD DL-UL configuration |  |  | FR2.120-2 as specified in Annex A | FR2.120-1 as specified in Annex A |
| DL BWP configuration #1 | First PRB |  | 0 | 0 |
|  | Number of contiguous PRB |  | 66 | 66 |
|  | Subcarrier spacing | kHz | 120 | 120 |
| Propagation channel |  |  | [TDLA30-35] | [TDLA30-35] |
| Antenna configuration |  |  | 2 x 2 [ULA Low] | 2 x 2 [ULA Low] |
| Beamforming Model |  |  | TBD | TBD |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (8,-) | Row 4, (8,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) | (13,-) |
|  | CSI-RS interval and offset | slot | 8/1 | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3, (6,-) | Row 3, (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) | (13,-) |
|  | CSI-RS interval and offset | slot | 8/1 | 5/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Patten 0 | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (8,13) | (8,13) |
|  | CSI-IM timeConfig interval and offset | slot | 8/1 | 5/1 |
| ReportConfigType |  |  | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 1 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband |
| CSI-Report interval and offset |  | slot | 8/1 | 8/1 |
| aperiodicTriggeringOffset |  |  | 0 | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A |
|  | CodebookSubsetRestriction |  | 001111 | 001111 |
|  | RI Restriction |  | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 1.375 | 1.75 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Measurement channel |  |  | R.PDSCH.5-8.1 TDD | R.PDSCH.5-7.1 TDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.125 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-4)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+4)]. Note 3: Randomization of the principle beam direction shall be used as specified in TBD. |  |  |  |  |

Table 8.3.2.2.1-2: Minimum requirement

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
|  | [1.05] | [1.05] |

< ------------------------------Start of next  change ------------------------------- >

##### A.3.2.2.2	Reference measurement channels for SCS 30 kHz FR1

Table A.3.2.2.2-1: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 2-1.1 TDD | R.PDSCH. 2-1.2 TDD | R.PDSCH. 2-1.3 TDD |  |  |
| Channel bandwidth | MHz | 40 | 40 | 40 |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 |  |  |
| Allocated resource blocks | PRBs | 106 | 6 | 106 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | [N/A] |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} |  | 12 | 12 | 7 |  |  |
| Allocated slots per 2 frames |  | 31 | 31 | [27] |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 4 | 4 | 4 |  |  |
| Modulation |  | QPSK | QPSK | QPSK |  |  |
| Target Coding Rate |  | 0.30 | 0.30 | 0.30 |  |  |
| Number of MIMO layers |  | 1 | 1 | 1 |  |  |
| Number of DMRS rEs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | [N/A] |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} |  | 18 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 2664 | 144 | [N/A] |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | Bits | 8064 | 480 | 4608 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 16 | 16 | [N/A] |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | Bits | 24 | 16 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 1 | 1 | [N/A] |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | CBs | 2 | 1 | 2 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 20, 21 | Bits | 25440 | 1512 | 13992 |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 8904 | 504 | [N/A] |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,19,22,…,39} | Bits | 26712 | 1584 | 15264 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 11.419 | 0.677 | [6.221] |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-2: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 2-2.1 TDD | R.PDSCH. 2-2.2 TDD | R.PDSCH. 2-2.3 TDD | R.PDSCH. 2-2.4 TDD |  |
| Channel bandwidth | MHz | 40 | 40 | 40 | 40 |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |  |
| Allocated resource blocks | PRBs | 106 | 106 | 106 | 106 |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 | 4 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} |  | 12 | 12 | 12 | 12 |  |
| Allocated slots per 2 frames |  | 31 | 31 | 31 | 31 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 13 | 13 | 13 | 13 |  |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 |  |
| Number of MIMO layers |  | 1 | 2 | 3 | 4 |  |
| Number of DMRS rEs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 | 12 | 12 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} |  | 12 | 12 | 24 | 24 |  |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 8456 | 16896 | 22032 | 29192 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | Bits | 26632 | 53288 | 73776 | 98376 |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | Bits | 24 | 24 | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 1 | 2 | 3 | 4 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | CBs | 4 | 7 | 10 | 13 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 20, 21 | Bits | 53424 | 106848 | 144008 | 193344 |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 17808 | 35616 | 45792 | 61056 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,19,22,…,39} | Bits | 55968 | 111936 | 152640 | 203520 |  |
| Max. Throughput averaged over 2 frames | Mbps | 37.644 | 75.318 | 104.719 | 138.646 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-3: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 2-3.1 TDD | R.PDSCH. 2-3.2 TDD |  |  |  |
| Channel bandwidth | MHz | 40 | 20 |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 |  |  |  |
| Allocated resource blocks | PRBs | 106 | 51 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames |  | 31 | 31 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 19 | 19 |  |  |  |
| Modulation |  | 64QAM | 64QAM |  |  |  |
| Target Coding Rate |  | 0.51 | 0.51 |  |  |  |
| Number of MIMO layers |  | 2 | 2 |  |  |  |
| Number of DMRS rEs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 27144 | 13064 |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | Bits | 83976 | 40976 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 3 | 2 |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | CBs | 10 | 5 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slots i = 20, 21 | Bits | 160272 | 77112 |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 53424 | 25704 |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,19,22,…,39} | Bits | 167904 | 80784 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 118.796 | 57.930 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-4: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (256QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 2-4.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 256QAM |  |  |  |  |
| MCS index |  | 24 |  |  |  |  |
| Modulation |  | 256QAM |  |  |  |  |
| Target Coding Rate |  | 0.82 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS rEs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Maximum number of HARQ transmissions |  | 4 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 29192 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | Bits | 92200 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 3 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | CBs | 11 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slots i = 20, 21 | Bits | 106848 |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 35616 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,19,22,…,39} | Bits | 111936 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 130.308 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-5: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-2

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 2-5.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} |  | 8 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS rEs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} | Bits | 5376 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,39} | Bits | 8456 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,39} | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i = 20, 21 | Bits | 26712 |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,39} | Bits | 17808 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,19,22,…,39} | Bits | 27984 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 11.875 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-6: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-3

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 2-6.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} |  | 8 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,}) for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 27 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS rEs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,}) for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Maximum number of HARQ transmissions |  | 4 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 10) = {4,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} | Bits | 5376 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,}) for i from {1,…,39} | Bits | 8456 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 10) = {4,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,}) for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 10) = {4,8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,}) for i from {1,…,39} | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 10) = {4,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i = 20, 21 | Bits | 26712 |  |  |  |  |
| For Slot i, if mod(i, 10) = {3,7} for i from {0,…,39} | Bits | 17808 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,5,}) for i from {1,…,19,22,…,39} | Bits | 27984 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 10.184 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-7: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 and CSI-RS overlapped with PDSCH

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 2-7.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 31 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS rEs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 16896 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | Bits | 53288 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 2 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,}) for i from {1,…,39} | CBs | 7 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,}) for i from {1,…,19,22,…,39} | Bits | 103456 |  |  |  |  |
| For Slots i = 20 | Bits | 98368 |  |  |  |  |
| For Slots i = 21 | Bits | 106848 |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 35616 |  |  |  |  |
| For Slot i, if mod(i, 10) = {1,2,3,4,}) for i from {1,…,19,22,…,39} | Bits | 111936 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 75.318 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.2-8: PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR1.30-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 2-8.1 TDD | R.PDSCH. 2-8.2 TDD |  |  |  |
| Channel bandwidth | MHz | 40 | 40 |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 |  |  |  |
| Allocated resource blocks | PRBs | 106 | 106 |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames |  | 23 | 23 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 13 | 13 |  |  |  |
| Modulation |  | 16QAM | 16QAM |  |  |  |
| Target Coding Rate |  | 0.48 | 0.48 |  |  |  |
| Number of MIMO layers |  | 1 | 2 |  |  |  |
| Number of DMRS rEs (Note 3) |  | 24 | 24 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slot i = 20 | Bits | 24576 | 49176 |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} |  | 24576 | 49176 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} |  | N/A | N/A |  |  |  |
| For Slot i = 20 | Bits | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | CBs | N/A | N/A |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} |  | N/A | N/A |  |  |  |
| For Slot i = 20 | CBs | 3 | 6 |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | CBs | 3 | 6 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slot i = 20 | Bits | 48336 | 96672 |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6}for i from {1,…,19,22,…,39} | Bits | 50880 | 101760 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 28.2624 | 56.5524 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS rEs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

< ------------------------------Start of next  change ------------------------------- >

##### A.3.2.2.5	Reference measurement channels for SCS 120 kHz FR2

Table A.3.2.2.5-1: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 5-1.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 127 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS rEs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 3624 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} | Bits | 5504 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 16 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slots i = 80, 81 | Bits | 17490 |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 12210 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,79,82,…,159} | Bits | 18282 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 31.942 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.5-2: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 5-2.1 TDD | R.PDSCH. 5-2.2 TDD | R.PDSCH. 5-2.3 TDD |  |  |
| Channel bandwidth | MHz | 100 | 100 | 200 |  |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |  |
| Allocated resource blocks | PRBs | 66 | 66 | 132 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 9 |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} |  | 13 | 13 | 13 |  |  |
| Allocated slots per 2 frames |  | 127 | 127 | 127 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 13 | 13 |  |  |
| Modulation |  | 16QAM | 16QAM | 16QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 |  |  |
| Number of MIMO layers |  | 1 | 2 | 2 |  |  |
| Number of DMRS rEs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 | 12 | 12 |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 6 | 6 | 6 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 11272 | 22536 | 45096 |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} | Bits | 17424 | 34816 | 69672 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 | 24 |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 2 | 3 | 6 |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} | CBs | 3 | 5 | 9 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 80, 81 | Bits | 34980 | 69960 | 139920 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24420 | 48840 | 97680 |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,79,82,…,159} | Bits | 36564 | 73128 | 146256 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 100.799 | 201.434 | 403.096 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.5-3: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 5-3.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 127 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 18 |  |  |  |  |
| Modulation |  | 64QAM |  |  |  |  |
| Target Coding Rate |  | 0.46 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS rEs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 16136 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} | Bits | 25104 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 3 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,159} | CBs | 3 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slots i = 80, 81 | Bits | 52470 |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 36630 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,}) for i from {1,…,79,82,…,159} | Bits | 54846 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 145.062 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.5-4: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-2 (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 5-4.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 6 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} |  | 10 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 119 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS rEs |  |  |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | Bits | 736 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} | Bits | 1032 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | Bits | 16 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} | Bits | 16 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80, 81 | Bits | 3180 |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {4,…, 159} | Bits | 2496 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,79,82,…,159} | Bits | 3324 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 5.548 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.5-5: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-2 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 5-5.1 TDD | R.PDSCH. 5-5.2 TDD |  |  |  |
| Channel bandwidth | MHz | 100 | 50 |  |  |  |
| Subcarrier spacing | kHz | 120 | 120 |  |  |  |
| Allocated resource blocks | PRBs | 66 | 32 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} |  | 10 | 10 |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} |  | 13 | 13 |  |  |  |
| Allocated slots per 2 frames |  | 119 | 119 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 13 | 13 |  |  |  |
| Modulation |  | 16QAM | 16QAM |  |  |  |
| Target Coding Rate |  | 0.48 | 0.48 |  |  |  |
| Number of MIMO layers |  | 2 | 2 |  |  |  |
| Number of DMRS rEs |  |  |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} |  | 12 | 12 |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 6 | 6 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | Bits | 25608 | 12552 |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} | Bits | 34816 | 16896 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | Bits | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | CBs | 4 | 2 |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} | CBs | 5 | 3 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i = 80, 81 | Bits | 69960 | 33920 |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {4,…, 159} | Bits | 54912 | 26624 |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,79,82,…,159} | Bits | 73128 | 35456 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 188.739 | 91.843 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.5-6: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-2 (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 5-6.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} |  | 10 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 119 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 17 |  |  |  |  |
| Modulation |  | 64QAM |  |  |  |  |
| Target Coding Rate |  | 0.43 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS rEs |  |  |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | Bits | 34816 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} | Bits | 47112 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {1,…, 159} | CBs | 5 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,159} | CBs | 6 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 4) = 3 for i from {0,…,159} | Bits | N/A |  |  |  |  |
| For Slot i = 80, 81 | Bits | 114940 |  |  |  |  |
| For Slot i, if mod(i, 4) = 2 for i from {4,…, 159} | Bits | 82368 |  |  |  |  |
| For Slot i, if mod(i, 4) = {0,}) for i from {1,…,79,82,…,159} | Bits | 109692 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 255.724 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.5-7: PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR2.120-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 5-7.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 63 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS rEs (Note 3) |  | 6 |  |  |  |  |
| Overhead for TBS determination |  | 4 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | 14344 |  |  |  |  |
| For Slot i = 20 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} |  | 14344 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} |  | 24 |  |  |  |  |
| For Slot i = 20 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} |  | 2 |  |  |  |  |
| For Slot i = 20 | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | 28776 |  |  |  |  |
| For Slot i = 20 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6}for i from {1,…,19,22,…,39} | Bits | 30360 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 45.1836 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS rEs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

Table A.3.2.2.5-8: PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR2.120-2 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH. 5-8.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 59 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS rEs (Note 3) |  | 6 |  |  |  |  |
| Overhead for TBS determination |  | 4 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | 14344 |  |  |  |  |
| For Slot i = 20 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} |  | 14344 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} |  | 24 |  |  |  |  |
| For Slot i = 20 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} |  | 2 |  |  |  |  |
| For Slot i = 20 | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6} for i from {1,…,19,22,…,39} | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i,10) =1 for i from {0,…,39} | Bits | 28776 |  |  |  |  |
| For Slot i = 20 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,2,3,4,5,6}for i from {1,…,19,22,…,39} | Bits | 30360 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 42.3148 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS rEs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |