3GPP TSG-RAN WG4 Meeting #90                                                   R4-1902457

Athens, Greece, 25 Feb – 1st Mar 2019

Agenda Item:	6.7.2.2

Source: 	MediaTek Inc.

Title: 	Revisit DC_25A-n41A MSD due to cross band isolation

Document for:	Approval

- 1. Introduction

The EN DC 25A-n41A has been completed in Release-15 in RAN4 meetings. The band combination is subjected to insufficient cross band isolation. This contribution is to characterize MSD due to cross band isolation with assumptions in R4-1902111, to calculate MSD comparing separate antenna and shared antenna.

- 2. Discussion

2.1 Separate antenna:

Figure 2-1 Reference architecture for DC_25A-n41A MSD with separate antenna

The following MSD assumptions are proposed (R4-1902111):

- Dual PA and separate antenna for each band (without separate diplexer) with 10 dB antenna isolation and 4 dB front-end loss,

- Full band n41 filter with Band 25 Rx band rejection of 32 dB, representing neither best case nor worst case of available filter performance,

- Band 25 rejection of Band n41 Tx is 50 dB

- Noise figure, IP2, IP3, transceiver spurious products according to each companies assumptions,

- Band n41 Tx bandwidth (uplink RB configuration) reduced to no more than approximately 60 MHz (160 RB’s for 30 kHz SCS) placed closest to B25 Rx band.  This is consistent with the approach taken for FDD reference sensitivity.

- Band n41 SCS is 30 kHz.  This is the only common SCS that is available for all bandwidths in n41.  Alternatively, the lowest SCS supported for a given channel bandwidth in the band could be used.  The difference in MSD between 30 kHz or lowest supported SCS is not expected to be significant; therefore, the simpler approach of fixed 30 kHz SCS is recommended.

- Evaluate both equal power of 20 dBm on each transmitter and 23 dBm on NR transmitter.  Specify the worst of the two as the MSD. (note: 23dBm shall not be limited on NR transmitter only, we also analyze impact on n41 receiver when 23dBm on LTE transmitter)

- The cross-band interference terms are correlated and equal power into each of the primary and diversity receivers of Band 25

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation Primary <-> Diversity | 10 | dB |
| Diplexer isolation at B25 uplink freq | 12 | dB |
| B41 SAW Ant to RX port rejection at 1850~1915MHz | 32 | dB |
| B25 DUP ANT to RX port rejection at 2496~2690MHz | 50 | dB |
| B41 front-end loss | 4 | dB |
| B25 front-end loss | 4 | dB |
| Victim band noise figure | 9 | dB |
| PA noise floor | -130 | dBm/Hz |
| Typical receiver IIP2 | 50 | dBm |
| Typical transmitter / receiver phase noise | -155 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 2-1 Typical receiver performance parameters for MSD analysis

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| B25 TX power at antenna port (Primary path) | 23 | 23 | dBm |
| B25 TX power at B41 SAW-ant port | 13 | 13 | dBm |
| Thermal noise at RX ant port | -165 | -165 | dBm/Hz |
| TX IM2 noise level refer to RX LNA input | -88 | -88 | dBm |
| Reciprocal mixing noise refer to RX LNA input | -174 | -174 | dBm/Hz |
| Total noise level refer to receiver ANT port | -83.6 | -83.6 | dBm |
| Combined |  |  |  |
| Combined total noise (correlated) | -77.8 |  | dBm |
| MSD (10MHz BW, SCS=30KHz) | 10.3 |  | dB |

Table 2-2 Link analysis for n41 10MHz REFSENS calculation

### 2.1.1 MSD for separate antenna

The MSD due to cross band isolation is proposed in table 6.65.5-1.

Table 6.65.5-1: Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

|  |  | E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| 25 | n41 | DL 30 |  | 10.3 | 8.4 | 7.5 |  |  | 4.5 | 3.1 | 3.1 | 2.1 | 1.8 | 1.5 |
| n41 | 25 |  | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.65.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

|  |  | E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| 25 | n41 | 30 |  | 50 | 50 | 50 |  |  | 50 | 50 | 50 | 50 | 50 | 50 |
| n41 | 25 | UL 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

2.2 Shared antenna:

We agree on the MSD analysis on B25 in R4-1902111, the following is to analyze MSD on n41.

Figure 2-2 Reference architecture for DC_25A-n41A MSD with separate antenna

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation Primary <-> Diversity | 10 | dB |
| Diplexer isolation at B25 uplink freq | 12 | dB |
| B41 SAW Ant to RX port rejection at 1850~1915MHz | 32 | dB |
| B25 DUP ANT to RX port rejection at 2496~2690MHz | 50 | dB |
| B41 front-end loss | 4 | dB |
| B25 front-end loss | 4 | dB |
| Victim band noise figure | 9 | dB |
| PA noise floor | -130 | dBm/Hz |
| Typical receiver IIP2 | 50 | dBm |
| Typical transmitter / receiver phase noise | -155 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 2-3 Typical receiver performance parameters for MSD analysis

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| B25 TX power at antenna port (Primary path) | 23 | 23 | dBm |
| B25 TX power at B41 SAW-ant port | 12 | 1 | dBm |
| Thermal noise at RX ant port | -165 | -165 | dBm/Hz |
| TX IM2 noise level refer to RX LNA input | -90 | -112 | dBm |
| Reciprocal mixing noise refer to RX LNA input | -174 | -178 | dBm/Hz |
| Total noise level refer to receiver ANT port | -85.4 | -95 |  |
| Combined |  |  |  |
| REFSENSE (referred to antenna)(10MHz BW) | -86.4 | -96 | dBm |
| Combined n41 REFSENS (10MHz BW) | -96.5 |  | dBm |
| MSD (10MHz BW, SCS=30KHz) | 0 |  | dB |

Table 2-4 Link analysis for n41 10MHz REFSENS calculation

### 2.2.1 MSD for shared antenna

Table 6.65.5-1: Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

|  |  | E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| 25 | n41 |  |  | 0 | 0 | 0 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| n41 | 25 | UL 30 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.65.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

|  |  | E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| 25 | n41 |  |  | 50 | 50 | 50 |  |  | 50 | 50 | 50 | 50 | 50 | 50 |
| n41 | 25 | UL 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

However shared antenna adopts new designed full n41 filter with better rejection may introduce additional insertion loss. Thus we propose ΔTIB,c and ΔRIB,c shall be further discussed.

3 Proposals

Proposal 1: To accommodate worst case scenario, separate antenna MSD result is adopted.

Proposal 2: If shared antenna MSD is adopted, since we only have filter data from single company, to accommodate general components, we propose to have additional 0.1dB on n41 ΔTIB,c

Table 6.65.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_25_n41 | 25 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

- References

[1] R4-151188 Way Forward Proposals for Inter-band CA Consisting of B41, 3GPP TSG-RAN WG4 Meeting #74.

[2] R4-1902111 MSD for EN-DC B25 + n41, 3GPP TSG-RAN WG4 Meeting #90

Annex:

Reference filter performance data:

Figure A-1 n41 Filter frequency response from different vendors

Figure A-2 New generation n41 filter frequency response from vendor C

Figure A-3 B25 Filter frequency response