3GPP TSG-RAN4 Meeting #90 	R4-1902560

# Athens, Greece, 25 Feb. – 1 March, 2019

Title:	Reply LS on UE behavior on reception of channels or RS in the same OFDM symbol

Response to:	R1-1809890

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

Name:	Jin-Yup Hwang

E-mail Address:	jinyup.hwang@lge.com

1. Overall Description:

RAN4 has discussed RAN1 LS on UE behavior on reception of channels or RS in the same OFDM symbol for FR2. Since RAN4 has not defined core requirements for CSI-RS based L3 measurement, CSI-RS based L3 measurement is precluded in this LS.

RAN4 would like to inform RAN1 the conclusion in RAN4 as follows:

RAN4 kindly remind RAN1 that additional to the following answers regarding restrictions due to FR2 beam sweeping, when performing reception of channels/RS in group 1 with different subcarrier spacing than channels/RS group 2, the UE which does not support simultaneousRxDataSSB-DiffNumerology cannot receive both channels/RS group 1 and group 2 simultaneously.

- Answer to Q1

Case 1 & 4

  - If the RS in group 1 is QCL-Type D with RS in group 2 and the QCL association is known to UE, UE may receive both RS in group 1 and group 2 in the same OFDM symbol where only UE is not expected to sweep its Rx beam in FR2. Otherwise, UE cannot receive both RS in group1 and group 2 simultaneously. The conditions that UE is expected to sweep its Rx beam are listed as follows:

    - L3 SSB-based measurement in SMTC window duration

    - RLM/BFD/CBD under certain conditions specified as the condition for N>1, which can be referred in Section 8.1/8.5 of TS38.133

    - For L1-RSRP measurement, UE may or may not receive SSB and CSI-RS simultaneously depending on UE implementation of Rx beam sweeping operation for L1-RSRP measurement in the OFDM symbol.

Case 2 & 3

  - If the TCI of the PDCCH/PDSCH includes at least one of the CSI-RS resource IDs of the CSI-RS resource set and QCL-Type D of the CSI-RS is configured, UE may receive a PDCCH/PDSCH that is overlapping with one or more symbols configured with CSI-RS resource(s) with repetition set to “OFF”. Otherwise, UE is not expected to receive PDCCH/PDSCH on CSI-RS symbol in FR2.

- Answer to Q2

  - For rate matching issue by scheduling restriction, RAN4 has already agreed in RAN4 NR-AH#4 meeting as follows:

    - It is up to RAN1 whether or not to define specific behavior for rate matching around scheduling restricted symbols.

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to take the above RAN4 conclusion into consideration for their further work.

3. Date of Next TSG-RAN4 Meetings:

3GPP RAN4 #90BIS 		8 - 12 April 2019, 		, China

3GPP RAN4 #91 		13 - 17 May 2019, 		, US