3GPP TSG-RAN WG4 Meeting #90	R4-1902683

# Athens, Greece, 25 February – 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.810 | CR | - | rev | - | Current version: | 16.1.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR38.810 on beam correspondence test procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_test_methods |  |  |  |  |  | Date: |  |  | 2018-02-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections in this draft CR follows the agreement on beam correspondence requirements in RAN#82 to specify relevant test procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise the test procedure of Tx beam peak search and EIRP spherical coverage to explicitly reflect the beam correspondence requirements based on UE’s beam correspondence capability; Add new test procedure of beam correspondence tolerance for the UE relying on uplink beam sweeping to fulfill minimum peak EIRP and EIRP spherical coverage requirements; Other minor changes on details. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test procedures will be unclear for beam correspondence requriements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.3.7, 5.2.1.3.9(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes 1 >>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”.

[3]	3GPP TS 36.133: “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”.

[4]	3GPP TS 37.144: “User Equipment (UE) and Mobile Station (MS) GSM, UTRA and E-UTRA over the air performance requirements”.

[5]	3GPP TR 37.977: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Verification of radiated multi-antenna reception performance of User Equipment (UE)".

[6]	3GPP TR 37.902: "Measurements of User Equipment (UE) radio performances for LTE/UMTS terminals; Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) test methodology".

[7]	3GPP TR 38.900: “Study on channel model for frequency spectrum above 6 GHz”.

[8]	3GPP TR 38.803: “Study on New Radio Access Technology; RF and co-existence aspects”.

[9]	3GPP TR 37.842: “Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”

[10]	3GPP TR 38.901: “Study on channel model for frequencies from 0.5 to 100 GHz”.

[11]	ETSI TR 102 273-1-1 V1.2.1 (2001-12): “Electromagnetic compatibility and Radio spectrum Matters (ERM); Improvement on Radiated Methods of Measurement (using test site) and evaluation of the corresponding measurement uncertainties; Part 1: Uncertainties in the measurement of mobile radio equipment characteristics; Sub-part 1: Introduction”.

[12]	3GPP TR 25.914: “Measurement of Radio Performances for UMTS terminals in speech mode”.

[13]	3GPP TS 34.114: “User Equipment (UE) / Mobile Station (MS) Over The Air (OTA) antenna performance; Conformance testing”.

[14]	3GPP TS 38.215: “Physical layer measurements”.

[15]	CTIA OTA Test Plan version 3.7, https://www.ctia.org/

[16]	3GPP TS 38.101-2: “UE radio transmission and reception; Part 2: Range 2 Standalone”

[17]	3GPP TS 38.133: “Requirements for support of radio resource management”

[18] 	3GPP TS 38.101-4: “5G; NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”.

[19]	3GPP TS 38.306: “NR; User Equipment (UE) radio access capabilities”.

<< End of Changes 1 >>

<< Start of Changes 2 >>

##### 5.2.1.3.7	TX Beam Peak direction search and EIRP Spherical Coverage

The beam peak search and spherical coverage test procedure apply to DUTs with different beam correspondence capability, as defined in TS38.306 [19]. The TX beam peak direction is found with a 3D EIRP scan (separately for each orthogonal downlink polarization). The TX beam peak direction search grid points for this single grid approach are defined in Annex G.2. Alternatively, a coarse and fine grid approach could be used according to the definition in Annex G.2.4.

The measurement procedure includes the following steps for each of the points in the grid:

1)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= to form the TX beam towards the measurement antenna.

2)	DUT refines its TX beam toward that direction depending on DUT’s beam correspondence capability which shall match OEM declaration: if  DUT’s beam correspondence capability is [bit-1], then DUT chooses autonomously (the corresponding TX beam) using downlink reference signals to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping; if DUT’s beam correspondence capability is [bit-0], then DUT chooses the TX beam using downlink reference signals which is based on beam correspondence with relying on uplink beam sweeping by its network-assisted uplink beam management capability, as defined in TS38.306 [19].

3)	Lock the beam.

4)	Measure the mean power Pmeas(PolMeas=PolLink=) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator).

5)	Calculate EIRP (PolMeas=PolLink=) by adding the composite loss of the entire transmission path for utilized signal path, LEIRP,θ, and frequency to the measured power Pmeas(PolMeas=PolLink=)

6)	Measure the mean power Pmeas (PolMeas=PolLink=) of the modulated signal arriving at the power measurement equipment.

7)	Calculate EIRP (PolMeas=PolLink=) by adding the composite losses of the entire transmission path for utilized signal path, LEIRP,φ, and frequency to the measured power Pmeas (PolMeas=PolLink=)

8)	Calculate total EIRP(PolLink=)  = EIRP(PolMeas=PolLink=)  + EIRP(PolMeas=PolLink=)

9)	Unlock the beam.

10)	Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink=polarization to form the TX beam towards the measurement antenna.

11)	Repeat steps 2) to 9).

The TX beam peak direction is where the maximum total component of EIRP(PolLink=) or EIRP(PolLink=) is found.

The EIRP results from the TX beam peak search using the minimum number of grid points as described in Annex G.2 can be re-used for EIRP spherical coverage. In case a coarse beam peak grid is used for TX beam peak search, using the minimum number of grid points defined in Annex G.3.3.2.3, the EIRP results can be re-used for EIRP spherical coverage.

In case a separate test is performed for EIRP spherical coverage, the procedure above shall be followed using the minimum number of grid points defined in Annex G.3.3.2.3 for spherical coverage.

The EIRPtarget-CDF is then obtained from the Cumulative Distribution Function (CDF) computed using [maximum(EIRP(PolLink=), EIRP(PolLink=)] for all grid points. When using constant step size measurement grids, a theta-dependent correction shall be applied, i.e., the PDF probability contribution for each measurement point is scaled by sin(θ).

Editor’s Note: For spherical coverage test, using maximum(EIRP(PolLink=), EIRP(PolLink=)) for each test point in the grid is considered as the baseline for performance evaluation and test procedure. Decision shall be made on April meeting.

<< End of Changes 2 >>

<< Start of Changes 3>>

##### 5.2.1.3.9	Beam Correspondence Tolerance

The beam correspondence requirement is fulfilled if the DUT satisfies one of the following conditions, depending on the beam correspondence capability, as defined in TS38.306 [19]:

-	If [bit-1], the DUT shall meet the minimum peak EIRP requirement and spherical coverage requirement with its autonomously chosen UL beams and without uplink beam sweeping.  Such a DUT is considered to have met the beam correspondence tolerance requirement.

-	If [bit-0], the DUT shall meet the minimum peak EIRP requirement and spherical coverage requirement with uplink beam sweeping.  Such a DUT shall meet the beam correspondence tolerance requirement and shall support uplink beam management, as defined in TS38.306 [19].

Thus, the beam correspondence tolerance test is only applicable to the DUT that has beam correspondence capability as [bit-0] (which shall match OEM declaratioin), such that DUT relies on uplink beam sweeping to fulfill the minimum peak EIRP and spherical coverage requirements.

ΔEIRPBC is introduced for beam correspondence tolerance based on two EIRP measurements (EIRP1 and EIRP2). EIRP1 is the measured total EIRP based on the beam which DUT chooses autonomously (corresponding beam) to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping. EIRP2 is the measured total EIRP based on the beam yielding highest EIRP in a given direction, which is based on beam correspondence with relying on UL beam sweeping. ΔEIRPBC shall be calculated over the link angles spanning a subset of the spherical coverage grid points which are corresponding to the top 50% of the EIRP2 measurement.

For each of the points in the grid:

1)	Follow the test procedures specified in subclause 5.2.1.3.7 with uplink beam sweeping disabled, obtain total EIRP1(PolLink=) and total EIRP1(PolLink=EIRP1 is calculated by [EIRP1 = maximum(EIRP1(PolLink=), EIRP1(PolLink=)]

2)	Follow the test procedures specified in subclause 5.2.1.3.7, with uplink beam sweeping enabled during DUT TX beam refinement, obtain total EIRP2(PolLink=) and total EIRP2(PolLink=EIRP2 is calculated by [EIRP2 = maximum(EIRP2(PolLink=), EIRP2(PolLink=)]

3)	Calculate the ΔEIRPBC=EIRP2– EIRP1.

The ΔEIRPtarget-CDF is then obtained from the Cumulative Distribution Function (CDF) computed using [ΔEIRPBC] for each of all top 50% of the EIRP2 measurement points in the grid. When using constant step size measurement grids, a theta-dependent correction shall be applied, i.e., the PDF probability contribution for each measurement point is scaled by sin(θ).

Editor’s Note: For beam correspondence tolerance test, using maximum(EIRP1(PolLink=), EIRP1(PolLink=)) and maximum(EIRP2(PolLink=), EIRP2(PolLink=)) for each test point in the grid is considered as the baseline for performance evaluation and test procedure. Decision shall be made on April meeting

Editor’s Note: The side conditions for downlink reference signals SSB and CSI-RS in beam correspondence tolearance test are FFS.

<< End of Changes 3>>