3GPP TSG-RAN WG4 Meeting #90bis 	R4-1902947

Xi’an, China, 8 - 12 April, 2019

Agenda item:       6.12.1.3.2

Source:                Intel Corporation

Title:	                     NR PBCH UE Demodulation Requirements

Document for:     Discussion

# Introduction

In RAN4#90 NR PBCH demodulation requirements were discussed and way forward [1] was approved. The agreements from RAN4#90 are summarized below:

- Introduce PBCH requirements for 4Rx - whether to apply 4Rx PBCH requirements is up to UE declaration - Note the PBCH demodulation requirements do not need to be tested in RAN5 as same as LTE PBCH demodulation requirements. (i.e. no conformance requirements) - Introduce PBCH requirements assuming with and without knowledge of SSB index.

In this contribution we provide updated simulation results for the agreed test cases and address the open issues related to PBCH demodulation requirements

# Simulation Results

## Simulation Assumptions

The simulation parameters for PBCH test cases agreed in [1] are summarized below.

Table 1: Common parameters for PBCH test cases

| Parameters | Unit | Value |
| --- | --- | --- |
| Cell ID |  | 0 |
| CP length |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index |  | 0 |
| SS burst periodicity | ms | 20 |
| MIB size | bits | 24 |
| PBCH payload size (including 24 bits CRC and 8 bits of SS/PBCH location information) | bits | 56 |
| Receiver assumption |  | LMMSE |

Table 2: Test cases for PBCH Demodulation

| Parameters | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| Frequency Range | FR1 | FR1 | FR2 | FR2 |
| Channel Bandwidth | 10MHz | 40MHz | 100MHz | 100MHz |
| SS/PBCH SCS | 15kHz | 30kHz | 120kHz | 240kHz |
| SS Block pattern (Note 1) | Case A | Case B Case C | Case D | Case E |
| Antenna Configuration | 1x2 1x4 | 1x2 1x4 | 1x2 | 1x2 |
| Propagation channel | TDLC300-100 | TDLA30-10 | TDLA30-300 | TDLA30-75 |
| Assumption on DMRS SSB index acquisition | a: SS/PBCH block index is known b: SS/PBCH block index is not known |  |  |  |

The UE combines PBCH symbols for the same SSB index within the MIB TTI.

## FR1 Results

In this section we present alignment results for FR1 test cases with 2Rx and 4Rx. The tables capture the SNR (dB) @ 1% Pm-bch. The simulations are run with and without PBCH-DMRS sequence detection.

Table 3: Alignment results for PBCH in FR1 with 2Rx

|  | Channel BW | SS/PBCH SCS | Antenna configuration | Propagation channel | SNR @ 1% Pm-bch |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | With DMRS Seq Det | Without DMRS Seq Det |
| Test 1 | 10MHz | 15kHz | 1x2 | TDL-C-300; 100Hz | -8.9 | -9.8 |
| Test 2 | 40MHz | 30kHz | 1x2 | TDL-A-30; 10Hz | -7.1 | -7.6 |

Table 4: Alignment results for PBCH in FR1 with 4Rx

|  | Channel BW | SS/PBCH SCS | Antenna configuration | Propagation channel | SNR @ 1% Pm-bch |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | With DMRS Seq Det | Without DMRS Seq Det |
| Test 1 | 10MHz | 15kHz | 1x4 | TDL-C-300; 100Hz | -11.7 | -12.2 |
| Test 2 | 40MHz | 30kHz | 1x4 | TDL-A-30; 10Hz | -10.4 | -10.8 |

## FR2 Results

In this section we present alignment results for FR2 test cases with 2Rx. The tables capture the SNR (dB) @ 1% Pm-bch. The simulations are run with and without PBCH-DMRS sequence detection.

Table 5: Alignment results for PBCH in FR2 with 2Rx

|  | Channel BW | SS/PBCH SCS | Antenna configuration | Propagation channel | SNR @ 1% Pm-bch |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | With DMRS Seq Det | Without DMRS Seq Det |
| Test 3 | 100MHz | 120kHz | 1x2 | TDL-A-30; 300Hz | -8.5 | -9.4 |
| Test 4 | 100MHz | 240kHz | 1x2 | TDL-A-30; 75Hz | -8.9 | -10.0 |

## Impairment Results

Taking into account implementation margin, we present the SNR (dB) @ 1% Pm-bch test cases in table below.

Table 6: Impairment results for NR PBCH tests

|  | Channel BW | SS/PBCH SCS | Antenna configuration | Propagation channel | SNR @ 1% Pm-bch |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | With DMRS Seq Det | Without DMRS Seq Det |
| Test 1 | 10MHz | 15kHz | 1x2 | TDL-C-300; 100Hz | -7.4 | -8.3 |
| Test 2 | 40MHz | 30kHz | 1x2 | TDL-A-30; 10Hz | -5.6 | -6.1 |
| Test 1 | 10MHz | 15kHz | 1x4 | TDL-C-300; 100Hz | -10.2 | -10.7 |
| Test 2 | 40MHz | 30kHz | 1x4 | TDL-A-30; 10Hz | -8.9 | -9.3 |
| Test 3 | 100MHz | 120kHz | 1x2 | TDL-A-30; 300Hz | -7.0 | -7.9 |
| Test 4 | 100MHz | 240kHz | 1x2 | TDL-A-30; 75Hz | -7.4 | -8.5 |

# PBCH Demodulation Requirements

In the Section 2 we present PBCH results with and without PBCH-DMRS sequence detection. In RAN4#90 results from companies were collected. For the test cases with span in results among within 2.5dB it was agreed to introduce tentative requirements in 38.101-4. For test cases with larger span the SNR requirements are still TBD. In order to define NR PBCH requirements for Rel-15, for results with large span in results we propose the following options:

Option1: Increase the margin added to derive SNR requirement from 0.5 dB to 1 dB

Option2: Eliminate outliers from results for SNR requirement derivation in order to bring the span in results within 2.5 dB.

Based on the results submitted in RAN4#90bis we recommend to use either approach and finalize requirements for NR PBCH.

Proposal #1: For test cases with span > 2.5dB – Option1: Increase margin to derive SNR requirement (or) Option2: eliminate outlier results to derive SNR requirements

In previous meetings (RAN4#90, RAN4#89, and RAN4 #88bis) it was agreed that there will be no conformance tests introduced for NR PBCH. Hence we recommend sending an LS to RAN5 that no conformance tests shall be introduced for NR PBCH.

Proposal #2: Send LS to RAN5 on agreement in RAN4 that no conformance tests for NR PBCH shall be introduced

# Conclusion

In this paper we present alignment and impairment results for PBCH test cases and also provide our views on NR PBCH demodulation requirements. Our proposals are summarized below.

Proposal #1: For test cases with span > 2.5dB – Option1: Increase margin to derive SNR requirement (or) Option2: eliminate outlier results to derive SNR requirements

Proposal #2: Send LS to RAN5 on agreement in RAN4 that no conformance tests for NR PBCH shall be introduced

# Reference

- R4-1902405, “Way Forward on NR PBCH demodulation requirements”, CMCC