3GPP TSG-WG RAN4 Meeting #90bis	R4-1903002

# Xian, China, 8th – 12th April, 2019

Source: 	Nokia, Nokia Shanghai Bell, T-Mobile USA

Title: 					TP for TR 38.716-02-00 Introduction of CA_n41-n261

Agenda Item: 			9.2.3 [NR_CADC_R16_2BDL_xBUL]

Document for:	Approval

# Introduction

This TP introduces two-band NR CA configurations of CA_n41A-n261A and CA_n41A-n261(2A) to TR 38.716-02-00.

**************************** Start of changes ********************************************

## 8.X	CA_n41-n261

### 8.X.1	Common for 1 band UL and 2 bands UL CA

#### 8.X.1.1		Operating bands for CA

Table 8.X.1.1-1: CA band combination of band n41+n261

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 | – | 2690 | 2496 | – | 2690 | FDD |
| n261 | 27500 | – | 28350 | 27500 | – | 28350 | TDD |

#### 8.X.1.2		Channel bandwidths per operating band for CA

Table 8.X.1.2-1: Supported bandwidths per CA band combination of band n41+n261

| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz |  | 10 MHz | 15 MHz |  | 20 MHz | 25 MHz |  | 30 MHz | 40 MHz |  | 50 MHz | 60 MHz |  | 80 MHz | 90 MHz |  | 100 MHz | 200 MHz | 400 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n261A | - | n41 | 15 |  |  | Yes | Yes |  | Yes |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  | Yes | Yes |  | Yes |  |  |  | Yes |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  | Yes | Yes |  | Yes |  |  |  | Yes |  | Yes | Yes |  | Yes | Yes |  | Yes |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes | Yes | Yes |  |
| CA_n41A-n261(2A) | - | n41 | 15 |  | Yes |  | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes |  | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes |  | Yes | Yes |  |  |  |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |
|  |  | n261 | See CA_ n261(2A) Bandwidth combination set in 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 8.X.1.3		UE co-existence studies

Harmonic relation between FR1 and FR2 band is not analyzed due to large frequency separation.

The addtional spurious emission level to coexisting bands is specified in Table 8.X.1.3-1.

Table 8.X.1.3-1: Requirements for uplink inter-band CA (two bands)

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n41-n261 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |

#### 8.X.1.4		∆TIB and ∆RIB values

For CA_n41-n261, the TIB,c and RIB,c values are all zero and thus do not need to be explicitly specified in TS 38.101-3.

#### 8.X.1.5		REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

**************************** End of changes ********************************************