3GPP TSG-RAN WG4 Meeting #90-bis                                R4-1903020

Xi’an, China, 8 – 12 April, 2019

Agenda Item:		9.20.2

Source: 		CMCC

Title: 	TP for TR 37.825 for PC2 EN-DC_39A-79A

Document for:		Approval

- Introduction

New WID on Power Class 2 UE for EN-DC (1 LTE band +1 NR band) were approved in RAN#82 meeting [1]. This TP summarizes the Band combination specific RF requirements for PC2 EN-DC_39A-79A.

- Reference

[1]  RP-182877 New WID on Power Class 2 UE for EN-DC (1 LTE band +1 NR band). CMCC

- Text Proposal

<Start of Text Proposal for TR 37.825>

  - Operating bands for PC2 EN_DC

# Table 2.1-1: DC band combination of LTE 1DL/1UL + one NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_39A_n79A 1 | 39 | n79 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

  - Channel bandwidths per operating band for PC2 EN_DC

# Table 2.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz] | 50 MHz | [60 MHz] | 80 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_39A-n79A | 39 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  | 120 |
|  | n79 | 15 |  |  |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |

  - Co-existence studies for PC2 EN_DC

Table 2.3-1 lists up to 7th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis for Band 39+Band n79 2UL DC.

# Table 2.3-1: Band 39 and Band n79 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1880 | 1920 |  | 4400 | 5000 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fy_low | 2* fy_high |  |
| 2nd harmonics frequency limits (MHz) | 3760-3840 |  |  | 8800-10000 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fy_low |  | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5640-5760 |  |  | 13200-15000 |  |  |
| 4th harmonics frequency limits | 4*fx_low |  | 4*fx_high | 4* fy_low |  | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7520-7680 |  |  | 17600-20000 |  |  |
| 5th harmonics frequency limits | 5*fx_low |  | 5*fx_high | 5*fy_low |  | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 9400-9600 |  |  | 22000-18000 |  |  |
| 6th harmonics frequency limits | 6*fx_low |  | 6*fx_high | 6*fy_low |  | 6*fy_high |
| 6th harmonics frequency limits (MHz) | 11280-11520 |  |  | 26400-3000 |  |  |
| 7th harmonics frequency limits | 7*fx_low |  | 7*fx_high | 7*fy_low |  | 7*fy_high |
| 7th harmonics frequency limits (MHz) | 13160-13440 |  |  | 30800-35000 |  |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2480-3120 |  |  | 6280-6920 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1240-560 |  |  | 6880-8120 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 8160-8840 |  |  | 10680-11920 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1780-2020 |  |  | 4380-5020 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 640-1360 |  |  | 11280-13120 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 6240-4960 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10040-10760 |  |  | 15080-16920 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 12560-13840 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 18120-15680 |  |  | 3280-2520 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11240-9360 |  |  | 3040-4360 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19480-21920 |  |  | 11920-12680 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 16960-18840 |  |  | 14440-15760 |  |  |

Based on Table 2.3-1, the 3rd 4th order IMD product may also fall into Band 39. However it should be noted that IMD will not be an issue for TDD bands (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 2.3-2 lists if up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

# Table 2.3-2: Band 39 and Band n79 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | NO |  |  |
| Galileo | 1559 | - | 1591 | NO |  |  |
| GLONASS | 1591 | - | 1610 | NO |  |  |
| GPS | 1563 | - | 1587 | NO |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YES | US/Europe | 2nd IMD |
|  | 2400 | - | 2494 | YES | Asia | 2nd IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic |
|  | 5150 | - | 5350 | NO | Europe |  |
|  | 5470 | - | 5725 | YES |  | 3rd Harmonic |
|  | 5150 | - | 5825 | YES | Asia | 3rd harmonic |

Table 2.3-3 lists the protected bands required for the dual connectivity configuration.

# Table 2.3-3: Protected bands for the dual connectivity configuration

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_39A-n79A | E-UTRA Band 1, 8, 34, 40, 41, 44, 45 or NR Band n1, n8, n34, n40, n41, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 18 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 18 |
| NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. |  |  |  |  |  |  |  |

  - ∆TIB and ∆RIB values for PC2 EN_DC

For PC2 EN_DC_39A-n79A, the TIB,c and RIB values are given in Table 2.4-1 Table 2.4-2

# Table 2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_39A-n79A | 39 | 0.3 |
|  | n79 | 0.8 |

# Table 2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_39A-n79A | 39 | 0 |
|  | n79 | 0.5 |

<Start of Text Proposal for TR 37.825>