3GPP TSG-RAN WG4 Meeting #90-bis                                 R4-1903021

Xi’an, China, 8 – 12 April, 2019

Agenda Item:		9.20.2

Source: 		CMCC

Title: 	TP for TR 37.825 for PC2 EN-DC_39A-41A

Document for:		Approval

- Introduction

New WID on Power Class 2 UE for EN-DC (1 LTE band +1 NR band) were approved in RAN#82 meeting [1]. This TP summarizes the Band combination specific RF requirements for PC2 EN-DC_39A-41A.

- Reference

[1]  RP-182877 New WID on Power Class 2 UE for EN-DC (1 LTE band +1 NR band). CMCC

- Text Proposal

<Start of Text Proposal for TR 37.825>

  - Operating bands for PC2 EN_DC

# Table 2.1-1: DC band combination of LTE 1DL/1UL + one NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_39_n411 | 39 | n41 | No |
| NOTE 1: Applicable for UE supporting inter-band EN-DC without simultaneous Rx/Tx. |  |  |  |

  - Channel bandwidths per operating band for PC2 EN_DC

# Table 2.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_39A_n41A | DC_39A_n41A | 39A | n41A |

  - Co-existence studies for PC2 EN_DC

Interference analysis for DC_39A-n41A is shown in Table 2.3-1.

# Table 2.3-1: Harmonics and IMD products for DC_39A-n41A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
|  | 1880 | 1920 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
|  | 3760 | 3840 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
|  | 5640 | 5760 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
|  | 7520 | 7680 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
|  | 9400 | 9600 | 12480 | 13450 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
|  | 576 | 810 | 4376 | 4610 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
|  | 1070 | 1344 | 3072 | 3500 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
|  | 6256 | 6530 | 6872 | 7300 |
| Two-tone 3rd order IMD products | ¦fx_low-max BW fy¦ | ¦fx_high+max BW fy¦ | ¦fy_low-max BW fx¦ | ¦fy_high+max BW fx¦ |
|  | 1780 | 2020 | 2476 | 2710 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
|  | 2950 | 3264 | 5568 | 6190 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
|  | 8136 | 8450 | 9368 | 9990 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
|  | -1620 | -1152 | 8752 | 9220 |
| Two-tone 4th order IMD products*1 | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
|  | 576 | 810 | 4376 | 4610 |
| Two-tone 4th order IMD products*1 | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
|  | 3760 | 3840 | 4992 | 5380 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
|  | -8880 | -8064 | -5184 | -4830 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
|  | 11864 | 12680 | 10016 | 10370 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
|  | -4310 | -3648 | -768 | -260 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
|  | 11248 | 11910 | 10632 | 11140 |
| Two-tone 5th order IMD products*2 | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
|  | 5640 | 5760 | 7488 | 8070 |
| Two-tone 5th order IMD products*2 | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
|  | 1070 | 1344 | 3072 | 3500 |
| Two-tone 5th order IMD products*2 | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
|  | 6256 | 6530 | 6872 | 7300 |
| NOTE1: The center frequency of this IMD is the same as that of 2nd harmonics and IMD. It is depending on the proponents of each band combination to include the impact of this IMD in MSD analysis. NOTE2: The center frequency of this IMD is the same as that of 3rd harmonics and IMD. It is depending on the proponents of each band combination to include the impact of this IMD in MSD analysis. |  |  |  |  |

From the above analysis it is seen that the 3rd order IMD will fall into band 39 and band n41 frequency range. However IMD will be an issue since this is a TDD band combination.

Table 2.3-2 lists the protected bands required for the 2UL bands CA configuration.

# Table 2.3-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_39A-n41A | E-UTRA Band 1, 8, 26, 34, 40, 42, 44, 45, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 3 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 3, 4, 5 |
| NOTE 1: To simplify Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 4: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 5: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. |  |  |  |  |  |  |  |

  - ∆TIB and ∆RIB values for PC2 EN_DC

For PC2 EN_DC_39A-n41A, the TIB,c and RIB values are given in Table 2.4-1 Table 2.4-2

# Table 2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_39A-n41 | 39 | 0.5 |
|  | n41 | 0.5 |

# Table 2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_39A-n41 | 39 | 0.2 |
|  | n41 | 0.2 |

<Start of Text Proposal for TR 37.825>