3GPP TSG-RAN WG4 Meeting #90bis	R4-1903062

# Xi’an, China, April 8 – 12, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-01 | CR | DRAFT | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.817-01 to align system level simulation assumptions with MPR definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current definition of the MPR=0 region in TS38.101-2 uses the center 1/3 of RBs for CBW=50,100,200 cases, and this is no longer aligned with the FR2 power class 3 system level simulation assumptions on the resource allocation in the uplink. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates the UL resource allocation for outage evaluation to 15 MHz and the UL resource allocation for mean throughput evaluation to 60 MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Future system level simulations for FR2 power class 3 may not be aligned with the MPR=0 assumptions in the RF specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< start of change >>

#### 7.2.1.4	UE Power class 3

Power class 3 is specified, assuming to be applied for Handheld UE like a smartphone.

##### 7.2.1.4.1	Minimum Peak EIRP requirement

How to derive the requirement

Several companies have provided minimum peak EIRP numbers based on feasible implementation assumptions. A summary of the reported minimum peak EIRP values is found in Table 7.2.1.4.3. After discussion, minimum peak EIRP was agreed as 22.4dBm for 28GHz and as 20.6dBm for 39GHz.

Table 7.2.1.4.1-1: Survey of reported minimum peak EIRP for Power class

| Parameter | Unit | Source 1 |  | Source 2 |  | Source 3 |  | Source 4 |  | Source 5 |  | Source 6 |  | Source 7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range | GHz | 24.2-29.5 | 37-40 | 24.2-29.5 | 37-40 | 24.2-29.5 | 37-40 | 24.2-29.5 | 37 - 40 | 24.2-29.5 | 37 – 40 | 24.2-29.5 | 37 – 40 | 24.2-29.5 | 37-40 |
| # ant elements |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Avg. element gain (per polarization) | dBi | 4.00 | 4.00 | 5.00 | 4.0 | 4.50 | 4.50 | 4.0 | 4.50 | 4.50 | 4.50 | 2.50 | 1.50 | 4.00 | 4.00 |
| Antenna roll-off loss vs frequency | dB | -2.00 | -2.50 | -1.00 | -1.50 | -3.00 | -1.50 | -1.00 | -1.00 | -1.00 | -1.50 | -0.50 | -0.50 | -1.00 | -1.50 |
| Realized antenna array gain | dBi | 8.00 | 7.50 | 10.00 | 8.5 0 | 7.50 | 9.00 | 9.00 | 9.50 | 9.50 | 9.00 | 8.00 | 7.00 | 9.00 | 8.50 |
| Polarization gain | dB | 2.80 | 2.80 | 2.50 | 2.50 | 2.50 | 2.50 | 2.00 | 2.00 | 2.50 | 2.80 | 2.80 | 2.80 | 2.80 | 2.80 |
| Total implementation loss (nominal) | dB | -6.75 | -7.95 | -7.25 | -8.50 | -5.10 | -6.10 | -4.85 | -5.85 | -6.75 | -7.75 | -4.25 | -4.25 | -7.95 | -9.15 |
| Total implementation loss (worst-case) | dB | -9.60 | -10.90 | -10.00 | -11.45 | -7.45 | -8.55 | -8.70 | -8.80 | -9.60 | -10.20 | -6.10 | -6.70 | -10.80 | -12.10 |
| P1d per PA (nominal) | dBm | 14.00 | 14.00 | 14.00 | 14.00 | 14.00 | 12.50 | 14.00 | 12.00 | 14.00 | 12.50 | 14.00 | 14.00 | 14.00 | 14.00 |
| P1d per PA (minimum) | dBm | 14.00 | 14.00 | 14.00 | 14.00 | 14.00 | 12.50 | 14.00 | 12.00 | 14.00 | 12.50 | 14.00 | 14.00 | 14.00 | 14.00 |
| Peak EIRP (nominal) | dBm | 24.05 | 22.35 | 25.25 | 22.5 | 24.90 | 23.90 | 26.15 | 23.65 | 25.25 | 24.05 | 26.55 | 25.55 | 23.85 | 22.15 |
| Tolerance | dB | 3.85 | 3.95 | 3.75 | 3.95 | 3.85 | 3.45 | 3.85 | 3.95 | 2.85 | 3.45 | 2.85 | 3.45 | 3.85 | 3.95 |
| Peak EIRP (minimum) | dBm | 20.20 | 18.40 | 21.50 | 18.55 | 21.05 | 20.45 | 22.30 | 19.70 | 22.40 | 20.60 | 23.70 | 22.10 | 20.00 | 18.20 |
| NOTE 1: We encourage companies to provide implementation losses and P1d numbers for nominal and worst cases to facilitate the analysis of nominal and minimum definitions of max EIRP; the current RAN4 agreement is to define power class as the minimum of the max EIRP without tolerance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Conclusion

The minimum Peak EIRP values are found in Table 7.2.1.4.1-2 below.

Table 7.2.1.4.1-2: UE Minimum Peak EIRP for Power class 3

| Operating Band | Min Peak EIRP (dBm) |
| --- | --- |
| n257 | 22.4 |
| n258 | 22.4 |
| n260 | 20.6 |
| n261 | 22.4 |
| NOTE 1: minimum peak EIRP is defined as the lower limit without tolerance |  |

##### 7.2.1.4.2	Maximum TRP/EIRP requirement

The maximum output power values for TRP and EIRP are found in Table 7.2.1.4.2-1 below. The maximum allowed EIRP is derived from regulatory requirements [12].

Table 7.2.1.4.2-1: UE Maximum Output Power Limits for Power class 3

| Operating Band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |

##### 7.2.1.4.3.	Spherical coverage requirement

How to derive the requirement

Due to study the sensitivity of the network performance to the EIRP percentile, outage performance and throughput performance were evaluated through system simulation. Network simulation assumptions are found in Table 7.2.1.4.3-1.

Table 7.2.1.4.3-1: Network simulation assumptions

| Scenario (in TR38.803) | Indoor hotspots | Dense Urban | Urban Macro |
| --- | --- | --- | --- |
| UE Elevation distribution | Uniform from 0 to 180 degree. |  |  |
| Indoor ratio | 100% | 0%(baseline), 20%, 100% |  |
| Resource Allocation (UL) | ・15 MHz for outage evaluation ・60 MHz for mean throughout evaluation |  |  |
| Blockage Modeling | No body blockage and hand grip modelling |  |  |
| Target UL SNR | 22dB |  |  |
| Antenna pattern Modeling | Option1: Based on mathematical modelling Option2: Based on measured simulated pattern |  |  |

To understand the sensitivity of the network performance to the 50%-tile value, the EIRP was degraded by several dB for all UEs whose EIRP percentile is lower than 50%. In another sets of simulations, the EIRP was reduced as same for all UEs whose EIRP percentile is lower than 20%. By comparing the outage and throughput loss performances, it was observed that the network performance was less sensitive to the variations of EIRP at 20%-tile and slightly decreased at 50%-tile case. Therefor several companies suggested that defining the spherical coverage requirement at 20%-tile value for Power class 3 is impractical but should be specified at not smaller than 50%-tile value. On the other hand, due to lack of sufficient measurement experience, there was the opinion not to be specified below 50%-tile.  After discussion, percentile of spherical coverage for Power class 3 was agreed as 50%-tile.

Based on above evaluation, several companies proposed 50%-tile value. But there was a variety of assumptions for the reported data and some was based of simulations, while a few were based on measurements. To help analyze the data, two tables are provided below summarizing the 50%-tile values for 1 panel and 2 panels. After discussion, spherical coverage in one power class is one specification. Finally, minimum EIRP at 50%-tile CDF is defined as 11.5dBm for 28GHz and as 8dBm for 39Hz.

Table 7.2.1.4.3-2: Summary of 50%-tile values – 1 panel

| Source | Data type | 28GHz |  | 39GHz |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Drop | EIRP | Drop | EIRP |
| Source 1 | Simulated | -14.50 | 8.00 | -18.0 | 2.5 |
| Source 2 | Measured | -14.00 | 8.40 | -15.4 | 5.2 |
| Source 3 | Simulated | -11.60 | 10.8 | -13.6 | 7.0 |
| Source 4 | Measured | -13.10 | 9.30 | - | - |
| Source 5 | Simulated | -11.70 | 10.7 | - | - |
| Average |  | -12.98 | 9.44 | [-18 to -13.6] | [2.5-7.0] |

Table 7.2.1.4.3-3: Summary of 50%-tile values – 2 panel

| Source | Data type | 28GHz |  | 39GHz |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Drop | EIRP | Drop | EIRP |
| Source 1 | Simulated | -13.0 | 9.5 | -16.5 | 4.0 |
| Source 2 | Simulated | -8.00 | 14.4 | -10.0 | 10.6 |
| Source 3 | Measured | -11.6 | 10.8 | -11.6 | 9.0 |
| Source 4 | Measured | -9.10 | 13.3 | -9.20 | 11.4 |
| Source 5 | Simulated | -10 | 12.4 | -12.2 | 8.4 |
| Source 6 | Simulated | -7.40 | 15.0 | [-8.90] | [11.7] |
| Source 7 | Simulated | -10.3 | 12.1 | - | - |
| Source 8 | Simulated | -12.2 | 10.2 | - | - |
| Source 9 | Simulated | -9.0 | 13.4 |  |  |
| Average |  | -10.1 | 12.3 | [-11.4] | [9.2] |

Conclusion

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.2.1.4.3-3 below.

Table 7.2.1.4.3-4: UE spherical coverage requirement for Power class 3

| Operating Band | Min EIRP at 50%-tile CDF [dBm] |
| --- | --- |
| n257 | 11.5 |
| n258 | 11.5 |
| n260 | 8 |
| n261 | 11.5 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are only applicable for UE which supports single band in FR2 |  |

<< end of change >>