3GPP RAN4 WG Meeting #90bis	R4-1903072
Xi’an, China, April 8 – 12, 2019

Agenda item:	9.23.1

Source:	Apple Inc.

Title:	Recommendations for the work plan on Band n259

Document for:	Discussion

# Introduction

During the RAN #83 meeting a new work item to introduce the new NR Band n259 was approved with the following objectives [1]:

The purpose of this work item is to specify a new NR TDD operating band, i.e. Band n259, covering at least the range 39.5-43.5 GHz taking into account the potential benefit of an overlap with Band n260. The lower limit of the band will be further analysed and decided by considering spectrum allocations in different regions, operator demands and RF implementation but should not exceed 39.5 GHz.

The objectives are to define:

- Determine the lower limit of Band n259 if lower than 39.5 GHz.

- Transmitter and receiver characteristics requirements for the UE (38.101-2)

- Transmitter and receiver characteristics requirements for the BS (38.104)

This paper provides recommendations to the work plan on this topic.

# Discussion

To determine the total work scope for UE RF associated with the introduction of the new Band n259, we consider the requirements currently listed in TS38.101-2 [2], identify requirements which are band-dependent, and further evaluate potential work effort needed to specify the related requirements for n259.  Table 1 below summarizes the evaluation.

Table 1: Potential work scope for n259

| Clause | Band dependent? | Potential work scope for n259 |
| --- | --- | --- |
| 6.2.1 UE maximum output power | yes | mmWave array EM sim, NW performance sim, multi-band framework |
| 6.2.2 UE maximum output power reduction | no |  |
| 6.2.3 UE maximum output power with additional requirements | yes | TBD whether NS values specific to n259 are needed |
| 6.2.4 Configured transmitted power | no |  |
| 6.2A.1 UE maximum output power for CA | yes | Reuse requirements from 6.2.1 under CA configuration |
| 6.2A.2 UE maximum output power reduction for CA | no |  |
| 6.2A.3 UE maximum output power with additional requirements for CA | yes | Reuse requirements from 6.2.3 under CA configuration |
| 6.2A.4 Configured transmitted power for CA | no |  |
| 6.2D.1 UE maximum output power for UL-MIMO | yes | Reuse requirements from 6.2.1 under UL-MIMO configuration |
| 6.2D.2 UE maximum output power for modulation / channel bandwidth for UL-MIMO | no |  |
| 6.2D.3 UE maximum output power with additional requirements for UL- MIMO | yes | Reuse requirements from 6.2.3 under UL-MIMO configuration |
| 6.2D.4 Configured transmitted power for UL-MIMO | no |  |
| 6.3.1 Minimum output power | no |  |
| 6.3.2 Transmit off power | no |  |
| 6.3.3 Transmit ON/OFF time mask | no |  |
| 6.3.4 Power control | no |  |
| 6.3A.1 Minimum output power for CA | no |  |
| 6.3A.2 Transmit off power for CA | no |  |
| 6.3A.3 Transmit ON/OFF time mask for CA | no |  |
| 6.3A.4 Power control for CA | no |  |
| 6.3D.1 Minimum output power for UL-MIMO | no |  |
| 6.3D.2 Transmit off power for UL-MIMO | no |  |
| 6.3D.3 Transmit ON/OFF time mask for UL-MIMO | no |  |
| 6.3D.4 Power control for UL-MIMO | no |  |
| 6.4 Transmit signal quality | no |  |
| 6.4 Transmit signal quality for CA | no |  |
| 6.4 Transmit signal quality for UL-MIMO | no |  |
| 6.5.1 Occupied bandwidth | no |  |
| 6.5.2.1 Spectrum emission mask | no |  |
| 6.5.2.3 Adjacent channel leakage ratio | yes | New round of coexistence simulations if ACLR from n260 cannot be reused |
| 6.5.3 Spurious emissions (general) | no |  |
| 6.5.3.1 Spurious emission band UE co-existence | yes | Requirements protecting the new band and existing bands are needed |
| 6.5.3.2 Additional spurious emissions | yes | TBD whether NS values specific to n259 are needed |
| 6.5A.1 Occupied bandwidth for CA | no |  |
| 6.5A.2.1 Spectrum emission mask for CA | no |  |
| 6.5A.2.3 Adjacent channel leakage ratio for CA | yes | Reuse requirements from 6.5.2.3 under CA configuration |
| 6.5A.3 Spurious emissions for CA (general) | no |  |
| 6.5A.3.1 Spurious emission band UE co-existence for CA | yes | Requirements protecting the new band and existing bands are needed |
| 6.5A.3.2 Additional spurious emissions for CA | yes | TBD whether NS values specific to n259 are needed |
| 6.5D.1 Occupied bandwidth for UL-MIMO | no |  |
| 6.5D.2.1 Spectrum emission mask for UL-MIMO | no |  |
| 6.5D.2.3 Adjacent channel leakage ratio for UL-MIMO | yes | Reuse requirements from 6.5.2.3 under UL-MIMO configuration |
| 6.5D.3 Spurious emissions for UL-MIMO (general) | no |  |
| 6.5D.3.1 Spurious emission band UE co-existence for UL-MIMO | yes | Reuse requirements from 6.5.3.1 under UL-MIMO configuration |
| 6.5D.3.2 Additional spurious emissions for UL-MIMO | yes | TBD whether NS values specific to n259 are needed |
| 6.6 Beam correspondence | yes | Beam correspondence tolerance sim |
| 6.6A Beam correspondence for CA | no |  |
| 7.3.2 Reference sensitivity power level | yes | Determine common assumption on UE receiver noise figure, reuse mmWave array sim and multi-band framework |
| 7.3.4 EIS spherical coverage | yes | Reuse mmWave array sim and multi-band framework |
| 7.3A.2 Reference sensitivity power level for CA | yes | Reuse requirements from 7.3.2 under CA configuration |
| 7.3D Reference sensitivity for UL-MIMO | yes | Reuse requirements from 7.3.2 under UL-MIMO configuration |
| 7.4 Maximum input level | no |  |
| 7.4A Maximum input level for CA | no |  |
| 7.4D Maximum input level for UL-MIMO | no |  |
| 7.5 Adjacent channel selectivity | yes | New round of coexistence simulations if ACS from n260 cannot be reused |
| 7.5 Adjacent channel selectivity for CA | yes | Reuse requirements from 7.5 under CA configuration |
| 7.5 Adjacent channel selectivity for UL-MIMO | yes | Reuse requirements from 7.5 under UL-MIMO configuration |
| 7.6.2 In-band blocking | yes | Analyze IBB scenario for n259 and define new requirements |
| 7.6A.2 In-band blocking for CA | yes | Analyze IBB scenario for n259 and define new requirements |
| 7.6D Blocking characteristics for UL-MIMO | yes | Reuse requirements from 7.6.2 under UL-MIMO configuration |
| 7.9 Spurious emissions | no |  |

It is possible to group the work tasks into three work flows, as shown in Table 2 below.

Table 2: Grouping of tasks into work flows

| Work flow | Clause | Potential work scope for n259 |
| --- | --- | --- |
| OTA perf | 6.2.1 UE maximum output power | mmWave array EM sim, NW performance sim, multi-band framework |
| Tx | 6.2.3 UE maximum output power with additional requirements | TBD whether NS values specific to n259 are needed |
| OTA perf | 6.2A.1 UE maximum output power for CA | Reuse requirements from 6.2.1 under CA configuration |
| Tx | 6.2A.3 UE maximum output power with additional requirements for CA | Reuse requirements from 6.2.3 under CA configuration |
| OTA perf | 6.2D.1 UE maximum output power for UL-MIMO | Reuse requirements from 6.2.1 under UL-MIMO configuration |
| Tx | 6.2D.3 UE maximum output power with additional requirements for UL- MIMO | Reuse requirements from 6.2.3 under UL-MIMO configuration |
| Tx | 6.5.2.3 Adjacent channel leakage ratio | New round of coexistence simulations if ACLR from n260 cannot be reused |
| Tx | 6.5.3.1 Spurious emission band UE co-existence | Requirements protecting the new band and existing bands are needed |
| Tx | 6.5.3.2 Additional spurious emissions | TBD whether NS values specific to n259 are needed |
| Tx | 6.5A.2.3 Adjacent channel leakage ratio for CA | Reuse requirements from 6.5.2.3 under CA configuration |
| Tx | 6.5A.3.1 Spurious emission band UE co-existence for CA | Requirements protecting the new band and existing bands are needed |
| Tx | 6.5A.3.2 Additional spurious emissions for CA | TBD whether NS values specific to n259 are needed |
| Tx | 6.5D.2.3 Adjacent channel leakage ratio for UL-MIMO | Reuse requirements from 6.5.2.3 under UL-MIMO configuration |
| Tx | 6.5D.3.1 Spurious emission band UE co-existence for UL-MIMO | Reuse requirements from 6.5.3.1 under UL-MIMO configuration |
| Tx | 6.5D.3.2 Additional spurious emissions for UL-MIMO | TBD whether NS values specific to n259 are needed |
| OTA perf | 6.6 Beam correspondence | Beam correspondence tolerance sim |
| OTA perf | 7.3.2 Reference sensitivity power level | Determine common assumption on UE receiver noise figure, reuse mmWave array sim and multi-band framework |
| OTA perf | 7.3.4 EIS spherical coverage | Reuse mmWave array sim and multi-band framework |
| OTA perf | 7.3A.2 Reference sensitivity power level for CA | Reuse requirements from 7.3.2 under CA configuration |
| OTA perf | 7.3D Reference sensitivity for UL-MIMO | Reuse requirements from 7.3.2 under UL-MIMO configuration |
| Rx | 7.5 Adjacent channel selectivity | New round of coexistence simulations if ACS from n260 cannot be reused |
| Rx | 7.5 Adjacent channel selectivity for CA | Reuse requirements from 7.5 under CA configuration |
| Rx | 7.5 Adjacent channel selectivity for UL-MIMO | Reuse requirements from 7.5 under UL-MIMO configuration |
| Rx | 7.6.2 In-band blocking | Analyze IBB scenario for n259 and define new requirements |
| Rx | 7.6A.2 In-band blocking for CA | Analyze IBB scenario for n259 and define new requirements |
| Rx | 7.6D Blocking characteristics for UL-MIMO | Reuse requirements from 7.6.2 under UL-MIMO configuration |

In this way, each work flow groups similar topics and expertise areas for coordinated discussion, can advance in parallel with the others, and can achieve progress based on incremental agreements.  This work flow grouping can be used to organize the sub-agenda for n259 UE RF work and to define the scope of adhoc or offline discussions.

Proposal 1: The Band n259 work item rapporteur is encouraged to take the scope and work flow task grouping shown in Table 2 into account when defining the work plan.

# Conclusions

Based on the analysis provided in this paper, the following observations and proposals can be made:

Proposal 1: The Band n259 work item rapporteur is encouraged to take the scope and work flow task grouping shown in Table 2 into account when defining the work plan.

# References

- RP-190765, “New WID: introduction of Band n259,” Ericsson, Huawei, HiSilicon, 3GPP RAN #83, March 2019

- 3GPP TS38.101-2, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone,” V15.5.0, 3GPP RAN #83, March 2019

- RP-190761, “NR RF requirements for FR2,” Nokia, Nokia Shanghai Bell, 3GPP RAN #83, March 2019

- 3GPP TR38.817-01, “General aspects for User Equipment (UE) Radio Frequency (RF) for NR,” V15.2.0, 3GPP RAN #82, December 2018