3GPP TSG-RAN WG4 Meeting #90Bis                 	R4-1903204

Xi’an, China, 8th – 12th April 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR PUSCH simulation results

Agenda item:	6.12.2.2.1

Document for:	Discussion

Introduction

In the RAN4#88-bis and RAN4#89 meetings, the way forward for contributing the input for NR PUSCH demodulation performance requirements was agreed [1, 2]. A procedure to arrive at performance requirement SNR numbers from the contributions of all companies was furthermore agreed in [3] and new test case configurations (FR1 type B, FR2 DMRS 1+1, FR2 without PT-RS for 4QAM/16QAM/64QAM) have been added in RAN4#90 [4].

In this contribution we deliver the new configuration results and corrections of our previously delivered results for the agreed basic simulation cases.

Simulation Results

Simulation settings - transform precoding disabled/CP

For transform precoding disabled case, the following simulation assumptions were agreed in [1, 2] and [4]:

Table 1: Simulation parameters – Transform precoding disabled

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1, 2 | 1, 2 |
| Number of Rx | 2, 4, 8 | 2 |
| Number of layers | 1, 2 | 1, 2 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1, 1+1 | 1, 1+1 |
| symbols length | 14 | 10 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A, type B | type B |
| Frequency domain resource | full RB allocation of the applicable BW | full RB allocation of the applicable BW |
| MCS index | 2, 16, 20 (20 only for 1Tx) | 2, 16, 20 (20 only for 1Tx) |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation condition | MCS 2: TDL-B 100ns, 400 Hz MCS 16: TDL-C 300ns, 100 Hz MCS 20: TDL-A 30ns, 10Hz | MCS 2: TDL-A 30ns, 300Hz MCS 16: TDL-A 30ns, 300Hz MCS 20: TDL-A 30ns, 75Hz |
| SCS and BW | 15kHz: 5MHz, 10MHz, 20MHz 30kHz: 10MHz, 20MHz, 40MHz, 100MHz 60kHz(FR2): 50MHz, 100MHz 120kHz: 50MHz, 100MHz, 200MHz |  |
| PTRS | Not configured | QPSK: Not configured 16 and 64 QAM: (Configured with KPTRS =2, LPTRS =1) and (not configured) |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group | Disabled | Disabled |
| Frequency hopping | Disabled | Disabled |
| Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |

Remark: Following the agreement in [2], the DMRS configuration 1+0 is not applicable in FR1 under 2T2R.

Simulation settings - transform precoding enabled/DFT

For transform precoding enabled case, the following simulation assumptions were agreed in [1] and refined in [2, 4]:

Table 2: Simulation parameters – Transform precoding enabled

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | enabled | enabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2, 4, 8 | 2 |
| Number of layers | 1 | 1 |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1+1, 1 | 1, 1+1 |
| symbols length | 14 | 10 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A, type B | type B |
| Frequency domain resource | 15kHz: 25 PRB (contig., middle of test BW) 30kHz: 24 PRB (contig., middle of test BW) | 60kHz: 30 PRB (contig., middle of test BW) 120kHz: 30 PRB (contig., middle of test BW) |
| MCS index | 2 | 2 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation condition | MCS 2: TDL-B 100ns, 400 Hz | MCS 2: TDL-A 30ns, 300Hz |
| SCS and BW | 15kHz: 5 MHz 30kHz: 10 MHz | 60kHz: 50 MHz 120kHz: 50 MHz |
| PTRS | Not configured | Not configured |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group | Disabled | Disabled |
| Frequency hopping | Disabled | Disabled |
| Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |

Simulation results PUSCH CP 1T2R

Simulation results for PUSCH CP 1T2R and with the parameters defined are shown in Table 3.

Table 3: Simulation results for PUSCH CP 1T2R

| PUSCH 1T2R |  |  |  |  |  | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impaired |
| CP-OFDM | FR1 Mappying type A | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -1,32 | 1,18 |
|  |  |  |  |  | DMRS 1+1 | -4,68 | -2,18 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,68 | 12,18 |
|  |  |  |  |  | DMRS 1+1 | 7,81 | 10,31 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 9,85 | 12,35 |
|  |  |  |  |  | DMRS 1+1 | 9,77 | 12,27 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,92 | -1,42 |
|  |  |  |  |  | DMRS 1+1 | -4,84 | -2,34 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 7,94 | 10,44 |
|  |  |  |  |  | DMRS 1+1 | 7,62 | 10,12 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 9,79 | 12,29 |
|  |  |  |  |  | DMRS 1+1 | 9,54 | 12,04 |
|  | FR1 Mappying type B | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | -4,41 | -1,91 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | 7,87 | 10,37 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 9,88 | 12,38 |
|  |  |  |  |  | DMRS 1+1 | 9,79 | 12,29 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,14 | -0,64 |
|  |  |  |  |  | DMRS 1+1 | -4,8 | -2,3 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 8,22 | 10,72 |
|  |  |  |  |  | DMRS 1+1 | 7,64 | 10,14 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 9,78 | 12,28 |
|  |  |  |  |  | DMRS 1+1 | 9,53 | 12,03 |
|  | FR2 with PTRS | 60kHz, 100MHz | MCS2 (To be removed) | TDLA30-300 | DMRS 1 | -4,46 | -1,36 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 8,98 | 12,08 |
|  |  |  |  |  | DMRS 1+1 | NaN | NaN |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 10,59 | 13,69 |
|  |  |  |  |  | DMRS 1+1 | NaN | NaN |
|  |  | 120kHz, 100MHz | MCS2 (To be removed) | TDLA30-300 | DMRS 1 | -5,08 | -1,98 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 7,69 | 10,79 |
|  |  |  |  |  | DMRS 1+1 | 7,53 | 10,63 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 10 | 13,1 |
|  |  |  |  |  | DMRS 1+1 | 10,11 | 13,21 |
|  | FR2 without PTRS | 60kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -4,65 | -1,55 |
|  |  |  |  |  | DMRS 1+1 | -4,78 | -1,68 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 8,6 | 11,7 |
|  |  |  |  |  | DMRS 1+1 | 7,58 | 10,68 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 10,2 | 13,3 |
|  |  |  |  |  | DMRS 1+1 | 10,08 | 13,18 |
|  |  | 120kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -5,26 | -2,16 |
|  |  |  |  |  | DMRS 1+1 | -5,28 | -2,18 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 7,22 | 10,32 |
|  |  |  |  |  | DMRS 1+1 | 7,11 | 10,21 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 9,56 | 12,66 |
|  |  |  |  |  | DMRS 1+1 | 9,57 | 12,67 |
| CP-OFDM | FR1 Mappying type A | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -0,28 | 2,22 |
|  |  |  |  |  | DMRS 1+1 | -4,4 | -1,9 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,16 | 11,66 |
|  |  |  |  |  | DMRS 1+1 | 7,84 | 10,34 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 9,7 | 12,2 |
|  |  |  |  |  | DMRS 1+1 | 9,63 | 12,13 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | -3,91 | -1,41 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 10,09 | 12,59 |
|  |  |  |  |  | DMRS 1+1 | 7,81 | 10,31 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 9,71 | 12,21 |
|  |  |  |  |  | DMRS 1+1 | 9,48 | 11,98 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,3 | -1,8 |
|  |  |  |  |  | DMRS 1+1 | -4,87 | -2,37 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 7,72 | 10,22 |
|  |  |  |  |  | DMRS 1+1 | 7,72 | 10,22 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 9,54 | 12,04 |
|  |  |  |  |  | DMRS 1+1 | 9,75 | 12,25 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,51 | -2,01 |
|  |  |  |  |  | DMRS 1+1 | -4,97 | -2,47 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 8 | 10,5 |
|  |  |  |  |  | DMRS 1+1 | 7,71 | 10,21 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 9,71 | 12,21 |
|  |  |  |  |  | DMRS 1+1 | 9,61 | 12,11 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,36 | -0,86 |
|  |  |  |  |  | DMRS 1+1 | -4,89 | -2,39 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 7,99 | 10,49 |
|  |  |  |  |  | DMRS 1+1 | 7,6 | 10,1 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 10,73 | 13,23 |
|  |  |  |  |  | DMRS 1+1 | 10,43 | 12,93 |
|  | FR1 Mappying type B | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | -4,1 | -1,6 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 10,77 | 13,27 |
|  |  |  |  |  | DMRS 1+1 | 7,89 | 10,39 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 9,67 | 12,17 |
|  |  |  |  |  | DMRS 1+1 | 9,61 | 12,11 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | -3,77 | -1,27 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | 7,91 | 10,41 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 9,76 | 12,26 |
|  |  |  |  |  | DMRS 1+1 | 9,5 | 12 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,78 | -1,28 |
|  |  |  |  |  | DMRS 1+1 | -4,79 | -2,29 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 7,95 | 10,45 |
|  |  |  |  |  | DMRS 1+1 | 7,71 | 10,21 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 9,54 | 12,04 |
|  |  |  |  |  | DMRS 1+1 | 9,73 | 12,23 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,04 | -1,54 |
|  |  |  |  |  | DMRS 1+1 | -4,92 | -2,42 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 8,23 | 10,73 |
|  |  |  |  |  | DMRS 1+1 | 7,7 | 10,2 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 9,7 | 12,2 |
|  |  |  |  |  | DMRS 1+1 | 9,6 | 12,1 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,18 | 0,32 |
|  |  |  |  |  | DMRS 1+1 | -4,82 | -2,32 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 8,28 | 10,78 |
|  |  |  |  |  | DMRS 1+1 | 7,61 | 10,11 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 10,7 | 13,2 |
|  |  |  |  |  | DMRS 1+1 | 10,43 | 12,93 |
|  | FR2 with PTRS | 60kHz, 50MHz | MCS2 (To be removed) | TDLA30-300 | DMRS 1 | -4,25 | -1,15 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 8,76 | 11,86 |
|  |  |  |  |  | DMRS 1+1 | 8,13 | 11,23 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 9,98 | 13,08 |
|  |  |  |  |  | DMRS 1+1 | 9,99 | 13,09 |
|  |  | 120kHz, 50MHz | MCS2 (To be removed) | TDLA30-300 | DMRS 1 | -4,64 | -1,54 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 7,61 | 10,71 |
|  |  |  |  |  | DMRS 1+1 | 7,24 | 10,34 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 10,02 | 13,12 |
|  |  |  |  |  | DMRS 1+1 | 9,98 | 13,08 |
|  |  | 120kHz, 200MHz | MCS2 (To be removed) | TDLA30-300 | DMRS 1 | -5,01 | -1,91 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 7,91 | 11,01 |
|  |  |  |  |  | DMRS 1+1 | 7,63 | 10,73 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 11,04 | 14,14 |
|  |  |  |  |  | DMRS 1+1 | 10,29 | 13,39 |
|  | FR2 without PTRS | 60kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -4,42 | -1,32 |
|  |  |  |  |  | DMRS 1+1 | -4,43 | -1,33 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 8,39 | 11,49 |
|  |  |  |  |  | DMRS 1+1 | 7,53 | 10,63 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 9,58 | 12,68 |
|  |  |  |  |  | DMRS 1+1 | 9,56 | 12,66 |
|  |  | 120kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -4,84 | -1,74 |
|  |  |  |  |  | DMRS 1+1 | -4,88 | -1,78 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 7,17 | 10,27 |
|  |  |  |  |  | DMRS 1+1 | 6,78 | 9,88 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 9,63 | 12,73 |
|  |  |  |  |  | DMRS 1+1 | 9,55 | 12,65 |
|  |  | 120kHz, 200MHz | MCS2 | TDLA30-300 | DMRS 1 | -5,2 | -2,1 |
|  |  |  |  |  | DMRS 1+1 | -5,23 | -2,13 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 7,45 | 10,55 |
|  |  |  |  |  | DMRS 1+1 | 7,15 | 10,25 |
|  |  |  | MCS20 | TDLA30-75 | DMRS 1 | 9,83 | 12,93 |
|  |  |  |  |  | DMRS 1+1 | 9,85 | 12,95 |

Simulation results PUSCH CP 1T4R

Simulation results for PUSCH CP 1T4R and with the parameters defined are shown in Table 4.

Table 4: Simulation results for PUSCH CP 1T4R

| PUSCH 1T4R |  |  |  |  |  | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impaired |
| CP-OFDM | FR1 Mappying type A | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -5,55 | -3,05 |
|  |  |  |  |  | DMRS 1+1 | -7,91 | -5,41 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,67 | 7,17 |
|  |  |  |  |  | DMRS 1+1 | 3,9 | 6,4 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,23 | 8,73 |
|  |  |  |  |  | DMRS 1+1 | 6,14 | 8,64 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -7,35 | -4,85 |
|  |  |  |  |  | DMRS 1+1 | -8,12 | -5,62 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 3,81 | 6,31 |
|  |  |  |  |  | DMRS 1+1 | 3,7 | 6,2 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,27 | 8,77 |
|  |  |  |  |  | DMRS 1+1 | 5,97 | 8,47 |
|  | FR1 Mappying type B | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | 7,65 | 10,15 |
|  |  |  |  |  | DMRS 1+1 | -7,74 | -5,24 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,36 | 7,86 |
|  |  |  |  |  | DMRS 1+1 | 3,92 | 6,42 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,21 | 8,71 |
|  |  |  |  |  | DMRS 1+1 | 6,14 | 8,64 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,67 | -4,17 |
|  |  |  |  |  | DMRS 1+1 | -8,09 | -5,59 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 3,93 | 6,43 |
|  |  |  |  |  | DMRS 1+1 | 3,7 | 6,2 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,27 | 8,77 |
|  |  |  |  |  | DMRS 1+1 | 5,96 | 8,46 |
| CP-OFDM | FR1 Mappying type A | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -5,21 | -2,71 |
|  |  |  |  |  | DMRS 1+1 | -7,76 | -5,26 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,29 | 6,79 |
|  |  |  |  |  | DMRS 1+1 | 3,74 | 6,24 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,12 | 8,62 |
|  |  |  |  |  | DMRS 1+1 | 6,11 | 8,61 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | 8,81 | 11,31 |
|  |  |  |  |  | DMRS 1+1 | -7,39 | -4,89 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,63 | 7,13 |
|  |  |  |  |  | DMRS 1+1 | 3,78 | 6,28 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,2 | 8,7 |
|  |  |  |  |  | DMRS 1+1 | 5,91 | 8,41 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -7,67 | -5,17 |
|  |  |  |  |  | DMRS 1+1 | -8,11 | -5,61 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 3,72 | 6,22 |
|  |  |  |  |  | DMRS 1+1 | 3,83 | 6,33 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 5,91 | 8,41 |
|  |  |  |  |  | DMRS 1+1 | 6,1 | 8,6 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -7,8 | -5,3 |
|  |  |  |  |  | DMRS 1+1 | -8,19 | -5,69 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 3,9 | 6,4 |
|  |  |  |  |  | DMRS 1+1 | 3,77 | 6,27 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,07 | 8,57 |
|  |  |  |  |  | DMRS 1+1 | 5,98 | 8,48 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,78 | -4,28 |
|  |  |  |  |  | DMRS 1+1 | -8,12 | -5,62 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 3,89 | 6,39 |
|  |  |  |  |  | DMRS 1+1 | 3,7 | 6,2 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,71 | 9,21 |
|  |  |  |  |  | DMRS 1+1 | 6,52 | 9,02 |
|  | FR1 Mappying type B | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | 3,94 | 6,44 |
|  |  |  |  |  | DMRS 1+1 | -7,57 | -5,07 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,85 | 7,35 |
|  |  |  |  |  | DMRS 1+1 | 3,72 | 6,22 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,13 | 8,63 |
|  |  |  |  |  | DMRS 1+1 | 6,1 | 8,6 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | -7,23 | -4,73 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,32 | 7,82 |
|  |  |  |  |  | DMRS 1+1 | 3,82 | 6,32 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,2 | 8,7 |
|  |  |  |  |  | DMRS 1+1 | 5,9 | 8,4 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -7,26 | -4,76 |
|  |  |  |  |  | DMRS 1+1 | -8,05 | -5,55 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 3,81 | 6,31 |
|  |  |  |  |  | DMRS 1+1 | 3,86 | 6,36 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 5,9 | 8,4 |
|  |  |  |  |  | DMRS 1+1 | 6,09 | 8,59 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -7,46 | -4,96 |
|  |  |  |  |  | DMRS 1+1 | -8,15 | -5,65 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4 | 6,5 |
|  |  |  |  |  | DMRS 1+1 | 3,78 | 6,28 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,05 | 8,55 |
|  |  |  |  |  | DMRS 1+1 | 6 | 8,5 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -5,89 | -3,39 |
|  |  |  |  |  | DMRS 1+1 | -8,06 | -5,56 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4 | 6,5 |
|  |  |  |  |  | DMRS 1+1 | 3,7 | 6,2 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 6,72 | 9,22 |
|  |  |  |  |  | DMRS 1+1 | 6,51 | 9,01 |

Simulation results PUSCH CP 1T8R

Simulation results for PUSCH CP 1T8R and with the parameters defined are shown in Table 5.

Table 5: Simulation results for PUSCH CP 1T8R

| PUSCH 1T8R |  |  |  |  |  | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impaired |
| CP-OFDM | FR1 Mappying type A | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -8,78 | -6,28 |
|  |  |  |  |  | DMRS 1+1 | -10,77 | -8,27 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,98 | 3,48 |
|  |  |  |  |  | DMRS 1+1 | 0,46 | 2,96 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 3,02 | 5,52 |
|  |  |  |  |  | DMRS 1+1 | 2,95 | 5,45 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -10,16 | -7,66 |
|  |  |  |  |  | DMRS 1+1 | -10,91 | -8,41 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,4 | 2,9 |
|  |  |  |  |  | DMRS 1+1 | 0,33 | 2,83 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 3 | 5,5 |
|  |  |  |  |  | DMRS 1+1 | 2,76 | 5,26 |
|  | FR1 Mappying type B | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,23 | -3,73 |
|  |  |  |  |  | DMRS 1+1 | -10,61 | -8,11 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 1,38 | 3,88 |
|  |  |  |  |  | DMRS 1+1 | 0,46 | 2,96 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 3,02 | 5,52 |
|  |  |  |  |  | DMRS 1+1 | 2,97 | 5,47 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -9,57 | -7,07 |
|  |  |  |  |  | DMRS 1+1 | -10,9 | -8,4 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,47 | 2,97 |
|  |  |  |  |  | DMRS 1+1 | 0,34 | 2,84 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 2,99 | 5,49 |
|  |  |  |  |  | DMRS 1+1 | 2,76 | 5,26 |
| CP-OFDM | FR1 Mappying type A | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -8,59 | -6,09 |
|  |  |  |  |  | DMRS 1+1 | -10,73 | -8,23 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,71 | 3,21 |
|  |  |  |  |  | DMRS 1+1 | 0,33 | 2,83 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 2,94 | 5,44 |
|  |  |  |  |  | DMRS 1+1 | 2,91 | 5,41 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,42 | -1,92 |
|  |  |  |  |  | DMRS 1+1 | -10,42 | -7,92 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,86 | 3,36 |
|  |  |  |  |  | DMRS 1+1 | 0,32 | 2,82 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 3,08 | 5,58 |
|  |  |  |  |  | DMRS 1+1 | 2,82 | 5,32 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -10,56 | -8,06 |
|  |  |  |  |  | DMRS 1+1 | -10,97 | -8,47 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,37 | 2,87 |
|  |  |  |  |  | DMRS 1+1 | 0,51 | 3,01 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 2,77 | 5,27 |
|  |  |  |  |  | DMRS 1+1 | 2,96 | 5,46 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -10,68 | -8,18 |
|  |  |  |  |  | DMRS 1+1 | -11,04 | -8,54 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,52 | 3,02 |
|  |  |  |  |  | DMRS 1+1 | 0,43 | 2,93 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 2,94 | 5,44 |
|  |  |  |  |  | DMRS 1+1 | 2,86 | 5,36 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -9,66 | -7,16 |
|  |  |  |  |  | DMRS 1+1 | -10,9 | -8,4 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,47 | 2,97 |
|  |  |  |  |  | DMRS 1+1 | 0,36 | 2,86 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 3,51 | 6,01 |
|  |  |  |  |  | DMRS 1+1 | 3,34 | 5,84 |
|  | FR1 Mappying type B | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -5,71 | -3,21 |
|  |  |  |  |  | DMRS 1+1 | -10,52 | -8,02 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 1,06 | 3,56 |
|  |  |  |  |  | DMRS 1+1 | 0,36 | 2,86 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 2,94 | 5,44 |
|  |  |  |  |  | DMRS 1+1 | 2,9 | 5,4 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | 21,74 | 24,24 |
|  |  |  |  |  | DMRS 1+1 | -10,28 | -7,78 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 1,33 | 3,83 |
|  |  |  |  |  | DMRS 1+1 | 0,35 | 2,85 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 3,09 | 5,59 |
|  |  |  |  |  | DMRS 1+1 | 2,83 | 5,33 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -10,2 | -7,7 |
|  |  |  |  |  | DMRS 1+1 | -10,93 | -8,43 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,43 | 2,93 |
|  |  |  |  |  | DMRS 1+1 | 0,52 | 3,02 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 2,78 | 5,28 |
|  |  |  |  |  | DMRS 1+1 | 2,94 | 5,44 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -10,34 | -7,84 |
|  |  |  |  |  | DMRS 1+1 | -11,01 | -8,51 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,58 | 3,08 |
|  |  |  |  |  | DMRS 1+1 | 0,44 | 2,94 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 2,95 | 5,45 |
|  |  |  |  |  | DMRS 1+1 | 2,86 | 5,36 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -8,86 | -6,36 |
|  |  |  |  |  | DMRS 1+1 | -10,85 | -8,35 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 0,57 | 3,07 |
|  |  |  |  |  | DMRS 1+1 | 0,38 | 2,88 |
|  |  |  | MCS20 | TDLA30-10 | DMRS 1 | 3,51 | 6,01 |
|  |  |  |  |  | DMRS 1+1 | 3,34 | 5,84 |

Simulation results PUSCH CP 2T2R

Simulation results for PUSCH CP 2T2R and with the parameters defined are shown in Table 6.

Table 6: Simulation results for PUSCH CP 2T2R

| PUSCH 2T2R |  |  |  |  |  | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impaired |
| CP-OFDM | FR1 Mappying type A | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,09 | 2,41 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,94 | 19,44 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -1,04 | 1,46 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 15,76 | 18,26 |
|  | FR1 Mappying type B | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 0,22 | 2,72 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 17,74 | 20,24 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,97 | 1,53 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 15,89 | 18,39 |
|  | FR2 with PTRS | 60kHz, 100MHz | MCS2 (To be removed) | TDLA30-300 | DMRS 1 | -0,9 | 2,2 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | 16,76 | 19,86 |
|  |  | 120kHz, 100MHz | MCS2 (To be removed) | TDLA30-300 | DMRS 1 | -1,29 | 1,81 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 18,06 | 21,16 |
|  |  |  |  |  | DMRS 1+1 | 15,44 | 18,54 |
|  | FR2 without PTRS | 60kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -1,13 | 1,97 |
|  |  |  |  |  | DMRS 1+1 | -1,16 | 1,94 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | 15,92 | 19,02 |
|  |  | 120kHz, 100MHz | MCS2 | TDLA30-300 | DMRS 1 | -1,52 | 1,58 |
|  |  |  |  |  | DMRS 1+1 | -1,59 | 1,51 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 15,93 | 19,03 |
|  |  |  |  |  | DMRS 1+1 | 14,77 | 17,87 |
| CP-OFDM | FR1 Mappying type A | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,73 | 1,77 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,27 | 18,77 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 0,3 | 2,8 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,79 | 19,29 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -1,33 | 1,17 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16 | 18,5 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,99 | 1,51 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 15,84 | 18,34 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,94 | 1,56 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,36 | 18,86 |
|  | FR1 Mappying type B | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,32 | 2,18 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,59 | 19,09 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 1,08 | 3,58 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 18,46 | 20,96 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -1,26 | 1,24 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,09 | 18,59 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,91 | 1,59 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 15,93 | 18,43 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1+1 | -0,85 | 1,65 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1+1 | 16,61 | 19,11 |
|  | FR2 with PTRS | 60kHz, 50MHz | MCS2 (To be removed) | TDLA30-300 | DMRS 1 | -0,72 | 2,38 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | 16,55 | 19,65 |
|  |  | 120kHz, 50MHz | MCS2 (To be removed) | TDLA30-300 | DMRS 1 | -1,38 | 1,72 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 16,77 | 19,87 |
|  |  |  |  |  | DMRS 1+1 | 15,47 | 18,57 |
|  |  | 120kHz, 200MHz | MCS2 (To be removed) | TDLA30-300 | DMRS 1 | -1,34 | 1,76 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 16,87 | 19,97 |
|  |  |  |  |  | DMRS 1+1 | 15,47 | 18,57 |
|  | FR2 without PTRS | 60kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -0,97 | 2,13 |
|  |  |  |  |  | DMRS 1+1 | -1,07 | 2,03 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | 15,78 | 18,88 |
|  |  | 120kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | -1,58 | 1,52 |
|  |  |  |  |  | DMRS 1+1 | -1,41 | 1,69 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 15,88 | 18,98 |
|  |  |  |  |  | DMRS 1+1 | 14,81 | 17,91 |
|  |  | 120kHz, 200MHz | MCS2 | TDLA30-300 | DMRS 1 | -1,57 | 1,53 |
|  |  |  |  |  | DMRS 1+1 | -1,51 | 1,59 |
|  |  |  | MCS16 | TDLA30-300 | DMRS 1 | 15,87 | 18,97 |
|  |  |  |  |  | DMRS 1+1 | 14,79 | 17,89 |

Simulation results PUSCH CP 2T4R

Simulation results for PUSCH CP 2T4R and with the parameters defined are shown in Table 7.

Table 7: Simulation results for PUSCH CP 2T4R

| PUSCH 2T4R |  |  |  |  |  | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impaired |
| CP-OFDM | FR1 Mappying type A | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | -4,1 | -1,6 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 21,61 | 24,11 |
|  |  |  |  |  | DMRS 1+1 | 8,73 | 11,23 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,01 | -0,51 |
|  |  |  |  |  | DMRS 1+1 | -4,72 | -2,22 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,04 | 11,54 |
|  |  |  |  |  | DMRS 1+1 | 8,46 | 10,96 |
|  | FR1 Mappying type B | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | -3,84 | -1,34 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | 8,82 | 11,32 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -1,8 | 0,7 |
|  |  |  |  |  | DMRS 1+1 | -4,63 | -2,13 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,51 | 12,01 |
|  |  |  |  |  | DMRS 1+1 | 8,46 | 10,96 |
| CP-OFDM | FR1 Mappying type A | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -1,04 | 1,46 |
|  |  |  |  |  | DMRS 1+1 | -4,41 | -1,91 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 12,91 | 15,41 |
|  |  |  |  |  | DMRS 1+1 | 8,51 | 11,01 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | -3,84 | -1,34 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | 8,57 | 11,07 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,22 | -1,72 |
|  |  |  |  |  | DMRS 1+1 | -4,91 | -2,41 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,07 | 11,57 |
|  |  |  |  |  | DMRS 1+1 | 8,54 | 11,04 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,62 | -1,12 |
|  |  |  |  |  | DMRS 1+1 | -4,68 | -2,18 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,06 | 11,56 |
|  |  |  |  |  | DMRS 1+1 | 8,52 | 11,02 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,56 | -0,06 |
|  |  |  |  |  | DMRS 1+1 | -4,66 | -2,16 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,57 | 12,07 |
|  |  |  |  |  | DMRS 1+1 | 8,76 | 11,26 |
|  | FR1 Mappying type B | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | -4,15 | -1,65 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | 8,63 | 11,13 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | -3,33 | -0,83 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | 8,74 | 11,24 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -3,74 | -1,24 |
|  |  |  |  |  | DMRS 1+1 | -4,83 | -2,33 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,51 | 12,01 |
|  |  |  |  |  | DMRS 1+1 | 8,57 | 11,07 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -2,74 | -0,24 |
|  |  |  |  |  | DMRS 1+1 | -4,62 | -2,12 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 9,51 | 12,01 |
|  |  |  |  |  | DMRS 1+1 | 8,54 | 11,04 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -1,04 | 1,46 |
|  |  |  |  |  | DMRS 1+1 | -4,58 | -2,08 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 10,13 | 12,63 |
|  |  |  |  |  | DMRS 1+1 | 8,8 | 11,3 |

Simulation results PUSCH CP 2T8R

Simulation results for PUSCH CP 2T8R and with the parameters defined are shown in Table 8.

Table 8: Simulation results for PUSCH CP 2T8R

| PUSCH 2T8R |  |  |  |  |  | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impaired |
| CP-OFDM | FR1 Mappying type A | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | 7,67 | 10,17 |
|  |  |  |  |  | DMRS 1+1 | -7,26 | -4,76 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,73 | 8,23 |
|  |  |  |  |  | DMRS 1+1 | 4,28 | 6,78 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,34 | -3,84 |
|  |  |  |  |  | DMRS 1+1 | -7,73 | -5,23 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,4 | 6,9 |
|  |  |  |  |  | DMRS 1+1 | 4,2 | 6,7 |
|  | FR1 Mappying type B | 15kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | -7,07 | -4,57 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 7,02 | 9,52 |
|  |  |  |  |  | DMRS 1+1 | 4,33 | 6,83 |
|  |  | 30kHz, 40MHz | MCS2 | TDLB100-400 | DMRS 1 | -5,45 | -2,95 |
|  |  |  |  |  | DMRS 1+1 | -7,68 | -5,18 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,57 | 7,07 |
|  |  |  |  |  | DMRS 1+1 | 4,2 | 6,7 |
| CP-OFDM | FR1 Mappying type A | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | -5,09 | -2,59 |
|  |  |  |  |  | DMRS 1+1 | -7,45 | -4,95 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,29 | 7,79 |
|  |  |  |  |  | DMRS 1+1 | 4,16 | 6,66 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | -7,08 | -4,58 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 5,77 | 8,27 |
|  |  |  |  |  | DMRS 1+1 | 4,17 | 6,67 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -7,28 | -4,78 |
|  |  |  |  |  | DMRS 1+1 | -7,81 | -5,31 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,56 | 7,06 |
|  |  |  |  |  | DMRS 1+1 | 4,33 | 6,83 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,8 | -4,3 |
|  |  |  |  |  | DMRS 1+1 | -7,72 | -5,22 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,46 | 6,96 |
|  |  |  |  |  | DMRS 1+1 | 4,27 | 6,77 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,04 | -3,54 |
|  |  |  |  |  | DMRS 1+1 | -7,7 | -5,2 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,7 | 7,2 |
|  |  |  |  |  | DMRS 1+1 | 4,41 | 6,91 |
|  | FR1 Mappying type B | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1 | 5,44 | 7,94 |
|  |  |  |  |  | DMRS 1+1 | -7,23 | -4,73 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 6,3 | 8,8 |
|  |  |  |  |  | DMRS 1+1 | 4,17 | 6,67 |
|  |  | 15kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | -6,66 | -4,16 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 7,16 | 9,66 |
|  |  |  |  |  | DMRS 1+1 | 4,2 | 6,7 |
|  |  | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,87 | -4,37 |
|  |  |  |  |  | DMRS 1+1 | -7,76 | -5,26 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,73 | 7,23 |
|  |  |  |  |  | DMRS 1+1 | 4,33 | 6,83 |
|  |  | 30kHz, 20MHz | MCS2 | TDLB100-400 | DMRS 1 | -6,11 | -3,61 |
|  |  |  |  |  | DMRS 1+1 | -7,68 | -5,18 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,64 | 7,14 |
|  |  |  |  |  | DMRS 1+1 | 4,28 | 6,78 |
|  |  | 30kHz, 100MHz | MCS2 | TDLB100-400 | DMRS 1 | -4,98 | -2,48 |
|  |  |  |  |  | DMRS 1+1 | -7,64 | -5,14 |
|  |  |  | MCS16 | TDLC300-100 | DMRS 1 | 4,89 | 7,39 |
|  |  |  |  |  | DMRS 1+1 | 4,42 | 6,92 |

Simulation results PUSCH DFT

Simulation results for PUSCH DFT with the parameters defined are shown in Table 9.

Table 9: Simulation results for PUSCH DFT

| PUSCH DFT-s-OFDM |  |  |  |  |  | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impaired |
| DFT-s-OFDM 1T2R | FR1 Mappying type A | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | -0,51 | 1,99 |
|  |  |  |  |  | DMRS 1+1 | -4,72 | -2,22 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -4,5 | -2 |
|  |  |  |  |  | DMRS 1+1 | -5,07 | -2,57 |
|  | FR1 Mappying type B | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | NaN | NaN |
|  |  |  |  |  | DMRS 1+1 | -4,42 | -1,92 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -3,93 | -1,43 |
|  |  |  |  |  | DMRS 1+1 | 5,06 | 7,56 |
|  | FR2 with PTRS | 60kHz, 50MHz (30 PRB) | TDLA30-300 | MCS2 (To be removed) | DMRS 1 | -4,42 | -1,32 |
|  |  | 120kHz, 50MHz (30 PRB) | TDLA30-300 | MCS2 (To be removed) | DMRS 1 | -4,89 | -1,79 |
|  | FR2 without PTRS | 60kHz, 50MHz (30 PRB) | TDLA30-300 | MCS2 | DMRS 1 | -4,62 | -1,52 |
|  |  |  |  |  | DMRS 1+1 | -4,57 | -1,47 |
|  |  | 120kHz, 50MHz (30 PRB) | TDLA30-300 | MCS2 | DMRS 1 | -5,13 | -2,03 |
|  |  |  |  |  | DMRS 1+1 | -5,03 | -1,93 |
| DFT-s-OFDM 1T4R | FR1 Mappying type A | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | -5,41 | -2,91 |
|  |  |  |  |  | DMRS 1+1 | -8,06 | -5,56 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -7,87 | -5,37 |
|  |  |  |  |  | DMRS 1+1 | -8,32 | -5,82 |
|  | FR1 Mappying type B | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | 4,93 | 7,43 |
|  |  |  |  |  | DMRS 1+1 | -7,82 | -5,32 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -7,44 | -4,94 |
|  |  |  |  |  | DMRS 1+1 | -8,28 | -5,78 |
| DFT-s-OFDM 1T8R | FR1 Mappying type A | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | -8,87 | -6,37 |
|  |  |  |  |  | DMRS 1+1 | -10,94 | -8,44 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -10,67 | -8,17 |
|  |  |  |  |  | DMRS 1+1 | -11,14 | -8,64 |
|  | FR1 Mappying type B | 15kHz, 5MHz (25PRB) | TDLB100-400 | MCS2 | DMRS 1 | -6,1 | -3,6 |
|  |  |  |  |  | DMRS 1+1 | -10,74 | -8,24 |
|  |  | 30kHz, 10MHz (24 PRB) | TDLB100-400 | MCS2 | DMRS 1 | -10,35 | -7,85 |
|  |  |  |  |  | DMRS 1+1 | -11,08 | -8,58 |

Conclusion

In this contribution we have provided simulation results for NR PUSCH test cases.

- References

- R4-1813942, WF on NR PUSCH demodulation requirements, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 RAN#88bis, Chengdu, China, 8 – 12 October 2018

- R4-1816347, WF on NR PUSCH demodulation requirements, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 RAN#89, Spokane, US, 12 – 16 November 2018

- R4-1816594, WF for test case SNR derivation procedure, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 RAN#89, Spokane, US, 12 – 16 November 2018.

- R4-1902434, WF for NR PUSCH demodulation, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 RAN#90, Athens, Greece, 25th Feb – 1st Mar, 2019.