3GPP TSG-RAN4 Meeting #90bis	R4-1903456

# Xian, China, 8th – 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR |  | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.141-2: Addition of RC test method for spurious emission in subclause 6.7 and 7.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current version of test procedures, it is stated that a spatial sampling grid needed for anechoic chamber (AC) test methods shall be selected. However for reverberation chamber test methods (RC), there are no need to select a sampling grid. Therefore, the test procedures are updated to allow for both AC and RC based test procedures. In this CR, the test procedured for spurious emission is updated to allow for both AC and RC based on input from feedback received last meeting. This CR is created on top of R4-1902653. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 6.7.5.2.4.2, the text is updated to upport both AC and RC test methods. - In subclause 6.7.5.4.4.2, the text is updated to upport both AC and RC test methods. - In subclause 7.7.4.2, the text is updated to upport both AC and RC test methods. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the specification is not updated, the current normative text excludeds all other test methods where a spatial grid is not needed (e.g. Reverberation chamber) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sub-clause 6.7 and 7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

###### 6.7.5.2.4.2	Procedure

1)	Place the BS at the positioner.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in subclause 6.7.5.2.5.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

5)	Set the BS type 1-O to transmit

-	For RIB declared to be capable of single carrier operation only, set the RIB to transmit a signal according to the applicable test configuration in subclause 4.8 using the corresponding test model NR-FR1-TM1.1 in subclause 4.9.2, at manufacturer's declared rated output power Prated,c,TRP.

-	For a RIB declared to be capable of multi-carrier and/or CA operation, set the RIB to transmit according to NR-FR1-TM1.1 on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.7.2 and 4.8.

6)	Mount the BS and the test antenna such that measurements to determine TRP can be performed (see annex I for more information about anechoic chamber measurements).

7)	Measure the emission at the specified frequencies with specified measurement bandwidth.

8)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for anechoic chamber TRP estimation (see annex I).

NOTE 1: the TRP measurement grid may not be the same for all measurement frequencies.

NOTE 2: the frequency sweep or the TRP measurement grid sweep may be done in any order.

NOTE 3: As an alternative, TRP can be measured in a reverberation chamber.

9)	For anechoic chamber, calculate TRP at each specified frequency using the directional measurements.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For BS type 1-O and multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

###### 6.7.5.4.4.2	Procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in subclause 6.7.5.2.5.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

5)	Set the BS type 1-O to transmit:

-	For RIB declared to be capable of single carrier operation only, set the RIB to transmit a signal according to the applicable test configuration in subclause 4.8 using the corresponding test model NR-FR1-TM1.1 in subclause 4.9.2, at manufacturer's declared rated output power Prated,c,TRP.

-	For a RIB declared to be capable of multi-carrier and/or CA operation, set the RIB to transmit according to NR-FR1-TM1.1 in subclause 4.9.2  on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.7.2 and 4.8..

6)	Mount the BS and the test antenna such that measurements to determine TRP can be performed (see annex I for more information about anechoic chamber measurements).

7)	Measure the emission at the specified frequencies with specified measurement bandwidth.

8)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for anechoic chamber TRP estimation (see annex I).

NOTE 1: the TRP measurement grid may not be the same for all measurement frequencies.

NOTE 2: the frequency sweep or the TRP measurement grid sweep may be done in any order.

NOTE 3: As an alternative, TRP can be measured in a reverberation chamber.

9)	For anechoic chamber, calculate TRP at each specified frequency using the directional measurements.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

#### 7.7.4.2	Procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in subclause 7.7.5.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

5)	Set the TDD BS to receive only.

6)	Mount the BS and the test antenna such that measurements to determine TRP can be performed (see annex I for more information about anechoic chamber measurements).

7)	Measure the emission at the specified frequencies with specified measurement bandwidth

8)	Repeat step 6-9 for all directions in the appropriated TRP measurement grid needed for anechoic chamber TRP estimation (see annex I).

NOTE 1: the TRP measurement grid may not be the same for all measurement frequencies.

NOTE 2: the frequency sweep or the TRP measurement grid sweep may be done in any order

NOTE 3: As an alternative, TRP can be measured in a reverberation chamber.

9)	For anechoic chamber, calculate TRP at each specified frequency using the directional measurements.

In addition, for multi-band RIB(s), the following steps shall apply:

10)	For BS type 1-O and multi-band RIB(s) and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.