3GPP TSG-RAN4 Meeting #90bis	R4-1903459

# Xian, China, 8th - 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR |  | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.141-2: Correction to TX requirements with respect to total EIRP in subclause 6.2.4.2, subclause 6.3.4.2 and Annex B.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | EIRP is always defined as total EIRP as the sum of two measured orthogonal polarizations. Measure EIRP by either a) or b) below: a) If the OTA test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the BS. Sum the EIRP measured on both polarizations. b) If the OTA test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2. This CR is created on top of R4-1902653. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 6.2.4.2, subclause 6.3.4.2 and Annex B.7.1, missing text about how to calculate EIRP is missing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the specification with respect to measureing EIRP is non consistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Subclause 6.2.4.2, subclasue 6.3.4.2 and Annex B.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 6.2.4.2	Procedure

For normal test environment conditions in OTA domain, the test procedure is as follows:

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Move the BS on the positioner in order that the direction to be tested aligns with the test antenna.

4)	Configure the beam peak direction of the BS according to the declared beam direction pair.

5)	Set the BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test model(s) in subclause 4.9.2.

For a BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8 using the corresponding test model(s) in subclause 4.9.2 on all carriers configured.

6)	Measure EIRP by either a) or b) below:

a)	If the OTA test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the BS. Sum the EIRP measured on both polarizations.

b)	If the OTA test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	Test steps 3 to 6 are repeated for all declared beams (D.3) and their reference beam direction pairs and maximum steering directions (D.8 and D.10).

For multi-band capable BS and single band tests, repeat the steps above per involved operating band where single band test configurations and test models shall apply with no carriers activated in the other band.

8)   For extreme conditions tests the methods in annex B.7 may be used where a representative power measurement is taken at both nominal conditions (Pmax,sample,nom) and extreme conditions (Pmax,sample,ext) and the delta (Δsample) is added to the nominal measurement from step 6 such that Pmax,c,EIRP, extreme = Pmax,c,EIRP + Δsample.

#### 6.3.4.2	Procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Set the BS in the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested.

4)	Configure the BS such that the beam peak direction(s) applied during the power measurement step 6 are consistent with the grid and measurement approach for the TRP test.

5)	Set the BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test model(s) in subclause 4.9.2.

For a BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8 using the corresponding test model(s) in subclause 4.9.2 on all carriers configured.

6)	Set the BS in the reference direction of the appropriated TRP measurement grid (see annex I).

7)	Measure EIRP by either a) or b) below:

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the BS. Sum the EIRP measured on both polarizations.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

8)	Repeat step 6-7 for all directions in the appropriated TRP measurement grid needed for full TRP estimation (see annex I).

9)	Calculate TRP using the EIRP measurements.

For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

## B.7.1	Direct far field method

The BS under test is placed inside a sealed RF transparent environmental enclosure, as showed in Figure B.7.1-1. This is connected to an environment control system which regulates the temperature inside the enclosure. The remaining equipment inside the OTA chamber (any suitable antenna test range chamber type is acceptable) is outside the environmental control and is at nominal temperature. Positioners, test antennas and all other OTA test equipment do not need to be specified over the extreme temperature range.

Figure B.7.1-1: Measurement set up for extreme conditions for EIRP accuracy using direct far field method

The presence of the environmental chamber inside the OTA chamber may affect the measurement accuracy due to additional reflections and refractions, also the loss through the environmental enclosure may not be consistent with direction as the path through the radome may vary with angle. Hence the system should be calibrated in all tested directions, frequencies and temperatures if necessary.

NOTE: Currently only a single direction is specified for extreme testing so a single calibration direction is sufficient.

Conformance may be demonstrated by measuring the difference between the nominal measurement and the extreme measurement (Δsample) or by measuring Pmax,c,EIRP, extreme directly.

As the measurement is done in the far field (or measured in near field transformed to far field):

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the BS. Sum the EIRP measured on both polarizations.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.