3GPP TSG-RAN WG4 Meeting #90bis												  	         R4-1903516

Athens, 8th – 12th of April, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | A-MPR definition harmonization in 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A-MPR definition harmonization |  |  |  |  |  |  |  |  |
| Summary of change: |  | A-MPR definitions are harmonized |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR definitions are not unified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************************** Start of Changes *******************************************

### 6.2.3	UE additional maximum output power reduction

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band,the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. In absense of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a PC3 UE, when IE [P-Boost-BPSK] is set to 1, PC2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms, the allowed A-MPR is TBD.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 |  |  | N/A |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70, n86 |  |  | Subclause 6.2.3.7 |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 |  |  | Subclause 6.2.3.7 |
| NS_04 | 6.5.2.3.2, 6.5.3.3.1 | n41 | 10, 15, 20, 40, 50, 60 80, 100 |  | Subclause 6.2.3.2 |
| NS_05 | 6.5.3.3.4 | n1, n84 | 5, 10, 15, 20 (NOTE 2) |  | Subclause 6.2.3.4 |
| NS_05U | 6.5.3.3.4, 6.5.2.4.2 | n1, n84 | 5, 10, 15, 20 |  | Subclause 6.2.3.4 |
| NS_06 | 6.5.2.3.4 | n12 | 5, 10, 15 |  | N/A |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3-1 | Table 6.2.3.3-1 |
| NS_17 | 6.5.3.3.2 | n28, n83 | 5,10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.13-1, A2 |
| NS_35 | 6.5.2.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.6 | n74 (NOTE 3) | 10, 15 | Table 6.2.3.8-1 | Table 6.2.3.8-1 |
| NS_38 | 6.5.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.9-1 | Table 6.2.3.9-1 |
| NS_39 | 6.5.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.10-1 | Table 6.2.3.10-1 |
| NS_40 | 6.5.3.3.9 | n51 | 5 |  | Table 6.2.3.5-1 |
| NS_41 | 6.5.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.11-1 |
| NS_42 | 6.5.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.12-1 |
| NS_43 | 6.5.3.3.5 | n8, n81 | 5, 10, 15 |  | Subclause 6.2.3.6 |
| NS_43U | 6.5.3.3.5, 6.5.2.4.2 | n8, n81 | 5, 10, 15 |  | Subclause 6.2.3.6 |
| NS_100 | 6.5.2.4.2 | n1, n2, n3, n5, n8, n20, n25, n66, n80, n81, n82, n84, n86 (NOTE 1) |  |  | Table 6.2.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed NOTE 2: No A-MPR is applied for 5 MHz CBW where the lower channel edge is ≥ 1930 MHz,10 MHz CBW where the lower channel edge is ≥ 1950 MHz and 15 MHz CBW where the lower channel edge is ≥ 1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

Table 6.2.3.1-1A: Mapping of Network Signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 |  |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n20 | NS_01 | NS_100 | NS_10 |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 |  |  |  |  |  |  |  |
| n39 | NS_01 |  |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 |  |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | NS_100 |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in sub-clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

Table 6.2.3.1-2: A-MPR for NS_100 (UTRA protection)

| Modulation |  |  |  | Outer |  |
| --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK |  | ≤ 2 |  |
|  |  | QPSK |  | ≤ 2 |  |
|  |  | 16 QAM |  | ≤ 2.5 |  |
|  |  | 64 QAM |  | ≤ 3 |  |
|  |  | 256 QAM |  | ≤ 4.5 |  |
| CP-OFDM |  | QPSK |  | ≤ 4 |  |
|  |  | 16 QAM |  | ≤ 4 |  |
|  |  | 64 QAM |  | ≤ 4 |  |
|  |  | 256 QAM |  | ≤ 6.5 |  |

#### 6.2.3.2	A-MPR for NS_04

For NS_04, A-MPR is not added to MPR. Also, when NS_04 is signalled, MPR shall be set to zero in the PCMAX equations to avoid double-counting MPR.

Allowed maximum power reduction is defined as A-MPR=max(MPR, A-MPR'),

Note that A-MPR'=0 dB means only MPR is applied,

where A-MPR' is defined as

if RBstart ≤ fstart,max,IMD3 / (12SCS) and LCRB ≤ AWmax,IMD3 / (12SCS) and FC - BWChannel/2 < FUL_low + offsetIMD3,
then

the A-MPR' is defined according to Table 6.2.3.2-2 PC3_A2 for Power Class 3 and PC2 A4 for Power Class 2,

else,

if RBstart ≤ LCRB/2 + start / (12SCS) and LCRB ≤ AWmax,regrowth / (12SCS) and FC - BWChannel/2 < FUL_low + offsetregrowth,
then

the A-MPR' is defined according to Table 6.2.3.2-2 PC3_A1 for Power Class 3 and PC2 A3 for Power Class 2,

else

A-MPR' = 0 dB and apply MPR.

With the parameters defined in Table 6.2.3.2-1.

Table 6.2.3.2-1: Parameters for region edges and frequency offsets

| Parameter | Symbol | Value |  |  | Related condition |
| --- | --- | --- | --- | --- | --- |
|  |  | CP-OFDM |  | DFT-s-OFDM |  |
| Max allocation start in IMD3 region | fstart,max,IMD3 | 0.33 BWChannel |  |  | RBstart ≤ fstart,max,IMD3 / (12SCS) |
| Max allocation BW in IMD3 region | AWmax,IMD3 | 4 MHz |  |  | LCRB ≤ AWmax,IMD3 / (12SCS) |
| Max freq. offset for IMD3 region | offsetmax,IMD3 | BWChannel – 6 MHz |  |  |  |
| Freq. offset required to avoid A-MPR in IMD3 region | offsetIMD3 | offsetmax,IMD3 |  |  | FC - BWChannel/2 ≥ FUL_low + offsetIMD3 |
| Right edge of regrowth region | start | 0.08 BWChannel |  |  | RBstart ≤ LCRB/2 + start / (12SCS) |
| Max allocation BW in regrowth region | AWmax,regrowth | 100 MHz |  |  | LCRB ≤ Min(LCRB,Max, AWmax,regrowth / (12SCS)) |
| Freq. offset required to avoid A-MPR in regrowth region | offsetregrowth | Max (10 MHz, 0.25* BWChannel MHz) | Max (10 MHz, 0.45* BWChannel MHz) |  | FC - BWChannel/2 ≥ FUL_low + offsetregrowth |

Table 6.2.3.2-2: A-MPR' valuesAccess

|  | Modulation | A-MPR’ (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | PC3_A1 | PC3_A2 | PC2_A3 | PC2_A4 |
| DFT-s-OFDM | Pi/2-BPSK | 3.5 | 3.5 | 3.5 | [5.5] |
|  | QPSK | 4 | 4 | 4.5 | [6] |
|  | 16 QAM | 4 | 4 | 5 | [6] |
|  | 64 QAM |  | 4.5 | 5 | [6.5] |
|  | 256 QAM | 4.5 | 6 | 6.5 | [8] |
| CP-OFDM | QPSK | 5.5 | 5.5 | 6.5 | [7.5] |
|  | 16 QAM | 5.5 | 5.5 | 6.5 | [7.5] |
|  | 64 QAM | 5.5 | 5.5 | 6.5 | [7.5] |
|  | 256 QAM | 6.5 | 8 | 7.5 | [10] |
| NOTE: The A-MPR’ values in this table apply for both A-MPR relative to 23 dBm for PC3 and A-MPR relative to 26 dBm for PC2 |  |  |  |  |  |

#### 6.2.3.3	A-MPR for NS_10

Table 6.2.3.3-1: A-MPR for NS_10

| Channel bandwidth (MHz) | Parameters | Region A |
| --- | --- | --- |
| 15 | RBstart | [0 – 10] |
|  | LCRB (RBs) | [1 – 20] |
|  | A-MPR (dB) | ≤ 36 |
| 20 | RBstart | [0 – 15] |
|  | LCRB (RBs) | [1 – 20] |
|  | A-MPR (dB) | ≤ 66 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects Region A, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects Region A, notes 1 and 2 apply on a Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersect Region A, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the subslot. NOTE 5: The total maximum output power reduction for NS_10 is obtained by taking the maximum value of MPR + A-MPR specified in Table 6.2.3-1 and Table 6.2.4-1 in TS 36.101 and A-MPR specified in Table 6.2.3.3-1. NOTE 6: The A-MPR for CP-OFDM shall also add the corresponding MPR specified in Table 6.2.2-1. |  |  |

#### 6.2.3.4	A-MPR for NS_05 and NS_05U

Table 6.2.3.4-1: A-MPR regions for NS_05 and NS_05U

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | Rbstart | LCRB | A-MPR |
| 5 MHz | 1922.5 ≤ FC < 1927.5 | < 1.62 MHz/12/SCS | > 2.52 MHz/12/SCS | A3 |  |  |  |  |  |  |
| 10 MHz | 1920 ≤ FC < 1935 | < 1.62 MHz/12/SCS | > 0 | A1 | > 1.62 MHz/12/SCS ≤ 3.60 MHz/12/SCS | > 5.4 MHz/12/SCS | A7 | > 7.74 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
| 10 MHz | 1935 ≤ FC < 1945 |  | >4.5 MHz/12/SCS | A4 |  |  |  |  |  |  |
| 15 MHz | 1927.5 ≤ FC < 1932.5 | < 3.24MHz/12/SCS | > 0 | A1 | > 3.24 MHz/12/SCS ≤ 5.40 MHz/12/SCS | > 8.1MHz/12/SCS | A7 | > 10.9 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
| 15 MHz | 1932.5 ≤ FC < 1942.5 | < 1.62 MHz/12/SCS | > 0 | A1 |  |  |  | > 12.6 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
| 15 MHz | 1942.5 ≤ FC < 1947.5 |  | > 7.2 MHz/12/SCS | A5 |  |  |  |  |  |  |
| 20 MHz | 1930 ≤ FC < 1950 | < 4.86 MHz/12/SCS | > 0 | A1 | > 4.86 MHz/12/SCS ≤ 7.20 MHz/12/SCS | > 9.0 MHz/12/SCS | A7 | > 14.22 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A2 |
| 20 MHz | 1950 ≤ FC < 1960 |  | > 9.0 MHz/12/SCS | A6 |  |  |  |  |  |  |
| NOTE 1: The A-MPR values are specified in Tables 6.2.3.4-2, 6.2.3.4-3 and 6.2.3.4-4. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-2: A-MPR NS_05 and NS_05U

| Modulation/Waveform |  | A1 | A2 | A3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 10 | ≤ 5 | ≤ 4 |  |
|  | QPSK | ≤ 10 | ≤ 5 | ≤ 4.5 |  |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 6 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 6 |  |
|  | 256 QAM | ≤ 13 | ≤ 5 | ≤ 7 |  |
| CP-OFDM | QPSK | ≤ 10 | ≤ 5 | ≤ 7.5 | ≤ 2 |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 8 |  |
|  | 256 QAM | ≤ 13 |  | ≤ 10 |  |

Table 6.2.3.4-3: A-MPR for NS_05

| Modulation |  | A4 |  | A5 |  | A6 |  | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer |  | Outer | Inner | Outer |  | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 1 |  | ≤ 1 |  | ≤ 1 |  | ≤ 6 |
|  | QPSK |  |  | ≤ 1.5 |  | ≤ 1.5 |  | ≤ 6 |
|  | 16 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 64 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 |
| CP-OFDM | QPSK | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 |
|  | 64 QAM |  |  |  | ≤ 4 |  |  | ≤ 6 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 |

Table 6.2.3.4-4: A-MPR NS_05U

| Modulation |  |  | A4 |  | A5 |  | A6 |  | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer |  | Outer | Inner | Outer |  | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK |  | ≤ 2 |  | ≤ 2 |  | ≤ 2 |  | ≤ 6 |
|  | QPSK |  | ≤ 2 |  | ≤ 2 |  | ≤ 2 |  | ≤ 6 |
|  | 16 QAM |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 6 |
|  | 64 QAM |  | ≤ 3 |  | ≤ 3 |  | ≤ 3 |  | ≤ 6 |
|  | 256 QAM |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 6 |
| CP-OFDM | QPSK |  | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 16 QAM |  | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 64 QAM |  | ≤ 4 |  | ≤ 4 | ≤ 4 | ≤ 4 |  | ≤ 6 |
|  | 256 QAM |  | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |

Table 6.2.3.4-9: Void

#### 6.2.3.5	A-MPR for NS_40

Table 6.2.3.5-1: A-MPR for NS_40

| Modulation |  | A-MPR |  |  |
| --- | --- | --- | --- | --- |
|  |  | Channel bandwidth 5 MHz |  |  |
|  |  | Outer |  | Inner |
| DFT-s-OFDM | QPSK | 15.5 |  | 12 |
|  | 16 QAM | 14.5 |  | 11 |
|  | 64 QAM | 14.5 |  | 10 |
|  | 256 QAM | 12.5 |  | 7.5 |
| CP-OFDM | QPSK | 14.5 |  | 10 |
|  | 16 QAM | 14.5 |  | 10 |
|  | 64 QAM | 14 |  | 8 |
|  | 256 QAM | 11 |  | 5.5 |
| NOTE 1: The total maximum output power reduction for NS_40 is obtained by taking the maximum value of MPR + A-MPR specified in Table 6.2.3-1 and Table 6.2.4-30a in TS 36.101 and MPR + A-MPR specified in Table 6.2.2-1 and Table 6.2.3.5-1. |  |  |  |

#### 6.2.3.6	A-MPR for NS_43

Table 6.2.3.6-1: A-MPR regions for NS_43

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | FC ≥ 902.5 |  | > 15 | A1 |  |  |  |
| 10 MHz | FC ≥ 910 |  | > 40 | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 45 | A3 |  | > 7.2 MHz/12/SCS | A5 |
| 15 MHz | FC = 907.5 | < 1.8 MHz /12/SCS > 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | > 7.2 MHz/12/SCS | A6 |
| NOTE 1: The A-MPR values are specified in Tables 6.2.3.6-2 and 6.2.3.6-3. NOTE 2: 15 kHz SCS unless otherwise stated |  |  |  |  |  |  |  |

Table 6.2.3.6-2: A-MPR for NS_43

| Modulation |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer | Outer | Outer | Outer | Outer and Inner |
| DFT-s-OFDM | Pi/2 BPSK |  | ≤ 1.5 |  |  |  | ≤ 9 |
|  | QPSK | ≤ 2 |  |  | ≤ 2.5 |  | ≤ 9 |
|  | 16 QAM |  |  |  |  | ≤ 2.5 | ≤ 9 |
|  | 64 QAM |  |  | ≤ 2.5 |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  | ≤ 9 |
| CP-OFDM | QPSK | ≤ 3.5 |  |  |  | ≤ 4 | ≤ 9 |
|  | 16 QAM | ≤ 3.5 |  |  |  | ≤ 4 | ≤ 9 |
|  | 64 QAM |  |  | ≤ 4 |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  | ≤ 9 |

Table 6.2.3.6-3: A-MPR for NS_43U

| Modulation |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer | Outer | Outer | Outer | Outer and Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | ≤ 2 | ≤ 2 | ≤ 2 | ≤ 2 | ≤ 9 |
|  | QPSK | ≤ 2 | ≤ 2 | ≤ 2 | ≤ 2.5 | ≤ 2 | ≤ 9 |
|  | 16 QAM | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 2.5 | ≤ 9 |
|  | 64 QAM | ≤ 3 | ≤ 3 | ≤ 3 | ≤ 3 | ≤ 3 | ≤ 9 |
|  | 256 QAM | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 4.5 | ≤ 9 |
| CP-OFDM | QPSK | ≤ 4 | ≤ 4 | ≤ 4 | ≤ 4 | ≤ 4 | ≤ 9 |
|  | 16 QAM | ≤ 4 | ≤ 4 | ≤ 4 | ≤ 4 | ≤ 4 | ≤ 9 |
|  | QAM | ≤ 4 | ≤ 4 | ≤ 4 | ≤ 4 | ≤ 4 | ≤ 9 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.5 | ≤ 9 |

#### 6.2.3.7	A-MPR for NS_03 and NS_03U

Table 6.2.3.7-1 A-MPR for NS_03

| Modulation |  | Outer |
| --- | --- | --- |
| DFT-s-OFDM |  | Pi/2 BPSK |  | ≤ 1.5 |  |  |
|  |  |  | QPSK |  |  | ≤ 2 |
|  |  |  | 16 QAM |  |  | ≤ 3 |
|  |  |  | 64 QAM |  |  | ≤ 3.5 |
|  |  |  | 256 QAM |  |  | ≤ 5.5 |
| CP-OFDM |  |  | QPSK |  |  | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4.5 |
|  | 256 QAM | ≤ 7.5 |

In case UE operates in a band where NS_03U applies and it receives additionalSpectrumEmission value of 3 then A-MPR values specified in Table 6.2.3.7-1 apply with an exception that DFT-s-OFDM Pi/2 PBSK A-MPR is 2 dB.

#### 6.2.3.8	A-MPR for NS_37

Table 6.2.3.8-1: A-MPR regions for B11/B21 protection (NS_37) for 10MHz, 15MHz (1447.9-1462.9MHz)

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A Outer/Inner |  |  | Region B Outer/Inner |  |  | Region C Outer/Inner |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart,(MHz/12/SCS) | LCRB (MHz/12/SCS) | A-MPR | RBstart (MHz/12/SCS) | LCRB (MHz/12/SCS) | A-MPR | RBstart (MHz/12/SCS) | LCRB (MHz/12/SCS) | A-MPR |
| 10 MHz | 1452.9 < FC ≤ 1457.9 | ≥ 0 | > 40 | ≤ A1 | N/A | N/A | N/A | N/A | N/A | N/A |
| 15 MHz | FC = 1455.4 | ≥ 0 | > 55 | ≤ A1 | < [0.54] | < [1.08] | [≤ A2] | > [13.86] | < [1.08] | [≤ A2] |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.8-2 NOTE 2: No A-MPR for SCS = 60 kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.8-2: A-MPR NS_37

| Modulation |  | A1 |  | A2 |
| --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 |  | [≤ 3] |
|  | QPSK | ≤ 1.5 |  | [≤ 3] |
|  | 16 QAM | ≤ 2.5 |  | [≤ 3] |
|  | 64 QAM | ≤ 3 |  | [≤ 3] |
|  | 256 QAM |  |  | [N/A] |
| CP-OFDM | QPSK | ≤ 3.5 |  | [≤ 3] |
|  | 16 QAM | ≤ 3.5 |  | [≤ 3] |
|  | 64 QAM |  |  | [N/A] |
|  | 256 QAM |  |  | [N\A] |

#### 6.2.3.9	A-MPR for NS_38

Table 6.2.3.9-1: A-MPR for EESS (NS_38) Protection (1430-1470MHz)

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A Outer/Inner |  |  | Region B Outer/Inner |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart+LCRB(MHz/12/SCS) | A-MPR |
| 10 MHz | [1435 ≤ FC < 1442] | [<= -1.8 MHz/12/SCS + LCRB/2] | [> 3.6] | [≤ 12] | [≤ 2.16] | [≤ 9] |
| 15 MHz | [1437.5 ≤ FC < 1447.5] | [<= -1.8 MHz/12/SCS + LCRB/2] | [> 3.6] | [≤ 13] | [≤ 3.6] | [≤ 10] |
| 20 MHz | [1440 ≤ FC < 1450] | [<= -1.8 MHz/12/SCS + LCRB/2] | [> 3.6] | [≤ 13] | [≤ 5.4] | [≤ 10] |

#### 6.2.3.10	A-MPR for NS_39

Table 6.2.3.10-1: A-MPR for own RX (NS_39) Protection for 10MHz,15MHz, 20MHz (1440-1470MHz)

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A Outer/Inner |  |
| --- | --- | --- | --- |
|  |  | RBstart+LCRB(MHz/12/SCS) | A-MPR |
| 10MHz | [1462 < FC ≤ 1465] | [> 7.9] | [≤ 6] |
| 15MHz | [1456.3 < FC ≤ 1462.5] | [> 11.2] | [≤ 6] |
| 20MHz | [1450.8 < FC ≤ 1460] | [> 14.4] | [≤ 6] |

#### 6.2.3.11	A-MPR for NS_41

Table 6.2.3.11-1: A-MPR for NS_41

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A Outer/Inner |  |  | Region B Outer/Inner |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBend | A-MPR |
| 5 MHz | - | - | - | NA | - | NA |
| 10 MHz | 1437 ≤ FC < 1442 | <= -4.5MHz/12/SCS + LCRB | > 4.5 MHz/12/SCS | ≤ 9 | LCRB < 1.8MHz/12/SCS | ≤ 9 |
| 15 MHz | 1439.5 ≤ FC < 1447.5 | <= -5.4MHz/12/SCS + LCRB | > 5.4 MHz/12/SCS | ≤ 11 | LCRB < 3.42 MHz/12/SCS | ≤ 9 |
| 20 MHz | 1442 ≤ FC < 1450 | <= -5.4MHz/12/SCS + LCRB | > 5.4 MHz/12/SCS | ≤ 12 | LCRB < 5.04 MHz/12/SCS | ≤ 9 |
| 40 MHz | 1452 ≤ FC < 1497 | <= -7.2MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | LCRB < 11.7 MHz/12/SCS | ≤ 13.5 |
| 50 MHz | 1457 ≤ FC < 1492 | <= -7.2MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | LCRB < 15.12 MHz/12/SCS | ≤ 13.5 |
| 60 MHz | 1462 ≤ FC < 1487 | <= -7.2MHz/12/SCS + LCRB | > 7.2 MHz/12/SCS | ≤ 13.5 | LCRB < 18.72 MHz/12/SCS | ≤ 13.5 |

#### 6.2.3.12	A-MPR for NS_42

Table 6.2.3.12-1: A-MPR for NS_42

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  | Region B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend | A-MPR Outer/Inner | RBstart | RBend5 | A-MPR Inner | A-MPR Outer |
| 5 MHz | 1512 ≤ FC ≤ 1514.5 | > 3.1 MHz / 12 / SCS | ≤ 7 | < 0.90 MHz / 12 / SCS | ≤ 3.1 MHz / 12 / SCS | ≤ 1.5 | ≤ 4 |
| 10 MHz | 1497 ≤ FC ≤ 1512 | > 6.2 MHz / 12 / SCS | ≤ 8 | < 0.90 MHz / 12 / SCS | ≤ 6.2 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 15 MHz | 1502 FC ≤ 1509.5 | > 9.3 MHz / 12 / SCS | ≤ 8 | < 3.06 MHz / 12 / SCS | ≤ 9.3 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 20 MHz | 1497 ≤ FC ≤ 1507 | > 12.4 MHz / 12 / SCS | ≤ 8 | < 4.50 MHz / 12 / SCS | ≤ 12.4 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 40 MHz | 1477 ≤ FC ≤ 1497 | > 24.8 MHz / 12 / SCS | ≤ 8 | < 5.40 MHz / 12 / SCS | ≤ 24.8 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 50 MHz | 1467 ≤ FC ≤ 1492 | > 31 MHz / 12 / SCS | ≤ 8 | < 7.20 MHz / 12 / SCS | ≤ 31 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| 60 MHz | 1462 ≤ FC ≤ 1487 | > 37.2 MHz / 12 / SCS | ≤ 8 | < 7.20 MHz / 12 / SCS | ≤ 37.2 MHz / 12 / SCS | ≤ 1.5 | ≤ 5 |
| NOTE 1: In region B, RBend > RBstart |  |  |  |  |  |  |  |

#### 6.2.3.13	A-MPR for NS_18

Table 6.2.3.13-1: A-MPR for NS_18

| Modulation |  |  | A1 | A2 |
| --- | --- | --- | --- | --- |
|  |  |  | Outer | Inner/Outer |
| DFT-s-OFDM |  | Pi/2 BPSK | ≤ 2 | ≤ 5 |
|  |  | QPSK | ≤ 2 | ≤ 5 |
|  |  | 16 QAM | ≤3 | ≤ 6 |
|  |  | 64 QAM | ≤ 4 | ≤ 7 |
|  |  | 256 QAM | ≤ 6 | ≤ 9 |
| CP-OFDM |  | QPSK | ≤ 5 | ≤ 6.5 |
|  |  | 16 QAM | ≤ 5 | ≤ 7 |
|  |  | 64 QAM | ≤ 5.5 | ≤ 8.5 |
|  |  | 256 QAM | ≤ 8.5 | ≤ 11.5 |

**************************** End of Changes *******************************************