3GPP TSG RAN WG4 Meeting #90bis		R4-1903549

Xian, China, 8th – 12th April, 2019

Source: 	NTT DOCOMO, INC.

Title: 	UE demodulation requirements for carrier aggregation in HST-SFN scenario

Agenda item:	7.15.4

Document for:	Discussion

- Introduction

At the last RAN4 meeting, RAN4 discussed whether HST-SFN CA requirements are specified or not. The summary of company’s view is as follows [1]:

- Define HST-SFN CA requirements based on Rel.14 requirements - Ericsson, Qualcomm, NTT DOCOMO, CMCC - Not Define HST-SFN CA requirements - Huawei, Intel, MTK

In this contribution, we provide our views on the necessity of HST-SFN CA requirements.

- Discussion

At first, RAN2 specified two types of RRC signalling regarding HST-SFN scenario in RadioResourceConfigCommon in Rel.14 WI as follows [2]:

HighSpeedConfig-r14 ::=			SEQUENCE {

highSpeedEnhancedMeasFlag-r14			ENUMERATED {true}				OPTIONAL,	-- Need OR

highSpeedEnhancedDemodulationFlag-r14	ENUMERATED {true}				OPTIONAL	-- Need OR

}

HighSpeedConfig-v1530 ::=		SEQUENCE {

highSpeedMeasGapCE-ModeA-r15			ENUMERATED {true}

}

HighSpeedConfigSCell-r14 ::=		SEQUENCE {

highSpeedEnhancedDemodulationFlag-r14	ENUMERATED {true}				OPTIONAL	-- Need OR

}

It means that the signalling regarding HST-SFN scenario can be configured separately between PCell and SCell. In Rel.14, only single carrier performance requirement was specified in TS36.101, therefore there is no requirements to ensure carrier aggregation performance in HST-SFN when HighSpeedConfigSCell-r14 is configured. Since carrier aggregation has already used in practical network in order to improve downlink throughput in high speed scenario including SFN scenario, such tests should be specified.

Observation 1:

RRC signalling regarding HST-SFN scenario can be configured separately between PCell and SCell.

Proposal 1:

HST-SFN CA performance requirements should be specified by extending Rel.14 single carrier requirements.

Regarding the applicability rule for above requirements, it is straightforward to reuse the same rule as single carrier requirements, i.e., such requirements are applied when highSpeedEnhancedDemodulationFlag in HighSpeedConfigSCell-r14 is configured.

Proposal 2:

HST-SFN CA performance requirements are applied when highSpeedEnhancedDemodulationFlag in HighSpeedConfigSCell-r14 is configured.

- Conclusion

In this contribution, we provided our view on the necessity of HST-SFN CA requirements. Our observation and proposals are as follows:

Observation 1:

RRC signalling regarding HST-SFN scenario can be configured separately between PCell and SCell.

Proposal 1:

HST-SFN CA performance requirements should be specified by extending Rel.14 single carrier requirements.

Proposal 2:

HST-SFN CA performance requirements are applied when highSpeedEnhancedDemodulationFlag in HighSpeedConfigSCell-r14 is configured.

- Reference

[1] R4-1902521, “Way forward on UE demodulation enhancement for HST”, NTT DOCOMO, INC.

[2] TS36.331