3GPP TSG-RAN WG4 Meeting #90bis	R4-1903561

Xian, China, 8th Apr – 12th Apr, 2019

Source: 	ZTE

Title:	discussion on frequency range for EMC RE test

Agenda Item:	6.9.2.1

Document for:	Approval

Introduction

The radiated emission test frequency for BS type 1-C and BS type 1-H are stated in subclause 8.2.1 in  BS EMC standards [1]. Those statements are not coincident with [2] and [3].

Discussion

Subclause 8.2.1 in [1] described the upper limit frequency for RE measurements was 12.75GHz.

Subclause 6.6.5 in [2] described the upper limit frequency for transmitter spurious emissions was 5th harmonic of the upper frequency edge of the DL operating band in GHz. While the 5th harmonic of the upper frequency edge of the DL operating band in GHz was lower than 12.75GHz, the upper limit frequency would be 12.75GHz.

The table 1 in [3] described the frequency range for measurement of unwanted emissions as the following table.

| Fundamental frequency range | Frequency range for measurements |  |
| --- | --- | --- |
|  | Lower limit | Upper limit (The test should include the entire harmonic band and not be truncated at the precise upper frequency limit stated) |
| 9 kHz-100 MHz | 9 kHz | 1 GHz |
| 100 MHz-300 MHz | 9 kHz | 10th harmonic |
| 300 MHz-600 MHz | 30 MHz | 3 GHz |
| 600 MHz-5.2 GHz | 30 MHz | 5th harmonic |
| 5.2 GHz-13 GHz | 30 MHz | 26 GHz |
| 13 GHz-150 GHz | 30 MHz | 2nd harmonic |
| 150 GHz-300 GHz | 30 MHz | 300 GHz |

Observation: The upper frequency range for NR FR1 BS was 6GHz. The upper frequency 12.75GHz for NR FR1 BS RE test can’t catch the enough harmonic spurious emission from the NR BS working on the higher frequency.

Proposal: Updated the upper frequency for NR BS RE test from 12.75GHz to 5th harmonic of the DL operating band.

Conclusion

In this contribution, we give the discussion of RE frequency range and the observation and proposal are:

Observation: The upper frequency range for NR FR1 BS was 6GHz. The upper frequency 12.75GHz for NR FR1 BS RE test can’t catch the enough harmonic spurious emission from the NR BS working on the higher frequency.

Proposal: Updated the upper frequency for NR BS RE test from 12.75GHz to 5th harmonic of the DL operating band.

- References

TS 38.113, 3rd Generation Partnership Project;Technical Specification Group Radio Access Network;NR;Base Station (BS) ElectroMagnetic Compatibility (EMC)

TS 38.104, 3rd Generation Partnership Project;Technical Specification Group Radio Access Network;NR;Base Station (BS) radio transmission and reception(Release 15)

ITU-R  SM.329-12, Unwanted emissions in the spurious domain