3GPP TSG-RAN WG4 Meeting #90Bis	R4-1903566

Xi’an, China, 8th Apr – 12th Apr, 2019

Source: 	ZTE

Title:	on test configuration of EMC test

Agenda Item:	6.9.3

Document for:	Approval

Introduction

So far, the EMC test configuration is based on the NRTC defined in TS 38.141-1[1] and TS 38.141-2[2]. How to choose the NRTC correctly for each EMC test hasn’t been fully discussed. In this contribution, we give some analysis between NRTC and EMC test and propose some change of the subclause 4.5 of TS 38.113[3] accordingly.

Discussion

As we all know, the EMC test can be divided into two parts as emission test and immunity test. The test configuration in current TS 38.113[3] is separated to these two parts, too. The principle of setting the test configuration for EMC emission test is to use the same NRTC of spurious emission. Here we gather the NRTC for spurious emission from [1] and [2] and list them below in table 1 and table 2 for convenience. Note that there will be no multi-band RIB for BS type 2-O in Rel-15.

Table 1: Test configurations for BS type 1-C and BS type 1-H

| BS test case | BS capable of multi-carrier and/or CA in a single band |  |  | BS capable of multi-band operation |  |
| --- | --- | --- | --- | --- | --- |
|  | Contiguous spectrum capable BS | C and NC capable BS with identical parameters | C and NC capable BS with different parameters | BS capable of multi-band operation with common connector | BS capable of multi-band operation with seperate connector |
| Spurious Emission tests | NRTC1 | NRTC3 | NRTC1, NRTC3 | NRTC1/3 (Note 1), NRTC5 | NRTC1/3 (Note 1), NRTC5(Note 2) |
| Note 1: NRTC1 and/or NRTC3 shall be applied in each supported operating band. Note 2: For single-band operation test, other antenna connector(s) is (are) terminated. |  |  |  |  |  |

Table 2: Test configurations for BS type 1-O and BS type 2-O

| BS test case | Single band RIB |  |  | Multi-band RIB |
| --- | --- | --- | --- | --- |
|  | Contiguous spectrum capable BS | C and NC capable BS with identical parameters | C and NC capable BS with different parameters |  |
| OTA spurious Emission tests | NRTC1 | NRTC3 | NRTC1, NRTC3 | NRTC1/3 (Note 1), NRTC5 |
| Note 1: NRTC1 and/or NRTC3 shall be applied in each supported operating band. |  |  |  |  |

To check the test configuration requirement of EMC emission test, the basic principle is to let the base station to produce highest emission. Hence, reusing the OTA spurious emission test configuration is quite reasonable.

However, for immunity test, currently in TS 38.113[3]  the NRTC is just simply copy the NRTC for emission test. The test configuration requirement of EMC immunity test should be different from emission test, as there is no need that the base station produces highest emission during an immunity test. The aim of an immunity test is to check the susceptibility of the base station from outside interfere. So the NRTC for out-of-band blocking is quite reasonable. Here we list them below in table 3 and 4:

Also note that there will be no multi-band RIB for BS type 2-O within Rel-15.

Table 3: Test configurations for BS type 1-C and BS type 1-H

| BS test case | BS capable of multi-carrier and/or CA in a single band |  |  | BS capable of multi-band operation |  |
| --- | --- | --- | --- | --- | --- |
|  | Contiguous spectrum capable BS | C and NC capable BS with identical parameters | C and NC capable BS with different parameters | BS capable of multi-band operation with common connector | BS capable of multi-band operation with seperate connector |
| Out-of-band blocking test | NRTC1 | NRTC3 | NRTC1, NRTC3 | NRTC1/3 (Note 1), NRTC5 | NRTC1/3 (Note 1), NRTC5(Note 2) |
| Note 1: NRTC1 and/or NRTC3 shall be applied in each supported operating band. Note 2: NRTC5 is only applicable for multi-band receiver. |  |  |  |  |  |

Table 4: Test configurations for BS type 1-O and BS type 2-O

| BS test case | Single band RIB |  |  | Multi-band RIB |
| --- | --- | --- | --- | --- |
|  | Contiguous spectrum capable BS | C and NC capable BS with identical parameters | C and NC capable BS with different parameters |  |
| OTA Out-of-band blocking test | NRTC1 | NRTC3 | NRTC1, NRTC3 | NRTC5 |
| Note 1: NRTC1 and/or NRTC3 shall be applied in each supported operating band. |  |  |  |  |

Proposal: To use the out-of-band blocking test configuration for EMC immunity test configuration.

Conclusion

In this contribution, we give analysis of EMC test configuration and proposals are:

Proposal: To use the out-of-band blocking test configuration for EMC immunity test configuration.

- References

TS 38.141-1

TS 38.141-2

TS 38.113