3GPP TSG-RAN WG4 RAN4#90-bis	R4-1903638

# Xi’an, China, 8 – 12 April, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.113 | CR |  | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.113 Correction to Tables in BS test configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction to text in tables in BS test configurations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction to text in tables in BS test configurations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of clarity if the tables in BS test configurations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

- ------------------------------ Start of Text ------------------------------

Table 4.5-1: Test configurations for BS type 1-C and BS type 1-H

| BS test case | BS with single-band connector capable of contiguous spectrum in single band only | BS with single-band connector capable of both contiguous and non-contiguous spectrum in single band | BS with single-band connector capable of multi-band operation | BS with Multi-band connector capable of multi-band operation |
| --- | --- | --- | --- | --- |
| Emission tests | NRTC1 | NRTC3 or NRTC1/3 (Note1) | NRCT1/3(Note 2), NRTC 5 | NRCT1/3(Note 2), NRTC 5 |
| Immunity tests | NRTC1 | NRTC3 or NRTC1/3 (Note1) | NRCT1/3(Note 2), NRTC 5 | NRCT1/3(Note 2), NRTC 5 |
| NOTE 1: NRTC 3 is used when parameters in the manufacturer's declaration according to TS 38.141-1 [3] subclause 4.6 are identical for contiguous (C) and non-contiguous (NC). , Otherwise, NRTC1/3 shall be used. NOTE 2: NRTC1 or NRTC3 shall be applied in each supported operating band. |  |  |  |  |

Table 4.5-2: Test configurations for BS type 1-O

| BS test case | BS with single-band RIB capable of contiguous spectrum in single band only | BS with single-band RIB capable of both contiguous and non-contiguous spectrum in single band | BS with single-band RIB capable of multi-band operation | BS withMulti-band RIB capable of multi-band operation |
| --- | --- | --- | --- | --- |
| Emission tests | NRTC1 | NRTC3 or NRTC1/3 (Note1) | NRCT1/3(Note 2), NRTC 5 | NRCT1/3(Note 2), NRTC 5 |
| Immunity tests | NRTC1 | NRTC3 or NRTC1/3 (Note1) | NRCT1/3(Note 2), NRTC 5 | NRCT1/3(Note 2), NRTC 5 |
| NOTE 1: NRTC 3 is used when parameters in the manufacturer's declaration according to TS 38.141-2 [4] subclause 4.6 are identical for contiguous (C) and non-contiguous (NC).Otherwise, NRTC1/3 shall be used. NOTE 2: NRTC1 or NRTC3 shall be applied in each supported operating band. |  |  |  |  |

Table 4.5-3: Test configurations for BS type 2-O

| BS test case | BS with single-band RIB capable of contiguous spectrum in single band only | BS with single-band RIB capable of both contiguous and non-contiguous spectrum in single band |
| --- | --- | --- |
| Emission tests | NRTC1 | NRTC3 or NRTC1/3 (Note1) |
| Immunity tests | NRTC1 | NRTC3 or NRTC1/3 (Note1) |
| NOTE 1: NRTC 3 is used when parameters in the manufacturer's declaration according to TS 38.141-2 [4] subclause 4.6 are identical for contiguous (C) and non-contiguous (NC). Otherwise, NRTC1/3 shall be used. |  |  |

------------------------------ End of proposed text ------------------------------