3GPP TSG-RAN4 Meeting #90bis	R4-1903721

# Xi’An, China, 8th April 2019 - 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of L1 indication period for RLM and BFD (section 8.1.6 and 8.5.4) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-03-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current definition of L1 indication period for RLM and BFD may cause frequent RLF/BF in certain scenarios |  |  |  |  |  |  |  |  |
| Summary of change: |  | N and P are considered in definition of L1 indication period. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will face frequent RLF/BF in certain scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6, 8.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

### 8.1.6	Minimum requirement for L1 indication

When the downlink radio link quality on all the configured RLM-RS resources is worse than Qout, Layer 1 of the UE shall send an out-of-sync indication for the cell to the higher layers. A Layer 3 filter shall be applied to the out-of-sync indications as specified in TS 38.331 [2].

When the downlink radio link quality on at least one of the configured RLM-RS resources is better than Qin, Layer 1 of the UE shall send an in-sync indication for the cell to the higher layers. A Layer 3 filter shall be applied to the in-sync indications as specified in TS 38.331 [2].

The out-of-sync and in-sync evaluations for the configured RLM-RS resources shall be performed as specified in clause 5 in TS 38.213 [3]. Two successive indications from Layer 1 shall be separated by at least TIndication_interval.

When DRX is not used TIndication_interval is max(10ms, mini(ceil(NiPi)*TRLM-RS,i)), where TRLM-RS,i is the periodicity of the i-th configured RLM-RS resource for the monitored cell, which corresponds to TSSB specified in section 8.1.2 if the RLM-RS resource is SSB, or TCSI-RS specified in section 8.1.3 if the RLM-RS resource is CSI-RS. Ni and Pi are Rx sweeping relaxing factor and collision releaxing factor for the i-th RLM-RS, which are specified in section 8.1.2.2 if the RLM-RS resource is SSB, or in section 8.1.3.2 if the RLM-RS resource is CSI-RS.

In case DRX is used, TIndication_interval is max(10ms, 1.5*DRX_cycle_length, 1.5* mini(ceil(NiPi)*TRLM-RS,i)) if DRX cycle_length is less than or equal to 320ms, and TIndication_interval is max(DRX_cycle_length, mini(ceil(NiPi)*TRLM-RS,i)) if DRX cycle_length is greater than 320ms. Upon start of T310 timer as specified in TS 38.331 [2], the UE shall monitor the configured RLM-RS resources for recovery using the evaluation period and Layer 1 indication interval corresponding to the no DRX mode until the expiry or stop of T310 timer.

<End of change 1>

<Start of change 2>

### 8.5.4	Minimum requirement for L1 indication

When the radio link quality on all the configured RS resources in set  is worse than Qout_LR, Layer 1 of the UE shall send a beam failure instance indication to the higher layers. A Layer 3 filter may be applied to the beam failure instance indications as specified in [2].

The beam failure instance evaluation for the configured RS resources in set  shall be performed as specified in section 6 in [3]. Two successive indications from Layer 1 shall be separated by at least TIndication_interval_BFD.

When DRX is not used, TIndication_interval_BFD is max(2ms, mini(ceil(NiPi)*TBFD-RS,i)), where TBFD-RS,i is the periodicity of the i-th configured RS resource in set  for the accessed cell, corresponding to either the  periodicity of the SSB  in the set  or CSI-RS resource in the set . Ni and Pi are Rx sweeping relaxing factor and collision releaxing factor for the i-th BFD-RS, which are specified in section 8.5.2.2 if the BFD-RS resource is SSB, or in section 8.5.3.2 if the BFD-RS resource is CSI-RS.

When DRX is used, TIndication_interval_BFD is max(1.5*DRX_cycle_length, 1.5* mini(ceil(NiPi)*TBFD-RS,i)) if DRX cycle_length is less than or equal to 320ms, and TIndication_interval is (DRX_cycle_length, mini(ceil(NiPi)*TBFD-RS,i)) if DRX cycle_length is greater than 320ms.

<End of change 2>