3GPP TSG-RAN WG4 Meeting #90bis	R4-1903889

Xi’an, China, 8 – 12 April 2019

Agenda Item:	6.5.11.5

Source:	Ericsson

Title:	Impact of maxUplinkDutyCycle to UE demodulation and RRM requirements

Document for:	Approval

# 1	Introduction

RAN plenary #83 agreed with the way forward on FR2 maximum UL duty cycle [1] and the following guideline to RAN4:

- RAN#83 has discussed RAN4’s agreement on FR2 maxUplinkDutyCycle capability values for PC3 UE in [1]. - RAN would like to inform RAN4 and RAN2 that it has updated the FR2 maxUplinkDutyCycle capability values as follows {2%, 10% (TBD), 20%, 25%, 30%, 40%, 50%, 60%, 70%, 80%, 90%, 100%} - RAN4 is asked whether any value <= 10% should be added - RAN asks - RAN2 to introduce FR2 maxUplinkDutyCycle capability signalling accordingly - RAN2 to evaluate backwards compatibility

Uplink duty cycle is a UE capability (maxUplinkDutyCycle) which facilitates the compliance described below with P-MPR < TBD [2]:

- ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

- ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

It was discussed in RAN4 the possible duty cycles, and RAN plenary askes RAN4 to add any value <= 10% or not. In this contribution, we discuss the impact of small uplink duty cycle from RRM and UE demodulation requirement point of view.

# 2	Discussion

Rel-15 TS38.101-2 assumes only TDD bands for FR2 and the evaluation period for maxUplinkDutyCycle is 10ms. Therefore we have reviewed the TDD UL-DL configuration and HARQ-ACK/CSI reporting timing for FR2 test discussed in RAN4 RRM and UE demodulation requirements in Rel-15.

## 2.1	RRM test

RAN4 has agreed to configure ‘DDDSU’ for all the RRM tests with FR2 SCS=120kHz as shown in Table 1. For RRM, only UL slots (marked as ‘U’) are used for UL signal transmission such as measurement report or UL signalling (HARQ-ACK or CSI reporting), and UL symbols in ‘S’ is not used for UL signal transmission. It is observed therefore the existing RAN4 RRM test needs 20% of uplink duty cycle.

Observation 1: RRM tests assume the uplink duty cycle of 20%.

Table 1	TDD UL/DL configuration used for FR2 RRM test cases (TS38.133).

Table A.3.1.4-3 TDD UL/DL configuration for SCS=120kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.3.1 |  |  |
| referenceSubcarrierSpacing | kHz | 120 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DDDSU’ S=’10DL:2GP:2UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 0.625 |  |  |
| nrofDownlinkSlots |  | 3 |  |  |
| nrofDownlinkSymbols |  | 9 |  |  |
| nrofUplinkSlot |  | 1 |  |  |
| nrofUplinkSymbols |  | 2 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | Not configured |  |  |
| dl-UL-TransmissionPeriodicity | ms | Not configured |  |  |
| nrofDownlinkSlots |  | Not configured |  |  |
| nrofDownlinkSymbols |  | Not configured |  |  |
| nrofUplinkSlot |  | Not configured |  |  |
| nrofUplinkSymbols |  | Not configured |  |  |
| Note 1: As specified in TS38.213 and TS 38.331. Note 2: For information |  |  |  |  |

## 2.2	UE demodulation test

RAN4 has agreed to consider two TDD configurations: ‘DDDSU’ and ‘DDSU’ for the UE demodulation requirements with FR2 SCS=120kHz as shown in Table 2. Figure 1 and Figure 2 illustrate the channel scheduling of DDDSU and DDSU, respectively. Same as the RRM test, only UL slots marked as ‘U’ are used for UL transmission, and therefore UL duty cycle is 20% for DDDSU and 25% for DDSU.

Observation 2: UE demodulation tests assume at least the uplink duty cycle of 20%.

Table 2	TDD UL/DL configuration used for FR1 UE demodulation and CSI reporting test cases (TS38.101-4).

Table A.3.1-5: TDD UL-DL pattern for SCS 120 kHz

| Parameter |  | Unit | UL-DL pattern |  |
| --- | --- | --- | --- | --- |
|  |  |  | FR2.120-1 | FR2.120-2 |
| TDD Slot Configuration pattern (Note 1) |  |  | DDDSU | DDSU |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U | 11D+3G+0U |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 120 | 120 |
|  | dl-UL-TransmissionPeriodicity | ms | 0.625 | 0.5 |
|  | nrofDownlinkSlots |  | 3 | 2 |
|  | nrofDownlinkSymbols |  | 10 | 11 |
|  | nrofUplinkSlot |  | 1 | 1 |
|  | nrofUplinkSymbols |  | 2 | 0 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | K1 = [4] if mod(i,5) = 0 K1 = [3] if mod(i,5) = 1 K1 = [2] if mod(i,5) = 2 K1 = [6] if mod(i,5) = 3 | K1 = [3] if mod(i,4) = 0 K1 = [2] if mod(i,4) = 1 K1 = [5] if mod(i,4) = 3 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,79} |  |  |  |  |

Figure 1	Scheduling of FR2 PDSCH demodulation with DDDSU.

Figure 2	Scheduling of FR2 PDSCH demodulation with DDSU.

## 2.3	CSI reporting tests

RAN4 agreed to configure DDSU for CSI reporting test with FR2 SCS=120kHz in Table 2. Figure 3 illustrates the scheduling of HARQ-ACK and CSI reporting according to Table 3. Note that CSI reporting uses DDSU but the HARQ-ACK timing (K1) is different from UE demodulation requirement. According to the ratio of UL slots, DDSU requires the UL duty cycle of 25%, but as is illustrated in Figure 3, UL transmission is scheduled every 8 slots for both HARQ-ACK and CSI reporting. Therefore the effective UL duty cycle is 12.5% (1/8).

Observation 3: CSI reporting test assume the uplink duty cycle of 12.5%.

Figure 3	Scheduling of FR1 CSI reporting test (CQI, PMI, RI) with DDSU.

## 2.4	Discussion

Based on the observations so far, the existing RAN4 RRM and UE demodulation/CSI test need at least the UL duty cycle of 20%. This means UE only supporting max uplink duty cycle of 10% cannot pass the RRM/ demodulation/CSI requirements because it cannot transmit all the HARQ-ACK, CSI reporting, or necessary signaling. If RAN4 needs to support the UE capable of the max UL duty cycle of 10%, RAN4 needs to revisit all the RRM and UE demodulation/CSI test cases, but the work cost is not small.

We should also emphasize the current RAN4 TDD configurations ‘DDDSU’ and ‘DDSU’ are based on the requests by operators. Many operators are interested in these configurations and are going use these configurations in their commercial networks. If RAN4 keeps the maximum UL duly cycle of 10% on DDDSU or DDSU, the network cannot schedule some slots to avoid UL transmission for the UL duty cycle capability. This means operators cannot utilize all the network resources. We therefore to propose to remove maxUplinkDutyCycle values < 20%.

Proposal: maxUplinkDutyCycle values < 20% are removed.

# 3	Summary

Observation 1: RRM tests assume the uplink duty cycle of 20%.

Observation 2: UE demodulation tests assume at least the uplink duty cycle of 20%.

Observation 3: CSI reporting test assume the uplink duty cycle of 12.5%.

Proposal: maxUplinkDutyCycle values < 20% are removed.

# References

- RP-190755, “WF on FR2 maximum UL duty cycle”, Nokia, Nokia Shanghai Bell.

- TS 38.101-2

# Appendix

Table 3	Test parameters used for FR2 CSI reporting tests.

Table 8.1.2-1: Test parameters for CSI test cases

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  |  | Transmission scheme 1 |
| Duplex Mode |  |  |  |  | TDD |
| EPRE ratio of PTRS to PDSCH |  |  |  | dB | [0] |
| Active DL BWP index |  |  |  |  | 1 |
| Cyclic prefix |  |  |  |  | Normal |
| Common serving cell parameters | Physical Cell ID |  |  |  | 0 |
|  | SSB position in burst |  |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  |  | Each slot |
|  | Symbols with PDCCH |  |  |  | 0,1 |
|  | Number of PDCCH candidates and aggregation levels |  |  |  | 1/[8] |
|  | DCI format |  |  |  | 1_1 |
|  | TCI state |  |  |  | TCI state #1 |
| Cross carrier scheduling |  |  |  |  | Not configured |
| PDSCH configuration | Mapping type |  |  |  | Type A |
|  | k0 |  |  |  | 0 |
|  | Starting symbol (S) |  |  |  | 2 |
|  | Length (L) |  |  |  | 12 |
|  | PDSCH aggregation factor |  |  |  | 1 |
|  | PRB bundling type |  |  |  | Static |
|  | PRB bundling size |  |  |  | 2 |
|  | Resource allocation type |  |  |  | 0 |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  |  | Type 1 |
|  | Number of additional DMRS |  |  |  | 1 |
|  | DMRS ports indexes |  |  |  | {1000} for Rank1 {1000,1001} for Rank2 |
|  | Length |  |  |  | Single-symbol DM-RS |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  |  | 2 |
| PTRS configuration | Frequency density (KPT-RS) |  |  |  | 2 |
|  | Time density (LPT-RS) |  |  |  | 1 |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | [0] |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | [4] |
|  |  | Number of CSI-RS ports (X) |  |  | 1 |
|  |  | CDM Type |  |  | No CDM |
|  |  | Density (ρ) |  |  | 3 |
|  |  | CSI-RS periodicity |  | slot | 120kHz SCS: 160 |
|  |  | CSI-RS offset |  | slot | 120 kHz SCS: 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
|  |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  |  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  |  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| CSI-RS for beam refinement |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2 |
|  |  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2 |
|  |  | CSI-RS periodicity |  | Slots | 120 kHz SCS: 160 for CSI-RS resource 1,2 |
|  |  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |
|  |  | Repetition |  |  | ON |
| TCI state #0 |  | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  |  | QCL Type |  | Type C |
|  |  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  |  | QCL Type |  | Type D |
| TCI state #1 |  | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | Type A |
|  |  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | Type D |
| Number of HARQ Processes |  |  |  |  | 8 |
| HARQ ACK/NACK bundling |  |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  |  | {0,2,3,1} |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  |  |  | For FR2.120-1: [3] if mod (i.5) = 0, [6] if mod(i,5) = 2 For FR2.120-2: [11] if mod(i,8) = 0, [7] if mod(i,8) = 4, [6] if mod(i,8) = 5, where i is slot index per radio fame with values 0-79. |
| Symbols for unused Res |  |  |  |  | OCNG as specified in A.5 |
| Note 1: PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL. Note 2: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. |  |  |  |  |  |