3GPP TSG-RAN WG4 Meeting #90-bis                                               R4-1904083

Xi’an, China, 8th to 12th Apr. 2019

Agenda Item:	9.4.2

Source: 	MediaTek Inc.

Title: 	TP for TR37.716-21-11 for DC_2A-66A _n5A

Document for:	Approval

- 1. Introduction

The EN DC 2A-66A _n5A has been requested in Release-16 basket WI in RAN4 meetings.

- 2. Discussion

### The contribution is to finish TP for DC_2A-66A _n5A. Here we analyze MSD for Scell due to dual uplink operation.

### Based on co-existence studies of DC_2_n5 and DC_66_n5, as captured in TR 37.863-01-01, own Rx impact on the

### 3rd band is	4th order IM generated by dual uplink of Band 2 and n5 may fall into Band 66. We found that the MSD

### due to IMD4 would be dominant on Band 2 PA side. IMD4 would be coherent on B66 primary and diversity receiving

### paths. FE architecture we used for the analysis is below:

Figure 2-1 Reference architecture for DC_25A-n41A MSD with shared antenna

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation Primary <-> Diversity | 10 | dB |
| Diplexer isolation at B2 uplink freq | 20 | dB |
| Diplexer LB to COM rejection at B66 RX freq | 25 | dB |
| B5 DUP isolation from COM to TX port at B2 TX freq | 30 | dB |
| B5 DUP isolation from TX to COM port at B66 RX freq | 27 | dB |
| QPX isolation from COM to PA at B5 freq | 35 | dB |
| QPX isolation from PA to COM at B66 RX freq | 40 | dB |
| QPX isolation from PA to B66 RX | 55 | dB |
| Typical transmitter / receiver phase noise | -150 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 2-1 Typical receiver performance parameters for MSD analysis

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| B2/B5 TX power at antenna port (Primary path) | 23 | 23 | dBm |
| B5 TX power at B2 QPX COM port | 4 | NA | dBm |
| Thermal noise at RX ant port | -165.5 | -165.5 | dBm/Hz |
| B2 TX IM4 spurious level refer to RX ANT input | -89.5 | -89.5 | dBm |
| Combined |  |  |  |
| Combined noise | -83 |  | dBm |
| Combined B66 REFSENS (5MHz BW) | -90 |  | dBm |
| MSD (10MHz BW, SCS=30KHz) | 9.5 |  | dB |

### Table 2-2 Link analysis for n41 10MHz REFSENS calculation

3. TP for TR37.716-21-11

- 5.1.x	DC_2-66_n5

### 5.1.x.1	Operating bands for DC_2-66_n5

Table 5.1.21.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n5 | CA_2-66 | n5 | No |

### 5.1.x.2	Configurations for DC_2-66_n5

Table 5.1.21.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-66A | n5 |

### 5.1.x.3	Co-existence Studies

Based on co-existence studies of DC_2_n5 and DC_66_n5, as captured in TR 37.863-01-01, own Rx impact on the 3rd band is the following:

- 4th order IM generated by dual uplink of Band 2 and n5 may fall into Band 66

### 5.1.x.4	∆TIB and ∆RIB values

Table 5.1.21.4-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n5 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n5 | 0.8 |

Table 5.1.21.4-1: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n5 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n5 | 0 |

### 5.1.x.5	REFSENS requirements

Table 5.1.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n5A | 2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | 9.5 | FDD | IMD4 |