3GPP TSG-RAN WG4 Meeting #90bis	R4-1904136

Xi’an, China, 8 - 12 April, 2019

Title: 					Introduction of enhanced measurements for high speed train for SCells

Source: 	Ericsson

Agenda item:	7.15.2

Document for:	Discussion

# Introduction

In this paper, we discuss the aspects of the way forward for HST enhancement related to

- RAN4 RRM shall focus on the following requirement impacts for HST feature: … - Measurement requirements of SCC with both active Scells and deactivated SCells …

# Discussion

## Active Scell, non DRX

Baseline cell detection requirements for active SCells in non DRX (and DRX) are specified in 36.133 section 8.3.3.1

#### 8.3.3.1 Measurements of a secondary component carrier with active SCell When the SCell is activated, measurement performance requirements for the frequency are those given in clause 8.1.2.2(E-UTRAN intra frequency measurements and E-UTRAN intra frequency measurements with autonomous gaps). If common DRX is in use, then the requirements for that secondary component carrier are given by the applicable DRX requirements (FDD or TDD) in clause 8.1.2.2, otherwise the non DRX requirements are applicable. When highSpeedEnhancedMeasFlag is configured, the enhanced measurement requirements apply only to measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. The applicable measurement accuracy requirements are in clause 9.1.11 (Carrier aggregation measurement accuracy)

In release 14, no enhancement was specified for non DRX requirements of the PCell.

This means that cells should be identified within 800ms, and 200ms measurement period is specified.

While the discussion will still take place for PCell HST non DRX requirements in release 16, our view is that there is limited scope to enhance SCC non DRX performance.

- The existing requirements assume measurement activity every 40ms; increasing the measurement duty cycle would significantly increase UE power consumption, and is also likely to have major design implications for the searcher hardware (e.g. the searcher has to perform offline processing more quickly due to the need to be ready in time to make the next measurement)

Proposal 1 : SCell  non DRX requirements are not enhanced in release 16

On the other hand, since LTE gives PSS/SSS every 5ms, and continuous CRS, there is undoubtedly scope for improved SCC (and PCC) mobility performance at the cost of additional power consumption, so we do not state this as a strong proposal if it is demonstrated that it is necessary to perform faster mobility measurements for 500km/h operation. In the final analysis, it is better to use power to maintain a connection at 500km/h that to have a very high call drop rate, which leads to unusable devices and even higher power drain.

## Active Scell, DRX

Considering the objective to “At least, extend Rel-14 RRM/demodulation enhancement to CA case”, it is necessary to define enhanced measurement requirements also for SCC. Considering that a relatively power efficient solution was chosen for release 14, it should be straightforward to apply the same requirements to SCC for the purposes of SCell management in high speed train operation. Naturally, there may be some UE power penalty, however it is typically impossible to reduce measurement delays without increased UE measurement activity, which automatically implies additional power consumption, and the enhancements are not requiring additional wakeup. Since SCell DRX in carrier aggregation is aligned with PCell DRX, there should be no significant concern on power consumption.

Proposal 2: Enhanced intrafrequency cell identification and measurement period are defined for SCC in release 16 with the same values as for PCC in release 14

## Dectivated Scell, no common DRX is used

Deactivated SCell measurement is performed according to a deactivated SCell measurement cycle, giving the requirements

Tidentify_scc = 20 measCycleSCell and Tmeasure_scc = 5 measCycleSCell.

Given our view on the feasibility of proposal 2,  we also think it is feasible to enhance deactivated SCell measurements for high speed operation according to

Proposal 3: For release 16, deactivated SCell requirements when no common DRX is used is based on Tidentify_scc = 15 measCycleSCell and Tmeasure_scc = 3 measCycleSCell.

## Dectivated Scell, common DRX is used

Following a similar and consistent approach, we propose

Proposal 4: For release 16, deactivated SCell requirements when common DRX is used is based on Tidentify_scc = max(15 measCycleSCell, Tidentify_scc1).  Tidentify_scc1 is given in table 1

Table 1: Requirement for Tidentify_scc1

| DRX cycle length (s) | Tidentify_scc1 (s) (DRX cycles) |
| --- | --- |
| ≤0.04 | 0.8 (Note1) |
| 0.04<DRX-cycle≤0.08 | Note2(15) |
| 0.128 | Note2(10) |
| 0.128<DRX-cycle≤2.56 | Note2(20) |
| Note1: Number of DRX cycle depends upon the DRX cycle in use Note2: Time depends upon the DRX cycle in use |  |

Proposal 5: For release 16, deactivated SCell requirements when common DRX is used is based on Tmeasure_scc =max( 3 measCycleSCell, Tmeasure_scc1).

Table 2: Requirement for Tmeasure_scc1

| DRX cycle length (s) | Tmeasure_scc1 (s) (DRX cycles) |
| --- | --- |
| ≤0.04 | 0.2 (Note1) |
| 0.04<DRX-cycle≤2.56 | Note2 (3) |
| Note1: Number of DRX cycle depends upon the DRX cycle in use Note2: Time depends upon the DRX cycle in use |  |

## Interruption aspects

Since the UE still needs to perform measurements according to the deactivated SCell measurement cycle, we do not consider there is an impact to interruption requirements.

## Signaling aspects

Next, we consider whether it is necessary for the network to independently configure high speed measurements for the SCC(s) and PCC. Given that the UE is either operating in high speed condition or it is not, we do not see a compelling reason for independent configuration, although there is also no significant problem with an independent configuration. However, we propose:

Proposal 6: Enhanced intrafrequency cell identification and measurement period for SCC are configured with the existing highSpeedEnhancedMeasFlag

Our final considerations are on UE capabilities. The existing release 14 enhancements are supported from release 13 onwards in a release independent manner in 36.307 according to the capability “measurementEnhancements-r14”. Since there are already UE in the field which support enhancement on the PCell and not on the SCell, and this support cannot be determined from the UE release, we support a new capability indicator such as

Proposal 7: UE capability for enhanced intrafrequency cell identification and measurement period for SCC is indicated using a new high speed enhancement capability such as SCCmeasurementEnhancements-r16

Note that this is not a discussion about whether the new capability is optional or mandatory, and from which release it applies. We anticipate this discussion will take place around the completion of release 16.

# Conclusion

In this contribution, we provide proposals for SCell measurement enhancement for the HST enhancement work item

Proposal 1 : SCell  non DRX requirements are not enhanced in release 16

Proposal 2: Enhanced intrafrequency cell identification and measurement period are defined for SCC in release 16 with the same values as for PCC in release 14

Proposal 3: For release 16, deactivated SCell requirements when no common DRX is used is based on Tidentify_scc = 15 measCycleSCell and Tmeasure_scc = 3 measCycleSCell.

Proposal 4: For release 16, deactivated SCell requirements when common DRX is used is based on Tidentify_scc = max(15 measCycleSCell, Tidentify_scc1).  Tidentify_scc1 is given in table 1

Table 1: Requirement for Tidentify_scc1

| DRX cycle length (s) | Tidentify_scc1 (s) (DRX cycles) |
| --- | --- |
| ≤0.04 | 0.8 (Note1) |
| 0.04<DRX-cycle≤0.08 | Note2(15) |
| 0.128 | Note2(10) |
| 0.128<DRX-cycle≤2.56 | Note2(20) |
| Note1: Number of DRX cycle depends upon the DRX cycle in use Note2: Time depends upon the DRX cycle in use |  |

Proposal 5: For release 16, deactivated SCell requirements when common DRX is used is based on Tmeasure_scc =max( 3 measCycleSCell, Tmeasure_scc1).

Table 2: Requirement for Tmeasure_scc1

| DRX cycle length (s) | Tmeasure_scc1 (s) (DRX cycles) |
| --- | --- |
| ≤0.04 | 0.2 (Note1) |
| 0.04<DRX-cycle≤2.56 | Note2 (3) |
| Note1: Number of DRX cycle depends upon the DRX cycle in use Note2: Time depends upon the DRX cycle in use |  |

Proposal 6: Enhanced intrafrequency cell identification and measurement period for SCC are configured with the existing highSpeedEnhancedMeasFlag

Proposal 7: UE capability for enhanced intrafrequency cell identification and measurement period for SCC is indicated using a new high speed enhancement capability such as SCCmeasurementEnhancements-r16

# References

- R4-1902520, “Way forward on HST enhancement for RRM”, Huawei, Hisilicon