3GPP TSG RAN WG4 #90-bis	R4-1904278

# Xi’an, China, 8 Apr 2019 - 12 Apr, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | CRNum | rev |  | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on FR2 CSI Reporting Tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-04-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TCI state is not defined for CSI-RS for beam refinement |  |  |  |  |  |  |  |  |
| Summary of change: |  | TCI state is defined for CSI-RS for beam refinement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TCI state for CSI-RS for beam refinement will not be clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-4 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

### 8.1.2	Common test parameters

Parameters specified in Table 8.1.2-1 are applied for all test cases in this section unless otherwise stated.

Table 8.1.2-1: Test parameters for CSI test cases

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  |  | Transmission scheme 1 |
| Duplex Mode |  |  |  |  | TDD |
| EPRE ratio of PTRS to PDSCH |  |  |  | dB | [0] |
| Active DL BWP index |  |  |  |  | 1 |
| Cyclic prefix |  |  |  |  | Normal |
| Common serving cell parameters | Physical Cell ID |  |  |  | 0 |
|  | SSB position in burst |  |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  |  | Each slot |
|  | Symbols with PDCCH |  |  |  | 0,1 |
|  | Number of PDCCH candidates and aggregation levels |  |  |  | 1/[8] |
|  | DCI format |  |  |  | 1_1 |
|  | TCI state |  |  |  | TCI state #1 |
| Cross carrier scheduling |  |  |  |  | Not configured |
| PDSCH configuration | Mapping type |  |  |  | Type A |
|  | k0 |  |  |  | 0 |
|  | Starting symbol (S) |  |  |  | 2 |
|  | Length (L) |  |  |  | 12 |
|  | PDSCH aggregation factor |  |  |  | 1 |
|  | PRB bundling type |  |  |  | Static |
|  | PRB bundling size |  |  |  | 2 |
|  | Resource allocation type |  |  |  | 0 |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  |  | Type 1 |
|  | Number of additional DMRS |  |  |  | 1 |
|  | DMRS ports indexes |  |  |  | {1000} for Rank1 {1000,1001} for Rank2 |
|  | Length |  |  |  | Single-symbol DM-RS |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  |  | 2 |
| PTRS configuration | Frequency density (KPT-RS) |  |  |  | 2 |
|  | Time density (LPT-RS) |  |  |  | 1 |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | [0] |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | [4] |
|  |  | Number of CSI-RS ports (X) |  |  | 1 |
|  |  | CDM Type |  |  | No CDM |
|  |  | Density (ρ) |  |  | 3 |
|  |  | CSI-RS periodicity |  | slot | 120kHz SCS: 160 |
|  |  | CSI-RS offset |  | slot | 120 kHz SCS: 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
|  |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  |  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  |  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| CSI-RS for beam refinement |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2 |
|  |  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2 |
|  |  | CSI-RS periodicity |  | Slots | 120 kHz SCS: 160 for CSI-RS resource 1,2 |
|  |  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |
|  |  | Repetition |  |  | ON |
|  |  | QCL info |  |  | TCI state #1 |
| TCI state #0 |  | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  |  | QCL Type |  | Type C |
|  |  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  |  | QCL Type |  | Type D |
| TCI state #1 |  | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | Type A |
|  |  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | Type D |
| Number of HARQ Processes |  |  |  |  | 8 |
| HARQ ACK/NACK bundling |  |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  |  | {0,2,3,1} |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  |  |  | For FR2.120-1: [3] if mod (i.5) = 0, [6] if mod(i,5) = 2 For FR2.120-2: [11] if mod(i,8) = 0, [7] if mod(i,8) = 4, [6] if mod(i,8) = 5, where i is slot index per radio fame with values 0-79. |
| Symbols for unused Res |  |  |  |  | OCNG as specified in A.5 |
| Note 1: PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL. Note 2: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. |  |  |  |  |  |

<< End of changes >