3GPP TSG-RAN WG4 Meeting #90bis	R4-1904545

Xi‘an, China, 8-12 April, 2019

Agenda item:	6.5.2

Source:	Sprint

Title:	UE coexistence table harmonization in 38.101-3

Document for:	Discussion

# 1	Introduction

All the UE coexistence tables in 38.101-1 list the E-UTRA band number for bands that support both NR and E-UTRA, and the NR band numbers only for NR-only bands. In 38.101-3 the tables contain many different combinations of E-UTRA and NR band numbers. This contribution proposes harmonizing the UE coexistence tables in 38.101-3 with those in 38.101-1.

# 2	Discussions

All the UE coexistence tables in 38.101-1 list the E-UTRA band number (X format) for bands that support both NR and E-UTRA, and the NR band numbers (nX format) only for NR-only bands. In 38.101-3 Table 6.5B.3.3.2-1 there are many different combinations of E-UTRA and NR band numbers as follows:

- E-UTRA/NR bands listed as NR bands when NR bands exist (which is currently a constantly moving target)

| DC_1_n28 | E-UTRA Band 18, 19, 27, 31, 32, 72 NR band n5, n7, n8, n20, n26, n38, n40, n41, n50, n51, n74 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA Band42, 43 NR band n78, n75, n76 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR band n3, n34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 65 NR band n1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |

- Calling them all “Band” without specifying E-UTRA or NR

| DC_1_n40 | Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 17 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 17 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 17 |

- Listing the number with and “n” for NR-only bands under the label “E-UTRA Band”

| DC_20_n8 | E-UTRA Band 1, 3, 7, 22, 28, 31, 32, 34, 38, 42, 43, 65, 75, 76, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Listing the label “E-UTRA” where there are both E-UTRA and NR bands, and NR where there are only NR bands (consistent with 38.101-1)

| DC_28_n51 | E-UTRA Band 2, 3, 5, 7, 8, 25, 26, 31, 34, 38, 40, 41, 66, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA Band 4, 10, 20, 22, 24, 32, 42, 43, 45, 46, 65, 66, 71, 73 NR band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 10 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |

- Listing both E-UTRA and some NR band numbers under the label “Bands”

| DC_30_n66 | Bands 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 66, 70, 71, n71 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bands 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |

- Listing both E-UTRA and NR band numbers separately under both “E-UTRA Band” and NR Band”

| DC_41_n79 | E-UTRA Band 1, 3, 5, 8, 9, 11, 18, 19, 21, 28, 34, 40, 42, 44, 45, 65 or NR Band n1, n3, n8, n28, n34, n40 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

- Some EN-DC band combinations seem to not have any requirement to protect any other bands:

| DC_42_n77 | N/A |
| --- | --- |
| DC_42_n78 | N/A |
| DC_42_n79 | N/A |

- Listing some rows as “Bands” and others as “E-UTRA bands”:

| DC_66_n5 | Bands 1, 2, 3, 4, 5, 6, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, n71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Bands 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |

Observation 1: the UE coexistence tables in 38.101-3 contain many unique ways of listing E-UTRA and NR band numbers.

We believe that RAN4 should harmonize the UE coexistence tables in 38.101-3 with those in 38.101-1. When things settle out and most if not all LTE bands are NR bands also, RAN4 can consider listing NR bands where there are both NR and E-UTRA bands.

Proposal 1: E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

The tables in 38.101-1 also contain a note like this one:

NOTE:	To simplify Table 6.5A.3.2.3-1, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

Only one of the UE coexistence tables in 38.101-3 contain a similar note, although most follow the format used in 38.101.

Proposal 2: Add the following note to each of the UE coexistence tables in 38.101-3:

NOTE:	To simplify the above Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

Proposal 3: Update the UE Coexistence tables in the EN-DC and NR-CA TRs as well.

There is an accompanying CR in R4-1904546 [1].

# 3	Conclusions

Observation 1: the UE coexistence tables in 38.101-3 contain many unique ways of listing E-UTRA and NR band numbers.

Proposal 1: E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

Proposal 2: Add the following note to each of the UE coexistence tables in 38.101-3:

NOTE:	To simplify the above Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

Proposal 3: Update the UE Coexistence tables in the EN-DC and NR-CA TRs as well.

# 4	References

- R4-1904546, “Draft CR for 38.101-3: Editorial harmonization of UE Coexistence tables,” Sprint