3GPP TSG-RAN WG4 Meeting #90bis	R4-1904781

Xi’an, China, 8 - 12 April, 2019

Title: 			Ad hoc minutes for RRM measurement

Source: 		Ericsson

Agenda item:	6.10.3

Document for:	Discussion

# 6.10.3.1 UE behaviour after measurement gap

| R4-1903748.zip | UE Tx behaviour after MG | Huawei, HiSilicon |
| --- | --- | --- |
| R4-1903749.zip | CR on UE Tx behavior after MG (section 9.1.2) | Huawei, HiSilicon |
| R4-1904484.zip | Discussion on UE behavior after MG | MediaTek inc. |
| R4-1904485.zip | CR on TS38.133 for UE behavior after MG (Section 9.1.2) | MediaTek inc. |
| R4-1904486.zip | CR on TS36.133 for UE behavior after MG (Section 8.1.2) | MediaTek inc. |
| R4-1904487 | CR on TS36.133 for UE behavior after MG (Section 8.1.2) | MediaTek inc. |

Handling of larger timing advance

- Discuss handling of TA > 1 slot e.g. from following options

- Option 1: ignore the case of TA larger than 1 slot (no change to the current requirements)

- Option 2: allow UE not to transmit in 2 UL slots

- Option 3: allow UE not to transmit in 1 or 2 UL slots based on UE’s TA

Status from main meeting Agreement: It is up to UE implemention to transmit in 1 or 2 UL slots based on UE’s TA if the TA is larger than [one slot] - Network should know the risk that the TA at UE side may be beyond one slot. - Figure out a way to capture the sub-bullet above in the spec. - Further working on the wording for the above agreement. - No test case for it in Rel-15 - Whether or not UE to inform the capability will be discussed in future release

Handling of MGTA

- Discuss if proposals in R4-1904484 are acceptable to RAN4

Proposal 1: UE behavior after measurement gap In TS36.133 should also be addressed separately when MGTA is not applied or when MGTA is applied.

Proposal 2: In TS36.133, when MGTA is applied, the uplink transmission occurring immediately after the sub-frame partially overlapped with measurement gap is up to UE implementation.

Status from main meeting Agreement: - UE behavior after measurement gap In TS36.133 should also be addressed separately when MGTA is not applied or when MGTA is applied. - In TS36.133, when MGTA is applied, the uplink transmission occurring immediately after the sub-frame partially overlapped with measurement gap is up to UE implementation Mediatek allocated 36.133 (Agreed), Huawei 38.133(4681)

# 6.10.3.2 Measurement gap maintenance

| R4-1903406.zip | CR on capability of measurement gap patterns for R15 | Intel Corporation |
| --- | --- | --- |
| R4-1903407.zip | CR on capability of measurement gap patterns for R16 | Intel Corporation |
| R4-1903802.zip | CR on applicability of GP with short MGL in 38.133 (section 9.1) | Huawei, HiSilicon |
| R4-1903803.zip | CR on applicability of gap in LTE, ENDC and NEDC in 36.133 | Huawei, HiSilicon |
| R4-1903804 | CR on applicability of gap in LTE, ENDC and NEDC in 36.133 R16 | Huawei, HiSilicon |
| R4-1904146.zip | Correction of supported gap pattern in setion 9.1.2 and 9.4.1 | Ericsson |
| R4-1904488.zip | Discussion on gap pattern signaling | MediaTek inc. |
| R4-1904489.zip | CR on TS38.133 for gap pattern signaling (Section 9.1.2) | MediaTek inc. |
| R4-1904490.zip | CR on TS36.133 for gap pattern signaling (Section 8.1.2.1) | MediaTek inc. |
| R4-1904491 | CR on TS36.133 for gap pattern signaling (Section 8.1.2.1) | MediaTek inc. |

Status in RAN2

In 38.306 supportedGapPattern Indicates measurement gap pattern(s) optionally supported by the UE. The leading / leftmost bit (bit 0) corresponds to the gap pattern 2, the next bit corresponds to the gap pattern 3, as specified in TS 38.133 [5] and so on.; UE; No; No; No. LS from RAN2 RAN2 would like to inform RAN4 that RAN2 has discussed the capability signalling of measurement gap patterns. RAN2 agreed that current RRC signaling for gap pattern capabilities as following: - Gap patterns 0 and 1 are mandatory for all cases. - In LTE SA - LTE RRC signalling shortMeasurementGap is used to signal whether UE supports gap patterns 2 and 3. - LTE RRC signalling measGapPatterns (8 bits) is used to signal whether UE supports gap patterns 4 to 11. - In EN-DC - LTE RRC signalling shortMeasurementGap is used to signal whether UE supports gap patterns 2 and 3 (for per-UE cap and FR1 gap of per-FR gap). - LTE RRC signalling measGapPatterns is used to signal whether UE supports gap patterns 4 to 11 (for per-UE cap and FR1 gap of per-FR gap). - NR RRC signalling supportedGapPattern (22 bits) is used to signal whether UE supports gap patterns 2 to 23 (for FR2 gap of per-FR gap). - In NR SA - NR RRC signalling supportedGapPattern (22 bits) is used to signal whether UE supports gap patterns 2 to 23. - If the UE supports an optional gap pattern (#2 to #23), it supports using the gap pattern according to the corresponding applicability defined in RAN4 specifications. RAN2 would like to confirm with RAN4 that the above agreements are consistent with TS 36.133 and TS 38.133.

Topics for discussion

Whether inclusion of a pattern in 38.133 gap applicability tables 9.1.2-2 and 9.1.2-3 indicates that the gap pattern is supported by all UEs

- Option 1 (from Mediatek, updated from R4-1904488)

- When there exists a non-NR RAT measurement object:

  - the gap pattern #0, 1, 4 are mandatory supported by UEs which support per-UE measurement gap, the gap patterns #0, 1, 4, 12-23 are mandatory supported by UEs which support per-FR measurement gap in EN-DC/NE-DC mode.

  - the gap pattern #0, 1, 4, 12-23 are mandatory supported by UEs which support per-UE measurement gap, the gap patterns #0, 1, 4, 12-23 are mandatory supported by UEs which support per-FR measurement gap in SA/NR-DC mode

- When there exist only NR measurement objects:

  - gap patterns #0-11 are mandatory supported by UE who supports only per-UE gap, gap patterns #0-11 are mandatory supported for FR1 measurement by UE who supports only per-FR gap, and gap patterns #12-23 are mandatory supported for FR2 measurement by UE who supports only per-FR gap.

  - gap patterns #0-23 are mandatory supported by UE who supports only per-UE gap, gap patterns #0-11 are mandatory supported for FR1 measurement by UE who supports only per-FR gap, and gap patterns #12-23 are mandatory supported for FR2 measurement by UE who supports only per-FR gap.

Correct the corresponding capability signalling of measurement gap patterns in EN-DC, NE-DC, NR SA, and NR-DC modes.

- Option 2 : Follow RAN2 agreements on mandatory and optional gap pattern

Based on the outcome, discuss CRs for 36.133, 38.133 and possible response to RAN2.

Ericsson : OK with more mandatory patterns but had assumed option 2

Samsung : Supports option 2, doesn’t see relationship between patterns as a reason to block RAN2 proposal

Qualcomm : RAN2 decides optional/mandatory. RAN4 specifies the requirements and when different GP can be used

ZTE : Don’t understand why optional/mandatory is decided by RAN2. In option 2 if UE only supports GP0,1 we always have to use 6ms MGL even if SMTC is only 1ms.

Mediatek : In 36.133 we already specify some gap patterns eg 2 and 3 are optional, and 0,1 are mandatory. We think applicability table needs to be modified if we follow RAN2 way

CMCC : Support option 1, RAN2 agreement may cause issues. For NR measuremement we did not send any LS but we sent LS on short MG case. If we follow RAN2 agreements wht does it mean eg if UE supports GP3? Can GP4 be used for LTE only or both NR and LTE.

Intel : Support option 2. Applicability table is guidance and we can add a note into applicability table. If positioning measurement is configured, only GP0 is mandatory.

Huawei : Support option 2. Late phase to modify.

Docomo : Support option 1, there is no mandatory per FR gap pattern.

Observations

- For both per UE and per FR gaps in the SA case that PCell is in FR2 and measurement is in FR2, there is no applicable requirement using the gap patterns that RAN2 decided to be mandatory. Based on current RAN4 spec a UE which is supporting SA operation on FR2 needs to support some of GP12-23 from a RAN4 requirements perspective.

- For UE which support per FR gaps, needs to support at least some of GP12-23 from a RAN4 requirements perspective based on current RAN4 spec.

  - Support of some common gap patterns by different UE implementations is important for network interoperability

Based on current spec, if  NR SA UE indicates support for any pattern with MGL<6ms  , need to have a common understanding whether it can do both NR and also LTE shorter measurement gap?

Proposed agreement : For SA case, if UE indicates support for any pattern 0-11 with  MGL<6ms and MGRP<160ms, it means the UE can do both NR and NR+LTE measurement

ZTE : For NR SA, shorter than 6ms gap is important

ZTE : Observations can be put in a reply LS and RAN2 can do work based on that.

Samsung : RAN2 can make some of the bits dummy bits.

ZTE : We can provide examples to RAN2. Eg SMTC <3ms

CMCC

Chair : From NW perspective, we may not wish to handle many different chipsets that all support a different GP from the range 12-23

Qualcomm : Makes sense then that the longest is supported.

- From RAN4 perspective, for UE supporting SA FR2 PCell, GP with 5.5ms MGL and all periodicities should be mandatory

ZTE : How hard to support other MGL if 5.5 is already supported?

CMCC : IF an NR SA UE indicates support for GP4 (4ms MGL) , is it guaranteed that it can do both NR and also LTE short measurement gap?

Intel : In RAN2 there are 2 ways to indicate : There is short measurement gap capability and also there is

Qualcomm : Assume so, but would like to check.

Qualcomm :Should reply to RAN2 about cases that need additional gap pattern than 0,1 based on current requiremenys.

# 6.10.3.2.1 collision of RRM measurement with UL transmission

| R4-1902893.zip | Further discussion on collision of RRM measurement with UL transmission | Intel Corporation |
| --- | --- | --- |
| R4-1902894.zip | CR on Scheduling availability for intra-freq meas wo gap for R15 (section 9.2.5.3.1) | Intel Corporation |
| R4-1902895.zip | Reply LS on collision of RRM measurement with UL transmission | Intel Corporation |
| R4-1903812.zip | Discussion on LS on collision of RRM measurement resources with uplink transmissions in FR1 TDD | Huawei, HiSilicon |
| R4-1903813.zip | CR on Scheduling availability for collision of RRM measurement resources with uplink transmissions in FR1 TDD (Section 9.2) | Huawei, HiSilicon |
| R4-1904492.zip | Discussion on collision of measurement resources with UL transmissions | MediaTek inc. |
| R4-1904493.zip | CR on TS38.133 for Scheduling availability (Section 9.2.5.3) | MediaTek inc. |

Based on the submitted papers there are the following open issues to discuss:

Open issue : RAN1 sent RAN4 an LS in August/October 2018 informing of a working assumption

Case 1: Unpaired spectrum on FR1

Neighbour SSB colliding with UL transmission

- Majority of contributions propose that SSB measurement may be prioritised over UL TDD transmission in FR1 (this assumption is implicit in the derivation of measurement procedure requirements).

- Whether to define this case as a scheduling restriction

  - Option 1 : Define scheduling restriction in this meeting

  - Option 2: Provide feedback on RAN4 preference to RAN1 by replying to LS, define scheduling restriction in RAN4 after RAN1 has had the opportunity to review

CSI-RS colliding with UL transmission

- Majority of contributions propose that as CSI-RS measurement for mobility is not included in release 15, RAN4 should not provide feedback on this case

Case 2: Half duplex CA on FR2 case

- Same approach as for case 1, ie RAN4 view is that SSB transmission may be prioritised over UL TDD transmission in FR1

- Scheduling restriction in RAN4 may need updates

  - Could be postponed to rel16 (Huawei) as RAN1 has a related agreement “directional collision handling for half duplex CA operation are postponed to Re1-16.”

Status from main meeting Agreement: delay the scheduling restriction requirement for inter-band simultaneous Rx/Tx limitation to Rel-16 Agreement: RAN4 understanding is that SSB measurement is prioritised over UL TDD transmission in FR1 in Rel-15. Agreement: The CSI-RS measurement for mobility is not included in release 15, and RAN4 should not provide feedback on this case. Agreement: The requirements for half duplex CA operation are postponed to Re1-16 Agreement: In a FR1 TDD band, UE is not requested to transmit UL on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. Intel preparing reply LS (R4-1904682) Mediatek preparing CR (R4-1904683) can be endorsed if LS sent in good time for RAN1 to review this week, otherwise noted/indicated technically correct from RAN4 point of view

# 6.10.3.2.2 UE behavior on receptions in the same OFDM symbol

Open issues

General principle should be agreeable to all, following discussion in RAN4#90

The concurrent reception of signal in group 1 and group 2 can be done as long as the UE can use the same Rx beam for reception (or Rx beam sweeping is not needed) and the SCS is same (unless UE can support simultaneousRxDataSSB-DiffNumerology).

The main issue to discuss is the condition on which RX beam sweep will be performed by UE or differnet RX beam will be used for L1 RSRP (SSB-RS, CSI-RS), RLM, BFD, CBD and whether this can be determined from the condition N>1 in QCL type D case.

| R4-1902864.zip | UE behavior on collision of downlink reception | Intel Corporation |
| --- | --- | --- |
| R4-1902865.zip | Reply LS on UE behavior on reception of channels or RS in the same OFDM symbol | Intel Corporation |
| R4-1903418.zip | Discussion on UE behavior on reception of channels or RS in the same OFDM symbol | LG Electronics Inc. |
| R4-1903419.zip | Reply LS on UE behavior on reception of channels or RS in the same OFDM symbol | LG Electronics Inc. |
| R4-1903737.zip | Further discussion on UE behaviour on reception of channels or RS in the same OFDM symbol | Huawei, HiSilicon |
| R4-1903738.zip | Reply LS on UE behaviour on reception of channels or RS in the same OFDM symbol | Huawei, HiSilicon |

Case 1 & 4

  - If the RS in group 2 is QCL-Type D with RS in group 1 and the QCL association is known to UE, UE can receive both RS in group 1 and group 2 in the same OFDM symbol when UE is not expected to sweep its Rx beam in FR2

    - Add CSI RS bullet

  - The conditions that UE is expected to sweep its Rx beam

    - L3 SSB-based measurement

      - All companies agree that RX beam sweep is used for L3 SSB measurement within the SMTC

    - L1 SSB-based measurement

      - Option 1 (Huawei): For L1-RSRP measurements, UE is expected to perform Rx beam sweeping on the configured RS under the conditions of N>1, which is defined in section 9.5 of TS38.133.

      - Option 2: (Intel, Huawei, Mediatek) : Always for SSB based measurements

      - Option 3: (LG) : When UE needs to sweep Rx beam for BM-RS, UE may not receive FDMed RSs in the same OFDM symbol depending on UE implementation even if the RSs are QCLed

    - L1 CSI-RS based measurement

      - Option 1 : (Huawei, Intel, Mediatek): For L1-RSRP measurements, UE is expected to perform Rx beam sweeping on the configured RS under the conditions of N>1, which is defined in section 9.5 of TS38.133.

      - Option 2: (LG) When UE needs to sweep Rx beam for BM-RS, UE may or may not receive FDMed RSs in the same OFDM symbol depending on UE implementation even if the RSs are QCLed.

    - RLM/BFD/CBD

      - Option 1 (Intel, Huawei, Meditatek) : RLM/BFD/CBD measurement under certain conditions specified as the condition for N>1, which can be referred in Section 8.1/8.5/9.5 of TS38.133

      - Option 2 (LG) : When UE needs to sweep Rx beam for RLM/BFD/BM-RS, UE may or may not receive FDMed RSs in the same OFDM symbol depending on UE implementation even if the RSs are QCLed.

Case 2 & 3

  - UE can receive a PDCCH/PDSCH that is overlapping with one or more symbols configured with CSI-RS resource(s) with repetition set to “OFF”, provided:

    - the TCI of the PDCCH/PDSCH includes at least one of the CSI-RS resource IDs of the CSI-RS resource set and QCL-Type D of the CSI-RS is configured, andUE is not expected to sweep its Rx beam in FR2.

Check whether case 2 and 3 are applicable to L3 CSI-RS measurement

Conditions for RX beam sweep

- Option 1 (Intel/Huawei/Mediatek):

    - For L1-RSRP measurements, UE is expected to perform Rx beam sweeping on the configured RS under the conditions of N>1, which is defined in section 9.5 of TS38.133.

Option 2 (LG):

    - When UE needs to sweep Rx beam for QCLed source refereeing the CSI-RS, UE may or may not receive PDCCH/PDSCH transmitted in the same OFDM symbol depending on UE implementation even if CSI-RS and PDCCH/PDSCH are QCLed.

Status from main meeting Agreement: The concurrent reception of signal in group 1 and group 2 can be done as long as the UE can use the same Rx beam for reception (or Rx beam sweeping is not needed) and the SCS is same (unless UE can support simultaneousRxDataSSB-DiffNumerology). - Case 1 & 4 - If the RS in group 2 is QCL-Type D with RS in group 1 and the QCL association is known to UE, UE can receive both RS in group 1 and group 2 in the same OFDM symbol when UE is not expected to sweep its Rx beam in FR2 - The agreement is not applicable to CSI-RS base L3 measurement in Rel-15. - The conditions that UE is expected to sweep its Rx beam - L3 SSB-based measurement - All companies agree that RX beam sweep is used for L3 SSB measurement within the SMTC - L1 SSB-based measurement - Option 1 (Huawei): For L1-RSRP measurements, UE is expected to perform Rx beam sweeping on the configured RS under the conditions of N>1, which is defined in section 9.5 of TS38.133. - Option 2: (Intel, Huawei, Mediatek) : Always for SSB based measurements - Option 3: (LG) : When UE needs to sweep Rx beam for BM-RS, UE may not receive FDMed RSs in the same OFDM symbol depending on UE implementation even if the RSs are QCLed - L1 CSI-RS based measurement - Option 1: (Huawei, Intel, Mediatek): For L1-RSRP measurements, UE is expected to perform Rx beam sweeping on the configured RS under the conditions of N>1, which is defined in section 9.5 of TS38.133. - Option 2: (LG) When UE needs to sweep Rx beam for BM-RS, UE may or may not receive FDMed RSs in the same OFDM symbol depending on UE implementation even if the RSs are QCLed. - RLM/BFD/CBD - Option 1 (Intel, Huawei, Mediatek) : RLM/BFD/CBD measurement under certain conditions specified as the condition for N>1, which can be referred in Section 8.1/8.5/9.5 of TS38.133 - Option 2 (LG) : When UE needs to sweep Rx beam for RLM/BFD/BM-RS, UE may or may not receive FDMed RSs in the same OFDM symbol depending on UE implementation even if the RSs are QCLed. - Case 2 & 3 - UE can receive a PDCCH/PDSCH that is overlapping with one or more symbols configured with CSI-RS resource(s) with repetition set to “OFF”, provided: - the TCI of the PDCCH/PDSCH includes at least one of the CSI-RS resource IDs of the CSI-RS resource set and QCL-Type D of the CSI-RS is configured, and UE is not expected to sweep its Rx beam in FR2. Check whether Case 2 and 3 are applicable to L3 CSI-RS measurement. - Conditions for RX beam sweep - Option 1 (Intel/Huawei, Mediatek): - For L1-RSRP measurements, UE is expected to perform Rx beam sweeping on the configured RS under the conditions of N>1, which is defined in section 9.5 of TS38.133. - Option 2 (LG): - When UE needs to sweep Rx beam for QCLed source refereeing the CSI-RS, UE may or may not receive PDCCH/PDSCH transmitted in the same OFDM symbol depending on UE implementation even if CSI-RS and PDCCH/PDSCH are QCLed. Reply LS allocated to LG in R4-1904684

# 6.10.3.2.3 Maintenance for intra-frequency measurement

| R4-1903445.zip | Draft CR on TS38.133 for Intrafrequency cell identification (section 9.2.5.1) | CMCC | Add condition on ssb-to-measure for Klayer1_measurement |
| --- | --- | --- | --- |
| R4-1903753.zip | Discussion on FR2 intra-freqeuncy measurement requriements | Huawei, HiSilicon | FR2 intra-frequency requirements with CSSF =1 for PCC/PSCC apply provided that SMTC period and offset is same in all CCs in the FR2 band. |
| R4-1903754.zip | CR on FR2 intra-freqeuncy measurement requriements (section 9.1.5.1) | Huawei, HiSilicon | FR2 intra-frequency requirements with CSSF =1 for PCC/PSCC apply provided that SMTC period and offset is same in all CCs in the FR2 band. |
| R4-1903914.zip | Measurement outside gap in FR2 for partially-overlapped SMTC occasions | MediaTek inc. | Similar issue as 3753/3754 : The requirements for measurement outside gap in is only applicable when the SMTCs of all CCs in FR2 are the same. |
| R4-1904494.zip | CR on TS38.133 for definition of detectable cell (Section 9.2.4.3, Section 9.3.6.3) | MediaTek inc. | Similar issue as 3753/3754 |
| R4-1903755.zip | Editorial correction on measurement period for de-activated SCell (section 9.2.5.2) | Huawei, HiSilicon | Misc editorial corrections for deactivated scells |

# 6.10.3.3 Finalization of inter-RAT measurement

      - Finalization of inter-RAT measurement	 [NR_newRAT-Core]

        - RSTD measurement: number of ACK/NACK during TECGI	[NR_newRAT-Core]

        - Whether to support cell detection with autonomous gap	[NR_newRAT-Core]

        - Maintenance for inter-RAT measurement	[NR_newRAT-Core]

# Contributions on: inter-RAT RRM in 38.133,  RSTD in 38.133, inter-RAT RRM in 36.133

| 6.10.3.3 | R4-1903756 | discussion | Discussion | Remaining issues in inter-RAT RSTD requirements | Huawei, HiSilicon | available |
| --- | --- | --- | --- | --- | --- | --- |
| 6.10.3.3 | R4-1903757 | draftCR | Endorsement | CR for inter-RAT RSTD measurement on FDD E-UTRA carrier (section 9.4.4.1) | Huawei, HiSilicon | available |
| 6.10.3.3 | R4-1903758 | draftCR | Endorsement | CR for inter-RAT RSTD measurement on TDD E-UTRA carrier (section 9.4.4.2) | Huawei, HiSilicon | available |
| 6.10.3.3 | R4-1903759 | draftCR | Endorsement | CR on gap sharing due to cell detection for inter-RAT RSTD (section 9.1.5.2) | Huawei, HiSilicon | available |
| 6.10.3.3 | R4-1904170 | draftCR | Endorsement | Clean up in inter-RAT E-UTRA requirements (sections 2 and 9.4) | Ericsson | available |
| 6.10.3.3 | R4-1904171 | discussion | Discussion | On remaining issues for inter-RAT RSTD measurements | Ericsson | available |
| 6.10.3.3.1 | R4-1904172 | draftCR | Endorsement | E-UTRAN time acquisition requirements for E-UTRA FDD RSTD measurements (section 9.4) | Ericsson | available |
| 6.10.3.3.1 | R4-1904173 | draftCR | Endorsement | E-UTRAN time acquisition requirements for E-UTRA TDD RSTD measurements (section 9.4) | Ericsson | available |
| 6.10.3.3.2 | R4-1902862 | discussion | Discussion | On inter-RAT RSTD measurement requirement | Intel Corporation | available |
| 6.10.3.3.2 | R4-1902863 | draftCR | Endorsement | CR for inter-RAT RSTD measurement requirement (section 9.4.4) | Intel Corporation | available |
| 6.10.3.3.3 | R4-1903625 | discussion | Discussion | Discussion on inter-RAT NR measurement requirements | NTT DOCOMO, INC. | available |
| 6.10.3.3.3 | R4-1903626 | CR | Agreement | Correction CR for inter-RAT FR2 measurement in 36.133 before EN-DC | NTT DOCOMO, INC. | available |
| 6.10.3.3.3 | R4-1903627 | CR | Agreement | Correction CR for inter-RAT FR2 measurement in 36.133 before EN-DC | NTT DOCOMO, INC. | not available |
| 6.10.3.3.3 | R4-1903719 | draftCR | Endorsement | Correction of inter-RAT EUTRA measurement requirements | Huawei, HiSilicon | available |

# inter-RAT RRM in 38.133

- Editorial corrections:

  - R4-1903719

    - CR correcting formulas after incorrect implementation of an earlier endorsed CR

    - Overlaps with R4-1904170

- Brackets removal and editorial corrections:

        - CR in R4-1901940

                - CR removing square brackets, correcting formulas and some more editorial corrections

                - Overlaps with R4-1903719

# Possible Way Forward: agree on R4-1903719, agree on R4-1904170 with removed the overlapping part with R4-1903719

# Inter-RAT RSTD in 38.133

- Issue #1: numbers of ACK/NACKs for ECGI

  - R4-1904171 (discussion), CRs in R4-1904172 (FDD) and R4-1904173 (TDD) – Ericsson

  - R4-1903756 (discussion), CRs in R4-1903757 (FDD) and R4-1903758 (TDD) - Huawei

  - R4-1902862 (discussion), CR in R4-1902863 - Intel

Summary of ACK/NACK numbers for ECGI:

|  | Ericsson | Huawei | Intel |
| --- | --- | --- | --- |
| 15 kHz FDD | 66 | 65 | 65 |
| 30 kHz FDD | 145 | 145 | 155 |
| 60 kHz FDD | 298 | 305 | 335 |
| 15 kHz TDD | 28 | - | - |
| 30 kHz TDD | 176 | - | - |
| 60 kHz TDD | 170 | - | - |

# Possible Way Forward: revise R4-1904172 (FDD, Ericsson) and revise R4-1903758 (TDD, Huawei) to capture agreements

- Issue #2: numbers of ACK/NACKs for MIB

  - Update some current numbers: CRs in R4-1903757/ R4-1903758 - Huawei

  - Keep current numbers: R4-1901940 - Ericsson

  - Update some current numbers: CR in R4-1902863 - Intel

|  | Ericsson | Huawei | Intel |
| --- | --- | --- | --- |
| 15 kHz FDD | [15] | 12 | 12 |
| 30 kHz FDD | [39] | 36 | 37 |
| 60 kHz FDD | [85] | - | 99 |
| 15 kHz TDD | [2] | - | - |
| 30 kHz TDD | [4] | - | - |
| 60 kHz TDD | [44] | - | - |

# Possible Way Forward: needs discussion, agreements to be captured in R4-1904170 (Ericsson), together with other corrections

- Issue #3: numbers of ACK/NACKs for MIB+ECGI

  - R4-1904171 (discussion), CRs in R4-1904172 (FDD) and R4-1904173 (TDD) – Ericsson

# Possible Way Forward: needs discussion, agreements can be captured in R4-1904172 (Ericsson) and R4-1903758 (Huawei), together with other agreements

- Issue #4: add 120 kHz in the ACK/NACK tables

# Possible Way Forward: can be included in R4-1904170 (Ericsson), together with other corrections

Agreement: add 120 kHz in the ACK/NACK tables

- Issue #5: gap sharing clarification during LTE cell detection for acquiring LTE timing prior to RSTD

  - CR in R4-1903759 (Huawei)

# Possible Way Forward: needs discussion, R4-1903759 can be revised to capture agreements

- Issue #6: requirements for LTE cell detection in autonomous gaps when acquiring LTE timing prior to RSTD

  - Not needed: R4-1904171 (discussion) – Ericsson

  - Needed, detection time is 400 ms: R4-1903756 (discussion), CRs in R4-1903757 (FDD) and R4-1903758 (TDD) - Huawei

  - Needed, detection time is 600 ms: R4-1902862 (discussion), CR in R4-1902863 - Intel

# Possible Way Forward: needs discussion, RAN2 procedure does not support this

# inter-RAT RRM in 36.133

- Clarification of gapless measurements support and applicable requirements for UE capable of per-FR gaps configured with FR2 inter-RAT measurements before being configured with EN-DC

  - CR in R4-1903625 (discussion), R4-1903626 (CR):

- Proposal 1: It should be captured in TS 36.133 that if the UE supporting per-FR gap is not yet configured with EN-DC and is configured to measure only FR2 inter-RAT MO, the UE can perform such measurement without gap.

- Proposal 2: If the UE supporting per-FR gap is not yet configured with EN-DC and is configured to measure only FR2 inter-RAT MO, the UE shall fulfil the measurement requirements based on effective MGRP = [20] ms.

- Proposal 3: If the LTE UE before EN-DC is configured to measure only LTE and/or inter-RAT NR measurement objects, CSSF reflecting SMTC configuration on NR carrier to be measured should apply to inter-RAT NR measurement requirement.

- Proposal 4: If GSM/UTRA measurement is contained in configured measurement objects, Nfreq could apply to all measurement requirements including inter-RAT NR measurement.

# Possible Way Forward: needs discussion, possibly revise R4-1903626

# Summary of possible agreements on AI 6.10.3.3:

- agree on R4-1903719, agree on R4-1904170 with removed the overlapping part with R4-1903719 captured agreements on 120 kHz and ACK/NACK for MIB reading

- revise R4-1904172 (FDD, Ericsson) and revise R4-1903758 (TDD, Huawei) to capture agreements

- possibly revise R4-1903759, based on the agreements

- possibly revise R4-1903626

Status from main meeting Agreement: add 120 kHz in the ACK/NACK tables CRs to return to R4-1904685 E-UTRAN time acquisition requirements for E-UTRA FDD RSTD measurements (section 9.4), Ericsson R4-1904686 CR for inter-RAT RSTD measurement on TDD E-UTRA carrier (section 9.4.4.2), Huawei R4-1904687 CR on gap sharing due to cell detection for inter-RAT RSTD (section 9.1.5.2), Huawei R4-1904688 Clean up in inter-RAT E-UTRA requirements (sections 2 and 9.4), Ericsson R4-1903719 Correction of inter-RAT EUTRA measurement requirements, Huawei