3GPP TSG-RAN4 Meeting #90bis	R4-1904864

# Xi’an, China, 8th Apr – 12th Apr, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TCI state switching delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-3-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The delay requirements for TCI state switching shall be provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the TCI state switching delay requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TCI state switching delay requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 8.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change >>

8.9	Active TCI state switching delay

8.9.1	Introduction

The requirements in this section apply for a UE configured with one or more TCI state configurations on serving cell in MR-DC or standalone NR. UE shall complete the switch of active TCI state within the delay defined in this section.

8.9.2	Known conditions for TCI state

The TCI state is known if it has been meeting the following conditions:

- The target TCI state remains detectable during the TCI state activation delay

- The UE reports L1-RSRP/L3-RSRP in last [X ms] for the target TCI state

Otherwise, the TCI state is unknown.

Note 1: FFS on X value and FFS the side condition for TCI state detection.

8.9.3	MAC-CE based TCI state switch delay

If the target TCI state is known to UE, for UE supporting power class 1, upon receiving PDSCH carrying MAC-CE activation command at slot n, UE shall be able to receive PDCCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+ THARQ +3ms, where THARQ is the timing between DL data transmission and acknowledgement as specified in [7].

Note2: If the target TCI state is known to UE, FFS the delay requirement for UE supporting power class 2/3/4.

Note 3: If the target TCI state is unknown to UE, FFS the delay requirement for UE supports power class 1/2/3/4.

8.9.4	DCI based TCI switch delay

If the target TCI state is known to UE, for UE supporting power class 1, when a UE is configured with the higher layer parameter tci-PresentInDCI which is set as 'enabled' for the CORESET scheduling the PDSCH at slot n, UE shall be able to receive PDSCH or transmit PUSCH on the new beam of the serving cell on which TCI state switch occurs no later than at slot n+timeDurationForQCL, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [26], the value of TQCL_time is defined in TS 38.306[26].

The known condition for TCI state defined in section 8.7.2 is applied.

Note 4: If the target TCI state is known to UE, FFS the delay requirement for UE supports power class 2/3/4.

<< End of Change >>