3GPP TSG-RAN WG4 #90BIS Meeting                                        	R4-1904970

Xi’an, China, 08 – 12 April, 2019

Source: 	LG Electronics

Title: 	MSD test results for new x bands DL (x=3, 4, 5) with 2 bands UL CA in Rel-16

Agenda item:	7.6.2

Document for:	Approval

1	Introduction

Based on the self-desense analysis from [1], MSD needs to be determined for those who have self-interference problems.

For 3 bands DL and 2 bands UL CA band combinations, MSD results are provided for one of three downlink bands which is impacted by harmonics and IMD products during dual uplink transmission.

For 4 bands DL with 2 bands UL and 5 bands DL with 2 bands UL CA band combinations, there is no need to study more for the case when harmonics and IMDs impact to their own receiving bands (4th and 5th) since all self-desense problems will be covered in 2 bands DL/2 bands UL and 3 bands DL/2 bands UL CA band combinations.

### 2.	Summary of interference studies

Table 1 shows 3 bands DL/ 2 bands UL CA band combinations with self-interference problems in Rel-16 from  [2] after RAN4#90 meeting.

Table 1: Summary of Self-interference analysis for 3 bands DL/ 2 bands UL CA

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | Intermodulation to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_3A-11A-18A | CA_3A-11A | - | 5th IMD | - | 4.9 dB |
| CA_3A-11A-26A | CA_3A-11A | - | 5th IMD | - | 4.9 dB |
| CA_1A-3A-42C | CA_1A-42C | - | - | - | N/A |
| CA_1A-3A-42C | CA_3A-42C | - | - | - | N/A |
| CA_2A-4A-13A | CA_2A-13A | - | 4th IMD | - | 7.6 dB |
|  | CA_4A-13A | - | 4th IMD | - | 6.2 dB |
| CA_2A-2A-4A-5A | CA_2A-5A | - | 4th IMD | - | 7.6 dB |
|  | CA_4A-5A | - | 2nd, 5th IMDs | - | - 2nd and 5th IMD problems were already covered in Table 7.3.1A-0f:2DL/2UL_4A-5A of TS36.101. |
| CA_2A-2A-5A-66A-66A | CA_2A-5A | - | 4th IMD | - | 7.2 dB |
| CA_2A-5B-66A-66A | CA_2A-5A | - | 4th IMD | - | 7.2 dB |
|  | CA_5A-66A | - | 2nd, 5th IMDs | - | 2nd and 5th IMD problems were already covered in Table 7.3.1A-0f:2DL/2UL_5A-66A of TS36.101 |
| CA_2A-5A-46D | CA_2A-5A | 3rd Harmonic | 4th, 5th IMDs | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 2.4 dB for IMD4 - 2.7 dB for IMD5 |
| CA_5A-46D-66A | CA_5A_46A | - | 5th IMD | - | 0.3 dB |
|  | CA_5A_66A | 3rd Harmonic | 4th, 5th IMDs | - | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 2.5 dB for IMD4 - 0 dB for IMD5 |
| CA_2A-13A-66A-66B | CA_2A-13A | - | 4th IMD | - | 7.2 dB |
|  | CA_13A-66A | - | 4th IMD | - | FFS |
| CA_2A-13A-48A-48C | CA_2A-13A | 2nd Harmonic at high frequency band edge | - | - | 2nd harmonic impact from B2 to B48 was covered in Table 7.3.1A-0a of TS 36.101. |
| CA_13A-46D-66A | CA_13A-66A | 3rd Harmonic | 4th, 5th IMDs | - | - No need to study for 3rd harmonic impact from B66 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 7.2 dB for IMD4 - 0 dB for IMD5 |
| CA_2A-13A-46D | CA_2A-13A | 3rd Harmonic | 4th IMD | - | - No need to study for 3rd harmonic impact from B2 to B46 since B46 is specified as reference measurement exclusion region in Table 7.3.1A-0eC of TS36.101. - 2.5 dB |
| CA_1A-3A-38A | CA_1A-3A | - | 3rd IMD into B1 | - | - 3rd IMD problem was already covered in Table 7.3.1A-0f: 2DL/2UL_1A-3A of TS36.101. |
| CA_2A-12A-66A | CA_2A-12A | 3rd Harmonic impact from B12 to B66 | - | - | - 3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. |
|  | CA_2A-66A | - | 3rd IMD into B2 5th IMD into B66 | - | - 3rd and 5th IMD problems were already covered in Table 7.3.1A-0f:2DL/2UL_2A-66A of TS36.101. |
|  | CA_12A-66A | 3rd Harmonic impact from B12 to B66 | 4th IMD into B66 4th IMD into B2 | - | - 3rd harmonic impact from B12 to B66 was covered in Table 7.3.1A-0a of TS36.101. - For 4th IMD into B66, no impact when considering fixed Tx-Rx band separation of 400 MHz in Band 66. - Need to study for the side-lobe impact of 4th IMD product |
| CA_1A-3A-42D | CA_1A-3A | 2nd Harmonic impact from B3 to B42 | 3rd IMD into B1 4th IMD into B42 | - | - 2nd harmonic impact from B3 to B42 was covered in Table 7.3.1A-0a of TS36.101. - 3rd IMD problem was already covered in Table 7.3.1A-0f:2DL/2UL_1A-3A of TS36.101. - 4th IMD problem was already covered in Table 7.3.1A-0g:3DL/2UL_CA_1A-3A-42A/CA_1A-3A of TS36.101. |

For MSD analysis, UE RF component parameters in Table 2 and each isolation level of RF components in Table 3 are considered.

Table 2: UE RF Front-end component parameters

| UE ref. architecture | Cascaded Diplexer Architecture |  |  |  |
| --- | --- | --- | --- | --- |
| Component | All CA band combos |  |  |  |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 112 | 68 | 55 | 55 |
| Diplexer | 115 | 87 | 55 | 55 |
| Duplexer | 100 | 75 | 55 | 53 |
| Quadplexer | 110 | 72 | 55 | 52 |
| PA Forward | 28.0 | 32 | 30 | 28 |
| PA Reversed | 40 | 30.5 | 30 | 30 |
| LNA | 10 | 0 | 0 | -10 |

Table 3: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Diplexer | 25 | High/low band isolation |
| Quadplexer | 15 | Adjacent Tx-Rx atten level |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | H-H cross-band |
| Duplexer | 50 | Tx band rejection at Rx band |

In Table 4, MSD levels are for 3 bands DL/2 bands UL inter-band CA band are proposed. If several interested companies brings different MSD levels, then they will be averaged to finalize the values.

Table 4: Proposed MSD levels for 3 bands DL/2 bands UL inter-band CA in rel-16

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-13A-66A-66B | CA_13A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | 6.2 | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 | N/A |  |  |
|  |  | 66 | 1762 | 5 | 25 | 2162 | 5 | N/A |  |  |
| CA_2A-12A-66A | CA_12A-66A | 2 | 1907.5 | 5 | 25 | 1987.5 | 5 | 0 | FDD | IMD4 |
|  |  | 12 | 713.5 | 5 | 25 | 743.5 | 5 | N/A |  |  |
|  |  | 66 | 1712.5 | 5 | 25 | 2112.5 | 5 | N/A |  |  |

3		Conclusion

In this contribution, we propose MSD levels for 3 bands DL/2 bands UL inter-band CAs that have potential IMD problems. Also, these MSD levels can be merged with other MSD levels from companies.

=== Text Proposal for TR36.716-03-02 section 6.x ===

6.9 LTE-A inter-band CA: Band 2 and Band 13 and Band 66 DL with 2 bands UL

----- Unchanged sections omitted -----

When uplink CA_13A_66A is paired with downlink CA_2A-13A-66A-66B, the 4th order IMD product by band 13 and band 66 falls into the own Rx frequency band 2. In Table 6.9.1.3-1, evaluated MSD values of 3 bands DL_CA_2A-13A-66A-66B with 2 bands UL is shown.

Table 6.9.1.3-2: MSD summary for 2 bands UL with 3 bands DL CA_2A-13A-66A-66B

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-13A-66A-66B | CA_13A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | 6.2 | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 | N/A |  |  |
|  |  | 66 | 1762 | 5 | 25 | 2162 | 5 |  |  |  |

----- Unchanged sections omitted -----

6.12 LTE-A inter-band CA: Band 2 and Band 12 and Band 66 DL with 2 bands UL

----- Unchanged sections omitted -----

6.12.1.5	MSD

When uplink CA configurations CA_2A-66A is paired with downlink CA configuration CA_2A-12A-66A, the 3rd order IMD products by the band 2 and the band 66 falls into the own Rx frequency of band 2, the 5th order IMD products by the band 2 and the band 66 falls into the own Rx frequency of band 66. For this case, the MSD value from 2DL/2UL CA_2A-66A in Table 7.3.1A-0f in TS 36.101 can be reused.

When uplink CA configurations CA_12A-66A is paired with downlink CA configuration CA_2A-12A-66A, the 4th order IMD products from uplink in band 12 and 66 to downlink in band 2 could be observed.  However, this 4th order IMD products could happen only in the case where uplink in the uppermost edge of band 12 with the lowest edge band 66 paired with downlink in the uppermost edge of band 2 is utilized.

In Table 6.12.1.5-1, the MSD analysis results from interested companies are summarized.

Table 6.12.1.5-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Nokia | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| CA_2A-12A-66A | B12 | IMD4 | ¦2*fB66 -2*fB12¦ | 0 | 0 | 0 |
|  | B66 |  |  |  |  |  |

Based on the provided MSD results, the MSD requirement shown in table 6.12.1.5-2 is proposed to capture in TS36.101.

Table 6.12.1.5-2: MSD summary for UL CA_12A-66A and DL CA_2A-12A-66A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_2A-12A-66A | CA_12A-66A | 2 | 1907.5 | 5 | 25 | 1987.5 | 5 | 0 | FDD | IMD4 |
|  |  | 12 | 713.5 | 5 | 25 | 743.5 | 5 | N/A |  | N/A |
|  |  | 66 | 1712.5 | 5 | 25 | 2112.5 | 5 | N/A |  | N/A |

----- Unchanged sections omitted -----

----- End of TP -----

References

- R4-1900198, “TP on summary of interference studies for new x bands DL (x=3,4,5) with 2 bands UL inter-band CA combinations in rel-16” LG Electronics

- TR 36.716-03-02 v0.4.0