3GPP TSG-RAN4 Meeting #90bis 	R4-1905112

# Xi’an, China, 8 April - 12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | - | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.141-2:Overview of radiated Tx requirements (4.1.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-4-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | OTA transmitter OFF power for FR2 was agreed to be verified by an EIRP measurement in the reference direction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | OTA transmitter OFF power and transient period requirements are classified as TRP. It is stated these requirements are verified by an EIRP measurement in the reference direction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Overview table of radiated Tx requirements remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--------------Start of change-------------

4	General radiated test conditions and declarations

4.1	Measurement uncertainties and test requirements

4.1.1	General

The requirements of this clause apply to all applicable tests in TS 38.141-2 (the present document), i.e. to all radiated tests defined in FR1 for BS type 1-H, BS type 1-O and radiated tests defined in FR2 for BS type 2-O. The frequency ranges FR1 and FR2 are defined in subclause 5.1 of TS 38.104 [2].

The minimum requirements are given in TS 38.104 [2]. Test Tolerances for the radiated test requirements (TTOTA) explicitly stated in the present document are given in annex C.

Test Tolerances are individually calculated for each test. Test Tolerances are used to relax the minimum requirements to create test requirements.

When a test requirement differs from the corresponding minimum requirement, then the Test Tolerance applied for the test is non-zero. The Test Tolerance for the test and the explanation of how the minimum requirement has been relaxed by the Test Tolerance are given in annex C.

Table 4.1.1-1: Overview of radiated Tx requirements

| Tx requirement |  | Classification (Note) | Notes |
| --- | --- | --- | --- |
| Radiated transmit power |  | Directional – OTA peak directions set | This requirement is based on Rel-13 AAS BS requirement for EIRP accuracy. |
| OTA BS output power |  | TRP |  |
| OTA total power dynamic range |  | Directional – OTA peak directions set | Conformance testing is carried in the reference direction. |
| OTA transmitter OFF power |  | Co-location | For FR1 only |
|  |  | Directional – OTA peak directions set | For FR2 only Core requirements are defined by TRP. Conformance requirements are verified by EIRP measurements in the reference direction. |
| OTA transient period |  | Co-location | For FR1 only |
|  |  | Directional – OTA peak directions set | For FR2 only Core requirements are defined by TRP. Conformance requirements are verified by EIRP measurements in the reference direction. |
| OTA modulation quality |  | Directional – OTA coverage range | Conformance testing is carried in the reference direction and the maximum directions of the OTA coverage range on each axis. |
| OTA frequency error |  | Directional – OTA coverage range | Conformance testing is carried out in the reference direction. |
| OTA time alignment error |  | Directional – OTA coverage range | Conformance testing is carried out in the reference direction. |
| OTA occupied bandwidth |  | Directional – OTA coverage range | Conformance testing is carried out in the reference direction. |
| OTA ACLR |  | TRP |  |
| OTA operating band unwanted emission |  | TRP |  |
| OTA transmitter spurious emission | General requirement | TRP | For FR1 and FR2. |
|  | Protection of the BS receiver of own or different BS | Co-location | For FR1 only. |
|  | Additional spurious emissions | TRP | For FR1 only. |
|  | Co-location with other base stations | Co-location | For FR1 only. |
| OTA transmitter intermodulation |  | Co-location | For FR1 only. |
| NOTE: Directional requirement does not imply one compliance direction only. The directional requirement applies to a single direction at a time. |  |  |  |

--------------End of change-------------